| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 2nd Meeting: Geneva, CH, 21-28 July, 2010 | Document: JCTVC-B053 |
| --- | --- |

| Title: | Summary report for TE3 on inter prediction in HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Andreas Krutz, Thomas Sikora Technische Universität Berlin, Sekr. EN1, Einsteinufer 17, 10587 Berlin, Germany | Email: | krutz@nue.tu-berlin.de sikora@nue.tu-berlin.de |
| Source: | Technische Universität Berlin, TE3: Inter prediction |  |  |

_____________________________

# Abstract

This document summarizes the activity of the TE3 [1]. The TE3 is separated into two subtests. Subtest 1 is regarded to parametric motion compensation. Subtest 2 is about flexible motion partitioning. For subtest1, two participants (LG, TUB) have conducted experiments and show a significant improvement of their tools. Three participants of subtest 2 (Technicolor, Huawei, Qualcomm) have contributed experimental results and show further improvement of coding efficiency and computational cost on flexible motion partitioning. Overall the TE3-activity has made a significant progress of the proposed technologies.

# Subtest 1

1.1 Motion compensation with adaptive warped reference [2]

The experimental results of TE3 subtest 1 [1] are presented, which aims to improve temporal prediction performance by using warping motion [2]. This tool uses the additional reference picture which is warped version of the original reference picture based on homography relation between the current picture and the reference picture. Experimental results can be seen in Table 1 and 2. It shows 0.64% overall bitsaving under constraints set 1 and 0.40% overall bitsaving under constraints set 2 compared with JM 17.0. And considering additional group of sequences having complex motion, overall bitsaving can be increased up to 12.38% and 3.83% respectively.

Encoder complexity is about 2.5 times. Decoder complexity is about 2.1 times.

| Table 1. BD rate (%) reduction of CS 1 & including extra sequences. Class; seq.; BD-rate(%). B; BasketballDrive; -0.16. BQTerrace; 1.13. Cactus; -8.48. Kimono; -0.01. ParkScene; 0.01. Class B Avg.; -1.50. C; BasketballDrill; 0.04. BQMall; 0.30. PartyScene; -0.38. RaceHorses; -0.09. Class C Avg.; -0.03. D; BasketballPass; -0.19. BlowingBubbles; 0.01. BQSquare; -1.24. RaceHorses; -0.09. Class D Avg.; -0.38. CS 1 Avg.; -0.64. Extra seq; City_720p; -6.64. Jets_720p; -17.54. Bluesky_1080p; -25.24. Station2_1080p; -47.06. Avg; -24.12. CS 1+ extra seq Avg; -12.38. | Table 2. BD rate (%) reduction of CS 2 & including extra sequences. Class; seq.; BD-rate(%). B; BasketballDrive; -0.04. BQTerrace; -1.28. Cactus; -3.41. Kimono; 0.04. ParkScene; 0.02. Class B Avg.; -0.93. C; BasketballDrill; -0.09. BQMall; -0.04. PartyScene; -0.04. RaceHorses; 0.02. Class C Avg.; -0.04. D; BasketballPass; -0.03. BlowingBubbles; -0.05. BQSquare; -0.82. RaceHorses; -0.01. Class D Avg.; -0.23. CS 2 Avg.; -0.40. Extra seq; City_720p; -0.64. Jets_720p; -2.18. Bluesky_1080p; -8.60. Station2_1080p; -17.59. Avg; -7.25. CS 2+ extra seq Avg; -3.83. |
| --- | --- |

1.2 Adaptive global motion temporal prediction [3]

The results for Subtest 1 of Tool Experiment 3 (Inter Prediction in HEVC) [1] are presented. The tool presented herein is a novel prediction mode, i.e. Adaptive Global Motion Temporal Prediction (GMTP), for hybrid video coding environments that is based on global motion compensation and optionally on temporal filtering of spatially aligned pictures. This often provides a better prediction signal than common INTRA or INTER modes, especially if complex motion like scaling, rotation, and perspective transformation occur. First, the encoder performs global motion estimation (GME) between neighboring pictures of the sequence, not necessarily in display order. Here, a sophisticated approach is used that is based on the Helmholtz tradeoff estimator and which utilizes the motion vector fields available in common hybrid video encoders. However, any GME method can be used. Second, the pictures inside the decoded picture buffer (DPB) are transformed into the coordinate system of the current picture to encode. This produces an image stack of spatially aligned pictures. If more than one picture is available inside the DPB, the encoder may choose to adaptively blend an arbitrary number of aligned pictures together, e.g. using median blending. The decoder is informed about that number N on a slice level inside the bitstream. Once the GMTP picture is available, the encoder chooses the macroblock types by means of rate-distortion optimization (RDO). If it chooses to encode a block using GMTP, only the type identifier is sent inside the macroblock header. No further information, e.g. coded block pattern, quantization parameters or coefficients are included in the bitstream for that block. This corresponds to the SKIP mode. However, the prediction quality is generally better.

The method has been evaluated using a set of 33 test sequences, including classes A to D of the CfP [3]. The evaluation follows the experimental conditions of [1]. The new prediction mode has been incorporated into the H.264/AVC reference software JM 17.0. It could be shown that using the new mode, the average bit rate spent on motion vector information, coded block pattern, and coefficients could be decreased while only marginally decreasing objective quality resulting in better rate-distortion performance. However, the results depend on the availability of global motion in the video sequences used and additionally correlate with its resolution.

# 2 Subtest 2

2.1 Simplified Geometry Block Partitioning

This contribution provides a simplification of Geometry Block Partitioning scheme as described in JCTVC-A121. Specifically, Most Valuable Partitions are proposed to achieve the best tradeoff between the complexity and coding efficiency. Most Valuable Partitions are derived from a statistical analysis of the actual used GEO partitions. For defined test conditions in TE3, average 3.12% bitrate saving is achieved for IPPP coding case, with a maximum gain of 6.65%, very close to the gain obtained with initial reference version with a reduced complexity.

# Simulation Results

The test conditions defined in TE3 are followed. The simulation is done using Qualcomm’s software (r1 version) in JCTVC-A121. Tests were done on the mandatory case for IPPP only. For the encoder, fast mode decision was used as Qualcomm did. The only change is that for GEO32, the same fast mode decision as GEO16 is used. For GEO64, AMP modes are simply tested.

|  |  | A121 reference bitrate Saving with/without GEO | proposal bitrate Saving with/without GEO |
| --- | --- | --- | --- |
| B | S03-Kimono | 0.84% | 0.74% |
| B | S04-ParkScene | 2.95% | 3.06% |
| B | S05-Cactus | 2.76% | 3.12% |
| B | S06-BasketballDrive | 1.36% | 1.72% |
| B | S07-BQTerrace | 1.48% | 2.66% |
| C | S08-BasketballDrill | 4.32% | 3.46% |
| C | S09-BQMall | 5.02% | 4.20% |
| C | S10-PartyScene | 2.21% | 2.17% |
| C | S11-RaceHorses | 3.92% | 2.98% |
| D | S12-BasketballPass | 3.24% | 2.99% |
| D | S13-BQSquare | 1.68% | 2.60% |
| D | S14-BlowingBubbles | 3.65% | 3.65% |
| D | S15-RaceHorses | 4.83% | 3.51% |
| E | S16-Vidyo1 | 3.41% | 3.34% |
| E | S17-Vidyo3 | 6.59% | 6.65% |
| E | S18-Vidyo4 | 3.33% | 3.01% |
| AVERAGE | AVERAGE | 3.22% | 3.12% |

Complexity wise, the speed of the encoder is in the same range of Qualcomm’s software. For decoder, there is almost no difference.

# Conclusion

GEO shows promising gain even with a big reduction of the partitions, average 3.12% bitrate saving in IPPP coding case. The simplifications proposed in this contribution bring very similar performance as the reference version considered for TE3 of the JCTVC. It is therefore proposed to continue the core experiment using the Test Model software.

2.2 Huawei & Hisilicon report on flexible motion partitioning coding [4]

Based on the concept that an object's boundary usually divides a block into 2 regions with arbitrary shape, flexible motion partitioning technique has been introduced. The representation method, and predictive coding of motion vector and partitioning representation parameters have been explored.

In order to represent the partitioning with flexibility, two parameters are coded and used to indicate two segments' shape. The probability of the usage of flexible partitioning modes is investigated, and used to code above two parameters.

Considering the connectivity property of image texture, information of neighboring blocks can be used to predict current block's coding elements, and improve compression efficiency. We have investigated to use neighboring blokcs' motion vectors and partitioning parameters to predict current block's motion vectors. The result shows potential coding efficiency improvement for non-AVC MB partitioning.

Besides, the partitioning representation parameters can also be predictive coded by using neighboring blocks' representation parameters.

Those preliminary research works show that the compression capability of flexible motion partitioning can be further improved.

2.3 Geometry motion partition

# 3 Conclusion and further activities of the TE

4 References

| [1] | A. Krutz, A. Glantz, T. Sikora, J. Park, S. Park, E. Francois, P. Yin, X. Zheng, H. Yu, W.-J. Han, and W.-H. Peng, “Tool Experiment 3: Inter Prediction in HEVC,” Doc. JCTVC-A303, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Dresden, Germany, Apr 2010 |
| --- | --- |
| [2] | S. Park, J. Sung, J. Young Park, B.-M. Jeon, “TE 3: Motion compensation with adaptive warped reference,” Doc. JCTVC-B022, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, Switzerland, Jul 2010 |
| [3] | A. Krutz, A. Glantz, T. Sikora, “TE 3: Adaptive Global Motion Temporal Prediction,” Doc. JCTVC-B052, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, Switzerland, Jul 2010 |
| [4] | X. Zheng (HiSilicon), H. Yu (Huawei) , “TE3: Huawei & Hisilicon report on flexible motion partitioning coding,” Doc. JCTVC-B041, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, Switzerland, Jul 2010 |
| [5] |  |
| [6] | ISO/IEC JTC1/SC29/WG11, “Joint Call for Proposals on Video Compression Technology,” MPEG Document N11113, Jan 2010 |