| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 2nd Meeting: Geneva, CH, 21-28 July, 2010 | Document: JCTVC-B054 |
| --- | --- |

| Title: | Proposed low complexity video encoder settings for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc. | Tel: Email: | +1 214 480 3816 zhou@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

In recent years camera phones, camcorders, video surveillance and video conferencing applications have grown fast and evolved into another major video market. Real-time, high quality video encoder and decoder at low-cost and low power consumption are required to serve this market’s needs.     In this contribution the low complexity video encoder settings are proposed. By considering characteristics of the low-complexity encoder designs and projected circuit design capability, it is proposed to limit the maximum CU size to 64 x 64 and maximum transform size to 32 x 32, enable CABAC, IBDI and simplified RDO, and turn off RDOQ and ALF in the settings.  It is advocated that the HEVC tools should be tested for both high complexity and low complexity encoder environments so that the final HEVC design can satisfy the requirements of a wide range of video applications.

# Introduction

When the MPEG4 AVC/H.264 was standardized in early 2000s, the video applications were dominated by applications such as video broadcasting, DVDs, Blue-Rays, etc. The emerging applications such as digital video surveillance and conferencing were still a niche market with very low volume. The camera phone market did not even exist at that time. Therefore, in the MPEG4 AVC/H.264 era, the decoder complexity was a major concern for the standardization, the encoder complexity was relatively less important because only a very small number of encoders would be built for those kinds of applications.

In the past few years, camera phones, camcorders, video surveillance and video conferencing applications have grown fast, and today they have evolved into another major video market. For example, the camera phone market and the video surveillance market are forecasted to reach one Billion and 20 Millions annual units in year 2010, respectively. In this fast growing market, the video encoder and decoder are equally important; the video encoders have to provide high video quality at low-cost and low power consumption, which pose great challenges to the low-complexity encoder design. Taking the MPEG-4 AVC/H.264 as example, due to design constraints today’s low-complexity MPEG-4 AVC/H.264 encoder designs are not much different from that of the MPEG4 Simple Profile encoder in terms of motion estimation and mode decision, the MPEG-4 AVC/H.264 provides better video quality mainly due to the advanced but deterministic coding tools such as spatial domain intra prediction, ¼ pel motion compensation, more efficient entropy coding and in-loop deblocking filter, but lots of other quality enhancement tools cannot be utilized due to constraints of computational power, memory bandwidth, on-chip memory size and high throughput requirements.

From the HEVC CfP testing results it is relatively clear how much the HEVC [1] can perform better than the MPEG4 AVC/H.264 High Profile for decoder only (high encoder complexity) applications. However, the HEVC quality improvement in the low-complexity encoder environment is not clear at this moment. Since the quality benefit from deterministic coding tools such as entropy coding, in-loop deblocking filter has already exhausted in the MPEG4 AVC/H.264, it will be more challenging to design a low-complexity HEVC encoder that would offer significant quality improvement over the MPEG4 AVC/H.264 High Profile.

Improved coding efficiency is the key for the HEVC to gain the market adoption. The HEVC has a unique opportunity to do a better job in the area of low-complexity video encoder design.  In addition to the traditional high complexity encoder test conditions, it is recommended to define low-complexity encoder coding test conditions as well by considering both the characteristics of state-of-art low-complexity MPEG4 AVC/H.264 encoder designs, and projected circuit design capability in the following years, tools should be tested for both high complexity and low complexity encoder environments so that the final HEVC design can satisfy the requirements of a wide range of video applications.

# Low complexity encoder settings

Based on lessons learned from the low-complexity MPEG4 AVC/H.264 encoder designs and projected circuit design capability, the following low complexity encoder settings are proposed for the HEVC (Table 1). Given the limited search capability of a low-complexity encoder, it is also recommended to limit the number of coding options/modes which may actually help improve the coding efficiency in low-complexity encoder environment.

| Parameters | Settings | Remark |
| --- | --- | --- |
| Maximum CU size | 64 x 64 | This will effectively increase the on-chip memory requirement by 8x compared to the current MPEG4 AVC/H.264 design, a very challenging goal. |
| Minimum PU size | 16 x 16 | Vector block size less than 8x8 is hard to support on the encoder side |
| Transform sizes | 8x8, 16x16, 32x32 | 4x4 and 2nd level 4x4 DC transform may have throughput issues at UHD due to dependency in intra prediction; 64x64 transform is too expensive |
| Transforms for intra | TBD | Transforms proposed for intra blocks need to be unified into a simpler design |
| Motion vector precision | ¼ pel | Low-complexity 1/8 pel vectors could be supported in the future |
| Search range | TBD | The search range is limited by on-chip memory size and available memory bandwidth |
| Slice type | I, P, B, HierP, HierB |  |
| Number of reference frames for P | 2 | In the DPB buffer there can be more than 2 reference frames, but for coding the current P frame the encoder choose up to 2 reference frames |
| Number of reference frames for B | 1 in each direction | In the DPB buffer there can be multiple reference frames, but for coding the current B frame the encoder choose the best reference frame in each direction |
| Reference picture re-ordering | Yes |  |
| LowDelayMode | 0 and 1 |  |
| Weighted prediction | yes |  |
| IBDI | On | n bits increase, deterministic increase in complexity and coding efficiency; needs to determine value n based on complexity and coding efficiency trade-offs; memory bandwidth reduction needs to be considered. |
| Interpolation Filter | TBD | Small number of filter tap is preferred for saving memory bandwidth; needs to consider filter settings when IBDI is on |
| Entropy coding | CABAC | It can be replaced by v2v if the final design of v2v can provide intended throughput improvements |
| Deblocking filter | On |  |
| ALF | Off | Memory bandwidth concerns |
| RDOQ | Off | Complexity is too high for low-complexity encoder design |
| RDO | On, fast mode | simplified cost metrics instead of brute force R + λ*D. Currently this feature is not addressed in the reference software [2] |
| CU partitioning for PUs and TUs | Fast mode | Currently this feature is not addressed in the reference software [2] |

Table 1. Proposed key settings for low-complexity encoder

The corresponding configuration file is provided in the table 2. The differences between the proposed settings and the low-complexity settings recommended by the TMuC reference software AHG are marked yellow.  The low-complexity RDO is not addressed in the configuration file below.

#======== File I/O ===================== InputFile : d:\test\origcfp\RaceHorses_416x240_30.yuv BitstreamFile : str.bin ReconFile : rec.yuv FrameRate : 30 # Frame Rate per second FrameSkip : 0 # Number of frames to be skipped in input SourceWidth : 416 # Input frame width SourceHeight : 240 # Input frame height FrameToBeEncoded : 9 # Number of frames to be coded #======== Unit definition ================ MaxCUWidth : 64 # Maximum coding unit width in pixel MaxCUHeight : 64 # Maximum coding unit height in pixel MaxPartitionDepth : 3 # Maximum coding unit depth QuadtreeTUFlag : 1 # Use quadtree-based TU coding QuadtreeTULog2MaxSize : 5 # Log2 of maximum transform size for # quadtree-based TU coding (2...6) QuadtreeTULog2MinSize : 3 # Log2 of minimum transform size for # quadtree-based TU coding (2...6) #======== Coding Structure ============= IntraPeriod : 32 # Period of I-Frame ( -1 = only first) GOPSize : 8 # GOP Size (number of B slice = GOPSize-1) RateGOPSize : 8 # GOP size used for QP assignment NumOfReference : 2 # Number of reference frames NumOfReferenceB_L0 : 1 # Number of reference frames for L0 for B-slices NumOfReferenceB_L1 : 1 # Number of reference frames for L1 for B-slices HierarchicalCoding : 1 # Hierarchical B coding ON/OFF LowDelayCoding : 0 or 1 # Low-delay coding structure GPB : 1 # Replace P-slice by B-slice using two same directions QBO : 1 # Reference reordering to make Skip refer # better quality reference NRF : 1 # Mark non-reference for highest temporal layer BQP : 0 # Use hier-B style QP assignment for hier-P structure #=========== Interpolation Filter ============= InterpFilterType : 3 # 0: DCT-IF, 1: 4-tap MOMS, 2: 6-tap MOMS, 3: DIF DIFTap : 12 # Number of filter taps for DCT-IF (4, 6, 8, 10, 12) #=========== Motion Search ============= FastSearch : 1 # 0:Full search 1:EPZS SearchRange : 64 # (0: Search range is a Full frame) HadamardME : 1 # Use of hadamard measure for fractional ME FEN : 1 # Fast encoder decision #======== Quantization ============= QP : 32 # Quantization parameter(0-51) MaxDeltaQP : 0 # CU-based multi-QP optimization DeltaQpRD : 0 # Slice-based multi-QP optimization RDOQ : 0 # RDOQ #=========== Entropy Coding ============ SymbolMode : 1 # 0:CAVLC, 1:CABAC, 2:PIPE, 3:V2V with load balancing # (only 1..3 supported, CAVLC implementation is not completed) MultiCodewordThreshold : 96000 # Threshold for multi-codeword coding #(for SymbolMode 0 and 1; 0->always single codeword) MaxPIPEBufferDelay : 4096 # max buffer delay in bits (0:no limit, >=64:down-rounded to # next unit of 64 bits [8 Bytes], 0<x<64: NOT SUPPORTED) # only applies if (SymbolMode = 2 and MultiCodewordThreshold = 0) BalancedCPUs : 8 # number of CPUs in V2V (SymbolMode = 3) #=========== Deblock Filter ============ LoopFilterDisable : 0 # Disable loop filter in slice header (0=Filter, 1=No Filter) LoopFilterAlphaC0Offset : 0 # Range: -26 ~ 26 LoopFilterBetaOffset : 0 # Range: -26 ~ 26 #=========== Misc. ============ BitDepth : 8 # Source bit-depth (should be 8 now) BitIncrement : 4 # Increased bit-depth #=========== Coding Tools ================= CIP : 0 # Combined intra prediction (0: OFF, 1: ON) ROT : 1 # Rotational transform (0: OFF, 1: ON) AIS : 0 # Adaptive intra smoothing (0: OFF, 1: ON) MRG : 0 # Merge mode (0: OFF, 1: ON) IMP : 0 # Interleaved motion vector predictor ALF : 0 # Adaptive loop filter (0: OFF, 1: ON) ALFSeparateTree : 0 # Separate quadtree for ALF ALFSymmetry : 1 # ALF symmetry (0:unsymmetric, 1:symmetric) ALFMinLength : 7 # Min. ALF filter length (must be odd and >= 3) ALFMaxLength : 9 # Max. ALF filter length (must be odd and > ALFMinLength) #GRefMode : w # generated reference mode (w = scale+offset, o=offset)

Table 2 Proposed low complexity configuration file settings (modified from encoder_loco.cfg)

# References

- JCT-VC, “Test Model under Consideration”, JCTVC-A205, Joint Collaborative Team on Video Coding meeting, 15-23 April 2010, Dresden, Germany.

- JCT-VC, TMuC-0.1 Reference Software, https://hevc.hhi.fraunhofer.de/svn/svn_TMuCSoftware/tags/0.1/  and http://hevc.kw.bbc.co.uk/git/w/jctvc-tmuc.git.

# Patent rights declaration(s)

Texas Instruments may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).