| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 2nd Meeting: Geneva, CH, 21-28 July, 2010 | Document: JCTVC-B057 |
| --- | --- |

| Title: | Performance report of DPCM-based memory compression on TE2 |  |  |
| --- | --- | --- | --- |
| Status: | [Input Document to JCT-VC] |  |  |
| Purpose: | [Report] |  |  |
| Author(s) or Contact(s): | Hirofumi Aoki Keiichi Chono Kenta Senzaki Junji Tajime Yuzo Senda 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 3757 5438 hao@bu.jp.nec.com chono@ct.jp.nec.com k-senzaki@bp.jp.nec.com tajime@ak.jp.nec.com y-senda@bc.jp.nec.com |
| Source: | [ Information and Media Processing Laboratories, NEC Corporation] |  |  |

_____________________________

# Abstract

This contribution presents the 1-D DPCM-based memory compression method proposed in JCTVC-A302 for Tool Experiment 2 (TE2) and reports its performance. The proposed method is designed to reduce actual memory bandwidth with relatively low complexity, in particular, in motion compensation. Motion compensation frequently and randomly accesses to frame memory for reading reference pixels. In consideration of the tradeoff between memory accessibility and image quality, the proposed method employs simple 1-D DPCM. It has been tested with two constraint sets in two experimental conditions: one is 3/4 memory compression without IBDI and another is memory retainment even with 12-bit IBDI. Experimental results have shown that average coding losses are 7.757 % for CS1 without IBDI, 11.552 % for CS2 without IBDI, 0.573 % for CS1 with IBDI and 1.157 % for CS2 with IBDI, respectively. Subjective quality degradation caused by such coding loss is invisible for most test cases. Additional experimental results have also shown that memory compression only at the decoder side causes significant coding loss. It is therefore suggested that memory compression tool should be adopted as one of optional codec components so as to be available for memory-conscious hardware implementations, such as SoCs. As for memory accessibility, the impact on actual memory bandwidth has not yet been evaluated in TE2. Hence it is recommended that it is next studied in TE/CE on memory compression for Test Model.

# Introduction

Thanks to technology advancement, high-definition contents and devices are affordable even for consumer usage today. Even for mobile phones, display devices have reached near-HD resolutions, such as 960x640. The next generation of contents and devices, such as 4Kx2K video for home use and HD video for mobile devices, are expected to be coming soon. For the future video coding standard, capability of handling such higher resolution should be considered.

For encoding and decoding higher resolution video contents, more memory size and bandwidth are required. In particular, memory bandwidth is limited in consumer devices because of its limited capacity of power consumption and cost. Therefore memory bandwidth should be considered in the development of the next-generation video coding standard, in addition to compression efficiency and codec complexity.

Reference frame memory compression is a promising approach to reduce memory bandwidth in video codec, since prediction from reconstructed pictures stored in frame memory is the most memory-consuming part of video codec, though the high compression efficiency of up-to-date video coding technologies depends highly on the prediction. Many memory compression methods at the decoder side have been proposed [1-7] but they cause mismatch between encoders and decoders and thereby severe image quality degradation potentially occurs. In this contribution, a memory compression method incorporated into codec process loop as in [8, 9] is proposed.

# Discussion on overhead in frame memory access

A memory compression algorithm should be simple. More specifically, it is required that the data structure is simple and its complexity is relatively low in order to reduce actual memory bandwidth in restoring compressed data in frame memory to images in MC. It is because MC needs frequent, random and unaligned access to frame memory. More complex or larger compression unit is, more overhead is caused by unaligned memory access. Figure 1 depicts this overhead when frame memory is compressed using a large compression unit versus a small compression unit.

| (a) Case with less overhead. | (b) Case with more overhead. |
| --- | --- |

Figure 1 Overhead caused by unaligned memory access.

The overhead caused in MC is hard to be masked due to possible randomness of memory accesses, i.e., that of motion vectors. This discussion can be applied not only for MC but also for other sophisticated prediction techniques, such as intra prediction based on template matching. In addition, this memory access is caused frequently by relatively small block. In consideration of these characteristics of the memory access in MC, a 1-D DPCM-based lightweight memory compression method as in [10] is introduced in this contribution.

# Algorithm Description

## Overview

As in [8, 9], reference frames are compressed before being stored in memory and they are decompressed after being read from the memory. The proposed method employs a 1-D DPCM for the compression and decompression. 1-D data structure is effective in suppressing the overhead caused by unaligned memory access described above.

Figure 1 shows an example for memory compression with the proposed 1-D DPCM. Here, a 1-D DPCM block consists of eight consecutive pixels. The left-most pixel is first coded with PCM and serves as a base point. The other seven pixels are then coded with DPCM from left to right, using a five-bit nonlinear quantizer. Actually the block size, i.e., base-pixel interval and bit allocation for prediction errors can be variable in addition to a nonlinear quantization matrix. In the implementation for TE2, these parameters are signaled to decoders as side information for each sequence parameter set. Detail syntax of the parameter signaling is shown in section 3.

Figure 3 1-D DPCM prediction structure and bit allocation.

## Compression

The proposed memory compression algorithm is described below.

Given a 1-D image block consisting of N consecutive pixels, let x(n) be the nth pixel value of the block. Compressed signal y(n) (n=0,.., N-1) from x(n) with the proposed 1-D DPCM is then derived by:

y(n)=x(n);n=0epqridx(n);n≠0

where epqridx(n) represents an index to a quantization representative value, and it is obtained as follows.

epqridx(n)=argmini=0,..,2M−1epqr(i)−ep(n)

ep(n)=(x(n)−xp(n)+2B) % 2B

xp(n)=xr(n−1)

xr(n)=(xp(n)+epqr(epqridx(n))+2B) % 2B

where xr(n) is the reconstructed pixel value of x(n), xp(n) is the predicted pixel value for x(n), ep(n) is the prediction error, epqr(i) is an i-th quantization representative, and B represents bit depth of input pixels.

## Decompression

Decompressed signal xd(n) from y(n) is also obtained by the following simple equation.

xd(n)=y(n);n=0(xd(n−1)+epqr(epqridx(n))+L)%L;n≠0

As shown here, in addition to simplicity of its data structure, decompression of the proposed method is simple and straightforward. Therefore its impact on memory accessibility in MC is negligibly small.

# Syntax Description

## Syntax

Syntax for embedding information about the proposed memory compression method is shown below. In the software submitted for cross-verification for TE2, mem_compression_param() is put into the KTA-revised version of seq_param_set_rbsp(). Of course it can also be put into pic_param_set_rbsp(), in order to fit the quantizer characteristics to each pictures or to switch memory compression on and off. This parameter set can be managed also in the TMuC in the same manner as in the KTA.

| mem_compression_param ( ) { | C | Descriptor |
| --- | --- | --- |
| mem_compression_flag | 0 | u(1) |
| if(mem_compression_flag) { |  |  |
| dpcm_luma_interval_minus1 | 0 | ue(v) |
| dpcm_luma_bit_depth_minus1 | 0 | ue(v) |
| dpcm_luma_qmatrix_present_flag | 0 | u(1) |
| if(dpcm_luma_qmatrix_present_flag){ |  |  |
| sizeOfLumaQmatrixList = 1 << (dpcm_luma_bit_depth_minus1+1) |  |  |
| for(i = 0; i < sizeOfLumaQmatrixList; i++) { |  |  |
| dpcm_luma_qmatrix[i] | 0 | u(v) |
| } |  |  |
| } |  |  |
| dpcm_chroma_interval_minus1 | 0 | ue(v) |
| dpcm_chroma_bit_depth_minus1 | 0 | ue(v) |
| dpcm_chroma_qmatrix_present_flag | 0 | u(1) |
| if(dpcm_chroma_qmatrix_present_flag){ |  |  |
| sizeOfChromaQmatrixList = 1 << (dpcm_chroma_bit_depth_minus1+1) |  |  |
| for(i = 0; i < sizeOfChromaQmatrixList; i++) { |  |  |
| dpcm_chroma_qmatrix[i] | 0 | u(v) |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

## Semantics

Semantics associated with the proposed syntax structure is described below.

mem_compression_flag equal to 1 specifies that parameters for frame memory compression are present. mem_compression_flag equal to 0 specifies that these parameters are not present. When mem_compression_flag is equal to 1, reference frames are compressed by the proposed memory compression method before being stored in memory and they are decompressed after being read from the memory. When mem_compression_flag is equal to 0, no frame memory compression is done.

dpcm_luma_interval_minus1 specifies the base-pixel interval for the luma component.

dpcm_luma_bit_depth_minus1 specifies the number of bits allocated for DPCM prediction error for the luma component.

dpcm_luma_qmatrix_present_flag equal to 1 specifies that user-defined quantization matrix is used for the luma component. dpcm_luma_qmatrix_present_flag equal to 0 specifies that the default quantization matrix is used for the luma component.

dpcm_luma_qmatrix[i] is the i-th element of a user-defiend quantization matrix for the luma component.

dpcm_chroma_interval_minus1 specifies the base-pixel interval for the chroma component.

dpcm_chroma_bit_depth_minus1 specifies the number of bits allocated for DPCM prediction error for the chroma component.

dpcm_chroma_qmatrix_present_flag equal to 1 specifies that the user-defined quantization matrix is used for the luma component. dpcm_luma_qmatrix_present_flag equal to 0 specifies that the default quantization matrix is used for the chroma component.

dpcm_chroma_qmatrix[i] is the i-th element of a user-defiend quantization matrix for the chroma component.

# Experiments

The proposed method has been implemented on JM11.0KTA2.6r1 and tested with common conditions described in JCTVC-A302r1[11]. The parameter settings of the proposed method are as follows.

- The base-pixel interval of DPCM is set to 8.

- The bit depth of quantized pixels is set to 5 for the case without IBDI and 7 for the case with 12-bit IBDI, respectively.

- Default quantization matrices are used.

5-bit DPCM without IBDI corresponds to roughly 3/4 memory compression. 7-bit DPCM in the case with 12-bit IBDI is selected for retaining the original memory size. These parameters are applied both for the luma component and the chroma component.

Default quantization matrices are defined as follows.

For 5-bit DPCM in order to compress 8-bit signal, the default nonlinear quantization matrix composed of 32 elements is defined by:

[0, 1, 2, 3, 6, 9, 14, 18, 24, 31, 39, 48, 58, 72, 86, 103, 128, 153, 170, 184, 198, 208, 217, 225, 232, 238, 242, 247, 250, 253, 254, 255]

For 7-bit DPCM in order to compress 12-bit signal, the default nonlinear quantization matrix composed of 128 elements is defined by:

[0, 4, 8, 12, 16, 20, 24, 28, 32, 36, 40, 44, 48, 60, 72, 84, 96, 108, 120, 132, 144, 164, 184, 204, 224, 240, 256, 272, 288, 312, 336, 360, 384, 412, 440, 468, 496, 528, 560, 592, 624, 660, 696, 732, 768, 808, 848, 888, 928, 984, 1040, 1096, 1152, 1208, 1264, 1320, 1376, 1444, 1512, 1580, 1648, 1748, 1848, 1948, 2048, 2148, 2248, 2348, 2448, 2516, 2584, 2652, 2720, 2776, 2832, 2888, 2944, 3000, 3056, 3112, 3168, 3208, 3248, 3288, 3328, 3364, 3400, 3436, 3472, 3504, 3536, 3568, 3600, 3628, 3656, 3684, 3712, 3736, 3760, 3784, 3808, 3824, 3840, 3856, 3872, 3892, 3912, 3932, 3952, 3964, 3976, 3988, 4000, 4012, 4024, 4036, 4048, 4052, 4056, 4060, 4064, 4068, 4072, 4076, 4080, 4084, 4088, 4092]

## Objective and subjective quality performance

As objective quality performance measure, BD-PSNRs and Rates are computed on the basis of fitting 5 test points with the fourth-order polynomial. Tables 5.1-4 show BD-PSNR and Rate for each test sequence with each condition against each type of anchor.

These results show that average coding losses are 7.757 % for CS1 without IBDI, 11.552 % for CS2 without IBDI, 0.573 % for CS1 with IBDI and 1.157 % for CS2 with IBDI, respectively. Unfortunately we also observed that subjective quality degradation is visible in vidyo3 sequence, where ringing artifacts around edges are emphasized in particular with low bit-rate cases. However, for most test cases, subjective quality degradation caused by such coding loss is invisible.

In order to evaluate effectiveness of incorporating memory compression process into codec loop itself, we have done an additional experiment. In this experiment, the encoder runs without memory compression but the decoder compress frame memory in order to reduce its memory bandwidth. Results are shown in Tables 5.5-9.

From these results, it is clear that severe quality degradation is caused by memory compression only at the decoder side in particular for CS2 cases, where IPPP-type GOP structure is used and only the first picture is coded as I picture. Average objective quality losses are 1.474 dB for the case without IBDI and 0.421 dB for the case with IBDI, although those of the proposed memory compression incorporated into codec loop are only 0.344 dB and 0.038 dB, respectively. It is therefore suggested that memory compression tool should be adopted as one of optional codec components so as to be available for memory-conscious hardware implementations, such as SoCs.

Table 5.1: BD-PSNR and Rate results against CS1 type anchor without IBDI.

| Class | Sequence | BD-PSNR dB |  |  | BD-Rate % |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
| A | Traffic | -0.296 | -0.089 | -0.074 | 9.681 | 4.306 | 3.507 |
|  | People on Street | -0.080 | -0.024 | -0.021 | 1.675 | 1.202 | 0.756 |
| B | Kimono | -0.119 | -0.018 | -0.024 | 4.227 | 1.031 | 1.189 |
|  | ParkScene | -0.156 | -0.046 | -0.028 | 4.828 | 2.191 | 1.495 |
|  | Cactus | -0.192 | -0.052 | -0.102 | 7.786 | 4.203 | 5.054 |
|  | BasketballDrive | -0.050 | -0.020 | -0.042 | 2.022 | 0.940 | 1.523 |
|  | BQTerrace | -0.350 | -0.067 | -0.067 | 24.336 | 6.220 | 7.356 |
| C | BasketballDrill | -0.189 | -0.105 | -0.132 | 4.999 | 3.494 | 3.828 |
|  | BQMall | -0.261 | -0.048 | -0.068 | 6.284 | 2.112 | 2.518 |
|  | PartyScene | -0.172 | -0.044 | -0.057 | 4.635 | 2.281 | 2.813 |
|  | RaceHorses WVGA | -0.101 | -0.046 | -0.047 | 2.799 | 2.458 | 2.185 |
| D | BasketballPass | -0.212 | -0.076 | -0.138 | 4.524 | 2.279 | 3.827 |
|  | BQSquare | -0.763 | -0.129 | -0.110 | 22.597 | 10.965 | 5.528 |
|  | BlowingBubbles | -0.369 | -0.130 | -0.140 | 9.810 | 5.049 | 5.135 |
|  | RaceHorses WQVGA | -0.297 | -0.146 | -0.159 | 6.146 | 4.977 | 5.155 |
| Class A average |  | -0.188 | -0.056 | -0.047 | 5.678 | 2.754 | 2.132 |
| Class B average |  | -0.174 | -0.041 | -0.053 | 8.640 | 2.917 | 3.323 |
| Class C average |  | -0.181 | -0.061 | -0.076 | 4.679 | 2.586 | 2.836 |
| Class D average |  | -0.410 | -0.120 | -0.137 | 10.769 | 5.818 | 4.911 |
| Overall average |  | -0.241 | -0.069 | -0.081 | 7.757 | 3.580 | 3.458 |

Table 5.2: BD-PSNR and Rate results against CS2 type anchor without IBDI.

| Class | Sequence | BD-PSNR dB |  |  | BD-Rate % |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
| B | Kimono | -0.137 | -0.005 | -0.022 | 4.610 | 0.299 | 1.088 |
|  | ParkScene | -0.226 | -0.043 | -0.015 | 7.238 | 2.464 | 1.149 |
|  | Cactus | -0.314 | -0.091 | -0.206 | 12.310 | 7.501 | 10.616 |
|  | BasketballDrive | -0.058 | -0.020 | -0.031 | 2.179 | 0.913 | 1.220 |
|  | BQTerrace | -0.461 | -0.073 | -0.057 | 28.216 | 7.906 | 7.075 |
| C | BasketballDrill | -0.288 | -0.181 | -0.207 | 8.387 | 6.761 | 7.326 |
|  | BQMall | -0.306 | -0.088 | -0.080 | 7.640 | 5.802 | 3.403 |
|  | PartyScene | -0.243 | -0.044 | -0.054 | 7.408 | 2.858 | 3.363 |
|  | RaceHorses WVGA | -0.121 | -0.047 | -0.070 | 3.531 | 3.448 | 4.118 |
| D | BasketballPass | -0.219 | -0.105 | -0.129 | 4.889 | 3.886 | 4.038 |
|  | BQSquare | -0.535 | -0.074 | -0.135 | 16.425 | -5.650 | 14.816 |
|  | BlowingBubbles | -0.344 | -0.173 | -0.180 | 9.713 | 7.636 | 7.443 |
|  | RaceHorses WQVGA | -0.313 | -0.147 | -0.167 | 6.598 | 5.349 | 5.714 |
| E | BasketballPass | -0.704 | -0.149 | -0.231 | 24.764 | 8.040 | 10.596 |
|  | BQSquare | -0.764 | -0.127 | -0.185 | 23.050 | 8.167 | 7.293 |
|  | RaceHorses WQVGA | -0.464 | -0.112 | -0.160 | 17.882 | 5.197 | 7.216 |
| Class B average |  | -0.239 | -0.047 | -0.066 | 10.910 | 3.817 | 4.230 |
| Class C average |  | -0.239 | -0.090 | -0.103 | 6.741 | 4.717 | 4.552 |
| Class D average |  | -0.353 | -0.125 | -0.153 | 9.406 | 2.806 | 8.003 |
| Class E average |  | -0.644 | -0.129 | -0.192 | 21.899 | 7.134 | 8.369 |
| Overall average |  | -0.344 | -0.092 | -0.121 | 11.552 | 4.411 | 6.030 |

Table 5.3: BD-PSNR and Rate results against CS1 type anchor with IBDI.

| Class | Sequence | BD-PSNR dB |  |  | BD-Rate % |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
| A | Traffic | -0.028 | -0.015 | -0.020 | 0.875 | 0.705 | 0.933 |
|  | People on Street | -0.015 | -0.018 | -0.014 | 0.293 | 0.840 | 0.722 |
| B | Kimono | -0.010 | -0.005 | -0.019 | 0.344 | 0.280 | 0.875 |
|  | ParkScene | -0.012 | -0.015 | -0.018 | 0.362 | 0.675 | 1.070 |
|  | Cactus | -0.012 | -0.011 | -0.018 | 0.455 | 0.775 | 0.854 |
|  | BasketballDrive | -0.004 | -0.017 | -0.015 | 0.131 | 0.794 | 0.529 |
|  | BQTerrace | -0.021 | -0.023 | -0.014 | 1.084 | 1.525 | 1.041 |
| C | BasketballDrill | -0.018 | -0.014 | -0.011 | 0.439 | 0.443 | 0.316 |
|  | BQMall | -0.016 | -0.018 | -0.028 | 0.376 | 0.793 | 1.165 |
|  | PartyScene | -0.011 | -0.017 | -0.014 | 0.287 | 0.935 | 0.732 |
|  | RaceHorses WVGA | -0.008 | 0.000 | -0.011 | 0.207 | -0.002 | 0.490 |
| D | BasketballPass | -0.017 | -0.010 | -0.002 | 0.352 | 0.223 | 0.010 |
|  | BQSquare | -0.085 | -0.035 | -0.030 | 2.264 | 1.840 | 1.409 |
|  | BlowingBubbles | -0.031 | -0.003 | -0.014 | 0.780 | -0.025 | 0.473 |
|  | RaceHorses WQVGA | -0.017 | -0.020 | -0.013 | 0.340 | 0.486 | 0.359 |
| Class A average |  | -0.022 | -0.016 | -0.017 | 0.584 | 0.773 | 0.828 |
| Class B average |  | -0.012 | -0.014 | -0.017 | 0.475 | 0.810 | 0.874 |
| Class C average |  | -0.013 | -0.012 | -0.016 | 0.327 | 0.542 | 0.676 |
| Class D average |  | -0.038 | -0.017 | -0.015 | 0.934 | 0.631 | 0.563 |
| Overall average |  | -0.020 | -0.015 | -0.016 | 0.573 | 0.686 | 0.732 |

Table 5.4: BD-PSNR and Rate results against CS2 type anchor with IBDI.

| Class | Sequence | BD-PSNR dB |  |  | BD-Rate % |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
| B | Kimono | -0.022 | -0.020 | -0.044 | 0.702 | 0.961 | 2.014 |
|  | ParkScene | -0.025 | -0.020 | -0.035 | 0.753 | 1.112 | 2.237 |
|  | Cactus | -0.032 | -0.064 | -0.077 | 1.174 | 4.689 | 3.568 |
|  | BasketballDrive | -0.008 | -0.022 | -0.018 | 0.288 | 1.029 | 0.620 |
|  | BQTerrace | -0.045 | -0.058 | -0.075 | 2.445 | 7.427 | 11.246 |
| C | BasketballDrill | -0.021 | -0.029 | 0.012 | 0.601 | 1.207 | -0.309 |
|  | BQMall | -0.028 | -0.037 | 0.022 | 0.661 | 1.865 | -1.412 |
|  | PartyScene | -0.015 | -0.001 | -0.019 | 0.443 | 0.010 | 1.085 |
|  | RaceHorses WVGA | -0.009 | -0.007 | 0.009 | 0.260 | 0.452 | -0.498 |
| D | BasketballPass | -0.028 | -0.046 | -0.077 | 0.634 | 1.651 | 2.204 |
|  | BQSquare | -0.047 | -0.052 | -0.121 | 1.329 | 3.807 | 8.735 |
|  | BlowingBubbles | -0.040 | -0.011 | 0.000 | 1.099 | 0.447 | -0.181 |
|  | RaceHorses WQVGA | -0.023 | -0.005 | -0.029 | 0.437 | -0.264 | 1.319 |
| E | BasketballPass | -0.098 | -0.158 | -0.296 | 2.973 | 9.376 | 12.350 |
|  | BQSquare | -0.094 | -0.186 | -0.171 | 2.483 | 10.256 | 7.929 |
|  | RaceHorses WQVGA | -0.065 | -0.236 | -0.251 | 2.231 | 11.748 | 11.462 |
| Class B average |  | -0.026 | -0.037 | -0.050 | 1.072 | 3.044 | 3.937 |
| Class C average |  | -0.018 | -0.018 | 0.006 | 0.491 | 0.884 | -0.283 |
| Class D average |  | -0.035 | -0.029 | -0.057 | 0.875 | 1.410 | 3.019 |
| Class E average |  | -0.086 | -0.193 | -0.239 | 2.562 | 10.460 | 10.580 |
| Overall average |  | -0.038 | -0.059 | -0.073 | 1.157 | 3.486 | 3.898 |

Table 5.5: BD-PSNR and Rate results against CS1 anchor without IBDI, where memory compression/decompression is performed only at the decoder side.

| Class | Sequence | BD-PSNR dB |  |  | BD-Rate % |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
| A | Traffic | -0.405 | -0.088 | -0.071 | 13.025 | 4.214 | 3.294 |
|  | People on Street | -0.102 | -0.017 | -0.015 | 2.132 | 0.760 | 0.605 |
| B | Kimono | -0.156 | -0.020 | -0.031 | 5.367 | 1.075 | 1.419 |
|  | ParkScene | -0.185 | -0.047 | -0.025 | 5.690 | 2.213 | 1.249 |
|  | Cactus | -0.274 | -0.057 | -0.110 | 10.833 | 4.613 | 5.409 |
|  | BasketballDrive | -0.090 | -0.042 | -0.076 | 3.590 | 2.004 | 2.775 |
|  | BQTerrace | -0.593 | -0.044 | -0.051 | 44.098 | 3.976 | 5.412 |
| C | BasketballDrill | -0.249 | -0.108 | -0.134 | 6.554 | 3.585 | 3.860 |
|  | BQMall | -0.414 | -0.050 | -0.083 | 9.864 | 2.143 | 3.040 |
|  | PartyScene | -0.239 | -0.049 | -0.059 | 6.469 | 2.558 | 2.894 |
|  | RaceHorses WVGA | -0.134 | -0.067 | -0.091 | 3.706 | 3.581 | 4.240 |
| D | BasketballPass | -0.313 | -0.097 | -0.166 | 6.635 | 2.928 | 4.580 |
|  | BQSquare | -1.381 | -0.056 | -0.067 | 45.095 | 4.310 | 3.330 |
|  | BlowingBubbles | -0.589 | -0.145 | -0.149 | 15.601 | 5.640 | 5.446 |
|  | RaceHorses WQVGA | -0.456 | -0.186 | -0.226 | 9.268 | 6.219 | 7.200 |
| Class A average |  | -0.253 | -0.053 | -0.043 | 7.579 | 2.487 | 1.949 |
| Class B average |  | -0.260 | -0.042 | -0.059 | 13.915 | 2.776 | 3.253 |
| Class C average |  | -0.259 | -0.069 | -0.092 | 6.648 | 2.967 | 3.509 |
| Class D average |  | -0.684 | -0.121 | -0.152 | 19.150 | 4.774 | 5.139 |
| Overall average |  | -0.372 | -0.072 | -0.090 | 12.528 | 3.321 | 3.650 |

Table 5.6: BD-PSNR and Rate results against CS2 anchor without IBDI, where memory compression/decompression is performed only at the decoder side.

| Class | Sequence | BD-PSNR dB |  |  | BD-Rate % |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
| B | Kimono | -0.971 | -0.083 | -0.115 | 34.471 | 3.906 | 5.293 |
|  | ParkScene | -1.367 | -0.256 | -0.058 | 54.270 | 14.955 | 3.898 |
|  | Cactus | -1.232 | -0.168 | -0.336 | 52.636 | 13.999 | 18.227 |
|  | BasketballDrive | -0.492 | -0.259 | -0.447 | 19.554 | 12.654 | 15.969 |
|  | BQTerrace | -2.001 | -0.266 | -0.123 | 260.048 | 93.660 | 13.893 |
| C | BasketballDrill | -0.911 | -0.376 | -0.444 | 28.907 | 18.086 | 15.618 |
|  | BQMall | -0.999 | -0.124 | -0.270 | 25.455 | 6.360 | 11.637 |
|  | PartyScene | -0.844 | -0.300 | -0.340 | 29.036 | 21.605 | 25.430 |
|  | RaceHorses WVGA | -0.612 | -0.329 | -0.618 | 19.134 | 25.879 | 44.212 |
| D | BasketballPass | -1.364 | -0.287 | -0.842 | 33.137 | 10.306 | 29.759 |
|  | BQSquare | -2.690 | -0.053 | -0.137 | 157.098 | 11.095 | 13.429 |
|  | BlowingBubbles | -1.934 | -0.630 | -0.473 | 74.144 | 24.197 | 15.174 |
|  | RaceHorses WQVGA | -2.090 | -0.725 | -1.444 | 52.165 | 29.396 | 86.530 |
| E | BasketballPass | -1.492 | -0.036 | -0.144 | 53.015 | 1.910 | 5.900 |
|  | BQSquare | -3.057 | -0.001 | -0.009 | 126.190 | 0.039 | 0.493 |
|  | RaceHorses WQVGA | -1.525 | -0.036 | -0.109 | 67.775 | 1.529 | 4.730 |
| Class B average |  | -1.213 | -0.207 | -0.216 | 84.196 | 27.835 | 11.456 |
| Class C average |  | -0.842 | -0.282 | -0.418 | 25.633 | 17.983 | 24.224 |
| Class D average |  | -2.019 | -0.424 | -0.724 | 79.136 | 18.749 | 36.223 |
| Class E average |  | -2.025 | -0.025 | -0.087 | 82.327 | 1.160 | 3.708 |
| Overall average |  | -1.474 | -0.246 | -0.369 | 67.940 | 18.099 | 19.387 |

Table 5.7: BD-PSNR and Rate results against CS1 type anchor with IBDI, where memory compression/decompression is performed only at the decoder side.

| Class | Sequence | BD-PSNR dB |  |  | BD-Rate % |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
| A | Traffic | -0.042 | -0.025 | -0.032 | 1.299 | 1.131 | 1.422 |
|  | People on Street | -0.009 | -0.026 | -0.014 | 0.179 | 1.166 | 0.659 |
| B | Kimono | -0.025 | -0.020 | -0.030 | 0.864 | 1.055 | 1.387 |
|  | ParkScene | -0.016 | -0.012 | -0.017 | 0.480 | 0.503 | 0.828 |
|  | Cactus | -0.024 | -0.020 | -0.034 | 0.942 | 1.489 | 1.572 |
|  | BasketballDrive | -0.015 | -0.028 | -0.039 | 0.594 | 1.306 | 1.406 |
|  | BQTerrace | -0.035 | -0.040 | -0.055 | 1.915 | 3.455 | 4.776 |
| C | BasketballDrill | -0.020 | -0.014 | -0.023 | 0.509 | 0.436 | 0.635 |
|  | BQMall | -0.034 | -0.033 | -0.027 | 0.803 | 1.460 | 0.956 |
|  | PartyScene | -0.013 | -0.011 | -0.021 | 0.328 | 0.565 | 1.012 |
|  | RaceHorses WVGA | -0.011 | -0.009 | -0.022 | 0.296 | 0.485 | 0.987 |
| D | BasketballPass | -0.034 | -0.023 | -0.031 | 0.722 | 0.679 | 0.885 |
|  | BQSquare | -0.115 | -0.042 | -0.049 | 2.976 | 2.031 | 2.276 |
|  | BlowingBubbles | -0.042 | -0.032 | -0.040 | 1.058 | 1.200 | 1.310 |
|  | RaceHorses WQVGA | -0.035 | -0.014 | -0.033 | 0.693 | 0.382 | 0.959 |
| Class A average |  | -0.025 | -0.025 | -0.023 | 0.739 | 1.149 | 1.040 |
| Class B average |  | -0.023 | -0.024 | -0.035 | 0.959 | 1.562 | 1.994 |
| Class C average |  | -0.020 | -0.017 | -0.023 | 0.484 | 0.736 | 0.898 |
| Class D average |  | -0.057 | -0.028 | -0.038 | 1.362 | 1.073 | 1.357 |
| Overall average |  | -0.031 | -0.023 | -0.031 | 0.910 | 1.156 | 1.405 |

Table 5.8: BD-PSNR and Rate results against CS2 type anchor with IBDI, where memory compression/decompression is performed only at the decoder side.

| Class | Sequence | BD-PSNR dB |  |  | BD-Rate % |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
| B | Kimono | -0.735 | -1.123 | -1.277 | 24.115 | 75.154 | 79.835 |
|  | ParkScene | -0.591 | -0.951 | -1.161 | 19.985 | 71.111 | 132.927 |
|  | Cactus | -0.363 | -0.561 | -0.811 | 13.902 | 53.079 | 47.308 |
|  | BasketballDrive | -0.529 | -1.172 | -1.033 | 18.386 | 67.330 | 40.351 |
|  | BQTerrace | -0.335 | -0.633 | -0.841 | 20.332 | 107.497 | 181.745 |
| C | BasketballDrill | -0.316 | -0.542 | -0.444 | 9.197 | 23.085 | 14.836 |
|  | BQMall | -0.243 | -0.740 | -0.657 | 5.815 | 51.360 | 33.132 |
|  | PartyScene | -0.096 | -0.259 | -0.326 | 2.842 | 20.393 | 26.399 |
|  | RaceHorses WVGA | -0.248 | -0.714 | -1.161 | 7.228 | 68.470 | 94.906 |
| D | BasketballPass | -0.417 | -0.708 | -0.627 | 9.352 | 27.492 | 19.858 |
|  | BQSquare | -0.305 | -0.427 | -0.456 | 9.025 | 42.256 | 37.793 |
|  | BlowingBubbles | -0.345 | -0.684 | -0.794 | 9.638 | 31.375 | 37.009 |
|  | RaceHorses WQVGA | -0.612 | -0.981 | -1.376 | 12.834 | 41.634 | 58.743 |
| E | BasketballPass | -0.571 | -0.755 | -1.038 | 17.550 | 47.236 | 51.723 |
|  | BQSquare | -0.411 | -0.989 | -0.659 | 10.802 | 60.600 | 33.151 |
|  | RaceHorses WQVGA | -0.627 | -1.519 | -1.236 | 23.009 | 93.575 | 63.204 |
| Class B average |  | -0.511 | -0.888 | -1.025 | 19.344 | 74.834 | 96.433 |
| Class C average |  | -0.226 | -0.564 | -0.647 | 6.271 | 40.827 | 42.318 |
| Class D average |  | -0.420 | -0.700 | -0.813 | 10.212 | 35.689 | 38.351 |
| Class E average |  | -0.536 | -1.088 | -0.978 | 17.121 | 67.137 | 49.359 |
| Overall average |  | -0.421 | -0.797 | -0.869 | 13.376 | 55.103 | 59.558 |

## Complexity

For complexity evaluation by encoding and decoding times, the executables built by Microsoft Visual C++ 2008 SP1 were run on following Windows-based computing platforms:

- OS: Windows XP 64-bit, CPU: Core i7 960 3.20GHz, Memory: 12GB.

- OS: Windows Vista 64-bit, CPU: Core i7 940 2.93GHz, Memory: 12GB.

- OS: Windows Vista 32-bit, CPU: Core i7 920 2.67GHz, Memory: 4GB.

The 64-bit executable was used for the platforms (a) and (b), and the 32-bit executable was used for the platform (c). The platform (a) was used for encoding Class A sequences and decoding all sequences. The platform (b) was used for encoding Class B and E sequences. The platform (c) was used for encoding Class C and D sequences. Tables 5.9-12 show the encoding and decoding times for each test case and each test sequence, by averaging results in five rate points.

It is observed that encoding time increase is negligible for all cases; however as for decoding time, a significant amount of time is consumed for the proposed memory compression process. The time consumption is due to the software implementation submitted to TE2. Since the implementation put a top priority on simplicity and readability, it is not fine-tuned in terms of the speed. The most time-consuming process in the implementation is search for an index to a quantization representative value.

Another implementation has then been tested, where the search process described above is done with a pre-expanded table. The results are shown in Table 5.13 and Table 5.14. It can be observed that the decoding time can get much faster by the implementation technique.

Table 5.9: Average encoding time without IBDI.

| Class | Sequence | Anchor sec |  | Proposal sec |  | Increase % |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | CS1 | CS2 | CS1 | CS2 | CS1 | CS2 |
| A | Traffic | 47722 |  | 47967 |  | 0.51 |  |
|  | People on Street | 54100 |  | 54824 |  | 1.34 |  |
| B | Kimono | 58102 | 21963 | 58233 | 21569 | 0.23 | -1.79 |
|  | ParkScene | 56865 | 20854 | 56218 | 20768 | -1.14 | -0.41 |
|  | Cactus | 54982 | 24315 | 55112 | 23042 | 0.24 | -5.24 |
|  | BasketballDrive | 61761 | 20235 | 61110 | 19613 | -1.05 | -3.07 |
|  | BQTerrace | 55393 | 21854 | 56062 | 22453 | 1.21 | 2.74 |
| C | BasketballDrill | 14536 | 5792 | 13332 | 5036 | -8.28 | -13.06 |
|  | BQMall | 14995 | 7036 | 15071 | 6657 | 0.51 | -5.38 |
|  | PartyScene | 15249 | 6387 | 15283 | 6126 | 0.23 | -4.08 |
|  | RaceHorses WVGA | 19315 | 9619 | 19423 | 9536 | 0.56 | -0.87 |
| D | BasketballPass | 2966 | 1494 | 2971 | 1459 | 0.16 | -2.33 |
|  | BQSquare | 2841 | 1124 | 2852 | 1140 | 0.38 | 1.43 |
|  | BlowingBubbles | 3203 | 1795 | 3203 | 1805 | -0.01 | 0.56 |
|  | RaceHorses WQVGA | 4245 | 2394 | 4262 | 2330 | 0.39 | -2.65 |
| E | Vidyo1 |  | 4816 |  | 4541 |  | -5.70 |
|  | Vidyo3 |  | 5115 |  | 4869 |  | -4.81 |
|  | Vidyo4 |  | 4712 |  | 4722 |  | 0.22 |
| Class A average |  |  |  |  |  | 0.91 |  |
| Class B average |  |  |  |  |  | -0.02 | -1.50 |
| Class C average |  |  |  |  |  | -1.89 | -5.63 |
| Class D average |  |  |  |  |  | 0.26 | -0.78 |
| Class E average |  |  |  |  |  |  | -3.18 |
| Overall average |  |  |  |  |  | -0.32 | -2.67 |

Table 5.10: Average encoding time with IBDI.

| Class | Sequence | Anchor sec |  | Proposal sec |  | Increase % |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | CS1 | CS2 | CS1 | CS2 | CS1 | CS2 |
| A | Traffic | 49597 |  | 49501 |  | -0.19 |  |
|  | People on Street | 57144 |  | 57112 |  | -0.06 |  |
| B | Kimono | 59521 | 26548 | 60176 | 27515 | 1.10 | 3.64 |
|  | ParkScene | 59126 | 23460 | 58447 | 23869 | -1.15 | 1.74 |
|  | Cactus | 58195 | 33024 | 59688 | 30187 | 2.56 | 0.54 |
|  | BasketballDrive | 64777 | 27179 | 64827 | 27818 | 0.08 | 2.35 |
|  | BQTerrace | 58661 | 26654 | 58521 | 26752 | -0.24 | 0.37 |
| C | BasketballDrill | 13822 | 6225 | 15479 | 6827 | 11.99 | 9.68 |
|  | BQMall | 15811 | 8341 | 15798 | 8589 | -0.08 | 2.98 |
|  | PartyScene | 15879 | 7848 | 16375 | 8213 | 3.12 | 4.65 |
|  | RaceHorses WVGA | 20141 | 11036 | 20249 | 11299 | 0.53 | 2.38 |
| D | BasketballPass | 3084 | 1666 | 3095 | 1682 | 0.37 | 0.97 |
|  | BQSquare | 2990 | 1419 | 3021 | 1590 | 1.05 | 12.05 |
|  | BlowingBubbles | 3308 | 2117 | 3317 | 2117 | 0.26 | 0.01 |
|  | RaceHorses WQVGA | 4362 | 2696 | 4364 | 2756 | 0.05 | 2.22 |
| E | Vidyo1 |  | 6414 |  | 6402 |  | -0.18 |
|  | Vidyo3 |  | 6569 |  | 6982 |  | 6.29 |
|  | Vidyo4 |  | 5905 |  | 5735 |  | -2.89 |
| Class A average |  |  |  |  |  | -0.11 |  |
| Class B average |  |  |  |  |  | 0.52 | 1.87 |
| Class C average |  |  |  |  |  | 4.28 | 6.71 |
| Class D average |  |  |  |  |  | 0.43 | 3.84 |
| Class E average |  |  |  |  |  |  | 1.57 |
| Overall average |  |  |  |  |  | 1.41 | 3.51 |

Table 5.11: Average decoding time without IBDI.

| Class | Sequence | Anchor sec |  | Proposal sec |  | Increase % |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | CS1 | CS2 | CS1 | CS2 | CS1 | CS2 |
| A | Traffic | 70.890 |  | 90.950 |  | 28.30 |  |
|  | People on Street | 70.062 |  | 84.319 |  | 20.35 |  |
| B | Kimono | 64.950 | 63.535 | 78.650 | 77.816 | 21.09 | 22.48 |
|  | ParkScene | 64.237 | 59.650 | 78.360 | 77.634 | 21.98 | 30.15 |
|  | Cactus | 46.543 | 42.097 | 63.153 | 65.016 | 35.69 | 54.44 |
|  | BasketballDrive | 62.466 | 58.962 | 74.262 | 71.462 | 18.89 | 21.20 |
|  | BQTerrace | 60.207 | 55.103 | 75.500 | 73.175 | 25.40 | 32.80 |
| C | BasketballDrill | 8.025 | 7.759 | 10.644 | 10.797 | 32.64 | 39.15 |
|  | BQMall | 8.072 | 7.512 | 10.937 | 10.806 | 35.50 | 43.85 |
|  | PartyScene | 10.006 | 8.519 | 12.444 | 11.465 | 24.36 | 34.59 |
|  | RaceHorses WVGA | 10.553 | 10.753 | 13.182 | 13.759 | 24.91 | 27.95 |
| D | BasketballPass | 2.037 | 1.794 | 2.659 | 2.459 | 30.53 | 37.12 |
|  | BQSquare | 2.519 | 2.284 | 3.097 | 2.897 | 22.95 | 26.80 |
|  | BlowingBubbles | 2.444 | 2.066 | 3.141 | 2.766 | 28.51 | 33.87 |
|  | RaceHorses WQVGA | 2.881 | 3.015 | 3.628 | 3.825 | 25.92 | 26.85 |
| E | Vidyo1 |  | 13.225 |  | 24.281 |  | 83.60 |
|  | Vidyo3 |  | 14.134 |  | 22.650 |  | 60.25 |
|  | Vidyo4 |  | 15.312 |  | 24.256 |  | 58.41 |
| Class A average |  |  |  |  |  | 24.32 |  |
| Class B average |  |  |  |  |  | 24.62 | 43.15 |
| Class C average |  |  |  |  |  | 29.48 | 51.47 |
| Class D average |  |  |  |  |  | 27.10 | 49.47 |
| Class E average |  |  |  |  |  |  | 62.55 |
| Overall average |  |  |  |  |  | 26.54 | 50.45 |

Table 5.12: Average decoding time with IBDI.

| Class | Sequence | Anchor sec |  | Proposal sec |  | Increase % |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | CS1 | CS2 | CS1 | CS2 | CS1 | CS2 |
| A | Traffic | 60.631 |  | 86.428 |  | 42.55 |  |
|  | People on Street | 59.032 |  | 83.753 |  | 41.88 |  |
| B | Kimono | 65.619 | 60.665 | 88.241 | 84.181 | 34.47 | 38.76 |
|  | ParkScene | 64.397 | 57.991 | 86.928 | 82.087 | 34.99 | 41.55 |
|  | Cactus | 46.468 | 42.482 | 67.966 | 64.978 | 46.26 | 52.96 |
|  | BasketballDrive | 62.541 | 58.273 | 84.825 | 81.288 | 35.63 | 39.50 |
|  | BQTerrace | 60.337 | 54.443 | 82.628 | 77.544 | 36.94 | 42.43 |
| C | BasketballDrill | 8.241 | 7.835 | 12.406 | 12.188 | 50.55 | 55.56 |
|  | BQMall | 8.124 | 7.615 | 12.428 | 11.897 | 52.97 | 56.23 |
|  | PartyScene | 9.985 | 8.634 | 14.284 | 13.041 | 43.06 | 51.03 |
|  | RaceHorses WVGA | 10.794 | 11.069 | 15.234 | 15.575 | 41.14 | 40.71 |
| D | BasketballPass | 2.068 | 1.816 | 3.144 | 2.897 | 51.98 | 59.55 |
|  | BQSquare | 2.550 | 2.313 | 3.606 | 3.375 | 41.45 | 45.94 |
|  | BlowingBubbles | 2.506 | 2.075 | 3.593 | 3.203 | 43.36 | 54.35 |
|  | RaceHorses WQVGA | 2.938 | 3.094 | 4.075 | 4.238 | 38.69 | 36.96 |
| E | Vidyo1 |  | 13.519 |  | 22.741 |  | 68.22 |
|  | Vidyo3 |  | 14.172 |  | 22.806 |  | 60.93 |
|  | Vidyo4 |  | 15.425 |  | 24.466 |  | 58.61 |
| Class A average |  |  |  |  |  | 42.26 |  |
| Class B average |  |  |  |  |  | 37.67 | 43.15 |
| Class C average |  |  |  |  |  | 47.14 | 51.47 |
| Class D average |  |  |  |  |  | 44.17 | 49.47 |
| Class E average |  |  |  |  |  |  | 62.55 |
| Overall average |  |  |  |  |  | 42.54 | 50.45 |

Table 5.13: Fast implementation results on average decoding time without IBDI.

| Class | Sequence | Anchor sec |  | Proposal sec |  | Increase % |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | CS1 | CS2 | CS1 | CS2 | CS1 | CS2 |
| A | Traffic | 70.890 |  | 77.681 |  | 9.58 |  |
|  | People on Street | 70.062 |  | 71.606 |  | 2.20 |  |
| B | Kimono | 64.950 | 63.535 | 68.025 | 67.203 | 4.73 | 5.77 |
|  | ParkScene | 64.237 | 59.650 | 67.128 | 66.209 | 4.50 | 11.00 |
|  | Cactus | 46.543 | 42.097 | 51.841 | 53.797 | 11.38 | 27.79 |
|  | BasketballDrive | 62.466 | 58.962 | 64.182 | 61.213 | 2.75 | 3.82 |
|  | BQTerrace | 60.207 | 55.103 | 64.078 | 61.881 | 6.43 | 12.30 |
| C | BasketballDrill | 8.025 | 7.759 | 8.440 | 8.491 | 5.18 | 9.43 |
|  | BQMall | 8.072 | 7.512 | 8.681 | 8.531 | 7.55 | 13.57 |
|  | PartyScene | 10.006 | 8.519 | 10.034 | 9.041 | 0.28 | 6.13 |
|  | RaceHorses WVGA | 10.553 | 10.753 | 10.841 | 11.375 | 2.73 | 5.78 |
| D | BasketballPass | 2.037 | 1.794 | 2.115 | 1.916 | 3.83 | 6.81 |
|  | BQSquare | 2.519 | 2.284 | 2.553 | 2.360 | 1.37 | 3.29 |
|  | BlowingBubbles | 2.444 | 2.066 | 2.522 | 2.150 | 3.19 | 4.07 |
|  | RaceHorses WQVGA | 2.881 | 3.015 | 3.022 | 3.194 | 4.88 | 5.91 |
| E | Vidyo1 |  | 13.225 |  | 19.734 |  | 49.22 |
|  | Vidyo3 |  | 14.134 |  | 18.365 |  | 29.93 |
|  | Vidyo4 |  | 15.312 |  | 19.897 |  | 29.94 |
| Class A average |  |  |  |  |  | 5.89 |  |
| Class B average |  |  |  |  |  | 5.96 | 12.18 |
| Class C average |  |  |  |  |  | 3.94 | 8.64 |
| Class D average |  |  |  |  |  | 3.28 | 5.02 |
| Class E average |  |  |  |  |  |  | 36.43 |
| Overall average |  |  |  |  |  | 4.70 | 14.05 |

Table 5.14: Fast implementation results on average decoding time with IBDI.

| Class | Sequence | Anchor sec |  | Proposal sec |  | Increase % |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | CS1 | CS2 | CS1 | CS2 | CS1 | CS2 |
| A | Traffic | 60.631 |  | 61.197 |  | 0.93 |  |
|  | People on Street | 59.032 |  | 59.113 |  | 0.14 |  |
| B | Kimono | 65.619 | 60.665 | 66.734 | 62.669 | 1.70 | 3.30 |
|  | ParkScene | 64.397 | 57.991 | 64.984 | 59.522 | 0.91 | 2.64 |
|  | Cactus | 46.468 | 42.482 | 47.253 | 44.278 | 1.69 | 4.23 |
|  | BasketballDrive | 62.541 | 58.273 | 63.388 | 59.340 | 1.35 | 1.83 |
|  | BQTerrace | 60.337 | 54.443 | 61.434 | 55.941 | 1.82 | 2.75 |
| C | BasketballDrill | 8.241 | 7.835 | 8.360 | 8.047 | 1.44 | 2.71 |
|  | BQMall | 8.124 | 7.615 | 8.397 | 7.860 | 3.36 | 3.21 |
|  | PartyScene | 9.985 | 8.634 | 10.078 | 8.756 | 0.94 | 1.41 |
|  | RaceHorses WVGA | 10.794 | 11.069 | 10.869 | 11.209 | 0.70 | 1.27 |
| D | BasketballPass | 2.068 | 1.816 | 2.134 | 1.881 | 3.18 | 3.61 |
|  | BQSquare | 2.550 | 2.313 | 2.603 | 2.375 | 2.09 | 2.69 |
|  | BlowingBubbles | 2.506 | 2.075 | 2.531 | 2.134 | 0.98 | 2.84 |
|  | RaceHorses WQVGA | 2.938 | 3.094 | 2.997 | 3.134 | 2.01 | 1.30 |
| E | Vidyo1 |  | 13.519 |  | 14.463 |  | 6.98 |
|  | Vidyo3 |  | 14.172 |  | 15.000 |  | 5.84 |
|  | Vidyo4 |  | 15.425 |  | 16.484 |  | 6.87 |
| Class A average |  |  |  |  |  | 0.54 |  |
| Class B average |  |  |  |  |  | 1.49 | 2.96 |
| Class C average |  |  |  |  |  | 1.62 | 2.15 |
| Class D average |  |  |  |  |  | 2.08 | 2.58 |
| Class E average |  |  |  |  |  |  | 6.48 |
| Overall average |  |  |  |  |  | 1.56 | 3.32 |

# Discussion

As described in section 2, the proposed method is designed to reduce actual memory bandwidth with relatively low complexity, in particular, in motion compensation. Frequent, random and unaligned accesses to frame memory are caused in MC and it is therefore required that the compression data structure is simple and its complexity is relatively low in order to reduce actual memory bandwidth in restoring compressed data in frame memory to images in MC.

In TE2, the actual memory bandwidth in particular in MC has not been evaluated, though memory size reduction has been evaluated. Current implementations of memory compression tools including the proposed method are not ready to evaluate this aspect. TE2 results have verified that memory compression should be a good option for memory-conscious implementations. Hence as the next step, it is desired that the performance in realistic situations including actual memory bandwidth will be next studied in TE/CE on memory compression for Test Model.

# Conclusion

This contribution has presented the 1-D DPCM-based memory compression method proposed in [3] for TE2 and has reported its performance. The proposed method is designed to reduce actual memory bandwidth with relatively low complexity, in particular, in motion compensation. Motion compensation frequently and randomly accesses to frame memory for reading reference pixels. In consideration of the tradeoff between memory accessibility and image quality, the proposed method employs simple 1-D DPCM. It has been tested with two constraint sets in two experimental conditions: one is 3/4 memory compression without IBDI and another is memory retainment even with 12-bit IBDI. Experimental results have shown that average coding losses are 7.757 % for CS1 without IBDI, 11.552 % for CS2 without IBDI, 0.573 % for CS1 with IBDI and 1.157 % for CS2 with IBDI, respectively. Subjective quality degradation caused by such coding loss is invisible for most test cases. Additional experimental results have also shown that memory compression only at the decoder side causes significant coding loss. It is therefore suggested that memory compression tool should be adopted as one of optional codec components so as to be available for memory-conscious hardware implementations, such as SoCs. As for memory accessibility, the impact on actual memory bandwidth has not yet been evaluated in TE2. Hence it is recommended that it is next studied in TE/CE on memory compression for Test Model.

# References

[1] U. Bayazit, et al., “A novel memory compression system for MPEG-2 decoders,” Proc. of IEEE International Conference on Consumer Electronics ’98, pp. 56-57, 1998.

[2] R. Bruni, et al., “A novel adaptive vector quantization method for memory reduction in MPEG-2 HDTV decoders,” Proc. of IEEE International Conference on Consumer Electronics ‘98, pp. 58-59, 1998.

[3] P. H. N. de With, et al., “An MPEG decoder with embedded compression for memory reduction,” Proc. of IEEE International Conference on Consumer Electronics ‘98, pp. 60-61, 1998.

[4] C. W. J. Shih, et al., “Memory reduction by Haar wavelet transform for MPEG decoder,” Proc. of IEEE International Conference on Consumer Electronics ‘99, pp. 116-117, 1999.

[5] T. Takizawa, et al., “High performance and cost effective memory architecture for an HDTV decoder LSI,” Proc. of 1999 IEEE International Conference on Acoustics, Speech, and Signal Processing, pp. 1981-1984, 1999.

[6] J. Tajime, et al., “Memory compression method considering memory bandwidth for HDTV decoder,” Proc. of 1999 IEEE International Conference on Image Processing, pp. 779-782, 1999.

[7] T. Y. Lee, “A new frame-recompression algorithm and its hardware design for MPEG-2 video decoders,” IEEE Tran. Circuits Syst. Video Technol., vol. 13, pp. 529-534, 2003.

[8] M. Budagavi and M. Zhou, “Video coding using compressed reference frames,” VCEG document VCEG-AE19, ITU-T Q.6/SG 16 31st VCEG meeting, Marrakech, 2007.

[9] M. Budagavi, V. Sze, M. U. Demircin, S. Dikbas, M. Zhou and A. P. Chandrakasan, “Description of video coding technology proposal by Texas Instruments Inc.,” JCT-VC document JCTVC-A101, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 1st meeting, Geneva, 2010.

[10] J. Tajime and Y. Miyamoto, "Memory compression method for avoiding perceivable distortion due to intra-prediction in H.264 decoders," Proc. of IEEE International Conference on Consumer Electronics, 2007.

[11] T. Chujoh, "Tool Experiment 2 on IBDI and memory compression,” JCT-VC document JCTVC-A302r1, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 1st Meeting, Geneva, 2010.

# Patent rights declaration(s)

NEC Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).