| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 2nd Meeting: Geneva, CH, 21-28 July, 2010 | Document: JCTVC-B059 Appendix |
| --- | --- |

| Title: | Adaptive frequency weighting quantization in macroblock level |  |  |
| --- | --- | --- | --- |
| Status: | [Input Document to JCT-VC] |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianhua Zheng Section B2, Huawei Industrial Base, Bantian Longgang District, Shenzhen 518129, China | Tel: Email: | Tel: +86-10-82836652 Email: zhengjianhua@huawei.com |
| Source: | Hisilicon Technologies & Huawei Technologies |  |  |

_____________________________

# Appendix

The process to derive the quantization mode of Block E：

if either A,B,C or D is not aviable, quantization_mode of E is derived as follows,

- if the macroblock is coded as I_SLICE macroblock, quantization_mode is set equal to q_mode_1;

- if the macroblock is coded as P_SLICE macroblock, quantization_mode is set equal to q_mode_0;

- if the macroblock is coded as B_SLICE macroblock, quantization_mode is set equal to q_mode_2;

if A,B,C,D are all avilable, then the following applies,

(1) if the macroblock is coded as I_SLICE macroblock,let quantization_mode of E be derived as follows,

-if mb_type for either A,B,C or D is equal to I_4x4, quantization mode  for E is set equal to q_mode_1;

- else if mb_types for A and B are both I_16x16, and mb_types for A and D are both I_16x16, quantization mode for E is set equal to q_mode_0;

- else if intra_predict_mode of A and B are the same mode, and intra_predict_mode of A and D are the same mode,quantization mode for E is set equal to q_mode_0;

- else quantization mode for E is set equal to q_mode_1.

(2) if the macroblock is coded as P_SLICE macroblock,let quantization_mode of E be derived as follows,

- if mb_type for either A,B,C or D is equal to P_4x4, quantization mode for E is set equal to q_mode_1;

- else if mb_type for either A,B,C or D is equal to P_8x8, if reference picture, quantization mode for E is set equal to q_mode_0, else quantization mode for E is set equal to q_mode_2;

-  if mb_type for A and D are P_Skip mode, or mb_type for B and D are P_Skip mode, or mb_type for A and B are P_Skip mode, if reference picture, quantization mode for E is set equal to q_mode_1, else quantization mode for E is set equal to q_mode_2;

-  if mb_type for A is P_16x8, and mb_type for B is P_8x16, if reference picture, quantization mode for E is set equal to q_mode_1,else quantization mode for E is set equal to q_mode_2;

-  if mb_type for either A,B or D is P_16x16, quantization mode for E is set equal to q_mode_2;

-  else if reference picture, quantization mode for E is set equal to q_mode_0,else quantization mode for E is set equal to q_mode_2;

(3) if the macroblock is coded as B_SLICE macroblock,let quantization_mode of E be derived as follows,

- if mb_type for either A,B,C or D is equal to B_4x4 or B_8x8, if reference picture, quantization mode for E is set equal to q_mode_0,else quantization mode for E is set equal to q_mode_2;

- else if mb_type for A and D are B_Skip mode, or mb_type for B and D are B_Skip mode, quantization mode for E is set equal to q_mode_2;

- else if mb_type for A and D are B_16x16 mode, or mb_type for B and D are B_16x16 mode, quantization mode for E is set equal to q_mode_2;

- else quantization mode for E is set equal to q_mode_2.