| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 2nd Meeting: Geneva, CH, 21-28 July, 2010 | Document: JCTVC-B083 |
| --- | --- |

| Title: | Study of MC interpolation filter for Bi-prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenji Kondo and Teruhiko Suzuki Gate City Osaki West 12F 1-11-1 Osaki, Shinagawa-ku Tokyo, 141-0032, JAPAN | Tel: Email: | +81-3-5435-3308 teruhikos@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Introduction

Bi-pred/Single pred MC filter switching was proposed in JCT-VC A103 as a response of Call for Proposal. This contribution provides the additional information. In this technique, MC interpolation filters are designed for single pred and bi-pred. The MC filter sets are switched if MB pred mode is single pred or bi-pred at MB level. Two tap bi-linear low pass filter is applied when MB is encoded as bi-pred and it removes high frequency component too much in some cases. The coding efficiency can be improved by enhancing mid frequency for bi-pred. This contribution provides simulation results for this tool. The average BD bitarte gain is about -1.0 % and the maximum gain is about -14.9 %. This contribution also provides complexity analysis.

## Bi-pred / Single pred filter switch

When Bi-pred is used, strong low pass filter is applied by 2 tap filter. It reduces the efficiency in B-picture. Coding efficiency can be improved to design interpolation filter for bi-pred taking such strong low pass characteristics into account. Two sets of interpolation filter coefficients are used in B-picture. One is for single pred and the other is for bi-pred.

### Interpolation filter for single pred and bi-pred

Due to the loss of high frequency using 2 tap average filter we propose to transmit two sets of filter coefficients for adaptive Bi/single separable filters. For bi-pred, we expect to improve the quality of reconstructed picture by designing interpolation filter that enhances the mid-frequency range in order to compensate for high frequency loss due to 2 tap filter.

For single pred, enhancement of mid frequency range enhances, unnecessarily, the high frequency, which could result in reduced coding efficiency. Suppression of high frequency is thus more desirable for single prediction.

### Frequency response of interpolation filter

The example of frequency response of interpolation filter for single pred and bi-pred is described in this sub clause. Figure 1 shows examples of frequency response of filter coefficient at half pel position. The frequency is normalized so that the Nyquist frequency is 0.5 rad. Blue, green and red line correspond with interpolation filter for bi-pred in B picture, single pred in B picture and single pred in P picture.

Error: Reference source not foundFigure 1: Frequency response for half pel position

As shown in this figure, the amplitude around 0.25 rad (mid frequency) is rather enhanced for bi-pred (blue line). The frequency response for single pred (red line) is similar to the conventional MC interpolation filter, e.g. AVC 6 tap interpolation filter. The filter for bi-pred in B picture has stronger low pass characteristics. It reduces high frequency component and enables to keep low frequency component more accurately.

# Proposal

The scheme to switch MC interpolation filter can be applied to various interpolation methods. In this contribution, two ways to apply bi-pred / single pred switching are proposed. One way is to apply this to conventional fixed interpolation filter, and the other is to apply to adaptive interpolation filter.

## Bi/Single filter switching in FIF case

### Separable fixed interpolation filter (SFIF)

In AVC, 2 tap interpolation filter is used for MC interpolation filter at 1/2 pel position and 6 tap filter is used at 1/4 pel position. In our 6 tap separable interpolation filter is used for MC interpolation at all pixel positions. The definition of the sub pel position is the same as Figure 2. Figure 2 indicates the sub pel position for MC interpolation. The light blue squares are the reference pixels stored in coded picture buffer. E, F, G, H, I, J are integer pixels. h[sub pel][z] is the z-th filter coefficient at the sub pel position.

Figure 2: Sub pel position

The interpolation filter is defined as Error: Reference source not found and Error: Reference source not found. In case of AVC, to calculate quarter pel, rounding and clipping is done to obtain half pel (b-position). It reduces the accuracy of the prediction, because of the accumulation of error. In our proposal, both quarter pel and half pel value are derived directly by separable interpolation filter as specified in Error: Reference source not found and Error: Reference source not found.

Step 1:

Horizontal interpolation is applied to derive pixels a, b and c using Error: Reference source not found.

a=h[a][0]×E+h[a][1]×F+h[a][2]×G+h[a][3]×H+h[a][4]×I+h[a][5]×Jb=h[b][0]×E+h[b][1]×F+h[b][2]×G+h[b][2]×H+h[b][1]×I+h[b][0]×Jc=h[c][0]×E+h[c][1]×F+h[c][2]×G+h[c][3]×H+h[c][4]×I+h[c][5]×J

Equation 1: Horizontal interpolation filter

The pixels a1-a5, b1-b5, c1-c5 are derived in the same way as specified in Error: Reference source not found. The following filter coefficients are used. Here, we know that Bi/Single filter has 3 sets of filter coefficient as introduced in Error: Reference source not found. The Bi/Single filter switch those filter set whether bi-pred or single-pred and slice type is used.

For single-pred in P slice

- h[a][0] = h[c][5] = 3 /128

- h[a][1] = h[c][4] = -14 /128

- h[a][2] = h[c][3] = 111 /128

- h[a][3] = h[c][2] = 36, /128

- h[a][4] = h[c][1] = -9 /128

- h[a][5] = h[c][0] = 1 /128

- h[b][0] = h[b][5] = 3 /128

- h[b][1] = h[b][4] = -15 /128

- h[b][2] = h[b][3] = 76 /128

For single-pred in B slice

- h[a][0] = h[c][5] = 0 /128

- h[a][1] = h[c][4] = -5 /128

- h[a][2] = h[c][3] = 97 /128

- h[a][3] = h[c][2] = 47 /128

- h[a][4] = h[c][1] = -15 /128

- h[a][5] = h[c][0] = 3 /128

- h[b][0] = h[b][5] = 3 /128

- h[b][1] = h[b][4] = -15 /128

- h[b][2] = h[b][3] = 76 /128

For bi-pred

- h[a][0] = h[c][5] = 8 /128

- h[a][1] = h[c][4] = -28 /128

- h[a][2] = h[c][3] = 129 /128

- h[a][3] = h[c][2] = 26 /128

- h[a][4] = h[c][1] = -7 /128

- h[a][5] = h[c][0] = 0 /128

- h[b][0] = h[b][5] = 6 /128

- h[b][1] = h[b][4] = -23 /128

- h[b][2] = h[b][3] = 81 /128

In order to obtain e, f, g, i, j, k, m, n, o positions, the values of a1-a5, b1-b5, c1-c5 positions are necessary and those values are stored at memory.

Step 2:

Vertical interpolation is applied to derive pixels d-o using

d=h[d][0]×G1+h[d][1]×G2+h[d][2]×G+h[d][3]×G3+h[d][4]×G4+h[d][5]×G5h=h[h][0]×G1+h[h][1]×G2+h[h][2]×G+h[h][2]×G3+h[h][1]×G4+h[h][0]×G5l=h[d][5]×G1+h[d][4]×G2+h[d][3]×G+h[d][2]×G3+h[d][1]×G4+h[d][0]×G5

e=h[e][0]×a1+h[e][1]×a2+h[e][2]×a+h[e][3]×a3+h[e][4]×a4+h[e][5]×a5h=h[i][0]×a1+h[i][1]×a2+h[i][2]×a+h[i][2]×a3+h[i][1]×a4+h[i][0]×a5m=h[e][5]×a1+h[e][4]×a2+h[e][3]×a+h[e][2]×a3+h[e][1]×a4+h[e][0]×a5

f=h[f][0]×b1+h[f][1]×b2+h[f][2]×b+h[f][3]×b3+h[f][4]×b4+h[f][5]×b5j=h[j][0]×b1+h[j][1]×b2+h[i][2]×b+h[j][2]×b3+h[j][1]×b4+h[j][0]×b5n=h[f][5]×b1+h[f][4]×b2+h[f][3]×b+h[f][2]×b3+h[f][1]×b4+h[f][0]×b5

g=h[g][0]×c1+h[g][1]×c2+h[g][2]×c+h[g][3]×c3+h[g][4]×c4+h[g][5]×c5k=h[k][0]×c1+h[k][1]×c2+h[k][2]×c+h[k][2]×c3+h[k][1]×c4+h[k][0]×c5o=h[g][5]×c1+h[g][4]×c2+h[g][3]×c+h[g][2]×c3+h[g][1]×c4+h[g][0]×c5

Equation 2: Vertical interpolation filter

The following filter coefficients are used.

For single-pred in P slice

- h[d][0] = h[e][0] = h[f][0] = h[g][0] = h[l][5] = h[m][5] = h[n][5] = h[o][5] = 3 /128

- h[d][1] = h[e][1] = h[f][1] = h[g][1] = h[l][4] = h[m][4] = h[n][4] = h[o][4] = -14 /128

- h[d][2] = h[e][2] = h[f][2] = h[g][2] = h[l][3] = h[m][3] = h[n][3] = h[o][3] = 111 /128

- h[d][3] = h[e][3] = h[f][3] = h[g][3] = h[l][2] = h[m][2] = h[n][2] = h[o][2] = 36 /128

- h[d][4] = h[e][4] = h[f][4] = h[g][4] = h[l][1] = h[m][1] = h[n][1] = h[o][1] = -9 /128

- h[d][5] = h[e][5] = h[f][5] = h[g][5] = h[l][0] = h[m][0] = h[n][0] = h[o][0] = 1 /128

- h[h][0] = h[i][0] = h[j][0] = h[k][0] = h[h][5] = h[i][5] = h[j][5] = h[k][5] = 3 /128

- h[h][1] = h[i][1] = h[j][1] = h[k][1] = h[h][4] = h[i][4] = h[j][4] = h[k][4] = -15 /128

- h[h][2] = h[i][2] = h[j][2] = h[k][2] = h[h][3] = h[i][3] = h[j][3] = h[k][3] = 76 /128

For single-pred in B slice

- h[d][0] = h[e][0] = h[f][0] = h[g][0] = h[l][5] = h[m][5] = h[n][5] = h[o][5] = 0 /128

- h[d][1] = h[e][1] = h[f][1] = h[g][1] = h[l][4] = h[m][4] = h[n][4] = h[o][4] = -5 /128

- h[d][2] = h[e][2] = h[f][2] = h[g][2] = h[l][3] = h[m][3] = h[n][3] = h[o][3] = 97 /128

- h[d][3] = h[e][3] = h[f][3] = h[g][3] = h[l][2] = h[m][2] = h[n][2] = h[o][2] = 47 /128

- h[d][4] = h[e][4] = h[f][4] = h[g][4] = h[l][1] = h[m][1] = h[n][1] = h[o][1] = -15 /128

- h[d][5] = h[e][5] = h[f][5] = h[g][5] = h[l][0] = h[m][0] = h[n][0] = h[o][0] = 4 /128

- h[h][0] = h[i][0] = h[j][0] = h[k][0] = h[h][5] = h[i][5] = h[j][5] = h[k][5] = 1 /128

- h[h][1] = h[i][1] = h[j][1] = h[k][1] = h[h][4] = h[i][4] = h[j][4] = h[k][4] = -10 /128

- h[h][2] = h[i][2] = h[j][2] = h[k][2] = h[h][3] = h[i][3] = h[j][3] = h[k][3] = 73 /128

For Bi-pred

- h[d][0] = h[e][0] = h[f][0] = h[g][0] = h[l][5] = h[m][5] = h[n][5] = h[o][5] = 8 /128

- h[d][1] = h[e][1] = h[f][1] = h[g][1] = h[l][4] = h[m][4] = h[n][4] = h[o][4] = -28 /128

- h[d][2] = h[e][2] = h[f][2] = h[g][2] = h[l][3] = h[m][3] = h[n][3] = h[o][3] = 129 /128

- h[d][3] = h[e][3] = h[f][3] = h[g][3] = h[l][2] = h[m][2] = h[n][2] = h[o][2] = 26 /128

- h[d][4] = h[e][4] = h[f][4] = h[g][4] = h[l][1] = h[m][1] = h[n][1] = h[o][1] = -7 /128

- h[d][5] = h[e][5] = h[f][5] = h[g][5] = h[l][0] = h[m][0] = h[n][0] = h[o][0] = 0 /128

- h[h][0] = h[i][0] = h[j][0] = h[k][0] = h[h][5] = h[i][5] = h[j][5] = h[k][5] = 6 /128

- h[h][1] = h[i][1] = h[j][1] = h[k][1] = h[h][4] = h[i][4] = h[j][4] = h[k][4] = -23 /128

- h[h][2] = h[i][2] = h[j][2] = h[k][2] = h[h][3] = h[i][3] = h[j][3] = h[k][3] = 81 /128

Therefore, the number of filter coefficients in Error: Reference source not found and Error: Reference source not found is 18. The filter coefficient is fixed for entire sequence for SFIF.

## Bi/Single filter switching in AIF case

### Separable adaptive interpolation filter (SAIF) and coefficients reduction in B-slice

#### Separable adaptive interpolation filter with overhead reduction

6 tap separable adaptive Wiener filter is used for MC interpolation filter. This method is referred to as “SAIF”. SAIF filter coefficients are encoded in each slice. The different symmetry assumption is applied to P and B slice to reduce the number of interpolation filter coefficients (overhead).

#### Background

The size of generated bits in B-slices is normally lower than that in P or I slices, especially at low bit rate (large QP). The size of SAIF filter coefficients (overhead) becomes a large portion of the generated bits in B-slice. The examples of the amount of generated bits in B-slice are shown in Figure 3.

Figure 3: The size of generated bits in B slice

The vertical axis is the average generated bits in logarithmic scale and the horizontal axis is QP. When QP is large, the size of generated bits is lower than 1000 bits/picture in some cases, especially for small picture.

Without SAIF filter coefficients reduction, the size of SAIF filter coefficients (51 coefficients by encoding UVLC) is about 400-500 bits/picture. The portion of the SAIF filter coefficients (overhead) is larger and can not therefore be ignored. Due to this overhead, R&D optimization does not select often SAIF thus resulting in lower gain in B-slice. Reduction of SAIF filter coefficients is therefore the key to improve coding efficiency of SAIF in B-slice.

In order to reduce the size of SAIF filter coefficients, strong symmetry assumption is applied.

#### Method to reduce filter coefficients

The method to reduce filter coefficients in B-slice is explained in this sub-clause. The definition of MC interpolation filter is the same as SFIF specified by Error: Reference source not found and Error: Reference source not found. But in case of SAIF, the number of filter coefficients of those equations is 51. The strong symmetry assumption shown in Error: Reference source not found is introduced for B picture to reduce the number of filter coefficients. If single/bi-pred filter separation is used in B slice, two sets of filter coefficients are encoded. In this case, total number of filter coefficients for B slice is 36.

| Sub pel position | P slice | B slice |
| --- | --- | --- |
| a | 6 | 6 |
| b | 3 | 3 |
| c | 6 | same as reflection of a |
| d | 6 | 6 |
| e | 6 | same as reflection of d |
| f | 6 | same as reflection of d |
| g | 6 | same as reflection of d |
| h | 3 | 3 |
| i | 3 | same as h |
| j | 3 | same as h |
| k | 3 | same as h |
| l | same as reflection of d | same as reflection of d |
| m | same as reflection of e | same as reflection of d |
| n | same as reflection of f | same as reflection of d |
| o | same as reflection of g | same as reflection of d |
| Total of number of coefficients | 51 | 18 (36 if two sets of filters are used) |

Table 1: Number of filter coefficients

The example of bit reduction of over head is shown in Figure 4.

Figure 4: The number of bits of filter coefficients

# Complexity analysis

The complexity of Bi/Single filter switching for FIF and AIF is investigated in this sub-clause. The additional complexity to FIF and AIF is small.

## FIF case

The complexity of Bi/Single filter switching for separable fixed interpolation filter is investigated.

### Encoder complexity

During ME, filter coefficients at sub pel position need to be changed, In case of software implementation, encoder needs to calculate reference image for bi-pred at sub pel position, in addition to conventional FIF.

### Decoder complexity

Decoder needs to switch MC interpolation filter depending on prediction mode (whether bi-pred or not). three sets of FIF are predefined and stored at register,

## AIF case

The complexity of Bi/Single filter switching for separable adaptive interpolation filter is investigated.

### Encoder complexity

Complexity and memory analysis

The filter coefficients of adaptive interpolation filter are determined by frame level 2 pass encoding. In the first pass,

In the first pass, SFIF interpolation filter is used for ME and MC. Then MV, reference picture index, and single/bi-pred for each MB are obtained. Using these information, wiener filter coefficients are derived by LMSE method solving Wiener-Hopf equation. In the second pass, interpolation mode (fixed interpolation filter or adaptive interpolation filter) is determined by Lagrange optimization.

Memory

For P slice, and for half-pel position, each auto-correlation matrix is 3x3 , and cross-correlation vector is 3 and its corresponding filter is 3. For quarter-pel position, each auto-correlation matrix is 6x6 , and cross-correlation vector is 6 and its corresponding filter is 6. For quarter-pel position, there is 6 filters ( each a, c, d, e, f, and g-position ). For half-pel position, there is 5 filters (each b, h, i, j, and k-position).

- For half-pel, 5 x (3x3) matrix and vector for auto-correlation and cross- correlation.

- For quarter pel 6 x (6x6) matrix and vector for auto-correlation and cross- correlation

- 5 x 3 filter coefficients for half-pel, and 6 x 6 filter coefficients for quarter-pel.

For B slice, and for quarter-pel position, there is 2 filters (each a, and d-position). For half-pel position, there is 2 filters (each b, and h -position). (Please see table 1)

And theses filter is added to bi-pred interpolation in B slice. Thus,

- For half-pel, 2 x 2 x (3x3) matrix and vector for auto-correlation and cross- correlation.

- For quarter pel 2 x 2 x (6x6) matrix and vector for auto-correlation and cross- correlation

- 2 x 2 x 3 filter coefficients for half-pel, and 2 x 2 x 6 filter coefficients for quarter-pel.

Computation

- Filter Coefficients Derivation: Solve a system of 6 linear equations. Using Gaussian elimination approach the number of operations (additions + multiplications + divisions) can be estimated as: (2x6x6x6)/3 x11 (for 11 directions)

- Filter Application: 63 multiplication, 84 addition, 6 shift as division per pixel. Please see table 20.

- Cross-Correlation calculation: 6X6 multiplications + 6X6 additions per pixel

- Auto-Correlation calculation: 6X6 multiplications + 6X6 additions per pixel

#### Additional Memory from conventional SAIF

Complexity and memory analysis

In addition to the complexity of SAIF, the derivation of filter coefficients for bi-pred is necessary at encoder. To obtain filter coefficients, Wiener-Hopf equation (6x6) is solved. Correlation function, auto-correlation function are necessary.

Memory

Required memory to store is double of conventional AIF.

Computation

Required computation is double of conventional AIF.

### Decoder complexity

Decoder needs to switch MC interpolation filter depending on prediction mode (whether bi-pred or not). three sets of FIF are predefined and stored at register,

# Simulation result

## Test conditions

Test conditions in this contribution follow the test condition for TE2 (IBDI). Common test conditions to compare all coding tools should be defined in Geneva. JMKTA 2.6r1 is used.

Anchor is the same as TE2 and the following setting is used.

•	MVCompetition			= 1  # Enabled with default parameters

•	UseIntraMDDT			= 1  # Use MDDT for intra blocks

•	UseHPFilter			= 1  # Use High Precision H.264 filter

•	UseAdaptiveLoopFilter		= 1  # Use adaptive loop filtering

•	UseExtMB				= 2  # Use extended block size (64x64)

•	InputBitDepth			= 8  # InputBitDepth for IBDI

•	BitDepthLuma			= 12 # Bit Depth for Luminance

•	BitDepthChroma			= 12 # Bit Depth for Chrominance

In addition to this, the results are compared with AVC (all new tools off).

## FIF case

Proposed method is compared with SIFO and FDIF in KTA2.6r1. For FDIF or SIFO, UseHPFilter is set to 2, 3.

### Comparison with JMKAT with new tools ON

Figure 5 BD% for constraint 1 (R1~R4)

Figure 6 BD% for constraint 1 (R2~R5)

Figure 7 BD% for constraint 2 (R1~R4)

Figure 8 BD% for constraint 2 (R2~R5)

| Cs | QP range | Class | Sequence name | a302_BiSingle | a302_SIFO | a302_FDIF |
| --- | --- | --- | --- | --- | --- | --- |
| c1 | R1-R4 | A | PeopleOnStreet | 0.00 | 0.27 | 0.11 |
|  |  |  | Traffic | -0.15 | 0.01 | 0.09 |
|  |  | A Avg. |  | -0.08 | 0.14 | 0.10 |
|  |  | B | BasketballDrive | -0.32 | -0.31 | -0.58 |
|  |  |  | BQTerrace | -0.43 | 0.13 | 1.94 |
|  |  |  | Cactus | -0.72 | -0.27 | 0.41 |
|  |  |  | Kimono1 | -0.16 | 0.06 | -0.06 |
|  |  |  | ParkScene | -0.24 | 0.20 | 0.32 |
|  |  | B Avg. |  | -0.37 | -0.04 | 0.41 |
|  |  | C | BasketballDrill | -0.51 | 0.21 | -0.16 |
|  |  |  | BQMall | -0.44 | 0.36 | -0.30 |
|  |  |  | PartyScene | -0.44 | 0.23 | 1.10 |
|  |  |  | RaceHorses | 0.01 | 0.18 | 0.45 |
|  |  | C Avg. |  | -0.34 | 0.25 | 0.27 |
|  |  | D | BasketballPass | -0.56 | 0.87 | 0.19 |
|  |  |  | BlowingBubbles | -0.77 | 1.03 | 2.11 |
|  |  |  | BQSquare | -10.54 | 0.83 | 5.11 |
|  |  |  | RaceHorses | -0.27 | 0.47 | 1.29 |
|  |  | D Avg. |  | -3.04 | 0.80 | 2.18 |
|  | R1-R4 Avg. |  |  | -1.04 | 0.29 | 0.80 |
|  | R2-R5 | A | PeopleOnStreet | -0.10 | 0.14 | 0.08 |
|  |  |  | Traffic | -0.06 | 0.01 | 0.03 |
|  |  | A Avg. |  | -0.08 | 0.08 | 0.06 |
|  |  | B | BasketballDrive | -0.25 | -0.39 | -0.72 |
|  |  |  | BQTerrace | 0.00 | 0.16 | 1.54 |
|  |  |  | Cactus | -0.95 | -0.45 | 0.54 |
|  |  |  | Kimono1 | -0.05 | -0.04 | 0.08 |
|  |  |  | ParkScene | -0.41 | 0.03 | 0.70 |
|  |  | B Avg. |  | -0.33 | -0.14 | 0.43 |
|  |  | C | BasketballDrill | -0.48 | 0.07 | -0.10 |
|  |  |  | BQMall | -0.70 | 0.09 | -0.33 |
|  |  |  | PartyScene | -0.95 | 0.00 | 1.63 |
|  |  |  | RaceHorses | -0.05 | -0.05 | 0.77 |
|  |  | C Avg. |  | -0.54 | 0.03 | 0.49 |
|  |  | D | BasketballPass | -0.94 | 0.12 | 0.12 |
|  |  |  | BlowingBubbles | -1.30 | 0.42 | 2.62 |
|  |  |  | BQSquare | -14.65 | 0.55 | 3.06 |
|  |  |  | RaceHorses | -0.30 | 0.07 | 2.01 |
|  |  | D Avg. |  | -4.30 | 0.29 | 1.95 |
|  | R2-R5 Avg. |  |  | -1.41 | 0.05 | 0.80 |
| c1 Avg. |  |  |  | -1.22 | 0.17 | 0.80 |
| c2 | R1-R4 | A | PeopleOnStreet | -0.19 |  |  |
|  |  |  | Traffic | -0.87 |  |  |
|  |  | A Avg. |  | -0.53 |  |  |
|  |  | B | BasketballDrive | 0.26 | -0.96 | -1.23 |
|  |  |  | BQTerrace | -1.79 | -0.30 | 3.05 |
|  |  |  | Cactus | -0.19 | -0.12 | 0.25 |
|  |  |  | Kimono1 | 0.12 | -0.18 | -0.43 |
|  |  |  | ParkScene | -0.06 | -0.45 | 0.40 |
|  |  | B Avg. |  | -0.33 | -0.40 | 0.41 |
|  |  | C | BasketballDrill | -0.35 | -0.75 | -1.29 |
|  |  |  | BQMall | 0.05 | -0.37 | -0.82 |
|  |  |  | PartyScene | 1.31 | -0.84 | 2.28 |
|  |  |  | RaceHorses | -0.09 | -0.82 | 0.02 |
|  |  | C Avg. |  | 0.23 | -0.69 | 0.05 |
|  |  | D | BasketballPass | 0.19 | -0.10 | -1.21 |
|  |  |  | BlowingBubbles | 0.42 | 0.84 | 1.90 |
|  |  |  | BQSquare | -3.88 | 0.99 | 10.92 |
|  |  |  | RaceHorses | 0.25 | -0.10 | 1.25 |
|  |  | D Avg. |  | -0.75 | 0.41 | 3.22 |
|  | R1-R4 Avg. |  |  | -0.32 | -0.24 | 1.16 |
|  | R2-R5 | A | PeopleOnStreet | -0.17 |  |  |
|  |  |  | Traffic | -1.01 |  |  |
|  |  | A Avg. |  | -0.59 |  |  |
|  |  | B | BasketballDrive | 0.33 | -0.86 | -1.20 |
|  |  |  | BQTerrace | -1.74 | -0.26 | 2.89 |
|  |  |  | Cactus | -0.37 | -0.18 | 0.23 |
|  |  |  | Kimono1 | 0.17 | -0.20 | -0.35 |
|  |  |  | ParkScene | -0.29 | -0.27 | 0.85 |
|  |  | B Avg. |  | -0.38 | -0.36 | 0.48 |
|  |  | C | BasketballDrill | -0.44 | -1.00 | -1.56 |
|  |  |  | BQMall | -0.29 | -0.49 | -0.91 |
|  |  |  | PartyScene | 0.99 | -0.76 | 2.83 |
|  |  |  | RaceHorses | -0.15 | -1.00 | 0.26 |
|  |  | C Avg. |  | 0.03 | -0.81 | 0.16 |
|  |  | D | BasketballPass | 0.38 | -0.07 | -0.78 |
|  |  |  | BlowingBubbles | -0.01 | 0.71 | 2.28 |
|  |  |  | BQSquare | -4.96 | 0.53 | 7.10 |
|  |  |  | RaceHorses | 0.29 | -0.28 | 1.60 |
|  |  | D Avg. |  | -1.07 | 0.22 | 2.55 |
|  | R2-R5 Avg. |  |  | -0.48 | -0.32 | 1.02 |
| c2 Avg. |  |  |  | -0.40 | -0.28 | 1.09 |

Table 2 BD% in FIF case, all KTA tools are ON

### Comparison with AVC ( All new tools OFF )

Figure 9 BD% for constraint 1 (R1~R4)

Figure 10 BD% for constraint 1 (R2~R5)

Figure 11 BD% for constraint 2 (R1~R4)

Figure 12 BD% for constraint 2 (R2~R5)

| Cs | QP range | Class | Sequence name | BiSingle | HPF | SIFO |
| --- | --- | --- | --- | --- | --- | --- |
| c1 | R1-R4 | A | PeopleOnStreet | -0.36 | -0.11 | -0.17 |
|  |  |  | Traffic | -2.70 | -2.61 | -2.76 |
|  |  | A Avg. |  | -1.53 | -1.36 | -1.47 |
|  |  | B | BasketballDrive | -1.22 | -0.86 | -2.69 |
|  |  |  | BQTerrace | -10.00 | -8.50 | -12.11 |
|  |  |  | Cactus | -1.48 | -0.95 | -1.11 |
|  |  |  | Kimono1 | -2.41 | -2.42 | -3.01 |
|  |  |  | ParkScene | -2.10 | -1.97 | -1.95 |
|  |  | B Avg. |  | -3.44 | -2.94 | -4.17 |
|  |  | C | BasketballDrill | -0.80 | -0.19 | -1.44 |
|  |  |  | BQMall | -1.64 | -0.73 | -1.52 |
|  |  |  | PartyScene | -2.76 | -0.62 | -0.82 |
|  |  |  | RaceHorses | -0.50 | -0.27 | -1.57 |
|  |  | C Avg. |  | -1.42 | -0.45 | -1.34 |
|  |  | D | BasketballPass | -1.62 | -0.81 | -1.15 |
|  |  |  | BlowingBubbles | -2.98 | -0.79 | -1.14 |
|  |  |  | BQSquare | -20.54 | -1.28 | -0.94 |
|  |  |  | RaceHorses | -0.72 | -0.50 | -1.45 |
|  |  | D Avg. |  | -6.46 | -0.84 | -1.17 |
|  | R1-R4 Avg. |  |  | -3.46 | -1.51 | -2.26 |
|  | R2-R5 | A | PeopleOnStreet | -0.55 | -0.26 | -0.47 |
|  |  |  | Traffic | -3.38 | -3.56 | -3.84 |
|  |  | A Avg. |  | -1.97 | -1.91 | -2.15 |
|  |  | B | BasketballDrive | -1.61 | -1.13 | -3.43 |
|  |  |  | BQTerrace | -10.99 | -9.57 | -12.50 |
|  |  |  | Cactus | -2.14 | -1.43 | -1.92 |
|  |  |  | Kimono1 | -3.35 | -3.38 | -4.50 |
|  |  |  | ParkScene | -2.81 | -2.60 | -2.65 |
|  |  | B Avg. |  | -4.18 | -3.62 | -5.00 |
|  |  | C | BasketballDrill | -1.43 | -0.38 | -1.94 |
|  |  |  | BQMall | -2.32 | -0.96 | -2.04 |
|  |  |  | PartyScene | -4.58 | -0.55 | -0.86 |
|  |  |  | RaceHorses | -0.81 | -0.40 | -1.71 |
|  |  | C Avg. |  | -2.29 | -0.57 | -1.64 |
|  |  | D | BasketballPass | -2.30 | -0.98 | -1.69 |
|  |  |  | BlowingBubbles | -5.26 | -1.03 | -1.65 |
|  |  |  | BQSquare | -23.60 | -1.72 | -1.77 |
|  |  |  | RaceHorses | -1.01 | -0.61 | -1.51 |
|  |  | D Avg. |  | -8.04 | -1.09 | -1.66 |
|  | R2-R5 Avg. |  |  | -4.41 | -1.90 | -2.83 |
| c1 Avg. |  |  |  | -3.93 | -1.71 | -2.54 |
| c2 | R1-R4 | B | BasketballDrive | -1.18 | -1.59 | -4.33 |
|  |  |  | BQTerrace | -9.64 | -8.94 | -14.87 |
|  |  |  | Cactus | -0.47 | -0.74 | -2.41 |
|  |  |  | Kimono1 | -3.92 | -4.33 | -6.55 |
|  |  |  | ParkScene | -5.72 | -6.22 | -6.86 |
|  |  | B Avg. |  | -4.19 | -4.37 | -7.00 |
|  |  | C | BasketballDrill | -0.45 | -0.95 | -2.18 |
|  |  |  | BQMall | -1.05 | -1.23 | -2.84 |
|  |  |  | PartyScene | -2.10 | -2.12 | -3.19 |
|  |  |  | RaceHorses | -0.54 | -0.98 | -2.51 |
|  |  | C Avg. |  | -1.04 | -1.32 | -2.68 |
|  |  | D | BasketballPass | -0.14 | -0.89 | -2.82 |
|  |  |  | BlowingBubbles | -2.20 | -1.92 | -2.82 |
|  |  |  | BQSquare | -7.53 | -2.64 | -2.28 |
|  |  |  | RaceHorses | 0.00 | -0.83 | -2.22 |
|  |  | D Avg. |  | -2.47 | -1.57 | -2.53 |
|  |  | E | vidyo1 | -3.78 | -4.17 | -10.43 |
|  |  |  | vidyo3 | -5.59 | -4.13 | -4.52 |
|  |  |  | vidyo4 | 0.93 | 0.13 | -2.86 |
|  |  | E Avg. |  | -2.81 | -2.72 | -5.94 |
|  | R1-R4 Avg. |  |  | -2.71 | -2.60 | -4.61 |
|  | R2-R5 | B | BasketballDrive | -1.27 | -1.65 | -5.15 |
|  |  |  | BQTerrace | -8.22 | -8.18 | -13.55 |
|  |  |  | Cactus | -0.72 | -0.79 | -2.93 |
|  |  |  | Kimono1 | -3.67 | -4.22 | -7.25 |
|  |  |  | ParkScene | -4.60 | -5.10 | -5.79 |
|  |  | B Avg. |  | -3.70 | -3.99 | -6.93 |
|  |  | C | BasketballDrill | -0.45 | -0.92 | -2.57 |
|  |  |  | BQMall | -1.21 | -1.19 | -3.30 |
|  |  |  | PartyScene | -1.99 | -1.70 | -2.66 |
|  |  |  | RaceHorses | -0.64 | -0.88 | -2.54 |
|  |  | C Avg. |  | -1.07 | -1.17 | -2.77 |
|  |  | D | BasketballPass | -0.39 | -1.00 | -3.21 |
|  |  |  | BlowingBubbles | -2.36 | -1.67 | -2.40 |
|  |  |  | BQSquare | -8.53 | -2.66 | -2.27 |
|  |  |  | RaceHorses | 0.12 | -0.77 | -1.93 |
|  |  | D Avg. |  | -2.79 | -1.52 | -2.45 |
|  |  | E | vidyo1 | -4.00 | -4.82 | -12.95 |
|  |  |  | vidyo3 | -5.39 | -3.61 | -4.93 |
|  |  |  | vidyo4 | 0.91 | 0.00 | -4.29 |
|  |  | E Avg. |  | -2.83 | -2.81 | -7.39 |
|  | R2-R5 Avg. |  |  | -2.65 | -2.45 | -4.86 |
| c2 Avg. |  |  |  | -2.68 | -2.52 | -4.73 |

Table 3 BD% in FIF case, all KTA tools are OFF

## AIF case

In addition to the test condition in TE2 (A302), the proposal is compared with JMKTA with new tools except AIF.

Note:

  - “AIF_BiSingle”: Separable FIF (No Bi/Single) + Separable AIF (used BiSingle)

  - “AIF_SAIF”: HPF + Separable AIF ( UseAdaptiveFilter = 2 )

  - “AIF_EAIF”: HPF + Enhanced AIF ( UseAdaptiveFilter = 5 )

### Comparison with JMKTA (All KTA new tools are OFF except AIF)

Figure 13 BD% for constraint 1 (R1~R4)

Figure 14 BD% for constraint 1 (R2~R5)

Figure 15 BD% for constraint 2 (R1~R4)

Figure 16 BD% for constraint 2 (R2~R5)

| Cs | QP range | Class | Sequence name | AIF_BiSingle | AIF_EAIF | AIF_SAIF | 総計 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| c1 | R1-R4 | A | PeopleOnStreet | -2.02 | -1.92 | -1.50 | -1.81 |
|  |  |  | Traffic | -4.89 | -7.78 | -4.25 | -5.64 |
|  |  | A Avg. |  | -3.45 | -4.85 | -2.88 | -3.73 |
|  |  | B | BasketballDrive | -3.57 | -2.48 | -3.15 | -3.07 |
|  |  |  | BQTerrace | -16.04 | -16.71 | -14.76 | -15.84 |
|  |  |  | Cactus | -2.42 | -3.03 | -1.84 | -2.43 |
|  |  |  | Kimono1 | -4.56 | -3.26 | -3.46 | -3.76 |
|  |  |  | ParkScene | -3.08 | -2.40 | -2.30 | -2.59 |
|  |  | B Avg. |  | -5.93 | -5.58 | -5.10 | -5.54 |
|  |  | C | BasketballDrill | -2.15 | -3.67 | -2.00 | -2.60 |
|  |  |  | BQMall | -2.92 | -3.04 | -2.48 | -2.81 |
|  |  |  | PartyScene | -4.36 | -2.77 | -3.18 | -3.43 |
|  |  |  | RaceHorses | -0.91 | -0.99 | -1.11 | -1.01 |
|  |  | C Avg. |  | -2.58 | -2.62 | -2.19 | -2.46 |
|  |  | D | BasketballPass | -1.51 | -2.17 | -1.82 | -1.83 |
|  |  |  | BlowingBubbles | -2.36 | -1.70 | -1.89 | -1.98 |
|  |  |  | BQSquare | -16.25 | -7.83 | -10.36 | -11.48 |
|  |  |  | RaceHorses | 0.08 | -0.82 | -0.75 | -0.50 |
|  |  | D Avg. |  | -5.01 | -3.13 | -3.70 | -3.95 |
|  | R1-R4 Avg. |  |  | -4.46 | -4.04 | -3.66 | -4.05 |
|  | R2-R5 | A | PeopleOnStreet | -2.40 | -2.15 | -1.66 | -2.07 |
|  |  |  | Traffic | -5.74 | -7.59 | -4.98 | -6.10 |
|  |  | A Avg. |  | -4.07 | -4.87 | -3.32 | -4.09 |
|  |  | B | BasketballDrive | -4.26 | -2.89 | -3.75 | -3.63 |
|  |  |  | BQTerrace | -16.24 | -16.08 | -14.69 | -15.67 |
|  |  |  | Cactus | -3.37 | -3.63 | -2.34 | -3.11 |
|  |  |  | Kimono1 | -6.18 | -4.18 | -4.72 | -5.03 |
|  |  |  | ParkScene | -3.49 | -2.92 | -2.69 | -3.03 |
|  |  | B Avg. |  | -6.71 | -5.94 | -5.64 | -6.09 |
|  |  | C | BasketballDrill | -2.98 | -4.14 | -2.76 | -3.29 |
|  |  |  | BQMall | -3.63 | -3.52 | -3.02 | -3.39 |
|  |  |  | PartyScene | -5.97 | -3.54 | -4.20 | -4.57 |
|  |  |  | RaceHorses | -1.25 | -1.16 | -1.25 | -1.22 |
|  |  | C Avg. |  | -3.46 | -3.09 | -2.81 | -3.12 |
|  |  | D | BasketballPass | -2.50 | -2.54 | -2.61 | -2.55 |
|  |  |  | BlowingBubbles | -4.39 | -2.28 | -2.58 | -3.08 |
|  |  |  | BQSquare | -18.62 | -8.94 | -11.89 | -13.15 |
|  |  |  | RaceHorses | -0.47 | -0.99 | -0.90 | -0.78 |
|  |  | D Avg. |  | -6.49 | -3.69 | -4.49 | -4.89 |
|  | R2-R5 Avg. |  |  | -5.43 | -4.44 | -4.27 | -4.71 |
| c1 Avg. |  |  |  | -4.95 | -4.24 | -3.96 | -4.38 |
| c2 | R1-R4 | B | BasketballDrive | -7.33 | -5.49 | -6.85 | -6.56 |
|  |  |  | BQTerrace | -19.59 | -20.97 | -19.07 | -19.88 |
|  |  |  | Cactus | -3.89 | -2.86 | -3.29 | -3.35 |
|  |  |  | Kimono1 | -9.57 | -8.44 | -8.93 | -8.98 |
|  |  |  | ParkScene | -7.52 | -7.50 | -7.13 | -7.38 |
|  |  | B Avg. |  | -9.58 | -9.05 | -9.05 | -9.23 |
|  |  | C | BasketballDrill | -5.27 | -6.25 | -4.68 | -5.40 |
|  |  |  | BQMall | -5.45 | -6.32 | -4.94 | -5.57 |
|  |  |  | PartyScene | -6.90 | -6.44 | -6.01 | -6.45 |
|  |  |  | RaceHorses | -2.38 | -2.14 | -2.14 | -2.22 |
|  |  | C Avg. |  | -5.00 | -5.29 | -4.44 | -4.91 |
|  |  | D | BasketballPass | -4.22 | -3.34 | -2.84 | -3.46 |
|  |  |  | BlowingBubbles | -4.16 | -4.34 | -4.37 | -4.29 |
|  |  |  | BQSquare | -12.68 | -9.26 | -13.05 | -11.67 |
|  |  |  | RaceHorses | -0.57 | -1.09 | -1.13 | -0.93 |
|  |  | D Avg. |  | -5.41 | -4.51 | -5.35 | -5.09 |
|  |  | E | vidyo1 | -9.18 | -13.23 | -8.09 | -10.16 |
|  |  |  | vidyo3 | -9.68 | -11.93 | -9.14 | -10.25 |
|  |  |  | vidyo4 | -8.72 | -8.16 | -7.89 | -8.26 |
|  |  | E Avg. |  | -9.19 | -11.11 | -8.37 | -9.56 |
|  | R1-R4 Avg. |  |  | -7.32 | -7.36 | -6.85 | -7.18 |
|  | R2-R5 | B | BasketballDrive | -8.35 | -5.54 | -7.92 | -7.27 |
|  |  |  | BQTerrace | -17.56 | -18.86 | -17.24 | -17.88 |
|  |  |  | Cactus | -4.44 | -2.69 | -3.91 | -3.68 |
|  |  |  | Kimono1 | -10.35 | -8.75 | -9.63 | -9.58 |
|  |  |  | ParkScene | -6.04 | -5.98 | -5.82 | -5.95 |
|  |  | B Avg. |  | -9.35 | -8.36 | -8.90 | -8.87 |
|  |  | C | BasketballDrill | -6.28 | -7.00 | -5.45 | -6.24 |
|  |  |  | BQMall | -6.00 | -6.11 | -5.61 | -5.91 |
|  |  |  | PartyScene | -6.48 | -5.91 | -5.87 | -6.09 |
|  |  |  | RaceHorses | -2.80 | -2.10 | -2.50 | -2.47 |
|  |  | C Avg. |  | -5.39 | -5.28 | -4.86 | -5.18 |
|  |  | D | BasketballPass | -4.74 | -3.85 | -3.77 | -4.12 |
|  |  |  | BlowingBubbles | -4.33 | -4.28 | -4.38 | -4.33 |
|  |  |  | BQSquare | -13.93 | -9.73 | -13.77 | -12.48 |
|  |  |  | RaceHorses | -0.66 | -0.82 | -1.07 | -0.85 |
|  |  | D Avg. |  | -5.91 | -4.67 | -5.75 | -5.44 |
|  |  | E | vidyo1 | -14.28 | -14.33 | -12.91 | -13.84 |
|  |  |  | vidyo3 | -10.71 | -11.79 | -10.30 | -10.93 |
|  |  |  | vidyo4 | -11.49 | -9.39 | -10.70 | -10.52 |
|  |  | E Avg. |  | -12.16 | -11.84 | -11.30 | -11.77 |
|  | R2-R5 Avg. |  |  | -8.03 | -7.32 | -7.55 | -7.63 |
| c2 Avg. |  |  |  | -7.67 | -7.34 | -7.20 | -7.40 |
| Overall |  |  |  | -6.35 | -5.84 | -5.63 | -5.94 |

Table 4 BD% in AIF case, all KTA tools are OFF

Figure 17 BD% for constraint 1 (R1~R4)

Figure 18 BD% for constraint 1 (R2~R5)

Figure 19 BD% for constraint 2 (R1~R4)

Figure 20 BD% for constraint 2 (R2~R5)

| Cs | QP range | Class | Seq | a302_AIF_BiSingle |
| --- | --- | --- | --- | --- |
| c1 | R1-R4 | A | PeopleOnStreet | -0.40 |
|  |  |  | Traffic | -0.01 |
|  |  | A Avg. |  | -0.21 |
|  |  | B | BasketballDrive | -1.11 |
|  |  |  | BQTerrace | -1.00 |
|  |  |  | Cactus | -0.27 |
|  |  |  | Kimono1 | -0.20 |
|  |  |  | ParkScene | 0.04 |
|  |  | B Avg. |  | -0.51 |
|  |  | C | BasketballDrill | -0.54 |
|  |  |  | BQMall | -0.82 |
|  |  |  | PartyScene | 0.11 |
|  |  |  | RaceHorses | -0.22 |
|  |  | C Avg. |  | -0.36 |
|  |  | D | BasketballPass | 0.14 |
|  |  |  | BlowingBubbles | 0.77 |
|  |  |  | BQSquare | -4.07 |
|  |  |  | RaceHorses | 0.67 |
|  |  | D Avg. |  | -0.62 |
|  | R1-R4 Avg. |  |  | -0.46 |
|  | R2-R5 | A | PeopleOnStreet | -0.50 |
|  |  |  | Traffic | -0.03 |
|  |  | A Avg. |  | -0.26 |
|  |  | B | BasketballDrive | -1.09 |
|  |  |  | BQTerrace | -1.15 |
|  |  |  | Cactus | -0.26 |
|  |  |  | Kimono1 | -0.11 |
|  |  |  | ParkScene | 0.04 |
|  |  | B Avg. |  | -0.52 |
|  |  | C | BasketballDrill | -0.58 |
|  |  |  | BQMall | -1.14 |
|  |  |  | PartyScene | -0.16 |
|  |  |  | RaceHorses | -0.45 |
|  |  | C Avg. |  | -0.58 |
|  |  | D | BasketballPass | -0.20 |
|  |  |  | BlowingBubbles | 0.20 |
|  |  |  | BQSquare | -6.63 |
|  |  |  | RaceHorses | 0.37 |
|  |  | D Avg. |  | -1.56 |
|  | R2-R5 Avg. |  |  | -0.78 |
| c1 Avg. |  |  |  | -0.62 |
| c2 | R1-R4 | B | BasketballDrive | -2.08 |
|  |  |  | BQTerrace | -4.91 |
|  |  |  | Cactus | -0.49 |
|  |  |  | Kimono1 | -0.87 |
|  |  |  | ParkScene | -0.46 |
|  |  | B Avg. |  | -1.76 |
|  |  | C | BasketballDrill | -1.56 |
|  |  |  | BQMall | -1.13 |
|  |  |  | PartyScene | -0.04 |
|  |  |  | RaceHorses | -0.82 |
|  |  | C Avg. |  | -0.89 |
|  |  | D | BasketballPass | -0.91 |
|  |  |  | BlowingBubbles | 0.82 |
|  |  |  | BQSquare | -10.05 |
|  |  |  | RaceHorses | 0.85 |
|  |  | D Avg. |  | -2.32 |
|  |  | E | vidyo1 | -0.28 |
|  |  |  | vidyo3 | 0.05 |
|  |  |  | vidyo4 | -0.27 |
|  |  | E Avg. |  | -0.17 |
|  | R1-R4 Avg. |  |  | -1.39 |
|  | R2-R5 | B | BasketballDrive | -2.09 |
|  |  |  | BQTerrace | -4.73 |
|  |  |  | Cactus | -0.60 |
|  |  |  | Kimono1 | -0.93 |
|  |  |  | ParkScene | -0.47 |
|  |  | B Avg. |  | -1.77 |
|  |  | C | BasketballDrill | -1.97 |
|  |  |  | BQMall | -1.86 |
|  |  |  | PartyScene | -0.57 |
|  |  |  | RaceHorses | -1.34 |
|  |  | C Avg. |  | -1.43 |
|  |  | D | BasketballPass | -0.47 |
|  |  |  | BlowingBubbles | -0.28 |
|  |  |  | BQSquare | -11.68 |
|  |  |  | RaceHorses | 0.40 |
|  |  | D Avg. |  | -3.01 |
|  |  | E | vidyo1 | -0.04 |
|  |  |  | vidyo3 | -0.21 |
|  |  |  | vidyo4 | -1.01 |
|  |  | E Avg. |  | -0.42 |
|  | R2-R5 Avg. |  |  | -1.74 |
| c2 Avg. |  |  |  | -1.56 |

Table 5 BD% in AIF case, Anchor : A302

## Simulation time

To estimate complexity, simulation time is measured. The additional simulation time for Bi/Single filter switching is ignorable.

### FIF case (TE2 test condition)

Encoding time is average of CPU time for 5 QPs.

#### Encoder simulation time

Figure 21 Encoder simulation time for Cs1

Figure 22 Encoder simulation time for Cs2

| Cs | Class | Seq | a302 | a302_BiSingle | a302_FDIF | a302_SIFO |
| --- | --- | --- | --- | --- | --- | --- |
| c1 | A | PeopleOnStreet | 20425 | 20534 | 25035 | 55073 |
|  |  | Traffic | 18087 | 18482 | 22149 | 32913 |
|  | A Avg. |  | 19256 | 19508 | 23592 | 43993 |
|  | B | BasketballDrive | 16706 | 17474 | 19094 | 114744 |
|  |  | BQTerrace | 15897 | 16241 | 19545 | 58915 |
|  |  | Cactus | 15848 | 16753 | 20301 | 53800 |
|  |  | Kimono1 | 16135 | 16740 | 22240 | 62259 |
|  |  | ParkScene | 16051 | 16517 | 18867 | 54394 |
|  | B Avg. |  | 16127 | 16745 | 20010 | 68822 |
|  | C | BasketballDrill | 3737 | 3798 | 3732 | 20397 |
|  |  | BQMall | 4128 | 4021 | 3974 | 19899 |
|  |  | PartyScene | 3995 | 3863 | 4000 | 16728 |
|  |  | RaceHorses | 4656 | 4757 | 4670 | 47140 |
|  | C Avg. |  | 4129 | 4110 | 4094 | 26041 |
|  | D | BasketballPass | 1062 | 1093 | 1067 | 3777 |
|  |  | BlowingBubbles | 1181 | 1145 | 1169 | 6992 |
|  |  | BQSquare | 1106 | 1159 | 1180 | 1849 |
|  |  | RaceHorses | 1388 | 1479 | 1478 | 12931 |
|  | D Avg. |  | 1184 | 1219 | 1223 | 6387 |
| c1 Avg. |  |  | 9360 | 9604 | 11233 | 37454 |
| c2 | B | BasketballDrive | 14653 | 14617 | 18227 | 23130 |
|  |  | BQTerrace | 15715 | 15614 | 17449 | 22541 |
|  |  | Cactus | 14301 | 14547 | 18200 | 20887 |
|  |  | Kimono1 | 14633 | 14850 | 18688 | 20850 |
|  |  | ParkScene | 15745 | 16166 | 20334 | 22699 |
|  | B Avg. |  | 15010 | 15159 | 18580 | 22021 |
|  | C | BasketballDrill | 3414 | 3507 | 3469 | 5438 |
|  |  | BQMall | 3635 | 3541 | 3761 | 5598 |
|  |  | PartyScene | 4098 | 4043 | 3965 | 6143 |
|  |  | RaceHorses | 4449 | 4601 | 4403 | 7941 |
|  | C Avg. |  | 3899 | 3923 | 3899 | 6280 |
|  | D | BasketballPass | 1026 | 1033 | 1004 | 1458 |
|  |  | BlowingBubbles | 1185 | 1154 | 1182 | 1821 |
|  |  | BQSquare | 1265 | 1297 | 1287 | 1743 |
|  |  | RaceHorses | 1330 | 1438 | 1410 | 2410 |
|  | D Avg. |  | 1202 | 1230 | 1221 | 1858 |
|  | E | vidyo1 | 6580 | 6550 | 6306 | 9343 |
|  |  | vidyo3 | 6486 | 6802 | 6505 | 9389 |
|  |  | vidyo4 | 6460 | 6850 | 6713 | 10015 |
|  | E Avg. |  | 6509 | 6734 | 6508 | 9582 |
| c2 Avg. |  |  | 7186 | 7288 | 8306 | 10713 |

Table 6 Encoder simulation time

#### Decoder simulation time

Figure 23 Decoder simulation time for Cs1

Figure 24 Decoder simulation time for Cs2

| Cs | Class | Sequence name | a302 | a302_BiSingle | a302_FDIF | a302_SIFO |
| --- | --- | --- | --- | --- | --- | --- |
| c1 | A | PeopleOnStreet | 83 | 73 | 77 | 123 |
|  |  | Traffic | 72 | 74 | 80 | 147 |
|  | A Avg. |  | 78 | 74 | 78 | 135 |
|  | B | BasketballDrive | 74 | 73 | 72 | 152 |
|  |  | BQTerrace | 74 | 71 | 76 | 159 |
|  |  | Cactus | 57 | 63 | 75 | 95 |
|  |  | Kimono1 | 79 | 75 | 114 | 152 |
|  |  | ParkScene | 77 | 74 | 78 | 162 |
|  | B Avg. |  | 72 | 71 | 83 | 144 |
|  | C | BasketballDrill | 12 | 13 | 11 | 22 |
|  |  | BQMall | 14 | 13 | 11 | 20 |
|  |  | PartyScene | 15 | 14 | 13 | 27 |
|  |  | RaceHorses | 16 | 15 | 15 | 27 |
|  | C Avg. |  | 14 | 14 | 12 | 24 |
|  | D | BasketballPass | 3 | 4 | 3 | 6 |
|  |  | BlowingBubbles | 4 | 4 | 4 | 8 |
|  |  | BQSquare | 4 | 5 | 4 | 7 |
|  |  | RaceHorses | 4 | 4 | 4 | 8 |
|  | D Avg. |  | 4 | 4 | 4 | 7 |
| c1 Avg. |  |  | 39 | 38 | 42 | 74 |
| c2 | B | BasketballDrive | 71 | 68 | 69 | 120 |
|  |  | BQTerrace | 69 | 67 | 62 | 130 |
|  |  | Cactus | 52 | 55 | 58 | 81 |
|  |  | Kimono1 | 95 | 75 | 78 | 124 |
|  |  | ParkScene | 72 | 68 | 75 | 129 |
|  | B Avg. |  | 72 | 67 | 68 | 117 |
|  | C | BasketballDrill | 11 | 12 | 11 | 20 |
|  |  | BQMall | 11 | 11 | 10 | 17 |
|  |  | PartyScene | 15 | 12 | 11 | 27 |
|  |  | RaceHorses | 17 | 18 | 14 | 26 |
|  | C Avg. |  | 13 | 13 | 12 | 23 |
|  | D | BasketballPass | 4 | 3 | 3 | 6 |
|  |  | BlowingBubbles | 4 | 3 | 3 | 6 |
|  |  | BQSquare | 4 | 4 | 3 | 8 |
|  |  | RaceHorses | 4 | 5 | 5 | 8 |
|  | D Avg. |  | 4 | 4 | 4 | 7 |
|  | E | vidyo1 | 24 | 24 | 19 | 31 |
|  |  | vidyo3 | 21 | 25 | 19 | 32 |
|  |  | vidyo4 | 22 | 24 | 20 | 37 |
|  | E Avg. |  | 22 | 24 | 19 | 33 |
| c2 Avg. |  |  | 31 | 30 | 29 | 50 |

Table 7 Decoder simulation time

### FIF case (All KTA tool off)

#### Encoder simulation time

Figure 25 Encoder simulation time for Cs1

Figure 26 Encoder simulation time for Cs2

#### Decode simulation time

Figure 27 Decoder simulation time for Cs1

Figure 28 Decoder simulation time for Cs2

### AIF case (All KTA tool off)

#### Encoder simulation time

Figure 29 Encoder simulation time for Cs1

Figure 30 Encoder simulation time for Cs2

| Cs | Class | Seq | AIF_SAIF | AIF_BiSingle | AIF_EAIF |
| --- | --- | --- | --- | --- | --- |
| c1 | A | PeopleOnStreet | 12073 | 13108 | 16335 |
|  |  | Traffic | 12766 | 13249 | 16848 |
|  | A Avg. |  | 12420 | 13178 | 16591 |
|  | B | BasketballDrive | 11361 | 11520 | 13107 |
|  |  | BQTerrace | 11751 | 11958 | 14188 |
|  |  | Cactus | 10898 | 11086 | 15158 |
|  |  | Kimono1 | 10943 | 11149 | 12969 |
|  |  | ParkScene | 11420 | 11240 | 13287 |
|  | B Avg. |  | 11275 | 11391 | 13742 |
|  | C | BasketballDrill | 2880 | 3058 | 3690 |
|  |  | BQMall | 2975 | 3054 | 3714 |
|  |  | PartyScene | 3030 | 3158 | 3601 |
|  |  | RaceHorses | 3076 | 3239 | 3477 |
|  | C Avg. |  | 2990 | 3127 | 3620 |
|  | D | BasketballPass | 725 | 756 | 879 |
|  |  | BlowingBubbles | 750 | 809 | 921 |
|  |  | BQSquare | 789 | 848 | 969 |
|  |  | RaceHorses | 794 | 843 | 887 |
|  | D Avg. |  | 764 | 814 | 914 |
| c1 Avg. |  |  | 6415 | 6605 | 8002 |
| c2 | B | BasketballDrive | 9702 | 10170 | 11694 |
|  |  | BQTerrace | 10461 | 10555 | 11305 |
|  |  | Cactus | 9314 | 9426 | 11961 |
|  |  | Kimono1 | 9737 | 9678 | 11779 |
|  |  | ParkScene | 9807 | 10068 | 11561 |
|  | B Avg. |  | 9804 | 9979 | 11660 |
|  | C | BasketballDrill | 2618 | 2645 | 3113 |
|  |  | BQMall | 2734 | 2646 | 3119 |
|  |  | PartyScene | 2819 | 2805 | 3141 |
|  |  | RaceHorses | 2865 | 2849 | 3144 |
|  | C Avg. |  | 2759 | 2736 | 3129 |
|  | D | BasketballPass | 665 | 666 | 762 |
|  |  | BlowingBubbles | 724 | 715 | 808 |
|  |  | BQSquare | 739 | 733 | 829 |
|  |  | RaceHorses | 744 | 755 | 815 |
|  | D Avg. |  | 718 | 717 | 803 |
|  | E | vidyo1 | 5700 | 5670 | 7089 |
|  |  | vidyo3 | 5685 | 5747 | 6770 |
|  |  | vidyo4 | 5764 | 5779 | 6973 |
|  | E Avg. |  | 5716 | 5732 | 6944 |
| c2 Avg. |  |  | 5005 | 5057 | 5929 |

Table 8 Encoder simulation time in AIF case

#### Decoder simulation time

Figure 31 Decoder simulation time for Cs1

Figure 32 Decoder simulation time for Cs2

| Cs | Class | Seq | AIF_SAIF | AIF_BiSingle | AIF_EAIF |
| --- | --- | --- | --- | --- | --- |
| c1 | A | PeopleOnStreet | 56 | 59 | 93 |
|  |  | Traffic | 59 | 59 | 130 |
|  | A Avg. |  | 57 | 59 | 112 |
|  | B | BasketballDrive | 53 | 52 | 75 |
|  |  | BQTerrace | 61 | 57 | 86 |
|  |  | Cactus | 42 | 45 | 91 |
|  |  | Kimono1 | 56 | 59 | 74 |
|  |  | ParkScene | 56 | 58 | 75 |
|  | B Avg. |  | 54 | 54 | 80 |
|  | C | BasketballDrill | 10 | 10 | 18 |
|  |  | BQMall | 11 | 10 | 14 |
|  |  | PartyScene | 12 | 12 | 16 |
|  |  | RaceHorses | 13 | 13 | 16 |
|  | C Avg. |  | 11 | 12 | 16 |
|  | D | BasketballPass | 3 | 3 | 3 |
|  |  | BlowingBubbles | 3 | 3 | 4 |
|  |  | BQSquare | 4 | 4 | 5 |
|  |  | RaceHorses | 4 | 4 | 4 |
|  | D Avg. |  | 3 | 3 | 4 |
| c1 Avg. |  |  | 29 | 30 | 47 |
| c2 | B | BasketballDrive | 46 | 39 | 54 |
|  |  | BQTerrace | 48 | 47 | 57 |
|  |  | Cactus | 38 | 36 | 51 |
|  |  | Kimono1 | 52 | 49 | 58 |
|  |  | ParkScene | 49 | 45 | 56 |
|  | B Avg. |  | 47 | 43 | 55 |
|  | C | BasketballDrill | 9 | 8 | 12 |
|  |  | BQMall | 10 | 8 | 13 |
|  |  | PartyScene | 11 | 11 | 14 |
|  |  | RaceHorses | 12 | 12 | 14 |
|  | C Avg. |  | 10 | 10 | 13 |
|  | D | BasketballPass | 3 | 2 | 3 |
|  |  | BlowingBubbles | 3 | 3 | 4 |
|  |  | BQSquare | 3 | 3 | 4 |
|  |  | RaceHorses | 4 | 4 | 4 |
|  | D Avg. |  | 3 | 3 | 3 |
|  | E | vidyo1 | 17 | 16 | 20 |
|  |  | vidyo3 | 17 | 17 | 24 |
|  |  | vidyo4 | 18 | 18 | 31 |
|  | E Avg. |  | 17 | 17 | 25 |
| c2 Avg. |  |  | 21 | 20 | 26 |

Table 9 Decoder simulation time in AIF case

## Consideration for combination with SIFO

Bi/Single filter switching for FIF can be implemented with SFIO. Example of syntax change is explained in this sub clause.

### Syntax

| slice_header( ) { | C | Descriptor |
| --- | --- | --- |
| first_lctb_in_slice | 2 | ue(v) |
| slice_type | 2 | ue(v) |
| … | 2 | ue(v) |
| slice_qp_delta | 2 | se(v) |
| alf_param() |  |  |
| if ( slice_type = = P ¦ ¦ slice_type = = B ) { |  |  |
| mc_interpolation_idc | 2 | ue(v) |
| if( slice_type = = B ) { |  |  |
| mc_interpolation_bi_idc | 2 | ue(v) |
| mc_weak_filter_pos_bi_idc | 1 | ue(v) |
| } |  |  |
| mv_competition_flag | 2 | u(1) |
| … |  |  |
| } |  |  |

### Semantics

- mc_interpolation_bi_idc : This index specifies MC interpolation filter set for Bi-pred.

- mc_filter_strength_pos_bi_idc : When this value is set to 1, MC interpolation filter at sub pel position =’g’ is set to “weak filter” for Bi-pred. Otherwise “Strong filter” is used.

# Conclusion

Bi/Single filter switching can improve coding efficiency for B-picture. This method can be applied to various interpolation methods. In this contribution, it is implemented on top of conventional 6 tap SFIF and SAIF. But this can be applied to other interpolation scheme discussed in MV resolution AHG, e.g. SIFO, 1/8 pel, 1/12 pel, 8 tap, or 12 tap)

CE to test this method is proposed.

Sony Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).