| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 2nd Meeting: Geneva, CH, 21-28 July, 2010 | Document: JCTVC-B091 |
| --- | --- |

| Title: | Some improvement of the arithmetic coding based on probability aggression |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbo Zhu | Tel: Email: | hurricanezhb@hotmail.com |
| Source: | Hongbo Zhu |  |  |

_____________________________

# Introduction

This proposal is an improvement version of the JCTVC-A027 [1]. The two improvements are as follows:

- Word based input and output

- Improved logProb table of logprobaddvl[64][2].

The basic procedure of the encoder is as follows:

If(MPS)

rngprob += logprobaddvl[probidx][1];

Else

{

unsigned int tmplgprb = rngprob + logprobaddvl[probidx][1];

rngprob += logprobaddvl[probidx][0];

low += ((subinterval_division[127&( tmplgprb >>5)]<<8)>>( tmplgprb >>12));

}

If(rngprob >= 0x10000)

{

rngprob -= 0x10000;

Output one word to bitstream

low = 0;

Renormalization of rngprob

}

The basic procedure of the decoder is as follows:

Unsigned int tmplgprb = rngprob + logprobaddvl[probidx][1];

Unsigned int threshold = ((subinterval_division[127&( tmplgprb >>5)] <<8)>>( tmplgprb >>12));

If(threshold <= value)

{

Output LPS

value -= threshold;

rngporb += logprobaddvl[probidx][0];

}

Else

{

Output MPS

rngporb += logprobaddvl[probidx][1];

}

If(rngprob >= 0x10000)

{

rngprob -= 0x10000;

value = (value<<16)+ReadOneWord;

Renormalization of rngprob

}

# Experiment results

The arithmetic coding is implemented in jm17.0 to get the results. Some basic configurations are as follows:

IntraPeriod = 32

FrameSkip = 0

SearchRange = 128

NumberReferenceFrames = 4

EnableIPCM = 0

NumberBFrames = 7

BList0References = 4

BList1References = 4

HierarchicalCoding = 2

HierarchyLevelQPEnable  =  1

BiPredMotionEstimation = 1

BiPredMERefinements    = 3

BiPredMESearchRange    = 16.

BiPredMESubPel         = 2

WeightedPrediction       =  0

WeightedBiprediction     =  2

RDOptimization =  1

SearchMode = 1

QP of I Slice = 22,27,32,37

QP of P Slice = 23,28,33,38

QP of B Slice = 24,29,34,39

Foreman_cif (300 frames), mobile_cif (300 frames) and carphone_cif (382 frames) are used to generated the results illustrated in table 1.

|  | BDPSNR | BDbitrate |
| --- | --- | --- |
| Mobile_cif | -0.01115 | 0.236667 |
| Foreman_cif | -0.01019 | 0.243659 |
| Carphone_cif | -0.00488 | 0.119895 |
| Average: | -0.00874 | 0.200073 |

# Reference

- Hongbo Zhu, “Arithmetic coding based on probability-aggregation and delayed-subdivision”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC TC1/SC29/WG11 1st Meeting: Dresden, DE, 15-23 April, 2010, Document JCTVC-A027.

# Patent rights declaration(s)

Hongbo Zhu may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).