| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 2nd Meeting: Geneva, CH, 21-28 July, 2010 | Document: JCTVC-B116 |
| --- | --- |

| Title: | TMuC performance evaluation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Rahul Panchal, Peisong Chen, Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: Email: | +1-858-845-1347 peisongc@qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution focuses on coding efficiency evaluation of TMuC 0.4. The average improvement of TMuC 0.4 over TMuC 0.1 obtained using the best combination of tools is 7.1% for the IPPP configuration.

# Tested Configuration

Following tools from TMuC 0.4, which were integrated on top of TMuC 0.1, were enabled:

- Block merging for Inter-Coded Block.

- Adaptive reference sample smoothing.

- Improved Transform Coefficient Coding with CABAC.

- Single pass switched Interpolation Filters with offsets (single pass SIFO)

  - Directional interpolation filters

- Adaptive Motion Vector Resolution.

- Mode dependent directional transforms for intra-prediction residuals.

- Adaptive Loop Filter (diamond filter support).

- Directional intra prediction: angular method.

All the tools inherited from TMuC 0.1 were enabled unless there is a corresponding method in TMuC 0.4 replacing them.

# Simulation results

Comparison between TMuC 0.4 and  TMuC 0.1 is given in Table 1. All the video sequences used in CfP from classes B, C, D, and E were tested, at QP values of 22, 27, 32, and 37, respectively. IPPP configuration was used.

Table 1: Simulation results comparing TMuC 0.4 and TMuC 0.1.

|  | BD Rate [%] |
| --- | --- |
| WQVGA30_RaceHorses | -1.75 |
| WQVGA50_Basketball | -1.27 |
| WQVGA50_PartyScene | -5.60 |
| WQVGA60_BQSquare | -19.45 |
| WQVGA AVE | -7.02 |
| WVGA30_RaceHorses | -5.47 |
| WVGA50_Basketball | -8.23 |
| WVGA50_PartyScene | -10.86 |
| WVGA60_BQSquare | -4.53 |
| WVGA AVE | -7.27 |
| 720p60_Vidyo1 | -3.76 |
| 720p60_Vidyo3 | -8.42 |
| 720p60_Vidyo4 | -4.72 |
| 720p AVE | -5.63 |
| 1080p24_ParkScene | -7.49 |
| 1080p24_Kimono | -1.73 |
| 1080p50_Basketball | -9.11 |
| 1080p50_Cactus | -6.12 |
| 1080p60_BQSquare | -15.51 |
| 1080p AVE | -7.99 |
| Average | -7.13 |

# Patent rights declaration(s)

Qualcomm Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).