| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 2nd Meeting: Geneva, CH, 21-28 July, 2010 | Document: JCTVC-B309 |
| --- | --- |

| Title: | Tool Experiment 9: Large Block Structure |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Tool Experiment Description |  |  |
| Author(s) or Contact(s): | Munchurl Kim Korea Advanced Institute of Science and Technology (KAIST)1 Hui Yong Kim ETRI2 Kazushi Sato SONY3 | Tel: Email: | +82-42-350-7419 mkim@ee.kaist.ac.kr +82-42-860-5069 hykim5@etri.re.kr +81-3-5435-3308 Kazushi.Sato@jp.sony.com |
| Source: | KAIST1, ETRI2, SONY3 |  |  |

_____________________________

# Introduction

Enlarged coding blocks and large block-sized transforms are introduced in the HEVC Test Model understand Consideration (TMuC) [1] as new coding tools to improve the coding efficiency. The structure of enlarged coding blocks is organized in terms of coding unit (CU), partition unit (PU) and transform unit (TU) in the HEVC TMuC. Furthermore, the CU, PU and TU are operated in conjunction with different transform kernel sizes. The transform kernel size in TMuC is maximally allowed up to 64 x 64.

The purpose of TE9 (Tool Experiment 9: Large Block Structure) is to investigate the coding efficiency for the large block structure in terms of CU, PU and TU in the HEVC TMuC version 0.6. Also, the non-square coding unit structure based on the proposal [6] at 2nd JCT-VC meeting is compared with the large block structure of the HEVC TMuC 0.6 in terms of BD-PSNR [3] and BD-BR [4] performances.

# Participants List

| Participants | Persons, Email addresses | P/C |
| --- | --- | --- |
| KAIST | Munchurl Kim, mkim@ee.kaist.ac.kr Jaeil Kim, jaeil1203@kaist.ac.kr Bumshik Lee, lbs@kaist.ac.kr Sangsoo Ahn, ssa@kaist.ac.kr | C |
| ETRI | Hui-Yong Kim, hykim5@etri.re.kr Jongho Kim, pooney@etri.re.kr | C |
| SONY | Kazushi Sato, Kazushi.Sato@jp.sony.com | P |
| Zhejiang University | Lu Yu, yul@zju.edu.cn Xiaolin Shen, babylion@zju.edu.cn | C |
| Motorola | Krit Panusopone, krit@motorola.com | C |
| Hanyang University | Kiho Choi, aikiho@dmlab.hanyang.ac.kr Euee S. Jang, esjang@hanyang.ac.kr | C |

P/C = Proponent / Crosschecking TE partner

# Tools under Test

## Large Block Structure in TMuC

The test points are identified in the table below with respect to the sizes of CTUs, PUs and TUs up to the maximum partition depths of CTU and TU.

Table 1 Performance Tests for TMuC 0.6

| Configurations | Controlled Features | Codebase | Test points |
| --- | --- | --- | --- |
| TMuC 0.6 encoder_intra.cfg [2] | Unit Definition: MaxCUWidth= MaxCUHeight=64 MaxPartitionDepth=4 QuadtreeTULog2MaxSize=2 | TMuC 0.6 reference SW | Max CU sizes: 16, 32, 64, 128 Variable TU sizes: - 6464, 3232 - 3232, 1616 - 1616, 88 - 88, 44 |
| TMuC 0.6 encoder_lowdelay.cfg [2] | Unit Definition: MaxCUWidth= MaxCUHeight=64 MaxPartitionDepth=4 QuadtreeTULog2MaxSize=6 | TMuC 0.6 reference SW | Max CU sizes: 16, 32, 64, 128 Variable TU sizes: - 6464, 3232, 1616, 88, 44 - 6464, 3232, 1616, 88 - 3232, 1616, 88, 44 - 3232, 1616, 88 - 1616, 88, 44 - 1616, 88 - 88, 44 |
| TMuC 0.6 encoder_randomaccess.cfg [2] | Unit Definition: MaxCUWidth= MaxCUHeight=64 MaxPartitionDepth=4 QuadtreeTULog2MaxSize=6 | TMuC 0.6 reference SW | Max CU sizes: 16, 32, 64, 128 Variable TU sizes: - 6464, 3232, 1616, 88, 44 - 6464, 3232, 1616, 88 - 3232, 1616, 88, 44 - 3232, 1616, 88 - 1616, 88, 44 - 1616, 88 - 88, 44 |

## Additional Tools

### Non-Square Coding Unit Structure (SONY) [6]

Enlarging the size of macroblock improves coding efficiency by saving motion information especially with larger size images like HD or 4k x 2k. TMuC employs CTB (Coding Tree Block) scheme, whose shape is square and whose size is not restricted to 16x16.

From implementation perspective with larger-sized macroblocks or CTB, the following two problems will be taken into consideration: Firstly, it causes the increase in buffer sizes to store pixels in the neighboring macroblocks for intra prediction or deblocking filtering; Secondly, the scan order of processing each of 16x16 blocks differs from the coding unit blocks in the current TMuC if larger CTB than 16x16 is employed. This would become bottleneck in developing multi-codec LSI for HEVC as well as other codecs like MPEG-2 or AVC.

In this TE, the method of non-square CTB will be investigated in low-cost implementation with respect to the following two scenarios: In scenario 1, only the top layer is non-square; In scenario 2, the ratio of non-square CTB is fixed to 2:1 but the boundary between non-square and square CTB can vary. In either scenario, additional parameters will be conveyed in the Sequence Parameter Set for the simplicity of syntax.

The experimental results will be examined in the trade-off between coding efficiency improvement and computational cost.

# Tool Experiment Tasks

The following tasks shall be performed in this tool experiment.

- Identify performance of the TMuC large block structure based on the default configurations of common test conditions [2]

- Analyze the experimental results with respects to different sizes of CU, PU and TU in the TMuC 0.6 [1] (as present in the software used for the evaluation)

- Communicate with TE12 for experiment coordination on large transforms

# Test Conditions

The TE experiments will be carried out based on the test conditions and reference configurations specified in [2].

As a codebase for TE9, we plan to use the latest TMuC software version (TMuC 0.6) which is scheduled to be released on 9th August. If not available, the final version of TMuC produced as an output from the 2nd JCT-VC meeting is to be used, instead. In either case, high-efficiency configuration files are to be only used for this TE.

# Time-line and Responsibilities

- 2010-08-05:	Assign the TE experiment works for TE Participants

- 2010-08-13:	Finalize and upload the TE description

- 2010-08-28:	Implement the proposed method [6] into TMuC 0.6 reference software for performance evaluation by the proponent (SONY)

- 2010-09-15: 	Complete all TE experiments in Section 3.1 by all participants

- 2010-09-15:	Complete all TE experiments in Section 3.2 by the proponent (SONY) and a cross-checker(s)

- 2010-09-22:	Perform cross-check of all TE results

- 2010-09-26:	Provide visual subjective assessments on all TE results by the TE participants

- 2010-10-01:	Upload input document for TE-9 activity

# Definition of Performance Measurement Criteria

The performance measurements are evaluated in different combinations of coding unit sizes in conjunction with transform sizes. Also, the performance of the proposed method [6] will be compared based on the same code base (TMuC 0.6) for the encoder configurations with encoder_intra.cfg, encoder_lowdelay.cfg and encoder_randomaccess.cfg released at the 2nd JCT-VC meeting in Geneva. The following measurements are considered to be used in this tool experiment.

## Coding Performance Measurements

Objective rate-distortion measurements using PSNR, BD-PSNR, BD-Rate [3][4].

The visual subjective quality will be assessed by the involved experts.

## Complexity Measurement

For the assessment of complexity impact, the following information should be provided:

- Algorithm description including tool configurations,

- Implementation cost in buffer size, memory access bandwidth, etc.

# References

- JCT-VC, “Draft Test Model under Consideration”, JCTVC-Bxxx, 2nd JCT-VC Meeting, Geneva, CH, July 2010.

- JCT-VC, “Common Test Conditions and Software Reference Configurations”, JCTVC-B300, 2nd JCT-VC Meeting, Geneva, CH, July 2010.

- Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.

- Gisle Bjøntegaard, “Improvements of the BD-PSNR model”, ITU-T SG16/Q6, 35th VCEG Meeting, Berlin, Germany, July 2008, Doc. VCEG-AI11.

- Minhua Zhou. Zhou, “Coding efficiency test on large block size transforms in HEVC”, JCTVC-B028, 2nd JCT-VC meeting, Geneva, CH, July. 2010.

- Peng Wang and Kazushi Sato, “Proposal of Non-Square Coding Unit Structure”, JCTVC-B082, 2nd JCT-VC meeting, Geneva, CH, July. 2010.