| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 2nd Meeting: Geneva, CH, 21-28 July, 2010 | Document: JCTVC-B311 |
| --- | --- |

| Title: | Tool Experiment 11: Motion Vector Coding |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Tool Experiment Description |  |  |
| Author(s) or Contact(s): | Joel Jung 38-40 rue du Général Leclerc 9279 ISSY LES MOULINEAUX | Tel: Email: | +33 1 45 29 53 66 joelb.jung@orange-ftgroup.com |
| Source: | Orange Labs |  |  |

_____________________________

# Introduction

Contributions JCTVC-B081, JCTVC-B094, JCTVC-B101, proposed to set up a tool experiment on MV coding, based either on results obtained with modifications of the current TMuC scheme, or on experimental results obtained from the TMuC itself, with different configurations. This tool experiment addresses this topic, and consequently targets at exploring the performance of the different motion vector coding methods proposed and their possible combinations.

# Participants List

| Participant | Person, Email address |
| --- | --- |
| Sony | K. Sato (Kazushi.Sato@jp.sony.com) |
| Sungkyunkwan University | B. Jeon (bjeon@skku.edu) J. Yang (binbak1@ece.skku.ac.kr) |
| Samsung | W.J. Han (wjhan.han@samsung.com) |
| NTT DOCOMO | F. Bossen (bossen@docomolabs-usa.com) |
| LGE | S. Park (seungwook.park@lge.com) |
| HHI | H. Schwarz (heiko.schwarz@hhi.fraunhofer.de) |
| Orange Labs | J. Jung (joelb.jung@orange-ftgroup.com) |

# Experiments

## Check the added value of MV-Competition

A first test, as requested during the meeting, will be performed to assess the gain provided by the competition, using as a reference a scheme that as only one median predictor, as in AVC.

## Merge / Skip

As a consequence of the discussions held during the breakout session on Thursday 22nd, the efficiency of the ‘Merge’ method and the ‘Skip’ method proposed respectively in [2] and [3] needs to be studied to check which one is the most appropriate, or if both  be applied together. And at which level (PU, CU).

## Adaptation of the set of predictors / pruning

In contribution JCTVC-B081 (Sony), it is mentioned that the temporal predictor does a good job between a moving object and stationary background. A dynamic code number assignment for each of the motion predictor candidate is proposed. It is observed that especially at lower bit-rate, RD-optimized mode decision tends to select mode with smaller code number. So with camera-fixed-type sequences assignment of smaller code number for the temporal predictor can improve visual quality as well as coding efficiency.

The contribution asks the ability to adapt code number assignment (that can be seen as the modification of the set of predictors) in the Slice Header (only possible at the SPS level with the current TMuC), as the best predictor may change if scene change occurs in the sequence.

Additionally, in the contribution [4] adopted in the KTA software in 2006, different set of predictors were used for Inter or Skip competition. The impact of such an adaptation can be studied in the TMuC.

## Reduction of the set of predictors / pruning

In contribution JCTVC-B094 (NTT DOCOMO), a method for coding motion vector data was presented. It reduces the number of motion vector predictors to 3, and simplifies the parsing of motion vector data, that does no more rely on motion vector data decoded for the current partition as in AMVP. In AMVP the set of predictors is pruned after decoding the motion vector difference. Thus a predictor construction process affecting parsing of the predictor index takes place while parsing motion vector data associated with the current partition. Such a dependency adversely affects the complexity of the finite state machine typically used for the parsing of the bitstream in hardware implementations. In the proposed method the dependency of the predictor index parsing is limited to the reference index which can take only a much small number of values and may thus be more easily pre-computed.

## Interleaved Motion Vector Prediction

Document JCTVC-B205 [6] describes Interleaved Motion Vector Prediction (IMVP). With IMVP, the prediction and coding of the components of a motion vector is interleaved. In a first step, the vertical motion vector component is predicted and the difference between the actual vertical component and its prediction is coded. Then, only the motion vectors of the neighborhood for which the absolute difference between their vertical component and the coded vertical component for the current motion vector is minimized are used for the prediction of the horizontal component. IMVP is further investigated in this TE.

# Test Sequences / Test Points / SW Configuration

Simulation results will be generated for sequences and coding conditions specified in the document JCTVC-B300 [5]. Two configurations are mandatory: 1- random access high efficiency, 2- low delay low complexity. Results with the two other Inter configurations may be provided. Latest version of the TMuC, available on August 9th will be used for evaluation.

# Definition of Performance Measurement Criteria

## Coding Performance Measurements

Measure impact on bitrate/PSNR using provided data (4 points, BDRate).

## Complexity Considerations

To provide an indicative assessment of complexity, the following numbers should be provided:

- Encoding time (compared to the default configuration)

- Decoding time (compared to the default configuration)

# Time-line

- 2010-August 13:	Final TE description, and description of proposals

- 2010-September 10:	Testing complete

- 2010-September 24: 	Cross-check complete

- 2010-October 01:	Input Document upload

# References

- ITU-T Q6/16 and ISO/IEC JTC1/SC29/WG11, Joint call for proposals on video compression technology, MPEG Document N11113, Kyoto, Japan, January 2010.

- JCTVC-A116 contribution, Description of video coding technology proposal by Fraunhofer HHI.

- JCTVC-A124 contribution, Samsung’s Response to the Call for Proposals on Video Compression Technology.

- G. Laroche, J. Jung, Competition-Based Scheme for Motion Vector Selection and Coding, VCEG-AC06 contribution, 31th Meeting: Klagenfurt, Austria, 17-18 July 2006.

- JCTVC-B300 contribution, Common test conditions and software reference configurations, F. Bossen.

- JCTVC-A205 contribution, Test Model under Consideration.