| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C007 WG11 Number: m18047 |
| --- | --- |

| Title: | JCT-VC AHG report: Memory compression |  |  |
| --- | --- | --- | --- |
| Status: | [Input Document] |  |  |
| Purpose: | [Report] |  |  |
| Author(s) or Contact(s): | Keiichi Chono (Chair) 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN Takeshi Chujoh 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, Japan ChongSoon Lim Blk 1022 Tai Seng Avenue #06-3530 Singapore 534415 | Tel: Email: | +81-50-431-7595 chono@ct.jp.nec.com +81-44-549-2288 takeshi.chujoh@toshiba.co.jp ChongSoon.Lim@sg.panasonic.com |
| Source: | [Memory Compression AHG] |  |  |

_____________________________

# Abstract

The following summarizes the Memory compression Ad hoc activities between the 2nd JCT-VC meeting in Geneva, Switzerland (21 to 28 July, 2010) and the current 3rd Meeting: Guangzhou, CN, 7-15 October, 2010.

# Mandate

The mandate of this AHG as specified in JCTVC-B200 is as follows:

| AHG Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| Memory compression (jct-vc@lists.rwth-aachen.de) - Study memory compression impact on video encoder and decoder: memory bandwidth reduction, graceful power degradation, etc. - Study memory compression schemes including adaptive scaling, DPCM, transform coding, entropy coding, checkerboard sampling, etc. | K. Chono (chair), T. Chujoh, C.S. Lim (vice chairs) | N |

# Status

The Ad hoc Group (AHG) on Memory Compression was formed during the July 2010 JCT-VC meeting in Geneva. The most of this AHG activity has been devoted to discuss and develop an MC read memory access bandwidth measure module for TMuC software in order to access the results of TE2: IBDI and Memory Compression. Approximately 70 e-mails were exchanged over the members from 4 parties/companies.

Related to Mandate 1:

The following items are listed as the benefits of memory compression for video encoder and decoder:

- Reduction in MC read memory access bandwidth;

- Reduction in DPB write memory access bandwidth; and

- Graceful power degradation by enabling the low resolution decoding of a bit-stream.

Figure 1		Conceptual block diagram of memory compression in video encoder and decoder

The second item can be reasonably assessed by comparing the compression ratios of memory compression schemes since the DPB write memory access is not random access. Similarly, the third item can be reasonably assessed by counting the number of low resolution pixels. Therefore, AHG activity has been devoted to discuss the first item. Indeed, it was mentioned by JCT-VC chairs during the last JCV-VC meeting that the compression ratios of memory compression schemes are not a comparable measure since overheads that come from the overlaps of the size of memory compression unit access in MC read random access are unclear.

Through the discussion on the first item as described in detail below, AHG developed a module that computes run-time memory access bandwidth measure for TMuC decoder.

In order to have a comparable measure, some TE2 members suggested developing a module that computes run-time memory access bandwidth for given the combination of MC block size, the tap-length of MC interpolation filter, Motion Vector (MV), sample-bits per pixel, etc. AHG members agreed to the suggestion and discussed a YUV storage structure in reference picture buffer. By considering the TMuC design in which the same MC interpolation filter is applied to UV components and a different MC interpolation filter is applied to Y component than UV's, the member decided to employ a YUV format comprising of a plane Y component and horizontally-interleaved UV components, as illustrated in Figure 2.

Figure 2		YUV format comprising of plane Y component and horizontally-interleaved UV components.

AHG members developed the module and updated it to account for the overlaps of a memory compression unit size. Given a combination of MC block size, the tap-length of MC interpolation filter, MV, sample-bits per pixel, memory compression unit size, the updated module computes the memory access samples as illustrated in Figure 3.

Figure 3		Example of memory access samples for a 4x4 MC block that points to sub-pel reference samples which are interpolated by a 2-D 6-tap filter. Magenta square is a set of MC reference samples associated with MC interpolation. Light-green-shaded square is a set of its memory access samples associated with a 4x4 memory compression unit.

Then AHG members discussed a model of compressed reference storage and decided to use a 1-D storage model, as illustrated in Figure 4. In the 1-D storage model, a memory compressor compresses each of compression units and stores its compressed versions, whose size is byte-aligned, into 1-D storage in the raster scan order. A memory decompressor accesses a set of compressed reference data units stored in the 1-D storage and outputs the decompressed compression units (MC reference samples) to a MC module. Since the 1-D storage access is restricted by memory alignment and memory burst sizes, the actual memory access size (hereafter, referred by run-time memory access bandwidth) is determined on the basis of the associated memory read overheads as well as the number of compressed reference data units.

Figure 4		Conceptual block diagram of compressed reference storage in a 1-D storage. Memory compressor compresses each of compression units represented by green-square and stores its compressed version into 1-D storage. A set of compressed reference data represented by orange-square is accessed through memory controller and decompressed by memory decompressor.

Although run-time memory access bandwidth is varied by memory allocation methods as depicted by Figure 5, AHG members agreed that the raster scan is used as a default setting and that TE2 proponents can report run-time memory access bandwidth measures obtained by their specific memory allocation methods in addition to those obtained by the default setting. The reason is that the optimal memory allocation method depends on a memory compression unit size as well as memory alignment and memory burst sizes

Figure 5		Example of run-time memory access bandwidths for different scan-order 1D-storages.

Although run-time memory access bandwidth is varied by memory allocation methods as depicted by Figure 5, AHG members agreed that the raster scan is used as a default setting and that TE2 proponents can report run-time memory access bandwidth measures obtained by their specific memory allocation methods in addition to those obtained by the default setting. The reason is that

After having the 1-D storage model, NEC offered their resources to implement the module and to integrate it into the top of TMuC0.7 decoder. The module can be turn on and off by setting a macro “#define MC_MEMORY_ACCESS_CALC 1” in TypeDef.h. Panasonic and TI inspected it prior to the module distribution to TE2 participants. (Note that current implementation supports the runtime memory access only for Anchor encoding settings, i.e., SIFO with 12tap and AMV res., and DIF without AMV res. Furthermore there are two restrictions: for a SIFO with 12tap and AMV res. case, the number of tap-lengths is not switched as the TMuC document specifies; for a DIF case, the shapes of memory access on diagonal fractional pixel positions are assumed to be the rectangular. However, the above restrictions presumably do not affect the assessment of MC read memory access bandwidth reduction obtained by memory compression schemes since Anchor and Proposal MC read memory access bandwidths are computed by the same way.)

A template excel sheet containing the memory access bandwidth measurement of the anchor streams generated based on JCTVC-B300.doc was created and distributed to TE2 participants along with the software. Using the module created by NEC, the total MC read memory bandwidth of the anchor streams for high efficiency and low complexity settings are computed based on combinations of DDR address and burst alignment bits, (8,8), (32, 64), (64, 128), (128,128), and (256, 256). Using the same memory bandwidth computation tool provided by NEC, the TE2 participants are requested to provide the total MC read memory bandwidth measurement of the same anchor streams based on each proponent’s proposed memory compression unit size and compression ratio.

Example:

Without Memory Compression (Anchor)

|  | QP | kbps | Y psnr | U psnr | V psnr | Enc T [s] | Dec T [s] | Enc T [h] | 32bit/ 64bit (Kbytes) | 64bit/ 128bit (Kbytes) | 128bit/ 128bit (Kbytes) | 256bit/ 256bit (Kbytes) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Vidyo1 | 22 | 2495.11 | 43.46 | 46.53 | 47.42 | 96709.92 | 102.89 | 26.86 | 3321 | 3576 | 3752 | 4665 |
|  | 27 | 880.55 | 41.13 | 45.08 | 45.71 | 90201.23 | 97.59 | 25.06 | 2904 | 3079 | 3179 | 3797 |
|  | 32 | 418.44 | 38.52 | 43.44 | 43.78 | 87725.76 | 90.98 | 24.37 | 2605 | 2736 | 2797 | 3252 |
|  | 37 | 216.32 | 35.77 | 41.97 | 41.89 | 86574.76 | 84.66 | 24.05 | 2390 | 2490 | 2526 | 2868 |

After Memory Compression (To be compared)

|  | QP | kbps | Y psnr | U psnr | V psnr | Enc T [s] | Dec T [s] | Enc T [h] | 32bit/ 64bit (Kbytes) | 64bit/ 128bit (Kbytes) | 128bit/ 128bit (Kbytes) | 256bit/ 256bit (Kbytes) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Vidyo1 | 22 | 2525.63 | 43.38 | 46.54 | 47.42 | 97795.28 | 105.16 | 27.17 | 2316 | 2316 | 2316 | 2444 |
|  | 27 | 890.85 | 40.95 | 45.08 | 45.70 | 91078.83 | 103.14 | 25.30 | 2017 | 2017 | 2017 | 2103 |
|  | 32 | 422.03 | 38.37 | 43.44 | 43.78 | 88535.85 | 97.33 | 24.59 | 1784 | 1784 | 1784 | 1846 |
|  | 37 | 216.66 | 35.72 | 41.99 | 41.80 | 86998.41 | 86.47 | 24.17 | 1622 | 1622 | 1622 | 1666 |

Results (Including MC read memory bandwidth savings)

| BD-rate Y | BD-rate U | BD-rate V | 32bit/ 64bit | 64bit/ 128bit | 128bit/ 128bit | 256bit/ 256bit |
| --- | --- | --- | --- | --- | --- | --- |
| 0.089 | 0.391 | 0.118 | -22.5% | -34.2% | -42.0% | -49.7% |
|  |  |  | -23.4% | -34.4% | -42.0% | -49.5% |
|  |  |  | -24.9% | -34.7% | -41.8% | -49.3% |
|  |  |  | -26.7% | -34.6% | -40.7% | -48.1% |

It was also discussed that having a simple cache model to measure memory bandwidth after the use of cache will be desirable since there are video coding solutions in the marketplace that use cache. Video coding solutions that do not use cache are also widely used in the marketplace, so the cache model needs to have the option of being switched on and off. TI offered their resources to implement a simple cache model on top of NEC’s memory bandwidth measurement module in TMuC0.7 decoder. After a quick informal survey of existing L1 and L2 cache architectures on recent microprocessors and DSP, a single level cache with following parameters was implemented in TMuC-0.7 decoder:

- Capacity (cache size): 128 KB

- Line size: 64 bytes

- Organization: 4-way set-associative

- Replacement strategy: FIFO (Least Recently Used (LRU) strategy is industry standard, but FIFO is easier to implement)

- DDR alignment: 64 bits

The aim of the cache model is to have a simple cache simulation that captures the most relevant parameters for comparing bandwidth consumed with and without memory compression and that is fair to all block structures used in memory compression. The goal was not to develop a very sophisticated cache model. The software cache model was released TE2 members after it was inspected by NEC and others.

The memory bandwidth usage is printed in the same format used by NEC's memory bandwidth measurement module. The code changes are isolated to be in #if MC_MEMORY_ACCESS_CACHE. MC_MEMORY_ACCESS_CALC needs to be enabled before cache can be used.

Related to Mandate 2:

There were the following proposals for TE2 :

- JCTVC-B044

- JCTVC-B057

- JCTVC-B089

- JCTVC-B103

- JCTVC-B114

[ JCTVC-B044 [1] (Toshiba)]

Proposal uses an adaptive scaling process block-by-block for memory compression. In the adaptive scaling process, the minimum pixel value and the maximum pixel values are derived in each of 4x4 blocks and those values are used to determine the use of either of adaptive scaling or fixed scaling. Currently proposal supports only an IBDI case, that is, 12 bit to 8 bit compression. The proposal does not require rate-control and is a loss less compression on 8-bit depth level.

[JCTVC-B057 [2] (NEC)]

Proposal uses a line-based non-linear 1-D PCM for memory compression. Given each of 8-pixel sets, the left-most pixel is first coded with PCM and serves as a base point, and the remaining pixels are then coded with DPCM from left to right. The proposal achieves 12 bit to 7.6 bit compression for an IBDI case and 8 bit to 5.4 bit compression for a non-IBDI case. It does not require rate-control.

[JCTVC-B089 [3] (TI)]

Proposal uses the 2-D DWT with an entropy coding. The 2-D DWT sizes for luma and chroma are 8x8 and 4x4. The proposal achieves 12 bit to 8 bit compression for an IBDI case and 8 bit to 4 bit compression for a non-IBDI case. The proposal requires rate-control. For a non-IBDI case, the proposal uses DC prediction to the DWT coefficients.

[JCTVC-B103 [4] (Panasonic)]

Proposal uses the 2-D Hadamard transform with a bit plane entropy coding. The 2-D Hadamard transform sizes for luma and chroma are 4x4 and 2x2. The luma and chroma bit planes are packed together in the same compression unit. The proposal achieves 12 bit to 5.33 bit compression for an IBDI case and 8 bit to 5.33 bit compression for a non-IBDI case. The proposal does not require rate-control.

[JCTVC-B114 [5] (Sharp)]

Proposal stores a low-resolution version of the full resolution frame. The low-resolution version consists of half the image pixels and corresponds to a quincunx sampling of the pixel data obtained without a low pas filter. The low-resolution version is coded using an adaptive moment block truncation coding (AMBTC) technique and stored in the frame buffer. Missing pixels are predicted from the stored values using bi-linear interpolation. The residual differences are coded by the AMBTC in a similar way. No rate control is required.

Table 1: Large block structures and classification

# Related input contributions

JCTVC-C077	A framework for standardization of memory compression

JCTVC-C094	An extension of DPCM-based memory compression to 2-D structure with ADPCM

JCTVC-C095	Performance improvement of DPCM-based memory compression by adaptive quantization

JCTVC-C228	A simple cache model for measuring motion compensation bandwidth

(Note that TE2 related contributions are excluded.)

# Recommendations

The recommendations of the AHG are to:

- Adopt the developed run-time memory access modules to the main branch of the software as an macro to be turn on or off by compilation

- Study MC memory access bandwidths of the combination of different minimum inter PU partition sizes and several TMuC MC interpolation schemes

- Study essential functionalities of memory compression schemes that need to be standardized in order to keep the interoperability of encoders and decoders

# Reference

  - T. Chujoh, T. Shiodera and T. Yamakage, “TE2: Adaptive scaling for bit depth compression on IBDI,” JCTVC-B044, 2nd. Meeting, Geneva, CH, 21-28 July, 2010.

  - H. Aoki, K. Chono, K. Senzaki, J. Tajime and Y. Senda, “Performance report of DPCM-based memory compression on TE2,” JCT-VC document JCTVC-B057, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 2nd meeting, Geneva, 2010.

  - M. U. Demircin, M. Budagavi, M. Zhou and S. Dikbas, “TE2: Compressed reference frame buffers (CRFB),” JCTVC-B089, 2nd. Meeting, Geneva, CH, 21-28 July, 2010.

  - C.S. Lim, H.W. Sun and V. Wahadaniah, “Reference Frame Compression Using Image Coder,” JCT-VC document JCTVC-B103, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 2nd meeting, Geneva, 2010.

  - Z. Ma and A. Segall, “System for Graceful Power Degradation,” JCT-VC document JCTVC-B114 Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 2nd meeting, Geneva, 2010.