| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C027 WG11 Number: m18038 |
| --- | --- |

| Title: | TE7: Verification results of Peking Univerity |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jungsun Kim Byeongmoon Jeon #221 Yangjae-dong, Seoucho-gu, Seoul 137-130, Korea | Tel: Email: | 82-2-6272-9179 jungsun.kim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This document reports the verification results of JCTVC-C089 on mode-dependent residual reordering for intra prediction residua [1] submitted by Peking University. The proponents provide their source codes.

Two separate binary files for each encoder and decoder are compiled on Window7 32bit for Win64 PC and Win32 PC.

The simulation follows the test conditions of TE7 [2] :

- Intra, high efficiency

- Random access, high efficiency

For test condition 1, the simulation succeeded on all platforms of Window7 32bit and WindowXP 64bit and produced as same results as the proponent provided. For test condition 2, the simulation succeeded and produced as same result as the proponent provided on same platforms the proponent used.

The spreadsheet contains the detailed data of the simulation is accompanied. The spreadsheet shows different result of V-PSNR for BasketballDrive_1920x1080_QP22 but it seems to be caused by rounding  off.

# Remarks

The results of test condition 2 on Window7 32bit and on WindowXP 64bit have differences for some sequences such as BQsquare_416x240 as shown below.

|  |  | Window XP 64bit |  |  |  | Window7 32bit |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | kbps | Y psnr | U psnr | V psnr | kbps | Y psnr | U psnr | V psnr |
| S13 | 22 | 1641.78 | 38.05 | 43.00 | 44.05 | 1640.88 | 38.05 | 43.02 | 44.04 |
| BQSquare | 27 | 638.17 | 34.85 | 40.96 | 41.88 | 637.33 | 34.85 | 40.98 | 41.88 |
|  | 32 | 291.76 | 32.09 | 39.42 | 40.12 | 291.37 | 32.08 | 39.43 | 40.13 |
|  | 37 | 148.24 | 29.40 | 38.12 | 38.45 | 148.71 | 29.41 | 38.14 | 38.46 |

The encoder crash was occurred for some sequences such as Racehorse_832x480_QP37 on WindowXP 64bit.

# References

- X. Zhao, L. Zhang, S. Ma, W. Gao, “TE7: Results for mode-dependent residual reordering for intra prediction residual”, JCTVC-C089, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Guangzhou,China, Oct. 2010 - R. Cohen, C. Ueo, R. Joshi, W. Ding, S. Ma, A. Tanizawa, H. Yang, X. Zhao, L. Zhang, “Tool Experiment 7: MDDT Simplification”, JCTVC-B307, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, Switzerland, Jul 2010