| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C030 WG11 Number: m18051 |
| --- | --- |

| Title: | TE12 : report on AMP evaluation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Tool Experiment Report |  |  |
| Author(s) or Contact(s): | Edouard Francois, Technicolor Laurent Guillo, INRIA, Atsuro Ichigaya, NHK, Haoping Yu, Huawei | Tel: Email: | +33-2-99-27-32-62 edouard.francois@technicolor.com laurent.guillo@inria.fr ichigaya.a-go@nhk.or.jp haopingyu@huawei.com |
| Source: | Huawei/Hisilicon, NHK, and Technicolor/INRIA |  |  |

_____________________________

# Abstract

This document reports the results of the evaluation of AMP performed in the context of TE12, as defined in 2nd JCTVC meeting. This evaluation was performed by Huawei / Hisilicon, NHK, and Technicolor / INRIA. Following performance are reported: when disabling AMP, average BD-rate loss is of 1.3% for Random Access High Efficiency, 1.8% for Random Access Low Complexity, 1.6% for Low Delay High Efficiency, 2.1% for Low Delay Low Complexity. Low delay presents higher losses on Class E, up to 3.8% for Low Complexity case. The related average encoding time is 65% compared to the anchors encoding time, while the decoding time is almost not impacted.

- Introduction

This document summarizes the Tool Experiment TE12 activities related to the evaluation of Asymmetric Motion Partitioning (AMP), [1].

This evaluation was performed by Huawei/Hisilicon, NHK, and Technicolor/INRIA.

- Tests conditions

The following platforms and compilers have been used:

| company | Platform | Compiler |
| --- | --- | --- |
| Huawei / Hisilicon | Windows Server 2003 64 bits | VS2005 |
| NHK | Windows XP 64bits | VS2008 |
| Technicolor / INRIA | Fedora Core 1164 bits | gcc 4.4.1 |

The testers have run each test case with ‘AMP on’ and ‘AMP off’ using similar processors, in order to obtain coherent Encoding and Decoding times for both configurations.

- Cross-checking results

The testers have observed the following points:

- There are mismatches between Linux or Windows platforms. But the differences between Linux and Windows results remain small, as depicted in the following table.

- Same simulations performed independently by Huawei / Hisilicon and by NHK using Window64 are perfectly matching.

- Similarly, anchors simulations made by Technicolor / INRIA using Linux are perfectly matching the anchors results provided by the TE12 chair.

Considering the minor differences between Linux or Windows platforms, the testers think that the evaluation results can be considered as relevant.

| Test | Tester1 | Tester2 | avg rate (%) |  |  | min rate (%) |  |  | max rate (%) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y | U | V | Y | U | V | Y | U | V |
| RA / HE / AMP on | Techni/INRIA | Huawei/HiSilicon | 0.0 | 0.0 | 0.0 | 0.0 | -0.3 | -0.1 | 0.0 | 0.1 | 0.1 |
| RA / HE / AMP off | Techni/INRIA | Huawei/HiSilicon | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 | 0.1 | 0.1 |
| RA / LC / AMP on | NHK | Huawei/HiSilicon | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| RA / LC / AMP off | NHK | Huawei/HiSilicon | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| LD / HE / AMP on | Techni/INRIA | NHK | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.3 | 0.0 | 0.0 | 0.0 |
| LD / HE / AMP off | Techni/INRIA | NHK | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| LD / LC / AMP on | NHK | Techni/INRIA | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.1 |
| LD / LC / AMP off | NHK | Techni/INRIA | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 |

Table legend:

- HE: High Efficiency

- LC: Low Complexity

- RA: Random Access

- LD: Low Delay

- Tester using linux is in Grey cell

- Tester using Windows64 is in White cell

- Perfect match is indicated in green font

- Mismatches are indicated in red font

# AMP performance

The full evaluation results can be found in files:

- TE12-AMP-HuaweiHisilicon.xls

- TE12-AMP-NHK.xls

- TE12-AMP-TechnicolorINRIA.xls

The following tables summarize the AMP performance evaluation obtained by Testers1:

It is observed that switching AMP off leads to an average BD-rate loss of 1.3% for Random Access High Efficiency, 1.8% for Random Access Low Complexity, 1.6% for Low Delay High Efficiency, 2.1% for Low Delay Low Complexity. Low delay presents higher losses on Class E, up to 3.8% for Low Complexity case. The related encoder time decrease of around 65% while the decoder time is not impacted.

# References

- Ken McCann, “Tool Experiment 12: Evaluation of TMuC Tools”, Doc. JCTVC-A312, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, Switzerland, July 2010.