| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C036_r3 WG11 Number: m18057 |
| --- | --- |

| Title: | Alternative performance measurement of MDDT and ROT in TMuC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Robert Cohen Anthony Vetro Huifang Sun Mitsubishi Electric Research Laboratories 201 Broadway Cambridge, MA 02139 U.S.A | Tel: Email: | +1-617-621-7503 cohen@merl.com avetro@merl.com hsun@merl.com |
| Source: | Mitsubishi Electric Research Laboratories |  |  |

_____________________________

# Abstract

This document evaluates the performance of ROT and MDDT both individually and combined in TMuC, using a reference in which both ROT and MDDT are disabled. The results show that for all-Intra high efficiency conditions in TMuC 0.7, these tools produce average BD-Rate improvements from 1.6% to 2.1%, where ROT slightly outperforms both MDDT and their combination. For random access high efficiency, the gains were between 0.7% and 0.9%, with a similar relationship among individual and combined tests. Encoder run times approximately doubled for ROT, whereas decoder run times doubled or tripled with MDDT, depending upon the configuration. Possible future improvements are addressed as well.

# Introduction

Tool Experiment 12 (TE12) [1] initially followed the paradigm of generating a reference with all tools enabled, and then disabling one tool at a time for comparison. For the closely related MDDT and ROT tools in TMuC [2], TE12 recently added a configuration where both were turned off. The purpose of this document is to report the results of an independent test for which the reference has MDDT and ROT both turned off, and then the performance gain of ROT and MDDT are tested individually and then together.

# Experimental conditions

These experiments were conducted using TMuC 0.7.1 [3], which is equivalent to using TMuC 0.7 for the configurations that we tested. We performed the “Intra high efficiency” and the “Random access high efficiency” experiments as outline in JCTVC-B300. The rate and PSNR values are reported in the accompanying spreadsheet.

# Experimental results

The BD-Rate results are summarized in Tables 1-4 and Figures 1 and 2. Encoding time ratios are shown in Table 3. The plots in Figures 1 and 2 also include results for 2560x1600 cropped versions of the SHV sequences Steam Locomotive and Nebuta, 150 frames each.

Table 1. Summary of Y BD-Rate results for Random access high efficiency

|  |  |  |  | Average for each resolution |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ROT alone | MDDT alone | ROT & MDDT | ROT alone | MDDT alone | ROT & MDDT |
| Traffic | -1.3 | -1.0 | -1.3 | -0.9 | -0.8 | -1.0 |
| PeopleOnStreet | -0.5 | -0.7 | -0.8 |  |  |  |
| Kimono | -0.7 | -0.2 | -0.6 | -0.9 | -0.6 | -0.8 |
| ParkScene | -0.7 | -0.8 | -0.9 |  |  |  |
| Cactus | -1.2 | -0.7 | -0.9 |  |  |  |
| BasketballDrive | -1.0 | -0.8 | -1.1 |  |  |  |
| BQTerrace: | -0.7 | -0.6 | -0.7 |  |  |  |
| BasketballDrill | -1.2 | -1.1 | -1.1 | -1.0 | -0.7 | -0.8 |
| BQMall | -1.0 | -0.6 | -0.7 |  |  |  |
| PartyScene | -1.0 | -0.7 | -0.7 |  |  |  |
| RaceHorsesC | -0.8 | -0.5 | -0.5 |  |  |  |
| BasketballPass | -0.7 | -0.8 | -0.9 | -0.8 | -0.6 | -0.7 |
| BQSquare | -0.9 | -0.5 | -0.6 |  |  |  |
| BlowingBubbles | -0.7 | -0.7 | -0.7 |  |  |  |
| RaceHorsesD | -0.8 | -0.5 | -0.6 |  |  |  |
| Average | -0.9 | -0.7 | -0.8 |  |  |  |

Table 2. Summary of Y BD-Rate results for Intra high efficiency

|  |  |  |  | Average for each resolution |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ROT alone | MDDT alone | ROT & MDDT | ROT alone | MDDT alone | ROT & MDDT |
| Traffic | -2.5 | -1.9 | -2.5 | -2.2 | -2.0 | -2.4 |
| PeopleOnStreet | -2.0 | -2.0 | -2.4 |  |  |  |
| Kimono | -1.9 | -0.5 | -1.7 | -1.9 | -1.2 | -1.8 |
| ParkScene | -1.5 | -1.5 | -1.8 |  |  |  |
| Cactus | -2.0 | -1.5 | -1.9 |  |  |  |
| BasketballDrive | -2.4 | -1.5 | -2.3 |  |  |  |
| BQTerrace: | -1.7 | -1.0 | -1.2 |  |  |  |
| BasketballDrill | -3.0 | -2.2 | -2.5 | -2.3 | -1.7 | -1.9 |
| BQMall | -2.2 | -2.0 | -2.2 |  |  |  |
| PartyScene | -1.8 | -1.6 | -1.6 |  |  |  |
| RaceHorsesC | -2.1 | -1.0 | -1.3 |  |  |  |
| BasketballPass | -2.2 | -1.7 | -2.1 | -2.1 | -1.6 | -1.8 |
| BQSquare | -2.1 | -1.6 | -1.7 |  |  |  |
| BlowingBubbles | -2.0 | -1.8 | -1.9 |  |  |  |
| RaceHorsesD | -2.2 | -1.4 | -1.6 |  |  |  |
| Vidyo1 | -2.6 | -2.1 | -2.6 | -2.4 | -2.1 | -2.6 |
| Vidyo3 | -2.2 | -2.1 | -2.6 |  |  |  |
| Vidyo4 | -2.4 | -2.1 | -2.7 |  |  |  |
| Average | -2.1 | -1.6 | -2.0 |  |  |  |

Figure 1. BD-Rate improvement of MDDT and ROT for Random access high efficiency

Figure 2. BD-rate improvement of MDDT and ROT for Intra high efficiency

Table 3. Encoding time ratios

|  | ROT+MDDT | ROT | MDDT |
| --- | --- | --- | --- |
| Intra | 181% | 241% | 110% |
| Random access | 104% | 104% | 101% |

Table 4. Decoding time ratios

|  | ROT+MDDT | ROT | MDDT |
| --- | --- | --- | --- |
| Intra | 227% | 102% | 227% |
| Random access | 304% | 101% | 302% |

# Summary and Conclusions

In this version of TMuC (0.7.1, equivalent to 0.7), using ROT alone in the all-Intra case yields an average improvement of 2.1%, MDDT alone gives a 1.6% improvement, and together the gain is 2.0%. For the random access case, gains are 0.9%, 0.7%, and 0.8% respectively. Encoding times are similar for the random access case, since these transforms operate on Intra frames and the overall encoding times are relatively long. For all-Intra coding, ROT required more than twice the encoding time as MDDT alone, and the combination required slightly less time than ROT alone since ROT only operates on block sizes larger than 8x8 when MDDT is enabled. For decoding, ROT has little impact on decoding time, whereas MDDT more than doubles the decode time for all-Intra and triples it for random access.

Although ROT alone yields better compression efficiency than MDDT in most cases for this particular experiment, the outcome may improve after the following tasks are completed:

- Resolve any issues between MDDT and Unified Intra Prediction, and address any issues related to adaptive scan for MDDT in TMuC.

- Artifacts in the decoded pictures should be examined and compared, especially for cases where the combination of ROT and MDDT produce the same BD-Rate gains as ROT or MDDT alone.

- Consider using the MDDT simplification techniques from Tool Experiment 7 and the ROT simplification methods presented at this meeting.

This contribution may also serve as an independent check of the corresponding tests in TE12.

# References

- K. McCann, “Tool Experiment 12: Evaluation of TMuC Tools”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-B312, Geneva, CH, July 2010.

- JCT-VC, “Draft Test Model under Consideration”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-B205, Geneva, CH, July 2010.

- TMuC 0.7.1 software, available at svn://hevc.kw.bbc.co.uk/svn/jctvc-tmuc

- F. Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-B300, Geneva, CH, July 2010.

- R. Cohen, C. Yeo, R. Joshi, et al., “TE7: Summary report for MDDT Simplification”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C310/m18568, Guangzhou, CN, October 2010.