| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C039 WG11 Number: m18060 |
| --- | --- |

| Title: | Choice of transforms in MDDT for Unified Intra Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chuohao Yeo Yih Han Tan Zhengguo Li Susanto Rahardja 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: +65 6408 2000 Email: chyeo@i2r.a-star.edu.sg |  |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

A simplification of Mode-Dependent Directional Transform (MDDT) was previously proposed in JCTVC-B024 that requires only two transform matrices: a DCT and a derived KLT. However, the choice of transforms in JCTVC-B024 was only specified for the 9 intra prediction modes present in H.264/AVC. In this proposal, a choice of transforms is presented for the 34 intra prediction modes possible in the unified intra prediction in TMuC. As in the earlier proposal, only 2 transform matrices are necessary, and no training is required.

# Introduction

In MDDT, separable transforms are used to approximate an ideal non-separable directional transform [1]. Specifically, if X is an N x N block of pixels, then its 2D transform coefficients are given by:

Y=CmXRmT

The subscript m in Cm and Rm denotes the dependence of the column and row transforms on the prediction mode. Note that in H.264, Cm = Rm = M, where M is an integer Discrete Cosine Transform (DCT) transform matrix. In the Mode-Dependent Directional Transform (MDDT) scheme, Karhunen-Loève Transforms (KLT) are used for Cm and Rm and are computed by performing SVD on residual blocks of prediction mode m collected from training video sequences.

In an earlier proposal, we derived using an image correlation model what the KLT should be for various intra prediction mode [2]. In particular, for vertical intra prediction, we found that the row transform should just be the DCT, and the KLT for the vertical transform is found to be:

Ki,j=22N+1sin(2i−1)jπ2N+1

We also proposed a set of choices of transforms for each intra prediction mode in H.264/AVC. Table 1 summarizes these choices.

Table 1. Choice of KLT/DCT for each intra prediction mode in H.264/AVC

| Mode | Column Transform | Row Transform |
| --- | --- | --- |
| 0 | KLT | DCT |
| 1 | DCT | KLT |
| 2 | DCT | DCT |
| 3 | KLT | DCT |
| 4 | KLT | KLT |
| 5 | KLT | KLT |
| 6 | KLT | KLT |
| 7 | KLT | DCT |
| 8 | DCT | KLT |

# Choice of transforms for Unified Intra Prediction

The Unified Intra Prediction scheme was adopted into TMuC [3]. This proposal replaces the previous combination of Angular Intra prediction and Arbitrary Directional Intra. The new intra prediction modes, compared to that in H.264/AVC, allow for directional prediction at finer granularity.

Our analysis indicates the following choice of transforms (Table 2):

Table 2. Choice of KLT/DCT for each intra prediction mode in Unified Intra Prediction (TMuC)

| “Logical” Mode | Mode Mnemonics | Column Transform | Row Transform |
| --- | --- | --- | --- |
| 0 | Vert | KLT | DCT |
| 1 | Horz | DCT | KLT |
| 2 | DC | DCT | DCT |
| 3 | Vert-8 | KLT | KLT |
| 4 | Vert-4 | KLT | KLT |
| 5 | Vert+4 | KLT | DCT |
| 6 | Vert+8 | KLT | DCT |
| 7 | Horz-4 | KLT | KLT |
| 8 | Horz+4 | DCT | KLT |
| 9 | Horz+8 | DCT | KLT |
| 10 | Vert-6 | KLT | KLT |
| 11 | Vert-2 | KLT | KLT |
| 12 | Vert+2 | KLT | DCT |
| 13 | Vert+6 | KLT | DCT |
| 14 | Horz-6 | KLT | KLT |
| 15 | Horz-2 | KLT | KLT |
| 16 | Horz+2 | DCT | KLT |
| 17 | Horz+6 | DCT | KLT |
| 18 | Vert-7 | KLT | KLT |
| 19 | Vert-5 | KLT | KLT |
| 20 | Vert-3 | KLT | KLT |
| 21 | Vert-1 | KLT | KLT |
| 22 | Vert+1 | KLT | DCT |
| 23 | Vert+3 | KLT | DCT |
| 24 | Vert+5 | KLT | DCT |
| 25 | Vert+7 | KLT | DCT |
| 26 | Horz-7 | KLT | KLT |
| 27 | Horz-5 | KLT | KLT |
| 28 | Horz-3 | KLT | KLT |
| 29 | Horz-1 | KLT | KLT |
| 30 | Horz+1 | DCT | KLT |
| 31 | Horz+3 | DCT | KLT |
| 32 | Horz+5 | DCT | KLT |
| 33 | Horz+7 | DCT | KLT |

# Implementation and Simulation Results

We have implemented our proposed approach on the TMuC v0.7 software, where as specified in TMuC [4], the 4x4 and 8x8 block transforms for intra prediction residual uses the proposed combinations of DCT and derived KLT transforms shown in Table 2. Note that in the proposed approach, only two transform matrices need to be stored for each block size (one of which is the DCT), instead of 18 transform matrices in MDDT.

We use the default software configuration as defined in the test conditions, JCTVC-B300 [5], and tested using “Intra, high efficiency” and “Random access, high efficiency” conditions.

We compare the TMuC anchor, our proposed method using the TMuC MDDT map [2], and the proposed choice of transforms. Detailed results are presented in our TE7 report (JCTVC-C037). Here, we provide the summary of BD-Rate [6] results. Table 3 shows the RD results for “Intra, high efficiency”. With the current TMuC MDDT map, our proposed set of transforms lead to a negligible rate increase. With the proposed choices of transforms in Table 2, the RD performance is virtually the same as TMuC 0.7 reference, even though only 2 transforms are used.

Table 3. RD results for all intra configuration

| Sequence | [2] vs TMuC | Proposed vs TMuC |
| --- | --- | --- |
|  | BD-Rate (%) | BD-Rate (%) |
| Average for Class A | 0.1 | 0.0 |
| Average for Class B | 0.1 | 0.1 |
| Average for Class C | 0.1 | 0.0 |
| Average for Class D | 0.1 | 0.0 |
| Average for Class E | 0.1 | 0.0 |

Table 3 shows the RD results for “Random access, high efficiency”. Again, with the current TMuC MDDT map, our proposed set of transforms has very negligible rate increase. With the proposed choices of transforms in Table 2, the RD performance is again virtually the same as TMuC 0.7 reference.

Table 4. RD results for hierarchical B configuration

| Sequence | [2] vs TMuC | Proposed vs TMuC |
| --- | --- | --- |
|  | BD-Rate (%) | BD-Rate (%) |
| Average for Class A | 0.1 | 0.0 |
| Average for Class B | 0.1 | 0.1 |
| Average for Class C | 0.0 | 0.0 |
| Average for Class D | 0.1 | 0.0 |

# Conclusion

This proposal presents an update of our previous proposal [2] to better suit the Unified Intra Prediction used in TMuC. As in our earlier proposal, only two transforms: DCT and a derived KLT. Experimental results show that there is no significant loss in coding efficiency. We recommend considering this proposal for adoption in the TM/TMuC.

# References

[1] Y. Ye and M. Karczewicz, “Improved Intra Coding,” ITU-T Q.6/SG11 VCEG, VCEG-AG11, Shenzhen, China, Oct 2007.

[2] C. Yeo, Y. H. Tan, Z. Li and S. Rahardja, “Mode-Dependent Fast Separable KLT for Block-based Intra Coding,” JCTVC-B024, Geneva, Switzerland, Jul 2010.

[3] J.-H. Min, S. Lee, I.-K. Kim, W.-J. Han, J. Lainema, K. Ugur, “Unification of the Directional Intra Prediction Methods in TMuC,” JCTVC-B100, Geneva, Switzerland, Jul 2010.

[4] JCT-VC, “Test Model under Consideration (TMuC),” JCTVC-B205, Geneva, Switzerland, Jul 2010.

[5] Frank Bossen, “Common test conditions and software reference configurations,” JCTVC-B300, Geneva, Switzerland, Jul 2010.

[6] Gisle Bjontegaard, “Calculation of Average PSNR Differences between RD curves,” ITU-T SC16/Q6, VCEG-M33, Austin, USA, Apr 2001.

# Patent rights declaration(s)

Institute for Infocomm Research may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).