| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C041 WG-11 Number:m18062 |
| --- | --- |

| Title: | TE12: Performance of Partition Based Illumination Compensation (PBIC) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal / Information |  |  |
| Author(s) or Contact(s): | Akira Fujibayashi 1 Sandeep Kanumuri 2 Frank Bossen 2 TK Tan 1 1, 3-6,Hikarinooka,Yokosuka,Kanagawa, 239-8536, Japan 2, 3240 Hillview Ave Palo Alto CA 94304 USA | Tel: Email: | +81-468-40-6833 fujibayashi@nttdocomo.co.jp tktan.p@rd.nttdocomo.co.jp +1-650-496-4723 skanumuri@docomolabs-usa.com +1-650-496-4742 bossen@docomolabs-usa.com |
| Source: | NTT DOCOMO, Inc. & DOCOMO Communications Laboratories USA, Inc. |  |  |

_____________________________

# Abstract

This is a TE12 contribution presenting the performance of Partition Based Illumination Compensation (PBIC) tool integrated into the TMuC software. In addition to the test conditions of TE12, the contribution compares PBIC to two other tools, namely SIFO and GRefMode, present in the TMuC software that can handle illumination changes. This comparison is also done when a linear fade is applied to the test sequences.

The results show that PBIC has no impact on coding efficiency for the regular test sequences and a large gain (11-20% depending on the config) for the test sequences with linear fade. Furthermore, PBIC has a minimal impact (2-5% increase) on encoder/decoder run times. The comparison indicates that PBIC is the top performing tool in terms of coding efficiency and has the least increase in encoder run time.

# Introduction

This contribution reports the performance of PBIC as part of Tool Experiment 12 described in JCTVC-B312r6[1]. This contribution also includes a performance comparison of PBIC with two other tools, SIFO and GRefMode. In addition to the regular test sequence set, the performance comparison is done for test sequences where a linear fade is applied, to illustrate the potential of these tools for situations where illumination level varies from frame to frame. The rest of the contribution is organized as follows: Section 2 gives a brief description of PBIC and Section 3 reports the results for TE12 test conditions. The comparison of PBIC with SIFO and GRefMode is done in Section 4 for regular test sequences and Section 6 for sequences with linear fade. Section 7 has concluding remarks.

# Partition Based Illumination Compensation (PBIC)

Illumination-compensation parameters are signaled at each partition using scale and offset parameters.

For single-list prediction, one transformed scale t0 and one offset o are transmitted. Scale s is derived from t0 as s = (64+t0). For bi-prediction, two transformed scales t0 and t1, and one offset o are transmitted. Scales s0 and s1 are derived from t0 and t1 as s0 = (64 + t0)/2+t1 and s1 = (64+t0) – s0.

The scales are applied to all components, whereas the offsets are applied only to the luma component. In addition, for chroma components, the value ‘128’ (chroma value corresponding to a grey pixel without color) is removed before the application of scale and this value ‘128’ is added after the application of scale.

In the case of luma component, illumination compensation is applied to motion-compensated samples as follows:

- Single-list Prediction

  - w = (s*v)/64+o

  - w is illumination-compensated sample

  - v is motion-compensated sample

- Bi-prediction

  - w = (w0+w1)/2

  - w0 = (2*s0*v0/64)+o

  - w1 = (2*s1*v1/64)+o

  - w is illumination-compensated sample

  - w0 and w1 are illumination-compensated samples for each list

  - v0 and v1 are motion-compensated samples for each list

Note that the implementation is done such that w0 and w1 are obtained first to be inline with the TMuC implementation of motion compensation for bi-prediction. The samples w0 and w1 are later averaged to obtain w.

In the case of chroma components, illumination compensation is applied to motion-compensated samples as follows (the terms v, v0, v1, w, w0, w1 have the same meaning as above, but represent chroma samples instead of luma samples):

- Single-list Prediction

  - w = (s*(v-128))/64+128

- Bi-prediction

  - w = (w0+w1)/2

  - w0 = (2*s0*(v0-128)/64)+128

  - w1 = (2*s1*(v1-128)/64)+128

Note that the above equations (for both luma and chroma) assume that the motion-compensated values are represented using an unsigned 8-bit notation. When IBDI is used, appropriate scaling modifications are used to do the operations at higher bit-depth.

The applicable PBIC parameters ({t0, o} or {t0, t1, o}) are coded differentially in combination with motion vector differences. A list of predictors is constructed using a default predictor (all parameters being 0) and predictors using PBIC parameters of neighbouring PUs (Left, Above and Corner). A predictor index is signaled to indicate the predictor to be used. A zero-tree is used to indicate which of the PBIC parameter differences and motion vector differences are zero. For single-list prediction, the zero-tree deals with 4 parameters (mvdx, mvdy, t0 and o). For bi-prediction, the zero-tree deals with 7 parameters (mvd0x, mvd0y, mvd1x, mvd1y, t0, t1 and o). For more details on zero-trees, please refer to section 2.6.1 of JCTVC-A114[2]. The non-zero PBIC parameter differences are coded similar to non-zero motion vector differences.

In the de-blocking filter, edges between blocks with different PBIC parameters are treated similar to the edges between blocks with MV differences.

In the encoder, after motion estimation is done for a PU, a maximum of 6 candidates are tested for selecting PBIC parameters. Two candidates are derived by minimizing the MSE of the residual with and without the constraint that the offset o is set to 0. The other candidates are the predictors in the predictor list. For each candidate, the corresponding PBIC parameters are used to apply illumination compensation on the motion-compensated samples (luma and chroma) and an RD cost is computed. The candidate with the least RD cost is selected.

# Experimental results

The experiments were conducted in accordance with the TE12 default and alternative settings for PBIC specified in the common test conditions, JCTVC-B300 [3].

Figures 1 to 4 indicate the change in coding efficiency (positive values indicate reduction in BD-rate) when PBIC is enabled on top of the inter reference configurations. For high efficiency configurations, PBIC has a tiny average gain (0.1% for Random Access (RA) and 0.2% for Low Delay (LD)) compared to the anchors. For low complexity configurations, PBIC performs worse than the anchors (average loss of 1.6% for RA and 1% for LD). However, it should be noted that this loss is due to the entropy coder (LCEC). To show this, the entropy coder is switched from LCEC to PIPE (used in high efficiency configs) for both the default and alternate settings and an experiment is done using the first 49 frames of classes B/C/D/E. This experiment shows that (Figures 5 and 6) PBIC has a negligible loss for RA and a tiny gain (0.3%) for LD.

Figure 1 Performance of PBIC (RA /High efficiency /10sec)

Figure 2 Performance of PBIC (LD /High efficiency /10sec)

Figure 3 Performance of PBIC (RA /Low complexity /10sec)

Figure 4 Performance of PBIC (LD /Low complexity /10sec)

Figure 5 Performance of PBIC (RA /Low complexity /PIPE /49frames)

Figure 6 Performance of PBIC (LD /Low complexity /PIPE /49frames)

To get a rough idea of PBIC complexity, we measured the user times reported by the linux tool “/usr/bin/time” for both the encoder and decoder. These measurements are done using a 64-bit linux cluster. Table 1 lists the increase in encoding and decoding time computed using the TE12 excel template. This result indicates that PBIC has minimal impact on encoder/decoder complexity.

Table 1 Increase (%) in encoding and decoding times

| Proponent | high efficiency |  | low complexity |  |
| --- | --- | --- | --- | --- |
| Condition | RA | LD | RA | LD |
| Encoding Time | 2 | 2 | 5 | 3 |
| Decoding Time | 5 | 2 | 4 | 5 |

# Comparison to similar tools in TMuC Software

In addition to the test conditions of TE12, a comparison is conducted between tools designed to handle illumination changes. Apart from PBIC, SIFO and GRefMode are two tools present in the TMuC software that fall into this category. In this section, for high efficiency configurations, we use DCT-IF (InterpFilterType=0) instead of SIFO (InterpFilterType=4) for generating the reference. The reference settings for low complexity configurations are not changed.

SIFO (Switched Interpolation Filters with Offsets) can handle illumination changes by using different offsets. In the following experiments involving SIFO, we enabled the bug fixes (BUGFIX48, BUGFIX50, BUGFIX67 and BUGFIX92) related to SIFO. These fixes include the fix to speedup SIFO.

“GRefMode” is a tool that enables the use of “virtual” reference frames to address illumination changes. The “GRefMode=w” option calculates the scale and offset for each reference frame in each list and a virtual reference frame is added for that reference frame when a non-trivial scale and offset are found. Though this tool is not part of the TMuC design as we understand it, this tool is highly similar to Weighted Prediction in H.264/AVC and can also handle partition-level illumination changes by having the ability to switch between “real” and “virtual” reference frames.

For the performance comparison, in addition to the regular test sequences, sequences with linear fade were also used to test the scenarios for which these tools are designed. In the past, for experiments involving weighted prediction in H.264/AVC, linear and sigmoid fades varying in length from 1 to 3 seconds were applied to the regular test sequences [5] [6] [7]. In the same spirit, a linear fade is applied to the first 2 seconds of the regular sequence set and the experiments are done with the 2-second sequences. A linear fade-out (Sequence to Black) is applied in the 1st half and a linear fade-in (Black to Sequence) in the 2nd half. The fade strength is limited to the range [0.25, 0.75] instead of [0, 1] to avoid extreme PSNR numbers at the frame level that cause abnormal rate-distortion graphs.

The BD-rate gain of each tool for the regular sequence set is summarized in Table 2. Table 3 indicates the increase in encoding and decoding times. With regard to coding efficiency, none of the tools have a significant impact for regular sequence set. While PBIC has a tiny gain for both RA and LD cases, GRefMode has a small loss for both RA and LD cases. SIFO has a tiny gain on LD and a tiny loss for RA. From a complexity viewpoint, the encoding time has increased significantly for SIFO (27-33%) and GRefMode (6-17%) while PBIC has a very modest increase (2-3%).

Table 2 BD-rate gain (%) of each tool for regular sequence set (high efficiency config)

|  | high efficiency |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tool | PBIC |  | SIFO |  | GRefMode=w |  |
| Condition | RA | LD | RA | LD | RA | LD |
| Ave [%] | 0.1 | 0.2 | -0.1 | 0.2 | -0.4 | -0.1 |
| Min [%] | -0.4 | -0.3 | -0.7 | -0.1 | -1.1 | -0.5 |
| Max [%] | 1.5 | 1.6 | 0.7 | 0.9 | 0 | 0.1 |

Table 3 Increase (%) in encoding and decoding times

|  | high efficiency |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tool | PBIC |  | SIFO |  | GRefMode=w |  |
| Condition | RA | LD | RA | LD | RA | LD |
| Encoding Time | 3 | 2 | 33 | 27 | 17 | 6 |
| Decoding Time | 0 | 2 | 1 | 1 | 0 | 1 |

Figures 7 to 8 indicate the change in coding efficiency obtained by PBIC (Red), SIFO (Yellow) and GRefMode=w (Blue).

Figure 7 Comparison of performance for regular sequences (RA /High efficiency /10sec)

Figure 8 Comparison of performance for regular sequences (LD /High efficiency /10sec)

# Comparison to similar tools on sequences with linear fade

Finally the performance comparison on 2-second sequences with linear fade is discussed here. The BD-rate gain of each tool is summarized in Table 4 and 5. These results indicate that PBIC brings much larger gain than that of SIFO for both RA and LD cases. Though GRefMode brings better gain than PBIC for RA case, PBIC has a much larger gain than GRefMode for LD case. If the average of RA and LD gain is considered, PBIC has a better average than GRefMode.

Tables 6 and 7 indicate the increase in encoding and decoding times. The results indicate that the encoding time has increased significantly for SIFO (26-134%) and GRefMode (16-45%) while PBIC does not cause an increase in encoder time. For some cases, PBIC has a decrease in encoder/decoder times and the reason for this behavior is not yet identified.

Table 4 BD-rate gain (%) of each tool for sequences with linear fade

(high efficiency config)

|  | high efficiency |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tool | PBIC |  | SIFO |  | GRefMode=w |  |
| Condition | RA | LD | RA | LD | RA | LD |
| Ave [%] | 10.9 | 13.7 | 4.7 | 5.1 | 14.2 | 5.8 |
| Min [%] | 0.9 | 5.2 | -1.5 | 1.4 | 7.5 | 1.1 |
| Max [%] | 32.4 | 30.0 | 11.4 | 13.6 | 28.4 | 17.7 |
| Ave of RA & LD[%] | 12.3 |  | 4.9 |  | 9.9 |  |

Table 5 BD-rate gain (%) of each tool for sequences with linear fade

(low complexity config)

|  | Low complexity |  |  |  |
| --- | --- | --- | --- | --- |
| Tool | PBIC |  | GRefMode=w |  |
| Condition | RA | LD | RA | LD |
| Ave [%] | 11.3 | 19.6 | 15.2 | 15.9 |
| Min [%] | 0.4 | 6.7 | 8.3 | 6.4 |
| Max [%] | 33.5 | 38.6 | 32.3 | 33.8 |
| Ave of RA & LD[%] | 15.6 |  | 15.6 |  |

Table 6 Increase (%) in encoding and decoding times (high efficiency config)

|  | high efficiency |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tool | PBIC |  | SIFO |  | GRefMode=w |  |
| Condition | RA | LD | RA | LD | RA | LD |
| Encoding Time | -1 | -8 | 134 | 26 | 45 | 16 |
| Decoding Time | 1 | -7 | 3 | -1 | 2 | -2 |

Table 7 Increase (%) in encoding and decoding times (low complexity config)

|  | low complexity |  |  |  |
| --- | --- | --- | --- | --- |
| Tool | PBIC |  | GRefMode=w |  |
| Condition | RA | LD | RA | LD |
| Encoding Time | 1 | -6 | 44 | 17 |
| Decoding Time | 7 | 4 | 3 | 1 |

Figures 9 to 12 indicate the change in coding efficiency obtained by PBIC (Red), SIFO (Yellow only for high efficiency setting) and GRefMode=w (Blue) for sequences with linear fade.

Figure 9 Comparison of performance for sequences with linear fade (RA /High efficiency/2sec)

Figure 10 Comparison of performance for sequences with linear fade (LD /High efficiency/2sec)

Figure 11 Comparison between PBIC and GRefMode for sequences with linear fade (RA /low complexity/2sec)

Figure 12 Comparison between PBIC and GRefMode for sequences with linear fade (LD /low complexity/2sec)

# Conclusion

In this contribution, TE12 experimental results for PBIC are presented. In addition, comparison to other tools, SIFO and GRefMode, that can handle illumination changes were shown. While PBIC has no impact for regular sequence set, large gains (11-20%) can be obtained for sequences with linear fade with minimal impact on encoder/decoder run times. The comparison to SIFO and GRefMode indicates that PBIC is the top performing tool in terms of coding efficiency and incurs a negligible overhead in complexity while the other tools have significant overhead in encoder complexity. Therefore, we recommend that PBIC be enabled in default settings of TMuC/TM software.

# Reference

[1] Ken McCann, “Tool Experiment 12: Evaluation of TMuC Tools,” JCT-VC Document, JCTVC-B312_r6, Geneva, July 2010.

[2] I. Amonou et.al., “Description of video coding technology proposal by France Telecom, NTT, NTT DOCOMO, Panasonic and Technicolor,” JCT-VC Document, JCTVC-A114, Dresden, April 2010.

[3]	Frank Bossen, “Common test conditions and software reference configurations,”　JCT-VC Document, JCTVC-B300, Geneva, July 2010.

[4]	S. Pateux, “Tools for proposal evaluations,” JCT-VC Document, JCTVC-A031, Dresden, April 2010.

[5] Jill Boyce, “Changes to Adaptive Reference Picture Weighting,” JVT Document, JVT-E060, Geneva,　October 2002.

[6] Jill Boyce, “Weighted Prediction Clean-up,” JVT Document, JVT-F034, Awaji, December, 2002.

[7]	Yoshihiro Kikuchi and Takeshi Chujoh,” Simplification of the Weighted Prediction and Verification result,” JVT Document, JVT-F077, Awaji, December, 2002	.

# Patent rights declaration(s)

NTT DOCOMO, Inc. and DOCOMO Communications Laboratories USA, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).