| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C056 WG11 Number: m18079 |
| --- | --- |

| Title: | TE 12: Evaluation of Transform Unit (TU) Size |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA Vivienne Sze Texas Instruments Inc., USA | Tel: Email: Tel: Email: | +1-214-480-3816 zhou@ti.com +1 214-567-4005 sze@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This document reports the evaluation results of the Transform Unit (TU) size for TE12, and provides a brief complexity analysis of transforms. Experimental results show that the average coding loss caused by disabling the 64x64 transform is around 0.1% to 0.6%; disabling both 32x32 and 64x64 transforms lead to an average coding loss of 1.1% to 4%. The results also show that disabling 4x4 transform leads to huge coding loss. When both the 4x4 and 64x64 transforms are disabled, the average coding loss is increase to 3.9% to 8.3%, and disabling 4x4, 32x32 and 64x64 transforms (i.e. only 8x8 and 16x16 transforms are enabled) can lead to quality degradation of 5.2% to 10.6% on average. Due to high complexity and limited coding efficiency gain of the 64x64 transform, it is recommended that the 64x64 transform be removed from reference configurations. It also recommended that the quality issues of disabling the 4x4 transform be investigated in the future.

# Introduction

In the TMuC software [2], 4x4, 8x8, 16x16, 32x32 and 64x64 transforms are supported. Large block size transforms (16x16, 32x32 and 64x64 transforms) improve the coding efficiency, but also increase the implementation complexity significantly. The trade-off between the coding efficiency and complexity needs to be studied for the large block size transforms. In addition, small block size transforms (e.g. 4x4 transform) may be disabled in the low-cost video encoder implantations to reduce the cost, the quality impact of disabling 4x4 transform is also investigated in this report.

# Transform complexity analysis

A bit-width estimate is performed for the HEVC transforms by using the TMuC software. For the forward transforms, the results are obtained by setting the sample values of input data blocks to 255 (or 255<<4 for IBDI on case). For the inverse transforms, the worst possible input data range for the inverse transforms is acquired by performing transform, quantization and inverse quantization with the worst possible transform input and quantization scale QP = 0, then the  intermediate inverse transform transpose buffer bit-width is obtained by setting all the values of the inverse transform input data to the worst possible value. The estimate results are provided in Table 1. While the bit-width estimate for the forward transforms are accurate, the estimate for the backward transforms should be viewed as rough, and be treated as the low-bound estimate. The actual inverse transform data bit-width could be higher for the worst cases.

| TU Size | Forward transform transpose buffer bit-width (after horizontal OR vertical forward transform ) |  | Forward transform output bit-width (after horizontal AND vertical forward transform) |  | Inverse transform input bit-width (after transform, quantization, inverse quantization) |  | Inverse transform transpose buffer bit-width (after horizontal OR vertical inverse transform ) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | IBDI on | IBDI off | IBDI on | IBDI off | IBDI on | IBDI off | IBDI on | IBDI off |
| 64x64 | 19 | 15 | 24 | 20 | 23 | 19 | 23 | 19 |
| 32x32 | 18 | 14 | 22 | 18 | 24 | 20 | 24 | 20 |
| 16x16 | 17 | 13 | 20 | 16 | 24 | 20 | 24 | 20 |
| 8x8 | 16 | 12 | 19 | 15 | 19 | 15 | 20 | 16 |
| 4x4 | 15 | 11 | 17 | 13 | 19 | 15 | 21 | 17 |

Table 1  Bit-width estimate of forward and inverse transform

A 2-D transform involves two stages, i.e. horizontal and vertical transform. The intermediate data (i.e. after the first stage horizontal or vertical transform) needs to be buffered and transposed as the input for the second stage of transform. This transpose data buffer has significant impact on the design area since it is normally implemented in register files to meet high-throughput requirements (e.g. 1080p@30, 4kx2k@30).  Figure 1 illustrates such register file architecture, which enables to write the intermediate data of the first stage transform horizontally into the transpose data buffer and read-out the data vertically for the second stage transform at required high throughputs.

Figure 1 Register file architecture for transform transpose data buffer

The 64x64 transform has very high implementation complexity. Table 2 lists ballpark gate count estimate of the transpose buffer for forward and backward transforms by using 45nm process technology.  As shown in Table 2, the gate counts of the 64x64 transpose buffer alone (not counting the logic gates) range from 600K to 900K gates, depending on forward or inverse transform, and IBDI on/off, while for a standard H.264/AVC encoder the gate count ranges from 500K to 1M gates based on the information from published papers (see Table 3).

| Transform Size | Entires | Bitwidth | Memory size (kB) | Gate count RTL/ Synthesis Generic reg file (kgates) |
| --- | --- | --- | --- | --- |
| FORWARD TRANSFORM |  |  |  |  |
| IBDI off |  |  |  |  |
| 64 | 4096 | 15 | 7.50 | 632.65 |
| 32 | 1024 | 14 | 1.75 | 148.40 |
| 16 | 256 | 13 | 0.41 | 34.15 |
| 8 | 64 | 12 | 0.09 | 8.27 |
| 4 | 16 | 11 | 0.02 | 1.94 |
| IBDI on |  |  |  |  |
| 64 | 4096 | 19 | 9.50 | 799.52 |
| 32 | 1024 | 18 | 2.25 | 192.71 |
| 16 | 256 | 17 | 0.53 | 45.41 |
| 8 | 64 | 16 | 0.13 | 10.80 |
| 4 | 16 | 15 | 0.03 | 2.61 |
| INVERSE TRANSFORM |  |  |  |  |
| IBDI off |  |  |  |  |
| 64 | 4096 | 19 | 9.50 | 799.52 |
| 32 | 1024 | 20 | 2.50 | 207.77 |
| 16 | 256 | 20 | 0.63 | 52.55 |
| 8 | 64 | 16 | 0.13 | 10.80 |
| 4 | 16 | 17 | 0.03 | 2.83 |
| IBDI on |  |  |  |  |
| 64 | 4096 | 23 | 11.50 | 947.38 |
| 32 | 1024 | 24 | 3.00 | 247.37 |
| 16 | 256 | 20 | 0.63 | 61.68 |
| 8 | 64 | 20 | 0.16 | 13.17 |
| 4 | 16 | 21 | 0.04 | 3.39 |

Table 2  Number of gate count estimate for the transpose buffer of forward and backward transforms

| Typical Gate Sizes |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Encoder | Profile | Publication | logic kgates | memory (kB) | resolution |
| H.264/AVC | High, L4 | ISSCC 2008 | 593 | 22.00 | 1080p |
| H.264/AVC | Baseline | ISSCC 2007 | 470 | 13.30 | 720p |
| H.264/AVC | Baseline | ISSCC 2005 | 923 | 34.72 | 720p |
| MVC | High, L5.1 | JSSC 2010 | 1732 | 20.10 | 4kx2k |
| Decoder |  |  |  |  |  |
| Standard |  | Publication | logic kgates | memory (kB) | resolution |
| MPEG-2, H.264/AVC | SP, Baseline | JSSC 2007 | 304 | 22.75 | 1080p |
| H.264/AVC | Baseline | JSSC 2009 | 315 | 17.00 | 720p |
| H.264/AVC | Main | JSSC 2007 | 160 | 4.50 | 1080p |
| MPEG-1/2/3, H.264/AVC, JPEG | SP, Baseline, Baseline | ISSCC 2007 | 252 | 4.90 | 1080p |
| MPEG-1/2, AVS, H.264/AVC | MP, JP, High | VLSI 2009 | 367 | 11.00 | 1080p |
| MVC, SVC, H.264 | High | ISSCC 2010 | 414 | 9.00 | 4kx2k |

Table 3  Typical encoder/decoder gate counts from publications

It is worth to mention here that the 64x64 transform is even more changeling for the embedded programmable implementations, due to the fact of large code size, large than 16-bit multiplications and large on-chip memory requirements. For example, the transpose buffer alone needs 16K bytes, which can already cause problems for the platforms with small cache size.

# Test Conditions

The TMuC0.7.1 and TMuC0.7.3 software are used for the evaluation of HE configurations and LOCO configurations, respectively. Using TMuC0.7.3 for LOCO configurations solves the quality issues when the 4x4 transform is disabled. The simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines, and the common test conditions and reference configurations specified in [2] are followed except for the QuadtreeTULog2MaxSize and QuadtreeTULog2MinSize that are configured differently for testing purpose. The following four settings are tested.

| TU size under test | Settings |
| --- | --- |
| Disabled 64x64 transform | QuadtreeTULog2MaxSize = 5 QuadtreeTULog2MinSize = 2 |
| Disabled 32x32 and 64x64 transforms | QuadtreeTULog2MaxSize = 4 QuadtreeTULog2MinSize = 2 |
| Disabled 4x4 and 64x64 transforms | QuadtreeTULog2MaxSize = 5 QuadtreeTULog2MinSize = 3 |
| Disabled 4x4, 32x32 and 64x64 transforms | QuadtreeTULog2MaxSize = 4 QuadtreeTULog2MinSize = 3 |

Table 4  Test settings for TU size evaluation

The test settings are designed to evaluate performance of the following cases:

- Coding loss caused by disabling the 64x64 transform

- Coding loss caused by disabling both the 32x32 and 64x64 transforms

- Coding loss caused by disabling the 4x4 and 64x64 transforms

- Coding loss caused by disabling the 4x4, 32x32 and 64x64 transforms

# Testing Results

## Coding loss when 64x64 transform is disabled

As shown in the table below, the average coding loss by disabling the 64x64 transform ranges 0.1% to 0.6%. The detailed results can be found in

TE12-TUSize-TI-maxTU32x32-minTU4x4.xls

|  | Intra |  |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR |  | V BDR | Y BDR | U BDR | V BDR |
| Class A | 0.1 | 0.3 |  | 0.3 | 0.0 | 0.1 | 0.1 |
| Class B | 0.4 | 1.0 |  | 1.0 | 0.1 | 0.2 | 0.3 |
| Class C | 0.1 | 0.2 |  | 0.1 | 0.0 | 0.0 | 0.0 |
| Class D | 0.1 | 0.1 |  | 0.1 | 0.0 | 0.0 | 0.0 |
| Class E | 0.3 | 1.8 |  | 1.3 | 0.4 | 0.8 | 0.7 |
| All | 0.2 | 0.7 |  | 0.6 | 0.1 | 0.2 | 0.2 |
| Enc Time[%] | 101% |  |  |  | 96% |  |  |
| Dec Time[%] | 102% |  |  |  | 93% |  |  |
|  | Random access |  |  |  | Random access LoCo |  |  |
|  | Y BDR | U BDR |  | V BDR | Y BDR | U BDR | V BDR |
| Class A | 0.2 | 0.4 |  | 0.5 | 0.1 | 0.2 | 0.2 |
| Class B | 0.6 | 1.3 |  | 1.3 | 0.4 | 1.0 | 1.6 |
| Class C | 0.3 | 0.3 |  | 0.4 | 0.2 | 0.2 | 0.1 |
| Class D | 0.1 | 0.2 |  | 0.2 | 0.0 | 0.4 | 0.1 |
| Class E |  |  |  |  |  |  |  |
| All | 0.3 | 0.6 |  | 0.7 | 0.2 | 0.5 | 0.6 |
| Enc Time[%] | 106% |  |  |  | 98% |  |  |
| Dec Time[%] | 109% |  |  |  | 98% |  |  |
|  | Low delay |  |  |  | Low delay LoCo |  |  |
|  | Y BDR | U BDR | V BDR |  | Y BDR | U BDR | V BDR |
| Class A |  |  |  |  |  |  |  |
| Class B | 0.5 | 3.9 | 4.8 |  | 0.8 | 2.0 | 3.1 |
| Class C | 0.2 | 1.2 | 1.3 |  | 0.3 | 0.4 | 0.4 |
| Class D | 0.1 | 0.8 | 0.9 |  | 0.1 | 0.7 | 0.4 |
| Class E | 1.7 | 9.6 | 7.3 |  | 1.3 | 4.3 | 3.0 |
| All | 0.5 | 3.5 | 3.4 |  | 0.6 | 1.7 | 1.7 |
| Enc Time[%] | 104% |  |  |  | 98% |  |  |
| Dec Time[%] | 102% |  |  |  | 94% |  |  |

Table 5  Average coding loss when 64x64 transform is disabled

## Coding loss when both 32x32 and 64x64 transforms are disabled

As shown in Table 6, the average coding loss by disabling both the 32x32 and 64x64 transforms is 1.1% to 4%, that is, disabling 32x32 transform alone leads to 1% to 3.4% coding loss. The detailed results can be found in

TE12-TUSize-TI-maxTU16x16-minTU4x4.xls

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.6 | 1.4 | 1.6 | 0.6 | 1.0 | 1.5 |
| Class B | 2.1 | 3.9 | 4.3 | 3.1 | 3.2 | 3.8 |
| Class C | 0.4 | 1.0 | 0.8 | 0.4 | 0.5 | 0.6 |
| Class D | 0.4 | 0.8 | 0.7 | 0.2 | 0.4 | 0.4 |
| Class E | 1.8 | 8.0 | 6.8 | 3.1 | 7.2 | 6.3 |
| All | 1.1 | 3.0 | 2.8 | 1.5 | 2.4 | 2.5 |
| Enc Time[%] | 94% |  |  | 94% |  |  |
| Dec Time[%] | 102% |  |  | 96% |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 1.1 | 2.9 | 3.3 | 0.7 | 1.3 | 1.6 |
| Class B | 3.3 | 6.9 | 6.9 | 4.2 | 5.2 | 7.2 |
| Class C | 1.5 | 2.8 | 2.6 | 1.5 | 1.3 | 1.3 |
| Class D | 0.7 | 2.0 | 1.7 | 0.4 | 1.8 | 0.9 |
| Class E |  |  |  |  |  |  |
| All | 1.8 | 4.0 | 3.9 | 2.0 | 2.7 | 3.2 |
| Enc Time[%] | 105% |  |  | 98% |  |  |
| Dec Time[%] | 111% |  |  | 104% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 2.9 | 15.8 | 20.5 | 6.4 | 13.7 | 19.5 |
| Class C | 1.4 | 6.7 | 7.8 | 2.4 | 3.8 | 4.2 |
| Class D | 0.5 | 6.2 | 6.5 | 1.0 | 4.8 | 3.3 |
| Class E | 8.0 | 35.8 | 26.7 | 6.4 | 23.2 | 17.0 |
| All | 2.9 | 14.9 | 15.0 | 4.0 | 10.8 | 11.2 |
| Enc Time[%] | 103% |  |  | 99% |  |  |
| Dec Time[%] | 106% |  |  | 102% |  |  |

Table 6  Average coding loss when 32x32 and 64x64 transforms are disabled

## Coding loss when both 4x4 and 64x64 transforms are disabled

As shown in the table below, the average coding loss by disabling both the 4x4 and 64x64 transforms is 3.9% to 8.3%, that is, disabling 4x4 transform alone leads to about 3.8% to 7.7% coding loss. The detailed results can be found in

TE12-TUSize-TI-maxTU32x32-minTU8x8.xls

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | 3.5 | 5.4 | 4.6 | 0.6 | 3.4 | 1.8 |
| Class B | 3.6 | 5.8 | 5.7 | 1.2 | 4.8 | 4.0 |
| Class C | 8.8 | 9.2 | 9.5 | 6.5 | 10.4 | 9.5 |
| Class D | 10.5 | 12.3 | 13.6 | 8.6 | 12.3 | 13.5 |
| Class E | 4.4 | 3.7 | 4.0 | 1.0 | -7.4 | -3.8 |
| All | 6.4 | 7.6 | 7.9 | 3.9 | 5.5 | 5.8 |
| Enc Time[%] | 48% |  |  | 47% |  |  |
| Dec Time[%] | 95% |  |  | 91% |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | 5.3 | 10.0 | 8.6 | 2.9 | 9.7 | 9.3 |
| Class B | 4.8 | 8.8 | 8.6 | 2.3 | 8.4 | 8.3 |
| Class C | 9.9 | 13.5 | 14.4 | 9.1 | 16.3 | 16.0 |
| Class D | 12.4 | 17.8 | 19.3 | 11.6 | 19.2 | 20.9 |
| Class E |  |  |  |  |  |  |
| All | 8.3 | 12.6 | 13.0 | 6.7 | 13.5 | 13.9 |
| Enc Time[%] | 64% |  |  | 60% |  |  |
| Dec Time[%] | 110% |  |  | 98% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A |  |  |  |  |  |  |
| Class B | 4.7 | 9.4 | 11.4 | 2.6 | 6.1 | 6.6 |
| Class C | 9.2 | 11.6 | 11.6 | 9.3 | 11.8 | 11.2 |
| Class D | 10.8 | 14.3 | 15.4 | 11.7 | 12.5 | 13.5 |
| Class E | 5.8 | 11.8 | 12.1 | 3.0 | -0.3 | 2.8 |
| All | 7.6 | 11.6 | 12.6 | 6.6 | 7.9 | 8.7 |
| Enc Time[%] | 63% |  |  | 58% |  |  |
| Dec Time[%] | 103% |  |  | 90% |  |  |

Table 7  Average coding loss when 4x4 and 64x64 transforms are disabled

## Coding loss when 4x4, 32x32 and 64x64 transforms are disabled

As shown in the table below, the average coding loss by disabling the 4x4, 32x32 and 64x64 transforms is 5.2% to 10.1%. The detailed results are provided in

TE12-TUSize-TI-maxTU16x16-minTU8x8.xls

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | 4.0 | 6.5 | 6.0 | 1.0 | 4.1 | 3.0 |
| Class B | 5.4 | 8.9 | 9.2 | 4.0 | 7.4 | 7.0 |
| Class C | 9.2 | 10.1 | 10.3 | 6.8 | 10.8 | 10.0 |
| Class D | 10.8 | 13.2 | 14.3 | 8.7 | 12.6 | 13.8 |
| Class E | 6.0 | 9.9 | 9.9 | 3.4 | -1.7 | 1.1 |
| All | 7.4 | 10.0 | 10.3 | 5.2 | 7.4 | 7.8 |
| Enc Time[%] | 43% |  |  | 45% |  |  |
| Dec Time[%] | 101% |  |  | 95% |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | 6.3 | 12.6 | 11.6 | 3.6 | 10.7 | 10.6 |
| Class B | 7.7 | 14.6 | 14.5 | 5.9 | 12.3 | 13.0 |
| Class C | 11.4 | 16.3 | 16.9 | 10.3 | 17.4 | 17.2 |
| Class D | 13.1 | 19.9 | 20.7 | 12.0 | 20.0 | 21.6 |
| Class E |  |  |  |  |  |  |
| All | 9.9 | 16.2 | 16.4 | 8.4 | 15.5 | 16.1 |
| Enc Time[%] | 62% |  |  | 57% |  |  |
| Dec Time[%] | 108% |  |  | 98% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A |  |  |  |  |  |  |
| Class B | 7.3 | 22.9 | 28.0 | 7.9 | 18.7 | 22.1 |
| Class C | 10.6 | 18.4 | 19.0 | 11.3 | 15.8 | 15.6 |
| Class D | 11.4 | 20.4 | 21.5 | 12.5 | 17.2 | 16.9 |
| Class E | 12.2 | 38.3 | 31.1 | 7.5 | 19.3 | 17.2 |
| All | 10.1 | 24.1 | 24.7 | 9.8 | 17.7 | 18.3 |
| Enc Time[%] | 61% |  |  | 59% |  |  |
| Dec Time[%] | 101% |  |  | 93% |  |  |

Table 8  Average coding loss when 4x4, 32x32, and 64x64 transforms are disabled

# Summary

The evaluation results are summarized as below in the table 6 below.

| Anchor | TU size under Test | Average loss (BD-rate) |
| --- | --- | --- |
| Up to 64x64 transforms | 64x64 transform disabled | 0.1% to 0.6% |
| Up to 64x64 transforms | 32x32 and 64x64 transforms disabled | 1.1% to 4.0% |
| Up to 64x64 transforms | 4x4 and 64x64 transforms disabled | 3.9% to 10.8% |
| Up to 64x64 transforms | 4x4, 32x32 and 64x64 transforms disabled | 5.2% to 10.1% |

Table 9.  Summary of TE12 testing on TU Size

The major observations are

- The coding gain from 64x64 transform is marginable

- Disabling 4x4 transform has huge quality impact, which is unexpected.

It is recommend removing 64x64 transform from the reference configurations due to it high complexity and limited coding efficiency gain, and launching the investigation on the quality issues when the 4x4 transform is disabled.

# References

- JCT-VC, “Tool Experiment 12: Evaluation of TMuC Tools”, JCTVC-B312, 2nd. JCT-VC Meeting, Geneva, CH, July 2010.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-B300, 2nd. JCT-VC Meeting, Geneva, CH, July 2010.