| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C057 WG11 Number: m18080 |
| --- | --- |

| Title: | TE 12: Evaluation of IBDI and TPE |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA | Tel: Email: | +1-214-480-3816 zhou@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This document reports the evaluation results of the IBDI and TPE (Transform Precision Extension) for TE12. Experimental results on TMuC-0.7 shows that the total gain with both the IBDI and TEP turned on is 0.85% to 5.0% on average; the average IBDI gain when TPE is off is 0.8% to 4.7%, and it is reduced to -0.1% to 3.3% when TPE is turned on; the average TPE gain when IBDI is off is 0.8% to 1.4%, but it totally vanishes when IBDI is on; The results also reveal that the additional left shift bits in the TPE (1 bit shift in T32x32, 2 bits shift in T16x16, and 3 bits shift in T8x8, respectively) do not provide additional gain compared to the 4 bits left shift across the transforms.

# Introduction

This document reports the performance evaluation results of the IBDI (Internal Bit Depth Increase) in the Test Model under Consideration (TMuC). Since TPE (Transform Precision Extension) closely interacts with the IBDI in the transforms, the performance of the TPE is also evaluated in the test.

# Tool Description

IBDI is a tool aimed to improve coding efficiency by left-shifting the input data by N bits before feeding the data into the video encoder. The complexity of the IBDI lies in two aspects: the internal data precision of each video codec processing stage needs to be increased by N bits, which leads to higher on-chip memory requirements and increased processing precision and power consumption; the reconstructed frames are stored with expanded bit-depth (N bits more), which consumes more memory bandwidth and off-chip storage for motion estimation and compensation. In the TMuC software, the IBDI on/off is controlled in the configuration files by setting the flag named “BitIncrement” to 4 or 0 (4 means IBDI is on in which the input data is left-shifted by 4 bits, and 0 means IBDI is off)

The TPE just expands the bit-depth for the intermediate data in the transforms after horizontal or vertical transform is performed, but retains the original bit-precision for the final transform outputs. In the current TMuC software, the TPE parameters are defined in

TlibCommon\Source\CommonDef.h

const int g_iShift8x8    = 7;

const int g_iShift16x16  = 6;

const int g_iShift32x32  = 5;

const int g_iShift64x64  = 4;

If the IBDI is off, the TPE expends the intermediate data precision for 8x8, 16x16, 32x32 and 64x64 transforms by 7, 6, 5, 4 bits, respectively. The data precision of the final transform outputs is not increased. When the IBDI is on, on the top of IBDI 4 bits bit-depth increase the TPE expends the intermediate data precision for 8x8, 16x16, 32x32 and 64x64 transforms by 3, 2, 1, 0 bits, respectively. The TPE can be viewed as a localized IBDI in the transforms with the exception that the final transform output maintains its original fidelity. The complexity of the TPE lies in the bit-width increase in the transform intermediate data, which has significant area impact especially if the transform block size is large.

# Test Conditions

The TMuC0.7.1 software is used for the evaluation, the simulation platforms is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz  64 bits Linux machines, the common test conditions and reference configurations specified in [2] are followed except for the IBDI and TPE that are configured differently for testing purpose. The following five settings are tested (g_iShift64x64 is always set to 4 when the TPE is on).

| TMuC0.7.1 | IBDI (bits) |  | TPE (Transform Precision Extension) |  |
| --- | --- | --- | --- | --- |
|  | HE (BitIncrement) | LOCO (BitIncrement) | HE (g_iShift32x32, g_iShift16x16, g_iShift8x8) | LOCO (g_iShift32x32, g_iShift16x16, g_iShift8x8) |
| Setting 0 (HE anchor + IBDI on in LOCO) | 4 | 4 | 5,6,7 | 5,6,7 |
| Setting 1 (IBDI off in HE + LOCO ) | 0 | 0 | 5,6,7 | 5,6,7 |
| Setting 2 | 0 | 0 | off | off |
| Setting 3 | 4 | 4 | off | off |
| Setting 4 | 0 | 0 | 4,4,4 | 4,4,4 |

Table 1  Test settings for the IBDI and TPE evaluation

The test settings are designed to evaluate performance of the following cases:

- The total gain from IBDI and IPE ( setting 0 vs. setting 2)

- The IBDI gain when TPE is on     (setting 0 vs. setting 1)

- The IBDI gain when TPE is off    (setting 3 vs. setting 2)

- The TPE gain when IBDI is on     (setting 0 vs. setting 3)

- The TPE gain when IBDI is off    (setting 1 vs. setting 2)

- The gain of additional 3, 2, 1-bit left shift in 32x32, 16x16 and 8x8 transforms provided by TPE (setting 1 vs. setting 4)

# Testing Results

## Total coding gain from IBDI and IPE

As shown in the table below, the average gain from IBDI and TPE ranges from 0.8% to 5%. The detailed results are shown in

TE12-IBDI-TI-IBDIoff_TPEoff_vs_IBDIon_TPE567.xls .

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.6 | 0.9 | 0.0 | 0.8 | 0.6 | 0.0 |
| Class B | 1.1 | 2.0 | 1.5 | 1.4 | 1.7 | 1.2 |
| Class C | 0.2 | 0.3 | 0.2 | 0.4 | 0.2 | 0.2 |
| Class D | 0.1 | 0.5 | 0.3 | 0.1 | 0.1 | 0.0 |
| Class E | 2.2 | 0.9 | 1.0 | 2.9 | 1.8 | 1.6 |
| All | 0.8 | 1.0 | 0.7 | 1.1 | 0.9 | 0.6 |
| Enc Time[%] | 99% |  |  | 90% |  |  |
| Dec Time[%] | 96% |  |  | 83% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 2.3 | 4.4 | 3.7 | 2.3 | 2.3 | 2.8 |
| Class B | 3.4 | 10.3 | 10.4 | 3.2 | 8.0 | 9.9 |
| Class C | 2.0 | 3.8 | 3.8 | 1.6 | 2.5 | 2.3 |
| Class D | 1.2 | 4.3 | 4.0 | 0.8 | 3.3 | 2.5 |
| Class E |  |  |  |  |  |  |
| All | 2.3 | 6.1 | 6.1 | 2.0 | 4.5 | 5.0 |
| Enc Time[%] | 107% |  |  | 100% |  |  |
| Dec Time[%] | 109% |  |  | 98% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 5.1 | 10.6 | 12.8 | 3.8 | 10.3 | 16.0 |
| Class C | 3.1 | 5.4 | 5.8 | 2.1 | 3.9 | 4.2 |
| Class D | 1.8 | 8.8 | 9.1 | 1.1 | 4.5 | 6.0 |
| Class E | 11.7 | 20.9 | 14.8 | 6.5 | 10.0 | 5.1 |
| All | 5.0 | 10.8 | 10.5 | 3.2 | 7.2 | 8.5 |
| Enc Time[%] | 105% |  |  | 101% |  |  |
| Dec Time[%] | 105% |  |  | 98% |  |  |

Table 2 Average gain from the IBDI and TPE. Positive numbers in the table mean the IBDI and TPE provide the coding gain.

## Coding gain from IBDI

As shown in Table 3, when TPE is off, the IBDI provides 0.8% to 4.7% gain. However, the IBDI gain reduces to -0.1% to 3.3% when the TPE is on (see Table 4). For detailed results please refer to

TE12-IBDI-TI-IBDIon_TPEoff_vs_IBDIoff_TPEoff.xls

TE12-IBDI-TI-IBDIon_TPE567_vs_IBDIoff_TPE567.xls

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.6 | -0.9 | 0.0 | -0.8 | -0.6 | 0.0 |
| Class B | -1.1 | -1.9 | -1.5 | -1.4 | -1.6 | -1.2 |
| Class C | -0.2 | -0.3 | -0.2 | -0.4 | -0.2 | -0.1 |
| Class D | -0.1 | -0.5 | -0.4 | -0.1 | -0.1 | 0.0 |
| Class E | -2.1 | -0.8 | -0.8 | -2.8 | -1.7 | -1.6 |
| All | -0.8 | -1.0 | -0.7 | -1.0 | -0.9 | -0.6 |
| Enc Time[%] | 100% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.2 | -4.2 | -3.7 | -2.3 | -2.3 | -2.7 |
| Class B | -3.3 | -9.1 | -9.1 | -3.1 | -7.3 | -8.4 |
| Class C | -2.0 | -3.5 | -3.7 | -1.6 | -2.4 | -2.4 |
| Class D | -1.2 | -3.9 | -3.6 | -0.8 | -3.0 | -2.6 |
| Class E |  |  |  |  |  |  |
| All | -2.2 | -5.6 | -5.5 | -2.0 | -4.2 | -4.5 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 104% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -4.8 | -9.3 | -11.0 | -3.7 | -9.4 | -12.9 |
| Class C | -3.0 | -5.0 | -5.4 | -2.1 | -3.7 | -4.0 |
| Class D | -1.8 | -7.8 | -7.6 | -1.0 | -4.2 | -5.4 |
| Class E | -10.4 | -16.5 | -13.4 | -6.1 | -9.5 | -5.0 |
| All | -4.7 | -9.2 | -9.2 | -3.1 | -6.7 | -7.3 |
| Enc Time[%] | 97% |  |  | 100% |  |  |
| Dec Time[%] | 96% |  |  | 102% |  |  |

Table 3  Average gain from the IBDI when TPE is off. Negative numbers in the table mean the IBDI provides the coding gain.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 0.3 | 1.4 | 0.3 | 0.5 | 1.3 |
| Class B | 0.2 | 0.6 | 1.4 | 0.3 | 0.5 | 1.2 |
| Class C | 0.2 | 0.5 | 0.7 | 0.1 | 0.4 | 0.5 |
| Class D | 0.3 | 0.2 | 0.4 | 0.2 | 0.2 | 0.4 |
| Class E | -0.3 | 4.0 | 2.7 | -0.8 | 2.4 | 1.7 |
| All | 0.1 | 1.0 | 1.2 | 0.0 | 0.7 | 1.0 |
| Enc Time[%] | 97% |  |  | 108% |  |  |
| Dec Time[%] | 100% |  |  | 132% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.6 | -3.0 | -2.0 | -1.4 | -1.1 | -1.3 |
| Class B | -2.1 | -6.3 | -6.3 | -1.6 | -4.0 | -4.9 |
| Class C | -1.3 | -2.5 | -2.7 | -0.9 | -1.4 | -1.3 |
| Class D | -0.8 | -3.3 | -3.4 | -0.5 | -2.4 | -1.8 |
| Class E |  |  |  |  |  |  |
| All | -1.5 | -4.1 | -4.0 | -1.1 | -2.5 | -2.6 |
| Enc Time[%] | 93% |  |  | 103% |  |  |
| Dec Time[%] | 91% |  |  | 110% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -3.0 | -7.2 | -8.1 | -1.9 | -5.1 | -7.1 |
| Class C | -2.0 | -4.1 | -4.5 | -1.3 | -2.6 | -2.7 |
| Class D | -1.2 | -7.0 | -7.0 | -0.6 | -3.3 | -4.3 |
| Class E | -8.4 | -10.0 | -9.9 | -4.1 | -0.4 | 2.0 |
| All | -3.3 | -6.9 | -7.2 | -1.9 | -3.1 | -3.6 |
| Enc Time[%] | 96% |  |  | 105% |  |  |
| Dec Time[%] | 99% |  |  | 112% |  |  |

Table 4  Average gain from the IBDI when TPE (g_iShift32x32    = 5; g_iShift16x16  = 6;g_iShift8x8  = 7)) is on. Negative numbers in the table mean the IBDI provides the coding gain.

## Coding gain from TPE

As shown in Table 5, when IBDI is off, the TPE provides 0.8% to 1.4% gain. However, the TPE gain vanishes when the IBDI is on (see Table 6). For detailed results please refer to

TE12-IBDI-TI-IBDIoff_TPEoff_vs_IBDIoff_TPE567.xls

TE12-IBDI-TI-IBDIon_TPEoff_vs_IBDIon_TPE567.xls

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.9 | 1.3 | 1.3 | 1.1 | 1.1 | 1.3 |
| Class B | 1.3 | 2.6 | 3.0 | 1.7 | 2.2 | 2.5 |
| Class C | 0.4 | 0.8 | 0.9 | 0.4 | 0.6 | 0.7 |
| Class D | 0.3 | 0.7 | 0.7 | 0.3 | 0.3 | 0.4 |
| Class E | 1.9 | 4.9 | 3.8 | 2.1 | 4.2 | 3.4 |
| All | 0.9 | 2.0 | 2.0 | 1.1 | 1.6 | 1.6 |
| Enc Time[%] | 96% |  |  | 97% |  |  |
| Dec Time[%] | 96% |  |  | 109% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.7 | 1.2 | 1.7 | 0.9 | 1.1 | 1.5 |
| Class B | 1.2 | 3.0 | 3.2 | 1.5 | 3.7 | 4.0 |
| Class C | 0.7 | 1.1 | 1.0 | 0.7 | 1.1 | 1.0 |
| Class D | 0.3 | 0.7 | 0.4 | 0.3 | 0.8 | 0.6 |
| Class E |  |  |  |  |  |  |
| All | 0.8 | 1.7 | 1.7 | 0.9 | 1.9 | 2.0 |
| Enc Time[%] | 100% |  |  | 103% |  |  |
| Dec Time[%] | 99% |  |  | 108% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 1.9 | 2.4 | 3.2 | 1.8 | 4.6 | 6.9 |
| Class C | 1.0 | 1.0 | 1.1 | 0.8 | 1.2 | 1.4 |
| Class D | 0.6 | 0.7 | 1.0 | 0.4 | 0.9 | 1.2 |
| Class E | 2.3 | 8.8 | 3.4 | 2.1 | 9.7 | 7.0 |
| All | 1.4 | 2.8 | 2.2 | 1.3 | 3.8 | 4.1 |
| Enc Time[%] | 101% |  |  | 106% |  |  |
| Dec Time[%] | 103% |  |  | 109% |  |  |

Table 5  Average gain from the TPE (g_iShift32x32    = 5; g_iShift16x16  = 6;g_iShift8x8  = 7)) when IBDI is off. Positive numbers in the table mean the TPE provides the coding gain.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.1 | 0.1 | 0.0 | 0.0 | 0.0 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 99% |  |  | 91% |  |  |
| Dec Time[%] | 96% |  |  | 83% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | -0.1 | 0.0 | 0.0 | 0.1 | 0.0 |
| Class C | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | -0.1 |
| Class D | 0.0 | 0.1 | 0.2 | 0.0 | 0.1 | -0.2 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | -0.1 |
| Enc Time[%] | 107% |  |  | 100% |  |  |
| Dec Time[%] | 111% |  |  | 102% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.1 | -0.1 | 0.0 | -0.2 | -0.1 |
| Class C | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | -0.2 | 0.3 | 0.0 | 0.0 | 0.0 |
| Class E | 0.1 | 0.9 | -0.7 | 0.1 | -0.5 | -0.4 |
| All | 0.0 | 0.2 | -0.1 | 0.0 | -0.2 | -0.1 |
| Enc Time[%] | 102% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 6  Average gain from the TPE (g_iShift32x32  = 5; g_iShift16x16  = 6;g_iShift8x8  = 7)) when IBDI is on. Positive numbers in the table mean the TPE provides the coding gain.

## Coding gain from additional left-shift bits provided by TPE

Table 7 below shows that the additional 1, 2, 3 bits left shift in 32x32, 16x16 and 8x8 transformed provided by the TPE does not lead to additional coding gain. The detailed comparison is provided in

TE12-IBDI-TI-IBDIoff_TPE444_vs_IBDIoff_TPE567.xls

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 111% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | 0.2 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.1 | -0.1 | 0.0 | 0.1 | 0.0 |
| Class C | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.1 | 0.0 | -0.1 | 0.0 | 0.1 | 0.2 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.1 |
| Enc Time[%] | 100% |  |  | 103% |  |  |
| Dec Time[%] | 99% |  |  | 109% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | 0.4 | 0.0 | 0.1 | 0.0 |
| Class C | 0.0 | 0.1 | -0.1 | 0.0 | -0.1 | 0.1 |
| Class D | 0.0 | -0.2 | -0.2 | 0.1 | 0.0 | -0.1 |
| Class E | 0.0 | 0.8 | -0.4 | 0.1 | -0.1 | 0.1 |
| All | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 107% |  |  |
| Dec Time[%] | 101% |  |  | 115% |  |  |

Table 7  Coding gain from the TPE (g_iShift32x32  = 5; g_iShift16x16  = 6;g_iShift8x8  = 7)) w.r.t the TPE (g_iShift32x32  = 4; g_iShift16x16  = 4;g_iShift8x8  = 4)) when IBDI is off.  Positive numbers in the table mean the additional left shift in TPE provides the coding gain.

# Summary

The evaluation results are summarized as below in the table 8 below.

| Anchor combination | Combination under Test | Average Gain (BD-rate) |
| --- | --- | --- |
| IBDI off, TPE off | IBDI on, TPE (5,6,7) on | 0.8% to 5.0% |
| IBDI off, TPE off | IBDI on, TPE off | 0.8% to 4.7% |
| IBDI off, TPE (5,6,7) on | IBDI on, TPE (5,6,7) on | -0.1% to 3.3% |
| IBDI off, TPE off | IBDI off, TPE (5,6,7) on | 0.8% to 1.4% |
| IBDI on, TPE off | IBDI on, TPE (5,6,7) on | No gain |
| IBDI off, TPE (4,4,4) on | IBDI off, TPE (5,6,7) on | No gain |

Table 8.  Summary of TE12 testing on IBDI and TPE

The major observations are

- The total gain with both the IBDI and TPE turned on is 0.8% to 5.0%.

- The average IBDI gain when TPE is off is 0.8% to 4.7%, it is reduced to -0.1% to 3.3% when TEP is turned on.

- The average TPE gain when IBDI is off is 0.8% to 1.4%, but it totally vanishes when IBDI is on.

- Additional left shift bits in the TPE (1 bit shift in T32x32, 2 bits shift in T16x16, and 3 bits shift in T8x8) do not provide gain do not provide additional gain compared to the 4 bits left shift across the transforms.

Currently four bits left shift is used when the IBDI is turned on, the trade-off between the number of left shift bits (complexity) and coding efficiency needs to be investigated in the future.

# References

- JCT-VC, “Tool Experiment 12: Evaluation of TMuC Tools”, JCTVC-B312, 2nd. JCT-VC Meeting, Geneva, CH, July 2010.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-B300, 2nd. JCT-VC Meeting, Geneva, CH, July 2010.