| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 07-14 October, 2010 | Document: JCTVC-C059 WG11 Number: m18082 |
| --- | --- |

| Title: | TE12: Evaluation of transform coefficient coding (HHI_TRANSFORM_CODING) with tool breakdown |  |  |
| --- | --- | --- | --- |
| Status: | Information |  |  |
| Purpose: | Tool Experiment Description |  |  |
| Author(s) or Contact(s): | Vivienne Sze Texas Instruments Inc. Madhukar Budagavi Texas Instruments Inc. Minhua Zhou Texas Instruments Inc. | Email: Email: Email: | sze@ti.com |
|  |  |  | madhukar@ti.com |
|  |  |  | zhou@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

Abstract

The coding efficiency gain of HHI_TRANSFORM_CODING is evaluated with TMuC-0.7.3 to be between 1.3 to 2.0%.  A breakdown of this gain across the three main tools was also determined: adaptive scanning order of significance map (0.2 to 0.4%), proposed context modeling of significance map (0.9 to 1.4%) and proposed context modeling of coefficient level (0.1 to 0.2%).  However, the three tools increase the complexity of context modeling making it more difficult to parallelize for throughput.   Note: The coding efficiency gain of HHI_TRANSFORM_CODING was also evaluated with TMuC-0.7 and has been verified to be a match with results provided by Samsung.

# Introduction

This document reports the performance evaluation results of the transform coefficient coding in the Test Model under Consideration (TMuC) as part of TE12 [1].

# Tool Description

Transform coefficient coding is a context modeling scheme that aims to improve coding efficiency using three main modifications:

- Adaptive scanning order of significance map:  The direction of the diagonal sub-scan depends on the location of the majority of significant coefficients, either bottom-left or top-right part of transform block, of the previous diagonal scan (i.e. each diagonal scan can be in any direction)

- Proposed context modeling for significance map:  For 8x8 blocks, each 2x2 sub-block uses a separate context for significant_coeff_flag.  For transforms greater than 8x8, the context of significant_coeff_flag depends on the top and left neighboring coefficient positions. The context for last_ significant_coeff_flag depends on distance from top-left corner of transform block.

- Proposed context modeling for coefficient levels: Different contexts can be used for each 4x4 sub-block to encode coeff_abs_level_minus_one depending on the statistics of already coded sub-blocks (i.e. number of coefficients whose absolute value is greater than one in the previous sub-block within the same macroblock).

## Complexity Analysis

Parallel bin processing has been proposed in [1][3][4].  However, as highlighted in [5][6], the context modeling remains a challenge to parallelize.  The three modifications introduce additional complexity to the context selection; specifically adaptive scanning order and proposed context modeling for significance map when transform >8x8 make it difficult to process multiple significant_coeff_flag elements in parallel.  In H.264/AVC, the context selection for all  significant_coeff_flag elements in a transform could be done in parallel. With adaptive scanning order, each diagonal scan must be done serially due to their dependence. Note that within a given scan, the context selection of significant_coeff_flag elements can still be done in parallel.

Figure 1  Processing order of significance map with zig-zag (a) and adaptive scan (b). Note parallelism in (a) is higher.

With the proposed context modeling for significance map, it is difficult to perform the context selection of significant_coeff_flag in parallel since their context depends on up to 11 neighbors.  The dependencies are highlighted in Fig. 2 and derived from the code in Fig. 3 from TComTrQuant.cpp.  In particular, when used with zig-zag scan or adaptive scan, the context selection for the majority of significant_coeff_flag elements cannot be done in parallel (at the decoder).

Figure 2  To select context for significant_coeff_flag in position X requires information from up to 11 neighbors in position A, B, C, and D.  Positions A, B, C, and D were derived from code highlighted in Fig. 3.

UInt TComTrQuant::getSigCtxInc ( TCoeff* pcCoeff, const UInt uiPosX, const UInt uiPosY, const UInt uiLog2BlkSize, const UInt uiStride, const bool bDownLeft ) { UInt uiCtxInc = 0; UInt uiSizeM1 = ( 1 << uiLog2BlkSize ) - 1; . . . . . . . . . { const int* pData = &pcCoeff[ uiPosX + uiPosY * uiStride ]; int iStride = uiStride; int iStride2= iStride << 1; UInt uiCnt = ( pData[ -iStride ] ? 1 : 0 ); A uiCnt += ( pData[ -1 ] ? 1 : 0 ); uiCnt += ( pData[ -1 -iStride ] ? 1 : 0 ); if( uiPosX > 1 ) { uiCnt += ( pData[ -2 ] ? 1 : 0 ); uiCnt += ( pData[ -2 -iStride ] ? 1 : 0 ); B if( uiPosY < uiSizeM1 ) { uiCnt += ( pData[ -2 +iStride ] ? 1 : 0 ); } } if( uiPosY > 1 ) { uiCnt += ( pData[ -iStride2 ] ? 1 : 0 ); uiCnt += ( pData[ -1 -iStride2 ] ? 1 : 0 ); C if( uiPosX < uiSizeM1 ) { uiCnt += ( pData[ 1 -iStride2 ] ? 1 : 0 ); } } if( bDownLeft ) { if( uiPosX < uiSizeM1 ) { uiCnt += ( pData[ 1 -iStride ] ? 1 : 0 ); } } else D { if( uiPosY < uiSizeM1 ) { uiCnt += ( pData[ -1 +iStride ] ? 1 : 0 ); } } uiCtxInc = 10 + min<UInt>( 4, ( uiCnt + 1 ) >> 1 ); } return uiCtxInc; }

Figure 2 Source code (TComTrQuant.cpp) to process context selection for proposed context modeling of significance map.  Dependencies are highlighted.

Finally, the proposed context modeling for coefficients introduces dependencies between subblocks, which again makes parallelism difficult.  Due to the increased complexity in context selection, it is important to understand the coding gains of each tool.  Therefore, we will evaluate each independently as described in the next sections.

# Test Conditions

The TMuC0.7.3 software is used for the evaluation, the simulation platforms is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz  64 bits Linux machines, the common test conditions and reference configurations specified in [7] are followed except for HHI_TRANSFORM_CODING are configured differently for testing purpose.

## Code Modifications

The HHI_TRANSFORM_CODING portion of the code was also modified to separate the tools.

### Turning Off Adaptive Scan

To turn off adaptive scan, in TEncSbac.cpp (around line 1484)

//===== update scan direction ===== if( ( uiDownLeft == 1 && ( uiPosX == 0 ¦¦ uiPosY == uiHeight - 1 ) ) ¦¦ ( uiDownLeft == 0 && ( uiPosY == 0 ¦¦ uiPosX == uiWidth - 1 ) ) ) { // uiDownLeft = ( uiNumSigTopRight >= uiNumSigBotLeft ? 1 : 0 ); uiDownLeft = ( uiDownLeft ? 0 : 1 ); // implement zig-zag scan }

Similarly, in TDecSbac.cpp (around line 1597)

//===== update scan direction ===== if( ( uiDownLeft == 1 && ( uiPosX == 0 ¦¦ uiPosY == uiHeight - 1 ) ) ¦¦ ( uiDownLeft == 0 && ( uiPosY == 0 ¦¦ uiPosX == uiWidth - 1 ) ) ) { //uiDownLeft = ( uiNumSigTopRight >= uiNumSigBotLeft ? 1 : 0 ); uiDownLeft = ( uiDownLeft ? 0 : 1 ); // implement zig-zag scan }

### Turning Off Proposed Context Modeling for Coefficient Levels

To turn off proposed context modeling for coefficient levels, in TEncSbac.cpp (around line 1549)

//if ( bFirstBlock ) // turn off adaptive context per sub-block if ( 1 ) //always select same sub-block { bFirstBlock = false; uiCtxSet = uiNumOfSets + 1; } else { uiCtxSet = uiPrevAbsGreOne / uiNumOfSets + 1;//( ( uiGreOne * uiEqRangeSize ) >> 8 ) + 1; uiPrevAbsGreOne = 0; }

Similarly, in TDecSbac.cpp (around line 1666)

// if ( bFirstBlock ) disable context selection by sub-block if ( 1 ) { bFirstBlock = false; uiCtxSet = uiNumOfSets + 1; } else { //uiCtxSet = ( ( uiGreOne * uiEqRangeSize ) >> 8 ) + 1; uiCtxSet = uiPrevAbsGreOne / uiNumOfSets + 1; uiPrevAbsGreOne = 0; }

## Summary of Test Cases

The following four settings are tested.

| TMuC-0.7.3 | HHI_TRANSFORM_CODING | Adaptive Scan | Context Modeling Significance Map | Context Modeling Coefficient Level |
| --- | --- | --- | --- | --- |
| Setting 0 | 1 | ON | ON | ON |
| Setting 1 | 0 | OFF | OFF | OFF |
| Setting 2 | 1 | OFF | ON | ON |
| Setting 3 | 1 | OFF | ON | OFF |

Table 1  Test settings for the HHI_TRANSFORM_CODING evaluation

The test settings are designed to evaluate performance of the following cases:

- The total gain from HHI_TRANSFORM_CODING (setting 1 vs. setting 0)

- The Adaptive Scan gain (setting 2 vs. setting 0)

- The Context Modeling Significance Map gain (setting 1 vs. setting 3)

- The Context Modeling Coefficient Level gain  (setting 2 vs. setting 3)

# Testing Results

## Total coding gain from HHI_TRANSFORM_CODING

As shown in the table below, the average gain from HHI_TRANSFORM_CODING ranges from 1.3% to 2.0%. The detailed results are shown in TE12-TI-HHI_TRANSFORM_CODING.xls

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 1.5 | -1.8 | -2.6 |
| Class B | 1.7 | -1.1 | -1.5 |
| Class C | 1.1 | -1.4 | -1.5 |
| Class D | 0.7 | -1.6 | -1.3 |
| Class E | 2.1 | -2.5 | -1.4 |
| All | 1.4 | -1.6 | -1.6 |
|  |  | Random access |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 1.0 | -1.3 | -1.4 |
| Class B | 1.7 | -0.1 | -0.5 |
| Class C | 1.4 | -0.3 | -0.5 |
| Class D | 1.0 | -1.1 | -0.7 |
| Class E |  |  |  |
| All | 1.3 | -0.6 | -0.7 |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 2.6 | 2.4 | 2.4 |
| Class C | 2.1 | 1.1 | 0.8 |
| Class D | 1.4 | -0.2 | -0.4 |
| Class E | 1.9 | -1.1 | -1.1 |
| All | 2.0 | 0.7 | 0.6 |

Table 2  Average gain from the HHI_TRANSFORM_CODING. Positive numbers in the table mean the HHI_TRANSFORM_CODING provides the coding gain.

## Coding gain from Adaptive Scan

As shown in Table 3, Adaptive Scan provides 0.2% to 0.4% gain. For detailed results, please refer to

TE12-TI-HHI_TRANSFORM_CODING.xls

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.9 | 0.7 |
| Class B | 0.2 | 0.8 | 0.7 |
| Class C | 0.2 | 0.6 | 0.7 |
| Class D | 0.2 | 0.5 | 0.5 |
| Class E | 0.1 | 0.5 | 0.8 |
| All | 0.2 | 0.7 | 0.6 |
|  |  | Random access |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.3 | 0.9 | 0.8 |
| Class B | 0.3 | 0.8 | 0.8 |
| Class C | 0.3 | 0.5 | 0.6 |
| Class D | 0.4 | 0.5 | 0.5 |
| Class E |  |  |  |
| All | 0.3 | 0.7 | 0.7 |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.4 | 0.8 | 0.9 |
| Class C | 0.4 | 0.8 | 0.7 |
| Class D | 0.6 | 0.9 | 0.8 |
| Class E | 0.3 | 0.6 | 0.8 |
| All | 0.4 | 0.8 | 0.8 |

Table 3  Average gain from the Adaptive Scan. Positive numbers in the table mean the Adaptive Scan provides the coding gain.

## Coding gain from Proposed Context Modeling of Significance Map

As shown in Table 4, proposed context modeling of significance map provides 0.9 to 1.4% gain. For detailed results, please refer to TE12-TI-HHI_TRANSFORM_CODING

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 1.1 | -3.0 | -3.5 |
| Class B | 1.1 | -2.3 | -2.6 |
| Class C | 0.7 | -2.2 | -2.2 |
| Class D | 0.4 | -2.2 | -2.0 |
| Class E | 1.7 | -3.4 | -2.6 |
| All | 1.0 | -2.5 | -2.5 |
|  |  | Random access |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.6 | -2.3 | -2.0 |
| Class B | 1.2 | -1.0 | -1.2 |
| Class C | 1.0 | -1.0 | -1.2 |
| Class D | 0.5 | -1.6 | -1.3 |
| Class E |  |  |  |
| All | 0.9 | -1.3 | -1.3 |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 1.9 | 1.6 | 1.8 |
| Class C | 1.5 | 0.1 | -0.2 |
| Class D | 0.8 | -1.0 | -1.4 |
| Class E | 1.4 | -2.4 | -1.9 |
| All | 1.4 | -0.2 | -0.2 |

Table 4  Average gain from the proposed context modeling for significance map. Positive numbers in the table mean proposed context modeling for significance map provides the coding gain.

## Coding gain from Proposed Context Modeling of Coefficient Levels

As shown in Table 5, proposed context modeling of coefficient levels provides 0.1% to 0.2% gain. For detailed results, please refer to TE12-TI-HHI_TRANSFORM_CODING.

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 0.2 | 0.2 |
| Class B | 0.4 | 0.4 | 0.4 |
| Class C | 0.1 | 0.2 | 0.1 |
| Class D | 0.1 | 0.2 | 0.1 |
| Class E | 0.2 | 0.3 | 0.3 |
| All | 0.2 | 0.3 | 0.2 |
|  |  | Random access |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.1 | -0.2 |
| Class B | 0.2 | 0.1 | -0.1 |
| Class C | 0.1 | 0.2 | 0.1 |
| Class D | 0.1 | 0.0 | 0.1 |
| Class E |  |  |  |
| All | 0.1 | 0.1 | 0.0 |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.2 | -0.1 | -0.3 |
| Class C | 0.1 | 0.2 | 0.3 |
| Class D | 0.0 | -0.2 | 0.2 |
| Class E | 0.2 | 0.6 | 0.1 |
| All | 0.1 | 0.1 | 0.1 |

Table 5  Average gain from the proposed context modeling for significance map. Positive numbers in the table mean proposed context modeling for significance map provides the coding gain.

# Summary

The evaluation results are summarized as below in the Table 6 below.  The breakdown is show in Fig. 5.

| Tool Under Test | Anchor combination | Combination under Test | Average Gain (BD-rate) |
| --- | --- | --- | --- |
| HHI_TRANSFORM_CODING | HHI_TRANSFORM_CODING=1 (adaptive scan ON, proposed context modeling significance map ON, proposed context modeling coefficient levels ON) | HHI_TRANSFORM_CODING=0 (adaptive scan OFF, proposed context modeling significance map OFF, proposed context modeling coefficient levels OFF) | 1.3 to 2.0 % |
| Adaptive Scan | HHI_TRANSFORM_CODING=1 (adaptive scan ON, proposed context modeling significance map ON, proposed context modeling coefficient levels ON) | HHI_TRANSFORM_CODING=1 (adaptive scan OFF, proposed context modeling significance map ON, proposed context modeling coefficient levels ON) | 0.2 to 0.4% |
| Proposed context selection significance map | HHI_TRANSFORM_CODING=1 (adaptive scan OFF, proposed context selection significance map ON, proposed context selection coefficient levels OFF) | HHI_TRANSFORM_CODING=0 (adaptive scan OFF, proposed context selection significance map OFF, proposed context selection coefficient levels OFF) | 0.9 to 1.4 % |
| Proposed context selection coefficient level | HHI_TRANSFORM_CODING=1 (adaptive scan OFF, proposed context selection significance map ON, proposed context selection coefficient levels ON) | HHI_TRANSFORM_CODING=1 (adaptive scan OFF, proposed context selection significance map ON, proposed context selection coefficient levels OFF) | 0.1 to 0.2 % |

Table 6.  Summary of TE12 testing on HHI_TRANSFORM_CODING

Figure 4.  Breakdown of coding gains in HHI_TRANSFORM_CODING for intra, random access and low delay (left-to-right).

# Conclusions

The majority of the coding efficiency gains of HHI_TRANSFORM_CODING are due to the proposed context selection for significance map.  It is unclear whether this is due the modification to significant_coeff_flag or last_significant_coeff_flag. The former introduced much more complexity than the latter, particularly when zig-zag or adaptive scan is being used.  Further investigation is recommended.

Given that proposed context selection for significance map provides the majority of the coding efficiency gain in HHI_TRANSFORM_CODING, we feel it merits further investigation in terms of complexity reduction.  However, the other two tools have a poor trade-off between coding gain and increased complexity in context modeling which is already difficult to parallelize.

# References

- JCT-VC, “Tool Experiment 12: Evaluation of TMuC Tools”, JCTVC-B312, 2nd. JCT-VC Meeting, Geneva, CH, July 2010.

- “Description of video coding technology proposal by Fraunhofer HHI”, JCTVC-A116, 1st JCT-VC Meeting, Dresden, DE, April 2010.

- “Video coding technology proposal by RIM”, JCTVC-A120, 1st JCT-VC Meeting, Dresden, DE, April 2010.

- “Video coding technology proposal by Texas Instruments Inc”, JCTVC-A101, 1st JCT-VC Meeting, Dresden, DE, April 2010.

- “Parallel Context Processing techniques for high coding efficiency entropy coding in HEVC”, JCTVC-B088, 2nd. JCT-VC Meeting, Geneva, CH, July 2010.

- “Improving throughput for V2V coding in HEVC”, JCTVC-B036, 2nd. JCT-VC Meeting, Geneva, CH, July 2010.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-B300, 2nd. JCT-VC Meeting, Geneva, CH, July 2010.

# Patent rights declaration(s)

Texas Instruments Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).