| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C060 WG11 Number: m18083 |
| --- | --- |

| Title: | TE 2: TI reference frame compression proposal |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA Madhukar Budagavi Texas Instruments Inc., USA | Tel: Email: Tel: Email: | +1-214-480-3816 zhou@ti.com +1-214-480-1861 madhukar@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This document reports the testing results of the reference frame compression (RFC) algorithm [1] proposed by Texas Instruments at the Geneva JCT-VC meeting. The algorithm is integrated into the TMuC-0.7 reference software and tested with the JCT-VC common testing conditions specified in [2]. For the High Efficiency (HE) configurations in which IBDI is on, the proposed algorithm provides 12 bits to 8 bits compression on the cost of an average BD-rate increase of 0.3% for random access configurations and 1.7% for low-delay configurations, respectively. The average motion compensation memory bandwidth is reduced by 14.7% to 45.7%. For the low complexity (LOCO) configurations in which the IBDI is off, the proposed algorithm conducts 8 bits to 4 bits compression, the average BD-rate increase is 2.5% for random access LOCO and 3.4% for low-delay LOCO, respectively. The memory bandwidth saving is 50% if the growing window is employed.

# Introduction

At the last JCT-VC meeting held in Geneva in April 2010, an in-loop reference frame compression algorithm [1] is proposed to reduce the memory bandwidth requirements in the motion estimation and motion compensation. This document reports the performance evaluation results of TI reference frame compression algorithm by using the TMuC-0.7 reference software and common testing conditions.

# Algorithm Description

The proposed algorithm [1] can operate in two modes: for the HE configurations in which the IBDI is on, the algorithm performs 12 bits to 8 bits compression, on 8x8 luma blocks and 4x4 chroma blocks, each block is guaranteed to have fixed compression ratio so that the random access of the reference blocks is permitted; for the LOCO configurations in which the IBDI is off, the algorithm provides 8 bits to 4 bits compression (factor 2) on 8x8 luma blocks and 4x4 chroma blocks. However, the fixed compression ratio (factor of 2) is not guaranteed at block level, instead, it is guaranteed at every CU row (64 lines) level. For this mode to work, an encoder/decoder needs to have growing search window (i.e. horizontal size equals to frame width) to buffer the reference blocks for motion estimation and motion compensation. The detailed description of the algorithm can be found in [1].

# Test Conditions

The TMuC0.7 software is used for the evaluation, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines, the common test conditions and reference configurations specified in [2] are followed.

# Testing Results

## BD-rate increase resulted from the reference frame compression

The summary of the coding efficiency testing results is listed in the table below. For the high efficiency settings (12 bits to 8 bits compression), the average coding loss is 0.3% to 1.7%; for the low complexity settings (8bits to 4 bits compression) the loss ranges from 2.5% to 3.4%. For detailed results please refer to TIRFC-TMuC-0.7_RFCvs.anchor.xls.

|  | AVERAGE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Random access |  |  | Random access LoCo |  |
|  | BD-rate [Y] | BD-rate [U] | BD-rate [V] | BD-rate [Y] | BD-rate [U] | BD-rate [V] |
| Class A | 0.2 | 0.7 | 1.0 | 0.0 | 0.1 | 0.1 |
| Class B | 0.2 | 1.3 | 2.6 | 0.4 | 1.8 | 2.7 |
| Class C | 0.3 | 0.4 | 0.7 | 2.9 | 9.8 | 11.7 |
| Class D | 0.3 | 0.8 | 1.3 | 5.9 | 28.5 | 29.8 |
| Class E |  |  |  |  |  |  |
| All | 0.3 | 0.9 | 1.5 | 2.5 | 10.8 | 12.0 |
| Enc Time[%] | 108% |  |  | 103% |  |  |
| Dec Time[%] | 147% |  |  | 147% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | BD-rate [Y] | BD-rate [U] | BD-rate [V] | BD-rate [Y] | BD-rate [U] | BD-rate [V] |
| Class A |  |  |  |  |  |  |
| Class B | 1.0 | 3.1 | 6.5 | 0.3 | 1.2 | 2.1 |
| Class C | 1.0 | 1.2 | 1.7 | 4.2 | 13.0 | 16.0 |
| Class D | 0.6 | 1.4 | 2.8 | 9.1 | 38.9 | 40.2 |
| Class E | 5.4 | 8.1 | 8.6 | 0.0 | 0.0 | 0.0 |
| All | 1.7 | 3.2 | 4.8 | 3.4 | 13.4 | 14.7 |
| Enc Time[%] | 106% |  |  | 107% |  |  |
| Dec Time[%] | 136% |  |  | 155% |  |  |

Table 1  Average BD-rate increase resulted from reference frame compression.

## Memory bandwidth reduction

The average motion compensation memory bandwidth saving with and without cache is listed in Table 3. The cache settings used in the simulation are:

- Single level cache (for our purpose of studying cache and memory compression interaction, a single level cache should be sufficient)

- Capacity (cache size): 128 KB

- Line size: 64 bytes

- Organization: 4-way set-associative

- Replacement strategy: FIFO (Least Recently Used (LRU) strategy is industry standard, but FIFO is easier to implement)

- DDR alignment: 64 bits

As shown in the table below, the average motion compensation bandwidth saving by using 12bits to 8 bits is 14.7% to 45.7%, depending on the DDR alignment size and burst size, with or without cache.

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 32bit/ 64bit | 64bit/128bit | 128bit/128bit | 256bit/256bit | Cache (64/512) |
| Class A | -15.0% | -23.9% | -29.8% | -44.9% | -37.7% |
| Class B | -17.4% | -25.4% | -30.0% | -43.3% | -35.6% |
| Class C | -14.7% | -24.0% | -30.1% | -44.9% | -39.0% |
| Class D | -11.0% | -21.7% | -29.2% | -45.3% | -42.3% |
| Class E |  |  |  |  |  |
| All | -14.7% | -23.8% | -29.8% | -44.5% | -38.6% |
|  | Low delay |  |  |  |  |
|  | 32bit/ 64bit | 64bit/128bit | 128bit/128bit | 256bit/256bit | Cache (64/512) |
| Class A |  |  |  |  |  |
| Class B | -14.6% | -23.9% | -29.7% | -44.3% | -39.6% |
| Class C | -12.3% | -22.9% | -30.0% | -45.7% | -44.0% |
| Class D | -9.4% | -21.9% | -30.6% | -47.6% | -45.3% |
| Class E | -25.7% | -30.8% | -33.8% | -45.2% | -43.3% |
| All | -14.8% | -24.4% | -30.8% | -45.7% | -42.8% |

Table 3 Average motion compression bandwidth saving

## For low-complexity settings, the 8bits to 4 bits compression is used. The bandwidth saving should be fixed to 50% if the growing widow is employed.

## For detailed results, please refer to TIRFC-membandwidthSavings_8x8(4x4).xls.

# Conclusions

The reference frame compression algorithm proposed in [1] is integrated into TMuC-0.7 software and evaluated.  For the high efficiency settings (IBDI on 12 bits to 8 bits compression), the average motion compensation memory bandwidth saving is 14.7% to 45.7% on the cost of coding loss of 0.3% to 1.7%; for the low complexity settings (8bits to 4 bits compression) the coding loss is 2.5% to 3.4%, the memory bandwidth saving is fixed to 50% if the growing widow is employed.

# References

- JCT-VC, “TE2: Compressed Reference Frame Buffers (CRFB)”, JCTVC-B089, 2nd. JCT-VC Meeting, Geneva, CH, July 2010.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-B300, 2nd. JCT-VC Meeting, Geneva, CH, July 2010.

# Patent rights declaration(s)

Texas Instruments may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).