| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C061 WG11 Number: m18084 |
| --- | --- |

| Title: | TE5: TI evaluation of unified intra prediction simplifications |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi Texas Instruments Inc., USA | Tel: Email: | +1-214-480-1861 madhukar@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

In HEVC TMuC-0.7 Intra prediction, pixels in current block are predicted from their neighboring left (side reference) and top pixels (main reference) using angular intra prediction. Prediction of pixels from side reference involves division of angle which is computationally complex to implement. Two proposals were made at the last JCT-VC meeting to simplify intra prediction from side reference – JCTVC-B118 and JCTVC-B093. This contribution presents results of evaluation of these two proposals for simplification of intra prediction. The simplifications are achieved using the following two tools: division elimination (JCTVC-B118, JCTVC-B093) and fractional pixel position calculation optimization (JCTVC-B093). Based on the results presented in this contribution, it is recommended that these two tools be adopted into TMuC and Test Model (TM).

- Introduction

HEVC TMuC-0.7 Unified Intra Prediction (UIP) [3] uses prediction directions with angles of:                      +/- [0,2,5,9,13,17,21,26,32] / 32. Figure 1 illustrates intra prediction process for 4x4 block with positive intra prediction angle (intra prediction angle = 21/32). Black pixels are reference pixels. Colored pixels are predicted pixels. Intercept of colored lines with x-axis denotes the fractional pixel location to use for predicting the corresponding colored pixel. When intra prediction angle is positive, intra prediction is calculated using the following code snippet. deltaInt denotes the integer pel location and deltaFract denotes the fractional pel location. Note that deltaFract is calculated only once every row of pixels.

// Loop over different rows

for (k=0;k<blkSize;k++){

deltaInt   = ((k+1)*intraPredAngle) >> 5;

deltaFract = ((k+1)*absAng) % 32;

// Loop in a row

for (l=0;l<blkSize;l++){

refMainIndex = l+deltaInt+1;

pDst[k*iDstStride+l] = (Pel) ( ((32-deltaFract)*refMain[refMainIndex]

+deltaFract*refMain[refMainIndex+1]+16) >> 5 );

}

Figure 2 illustrates intra prediction process for 4x4 blocks with negative intra prediction values (intra prediction angle = -21/32). Black pixels are reference pixels. Colored pixels are predicted pixels. Intercept of colored lines with x- or y-axis denotes the fractional pixel location to use for predicting the corresponding colored pixel. Code snippet below shows the intra prediction calculations for negative intra prediction angle. Three important things to note from code below is that the top reference pixel or the left reference pixels can get used in prediction process unlike the case of positive intra prediction angle where only the top reference pixels get used (this can be seen by comparing Figures 1 and 2). When the side reference is used, deltaFractSide (the fractional pel location) may need to be calculated every pixel in a row. This can be seen in Figure 2 where the green lines intercept the y-axis at different fractional-pel location. When side reference is used, a division by absAng is needed to calculate deltaIntSide and deltaFractSide. Hence Intra prediction calculation from side reference is more complex than from main reference.

// loop over rows

for (k=0;k<blkSize;k++){

deltaInt   = ((k+1)*intraPredAngle) >> 5;

deltaFract = ((k+1)*absAng) % 32;

if (intraPredAngle < 0){

deltaFract = (32 - deltaFract) % 32;

}

// loop within a row

for (l=0;l<blkSize;l++){

refMainIndex = l+deltaInt+1;

if (refMainIndex >= 0) {

pDst[k*iDstStride+l] = (Pel) ( ((32-deltaFract)*refMain[refMainIndex]

+ deltaFract*refMain[refMainIndex+1]+16) >> 5 );

}

else {

// Interpolate from the side reference

deltaIntSide   = (32*32*(l+1)/absAng) >> 5;

deltaFractSide = (32*32*(l+1)/absAng)  % 32;

refSideIndex   = k+1-deltaIntSide;

pDst[k*iDstStride+l] = (Pel) ( ((32-deltaFractSide)*refSide[refSideIndex]

+ deltaFractSide*refSide[refSideIndex-1]+16) >> 5 );

}

}

}

There were two proposals made at the last JCT-VC meeting to simplify intra prediction from side reference [1] [2].

- JCTVC-B118 Angular intra prediction simplification

JCTVC-B118 eliminates the division by using table lookups and additions. The table uses 9 bit entries to represent reciprocals. Figure 3 shows the effect of this approximation.

Table below lists the BD-rate impact of this modification in TMuC-0.7. There is no average loss in coding efficiency. This data was cross-verified by both DOCOMO and Nokia.

Table: TI rate-distortion performance.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |

- JCTVC-B093 Simplified angular intra prediction

JCTVC-B093 projects the side (vertical) reference and copies the data into main (horizontal) reference. This process is normatively identical to the JCTVC-B118’s process. However, JCTVC-B093 uses 12-bit data to represent reciprocals. The second optimization that JCTVC-B093 introduces is that it used fixed value of deltaFract for all pixels in a row when side reference (in main reference array) is used. This process is depicted in Figure 4 below.

This simplification results in slight warping of prediction signal but this warping does not seem to effect the overall rate-distortion performance as can be seen in Table below. This data was cross-verified by both TI and Nokia.

Table: DOCOMO rate-distortion performance

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | -0.1 | -0.1 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.1 | 0.1 | 0.2 | 0.1 | 0.2 | 0.2 |
| Class D | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 |
| Class E | 0.0 | 0.1 | 0.1 | 0.0 | 0.0 | -0.1 |
| All | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 |

- Conclusions

From the results it was confirmed that the proposed methods do not result in significant difference in coding efficiency compared to the TMuC 0.7.

Both JCTVC-B093 and JCTVC-B118 successfully eliminate the division using a lookup table.

It is recommended that the division elimination (JCTVC-B118, JCTVC-B093) and deltaFract optimization (JCTVC-B093) be added to the TMuC and Test Model (TM).

- References

- F. Bossen et. al., “Simplified angular intra prediction”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-B093, 2nd Meeting: Geneva, CH, 21-28 July, 2010.

- M. Budagavi, “Angular intra prediction and ADI simplification”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-B118, 2nd Meeting: Geneva, CH, 21-28 July, 2010.

- Jung-Hye Min et. al., “Unification of the Directional Intra Prediction Methods in TMuC”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-B100, 2nd Meeting: Geneva, CH, 21-28 July, 2010.

# Patent rights declaration(s)

Texas Instruments Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).