| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C062 WG11 Number: m18085 |
| --- | --- |

| Title: | TE8: TI Parallel context processing (PCP) proposal |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi Texas Instruments Inc., USA | Tel: Email: | +1-214-480-1861 madhukar@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

Context-Adaptive Binary Arithmetic Coding (CABAC) is one of two entropy engines used by the AVC video coding standard. Processing in CABAC engine is highly serial in nature.  Consequently, in order to decode high bitrate video bit-streams in real-time, the CABAC engine needs to be run at extremely high frequencies which consumes a significant amount of power and in the worst case may not be feasible. Techniques to parallelize CABAC can be broadly classified into three categories: bin-level parallelism, syntax element-level parallelism, and slice-level parallelism. Bin-level parallelism techniques such as NBAC/PIPE/V2V parallelize binary arithmetic coder (BAC) of CABAC. However, due to serial bottlenecks in context processing, there is limited overall throughput improvement in the entropy coder. Hence techniques that parallelize context processing are needed. The following three techniques for parallelization of context processing (PCP) were presented at the last JCTVC meeting in JCTVC-B088: Coefficient Sign PCP, Coeff Level BinIdx 0 PCP, significance map PCP and a tool experiment was set-up to study these techniques. This contribution presents the test results of this tool experiment (TE8).

# Introduction

Context-Adaptive Binary Arithmetic Coding (CABAC) is one of two entropy engines used by the AVC video coding standard. Processing in CABAC engine is highly serial in nature.  Consequently, in order to decode high bitrate video bit-streams in real-time, the CABAC engine needs to be run at extremely high frequencies which consumes a significant amount of power and in the worst case may not be feasible.

Figure 1: CABAC block diagram (from [1]).

The serial nature in CABAC comes from the following three blocks:

- Binarizer: Bins from many syntax elements such as coefficient levels and motion vector differences are coded using variable length coding such as unary coding and exp-Golomb coding. Variable length codes are inherently serial in nature.

- Context modeler: The serial dependency comes about since the probability used in the context model for coding the next bin is updated depending on the current bin value. If the current bin value is Least Probable Symbol (LPS), the probability is increased and if the current bin value is Most Probable Symbol (MPS), the probability is decreased. Another source of serial dependency is the context index selection process, where the context index of bin may be determined by the value of previously coded bins.

- Binary arithmetic coder (BAC): Arithmetic coding uses interval subdivision. The range, value, offset used to determine the interval on [0, 1] that uniquely identifies the coded stream of bin values are updated in a serial fashion as and when bins get encoded/decoded.

Parallel entropy coding tools were proposed in several contributions in response to CfP [4-7]. The parallelism proposed in those contributions can be broadly classified into three categories:

- Bin-level parallelism [4][6][7], which parallelizes the BAC

- Syntax element-level parallelism [4], which parallelizes the BAC, the context modeler, and the binarizer

- Sub-slice-level parallelism [4][5].

# Parallel Context Processing (PCP) techniques

Bin-level parallelism techniques such as NBAC/PIPE/V2V reduce serial dependency in the BAC block. However, there is limited overall throughput improvement in the entropy coder due to serial bottlenecks in context processing and binarization. Hence techniques that parallelize context processing are essential for leveraging the throughput gains achieved at bin level. This section presents techniques for parallel context processing (PCP).

## Coefficient Sign PCP

In AVC, sign information is interleaved along with level information as shown in .

for (i = MaxNumCoeff(BlockType)-1;i >= 0;i--)

{

{

Encode coeff_abs_level_minus1[i];

Encode coeff_sign_flag[i];

}

}

Figure 2 : Coding of coefficient level information in AVC CABAC.

Coefficient sign PCP technique codes sign information in a separate plane as shown in Error: Reference source not found to improve parallel context processing efficiency.

for (i = MaxNumCoeff(BlockType)-1;i >= 0;i--)

{

Encode coeff_sign_flag[i];

}

for (i = MaxNumCoeff(BlockType)-1;i >= 0;i--)

{

Encode coeff_abs_level_minus1[i];

}

Figure 3: Coding of coefficient sign information in a separate bin-plane.

## Coeff Level BinIdx 0 PCP

Coefficient coding is carried out in AVC CABAC as shown in Figure 2Error: Reference source not found.

The context (ctxIdxInc) used for coeff_abs_level_minus1 depends on the position of the bin (binIdx) as follows: (extracted from AVC specification)

–	If binIdx is equal to 0, ctxIdxInc is derived by

ctxIdxInc = ( ( numDecodAbsLevelGt1  !=  0 ) ? 0: Min( 4, 1 + numDecodAbsLevelEq1 ) )	(9-1)

–	Otherwise (binIdx is greater than 0), ctxIdxInc is derived by

ctxIdxInc = 5 + Min( 4 − ( ( ctxBlockCat  = =  3 )  ?  1  :  0 ), numDecodAbsLevelGt1 )	(9-2)

binIdx = 0 context processing is different from binIdx 0 bins.

Coeff Level BinIdx 0 PCP technique transmits binIdx = 0 bins in a separate bin-plane. The advantage of doing this is that context processing of Equation (9-24) can be carried out in parallel to context processing of Equation (9-23) i.e. the context processing for all the bins with binIdx 0 for all the coeffs level in a block can be carried out in parallel to bin processing of binIdx = 0 and before the decoding of bins with binIdx 0.

Coefficient sign PCP and Coeff Level BinIdx 0 PCP tools were integrated into TMuC-0.7. Table below shows the BD-Rate degradation and context processing speed up for Coefficient sign PCP and Coeff Level BinIdx 0 PCP. As expected there is no coding loss. A context processing speed up of 2.6X to 3X is achieved while decoding coefficient level and sign information.

|  | Intra - TI-PCP-Run1 (Sign&Bin0) |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 |
| All | 0.0 | 0.0 | 0.0 |
| Avg speedup factor | 2.6 |  |  |
| Max speedup factor | 3.0 |  |  |
|  | Random access - TI-PCP-Run1 (Sign&Bin0) |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | 0.1 |
| Class B | 0.0 | -0.1 | -0.1 |
| Class C | 0.0 | -0.1 | 0.0 |
| Class D | 0.0 | -0.1 | 0.0 |
| Class E |  |  |  |
| All | 0.0 | -0.1 | 0.0 |
| Avg speedup factor | 2.6 |  |  |
| Max speedup factor | 3.0 |  |  |
|  | Low delay - TI-PCP-Run1 (Sign&Bin0) |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.0 | 0.1 | 0.2 |
| Class C | 0.0 | -0.1 | 0.1 |
| Class D | 0.0 | 0.0 | 0.2 |
| Class E | 0.0 | 0.2 | 0.5 |
| All | 0.0 | 0.1 | 0.2 |
| Avg speedup factor | 2.2 |  |  |
| Max speedup factor | 3.0 |  |  |

## Significance Map PCP

Significance map coding is carried out in AVC CABAC as shown in Error: Reference source not found below.

for (i = 0 ;i < MaxNumCoeff(BlockType)-1 ;i++)

{

Encode significant_coeff_flag[i];

if(significant_coeff_flag[i])

Encode last_significant_coeff_flag[i];

if (last_significant_coeff_flag[i])

break ;

}

Figure 4 : Significance map coding in AVC CABAC.

last_significant_coeff_flag is transmitted only when a coeff is significant. This introduces serial dependency in decoding of significance map. If Nx throughput improvement is needed, then speculative computation at every bin is needed which leads to complex logic as shown in Error: Reference source not found(a). Speculative computation at every bin also results in increased power consumption.

Figure 5: Parallelization of context processing for significance map

Significance map PCP technique parallelizes significance map coding by transmitting a last_significant_coeff_flag (LAST) only once per N significant_coeff_flag (SIG) bins.  If all of the N significant_coeff_flag are zero, last_significant_coeff_flag is not transmitted. Error: Reference source not found shows the pseudo-code of Significance map PCP technique labeled as N-SIG, 1-LAST (NSIG1L).

for (i = 0 ;i < MaxNumCoeff(BlockType)-1 ;i += K)

{

if(i+K < MaxNumCoeff(BlockType)-1)

j1 = K ;

else

j1 = K-1 ;

sig = 0 ;

for (j = 0 ;j < j1 ;j++)

{

Encode significant_coeff_flag[i*K+j];

sig += significant_coeff_flag[i*K+j]

}

if(sig)

Encode last_significant_coeff_flag[i];

if (last_significant_coeff_flag[i])

break ;

}

Figure 6: Pseduo-code of Significance map PCP technique (N-SIG, 1-LAST).

Significance map PCP reduces the number of LAST bins that need to be transmitted, but it increases the number of SIG bins that need to be transmitted. However, there is overall reduction in the number of significance map bins that need to be processed. This is in addition to reduction in number of context that need to be speculatively fetched for parallel processing.

Significance map PCP was integrated into TMuC-0.7 with HHI_TRANSFORM_CODING disabled. Also a last was transmitted once per 2 significance map bins (denoted as 2SIG1L). Table below lists the BD-rate performance and speed up factors due to reduction in number of bins. There is an average bitrate increase of around 0.1-0.3%. There is about 10% average reduction in bins for Intra and Random access configurations. There is no average reduction in bins for Low delay configuration.

|  | Intra - TI-PCP-Run2 (Sigmap PCP) |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.4 | 1.6 | 1.6 |
| Class B | 0.3 | 1.4 | 1.5 |
| Class C | 0.2 | 1.7 | 1.6 |
| Class D | 0.2 | 1.4 | 1.5 |
| Class E | 0.7 | 1.1 | 1.5 |
| All | 0.3 | 1.5 | 1.5 |
| Avg speedup factor due to reduction of bins | 1.1 |  |  |
| Max speedup factor due to reduction of bins | 1.2 |  |  |
|  | Random access - TI-PCP-Run2 (Sigmap PCP) |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 1.0 | 0.8 |
| Class B | 0.1 | 1.1 | 1.0 |
| Class C | 0.0 | 1.0 | 1.0 |
| Class D | 0.1 | 0.7 | 0.5 |
| Class E |  |  |  |
| All | 0.1 | 1.0 | 0.8 |
| Avg speedup factor due to reduction of bins | 1.1 |  |  |
| Max speedup factor due to reduction of bins | 1.3 |  |  |
|  | Low delay - TI-PCP-Run2 (Sigmap PCP) |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.1 | 0.7 | 1.0 |
| Class D | -0.1 | 1.5 | 1.4 |
| Class E | 0.1 | 3.4 | 2.1 |
| All | #VALUE! | #VALUE! | #VALUE! |
| Avg speedup factor due to reduction of bins | 1.0 |  |  |
| Max speedup factor due to reduction of bins | 1.2 |  |  |

# Conclusions

Coefficient sign PCP and Coeff Level BinIdx 0 PCP tools provide a 2.6X to 3X speed up in context processing of coefficient sign and levels with no coding loss. Hence we recommend that both these techniques be adopted into TMuC and TM.

Sigmap PCP reduces the number of bins generated by about 10% for Intra and Random access configurations. There is no reduction in number of bins generated for Low Delay configuration. In addition, sigmap PCP reduces number of context that need to be speculatively fetched for parallel processing. There is factor of 4X reduction in the worst case number of contexts that need to be speculatively fetched for achieving 3X parallelism with 2SIG1L Sigmap PCP scheme that was implemented.

# References

- http://iphome.hhi.de/marpe/cabac.html.

- JCT-VC, “Test Model under Consideration”, JCTVC-A205, Joint Collaborative Team on Video Coding meeting, 15-23 April 2010, Dresden, Germany.

- V. Sze and M. Budagavi, “Parallel CABAC,” ITU-T SG 16/Q.6 Doc. COM16-C334, Geneva, Switzerland, April 2008.

- M. Budagavi, V. Sze, M. U. Demircin, S. Dikbas, M. Zhou, and A. P. Chandrakasan, “Video coding technology proposal by Texas Instruments,” JCTVC-A101, Joint Collaborative Team on Video Coding meeting, 15-23 April 2010, Dresden, Germany.

- A. Segall, T. Yamamoto, J. Zhao, Y. Kitaura, Y. Yasugi, and T. Ikai, “Video coding technology proposal by Sharp,” JCTVC-A105, Joint Collaborative Team on Video Coding meeting, 15-23 April 2010, Dresden, Germany..

- M. Winken et. al., “Video coding technology proposal by Fraunhofer HHI,” JCTVC-A116, Joint Collaborative Team on Video Coding meeting, 15-23 April 2010, Dresden, Germany..

- D. He, G. Korodi, G. Martin-Cocher, E.-h. Yang, X. Yu, and J. Zan, “Video coding technology proposal by Research in Motion,” JCTVC-A120, Joint Collaborative Team on Video Coding meeting, 15-23 April 2010, Dresden, Germany.

- JCT-VC, TMuC-0.1 Reference Software, https://hevc.hhi.fraunhofer.de/svn/svn_TMuCSoftware/tags/0.1/  and http://hevc.kw.bbc.co.uk/git/w/jctvc-tmuc.git.

- V. Sze, M. Budagavi, and A. P. Chandrakasan, “Massively Parallel CABAC,” ITU-T SGI 6/Q.6 Doc. VCEG-AL21, Geneva, CH, July, 2009.

# Patent rights declaration(s)

Texas Instruments Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).