| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C065 WG11 Number: m18088 |
| --- | --- |

| Title: | Testing results on the simplified TMuC configurations |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA | Tel: Email: | +1-214-480-3816 zhou@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This document reports the evaluation results of simplified TMuC0.7.4 configurations, the tools included in the simplified configurations are identical to the ones proposed in JCTVC-C122, with the exception that the CABAC instead of PIPE is used in the high efficiency configurations and 12-tap instead of 6-tap DCTIF is used for the loc complexity configurations. Also tested is the performance of the DCTIF with different filter taps (6, 8, 10, and 12) under the simplified configurations. Based on the testing results, it is recommended to use CABAC in the HE reference configurations due to uncertainty of the PIPE development for throughput improvement purpose, and considering 8-tap DCIIF for both HE and LOCO reference configurations.

# Introduction

In this contribution the performance of the simplified TMuC-0.7.4 configurations are evaluated. The proposed simplified configurations in this contribution are identical to the ones proposed in Table 1 and Table 2 of JCTVC-C122, with the exception that the CABAC instead of PIPE is used in the high efficiency configurations and 12-tap instead of 6-tap DCTIF is used for the loc complexity configurations. As a single interpolation filter is desirable for the HEVC standard, the DCTIF performance of different filter taps are also evaluated in this contribution to determine the good trade-off of filter tap length for both HE and LOCO configurations.

# Test Conditions

The TMuC0.7.4 software is used for the evaluation. The simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines, and the common test conditions and reference configurations specified in [2] are followed to create anchor data for TMuC-0.7.4. The simplified configurations as proposed in JCTVC-C122 [1] with the exception of using CABAC in HE configurations and 12-tap DCTIF in LOCO configurations are used to create test data for the simplified configurations.

To evaluate the DCTIF performance of different filter taps, the simplified configurations are used except that the DCTIF filter tap is configured to 6, 8, 10, and 12 for testing purpose.

# Test Results

## Performance comparison of simplified settings against TMuC-0.7.4 anchor

As shown in the table below, for the HE configurations the coding loss of the simplified configurations ranges 4.1 to 7.7% compared to the TMuC-0.7.4 anchor (note that LCECPHASE2 is set to 1 in the anchor), but the encode and decode  time is reduced to 11% to 36% and 11% to 33% of anchor time, respectively. Note that the HE results in table 1 are identical to the HE results of Table 4 of JCTVC-C122  provided by Mitsubishi.

For the LOCO configurations, loss in the intra LoCo configurations is 6.8% but the encode time is reduced to 8% and decode time is reduced to 27% of anchor time; there are 5.8% to 6.6% gains in the random access and low-delay LoCo configurations, the encode/decode time is reduced to 66%,  and 23% to 26% of anchor, respectively.

For detailed results please refer to TMuC-0.7.4-TI-DCTIF12vsAnchor.xls.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 7.4 | 7.6 | 5.2 | 4.7 | 3.0 | 1.2 |
| Class B | 6.7 | 5.2 | 4.2 | 6.3 | 5.6 | 4.7 |
| Class C | 7.9 | 5.9 | 5.1 | 7.5 | 5.3 | 5.1 |
| Class D | 6.0 | 4.1 | 3.8 | 5.6 | 3.5 | 3.5 |
| Class E | 11.3 | 6.6 | 7.1 | 9.6 | 2.3 | 3.6 |
| All | 7.7 | 5.6 | 4.9 | 6.8 | 4.2 | 3.9 |
| Enc Time[%] | 11% |  |  | 8% |  |  |
| Dec Time[%] | 43% |  |  | 27% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 4.5 | 3.1 | -0.3 | -1.2 | -7.1 | -8.4 |
| Class B | 5.0 | 1.5 | -0.2 | -2.1 | -7.8 | -9.5 |
| Class C | 4.4 | 1.3 | 0.1 | -5.1 | -6.1 | -7.0 |
| Class D | 2.7 | 0.0 | -0.8 | -13.4 | -11.8 | -12.1 |
| Class E |  |  |  |  |  |  |
| All | 4.1 | 1.3 | -0.3 | -5.8 | -8.3 | -9.4 |
| Enc Time[%] | 36% |  |  | 66% |  |  |
| Dec Time[%] | 31% |  |  | 23% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 4.3 | 4.5 | 2.0 | -2.3 | -7.8 | -8.5 |
| Class C | 3.2 | 0.4 | -1.2 | -4.8 | -9.0 | -9.3 |
| Class D | 4.1 | -2.3 | -2.4 | -11.5 | -14.7 | -16.6 |
| Class E | 6.1 | 10.3 | 9.3 | -9.8 | -20.7 | -25.6 |
| All | 4.3 | 2.9 | 1.5 | -6.6 | -12.2 | -13.9 |
| Enc Time[%] | 36% |  |  | 66% |  |  |
| Dec Time[%] | 33% |  |  | 26% |  |  |

Table 1  BD-Rate and encode/decode time comparison  between the proposed simplified configurations and TMuC-0.7.4 anchor

## Performance comparison of simplified configurations against JCTVC-C122 configurations

As mentioned early, the only difference between the proposed simplified configurations and the configurations of JCTVC-C122 is that CABAC instead of PIPE is used in HE configurations and 12-tap DCTIF instead 6-tap DCTIF is used in LOCO configurations. As shown in the table 2 below, using CABAC in the HE configurations provides some marginal coding gains (0.1% to 0.2%). However, using the 12-tap DCTIF in LOCO configurations provides 3% to 4.3% coding gain. Results reveal that the PIPE is not providing coding gain against CABAC despite of more dependency has been added to the context modeling part. Adding the dependency to the context modeling part essentially departs from the original design PIPE goal of the improved throughputs.

For detailed results please refer to TMuC-0.7.4-TI-DCTIF12vsJCTVC-C122.xls.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | -0.1 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 101% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 | -0.2 | 0.4 | 0.3 |
| Class B | -0.1 | -0.1 | -0.1 | -0.4 | 0.3 | 0.3 |
| Class C | -0.1 | -0.1 | -0.1 | -3.5 | -0.3 | -0.5 |
| Class D | -0.2 | -0.2 | -0.2 | -7.1 | -2.4 | -2.1 |
| Class E |  |  |  |  |  |  |
| All | -0.1 | -0.1 | -0.1 | -3.0 | -0.6 | -0.5 |
| Enc Time[%] | 99% |  |  | 107% |  |  |
| Dec Time[%] | 96% |  |  | 112% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.1 | -0.1 | -0.1 | -1.7 | -0.5 | -0.4 |
| Class C | -0.2 | -0.2 | -0.2 | -4.5 | -2.8 | -3.0 |
| Class D | -0.3 | -0.3 | -0.3 | -7.8 | -7.2 | -7.4 |
| Class E | -0.4 | -0.4 | -0.4 | -3.6 | -2.0 | -1.0 |
| All | -0.2 | -0.2 | -0.2 | -4.3 | -3.0 | -2.9 |
| Enc Time[%] | 99% |  |  | 108% |  |  |
| Dec Time[%] | 97% |  |  | 114% |  |  |

Table 2  BD-Rate and encode/decode time comparison  between the simplified configurations and JCTVC-C122 configurations

## Performance evaluations of DCTIF filter taps (6, 8, 10, 12)

The DCTIF performance of different filter taps is evaluated under the proposed simplified configurations. The testing results against 12-tap DCTIF are summarized in table 3. As it is desirable to have a single interpolation filter for both the HE and LOCO configurations to simplify the design, 8-tap DCTIF appears to be a good trade-off between coding efficiency and memory bandwidth. Overall coding efficiency is improved, because using 8-tap DCTIF instead of 12-tap DCTIF in HE configuration causes a coding loss of 0.5% to 0.6%, which can be compensated by a coding gain of 1.8% to 2.2% in LOCO configurations by using 8-tap DCTIF instead of 6-tap DCTIF. For detailed results please refer to

TMuC-0.7.4-TI-DCTIF12vsDCTIF10.xls

TMuC-0.7.4-TI-DCTIF12vsDCTIF8.xls

TMuC-0.7.4-TI-DCTIF12vsDCTIF6.xls

|  | Random access (Y BD-rate) |  |  | Low delay (Y BD-rate) |  |  | Random access LoCo (Y BD-rate) |  |  | Low delay LoCo (Y BD-rate) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10-tap | 8-tap | 6-tap | 10-tap | 8-tap | 6-tap | 10-tap | 8-tap | 6-tap | 10-tap | 8-tap | 6-tap |
| Class A | -0.3 | 0.0 | -0.1 |  |  |  | -2.3 | -0.3 | -0.2 |  |  |  |
| Class B | -0.4 | 0.0 | -0.4 | -1.0 | 0.1 | -0.6 | -2.1 | -0.5 | -0.4 | -3.7 | -1.3 | -1.7 |
| Class C | -0.6 | -0.8 | -2.2 | -1.1 | -0.9 | -2.5 | -1.2 | -1.1 | -3.5 | -2.5 | -1.6 | -4.5 |
| Class D | -1.2 | -1.4 | -4.3 | -1.6 | -0.9 | -4.3 | -1.9 | -2.5 | -7.1 | -3.6 | -3.2 | -7.8 |
| Class E |  |  |  | -2.2 | -0.3 | -1.0 |  |  |  | -6.5 | -2.4 | -3.6 |
| All | -0.6 | -0.6 | -1.9 | -1.4 | -0.5 | -2.1 | -1.8 | -1.2 | -3.0 | -3.9 | -2.1 | -4.3 |

Table 3  BD-Rate comparison for DCTIF filter taps (6, 8, 10 taps vs. 12 taps). Negative values mean loss.

Overall memory bandwidth for motion estimation and motion compensation is reduced too because the filter tap in the HE is reduced from 12-tap to 8-tap, while it is only increased from 6-tap to 8-tap in the LOCO. As shown in Table 4, in no cache mode, the overall memory bandwidth is reduced by about 20% by using 8-tap filter instead of 12-tap in the HE configurations, while moving from 6-tap to 8-tap filter in the LOCO configurations the overall memory bandwidth only increases about 10%.

|  | Random access |  |  | Low delay |  |  | Random access LoCo |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10-tap | 8-tap | 6-tap | 10-tap | 8-tap | 6-tap | 10-tap | 8-tap | 6-tap | 10-tap | 8-tap | 6-tap |
| Class A | -13.5 | -19.7 | -30.2 |  |  |  | -10.2 | -15.1 | -23.7 |  |  |  |
| Class B | -12.4 | -18.0 | -27.5 | -13.3 | -19.3 | -30.2 | -10.2 | -14.6 | -22.4 | -11.4 | -16.3 | -25.7 |
| Class C | -13.4 | -19.4 | -30.4 | -14.1 | -20.5 | -32.4 | -11.1 | -16.2 | -25.2 | -12.8 | -18.8 | -28.8 |
| Class D | -15.2 | -21.9 | -34.8 | -15.2 | -23.3 | -37.7 | -12.9 | -18.7 | -29.3 | -14.8 | -21.0 | -32.4 |
| Class E |  |  |  | -8.1 | -11.9 | -18.3 |  |  |  | -5.4 | -7.9 | -12.5 |
| All | -13.5 | -19.6 | -30.6 | -13.0 | -19.2 | -30.4 | -11.2 | -16.2 | -25.2 | -11.5 | -16.5 | -25.7 |

Table 4 Decoder memory bandwidth savings (%) for DCTIF filter taps (6, 8, 10 taps vs. 12 taps). Negative values mean  saving. No Cache 64bit/128bit mode.

Table 5 shows the memory bandwidth compression in cache mode. About 6% memory bandwidth reduction is achieved by replacing 12-tap with 8-tap filter in HE, while memory bandwidth is increased by about 3% by using 8-tap instead of 6-tap filter in LOCO.

|  | Random access |  |  | Low delay |  |  | Random access LoCo |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10-tap | 8-tap | 6-tap | 10-tap | 8-tap | 6-tap | 10-tap | 8-tap | 6-tap | 10-tap | 8-tap | 6-tap |
| Class A | -3.7 | -5.6 | -8.9 |  |  |  | -5.4 | -8.5 | -13.7 |  |  |  |
| Class B | -3.7 | -5.5 | -8.8 | -4.1 | -6.2 | -9.8 | -3.3 | -4.8 | -8.0 | -3.5 | -5.2 | -8.7 |
| Class C | -3.9 | -5.8 | -9.0 | -4.6 | -6.9 | -10.8 | -3.0 | -4.5 | -7.2 | -3.5 | -5.2 | -8.5 |
| Class D | -4.2 | -6.2 | -9.2 | -4.3 | -6.4 | -9.9 | -1.5 | -2.2 | -3.4 | -1.9 | -2.8 | -4.7 |
| Class E |  |  |  | -3.0 | -4.3 | -6.4 |  |  |  | -2.1 | -3.2 | -5.2 |
| All | -3.9 | -5.8 | -9.0 | -4.1 | -6.1 | -9.4 | -3.0 | -4.5 | -7.3 | -2.8 | -4.2 | -7.0 |

Table 5 Decoder memory bandwidth savings (%) for DCTIF filter taps (6, 8, 10 taps vs. 12 taps). Negative values mean  saving. Cache mode.

The memory bandwidth numbers are measured on the motion vector fields of TMuC-0.7 reference configurations by looping through the DCTIF filter taps in the memory bandwidth measurement routines. The detailed results are provided in

TI_DecMemBW_DCTIF_6tapvs12tap.xls

TI_DecMemBW_DCTIF_8tapvs12tap.xls

TI_DecMemBW_DCTIF_10tapvs12tap.xls

# Conclusions

Based on the testing results, it is recommended to use CABAC in the HE reference configurations due to uncertainty of the PIPE development for throughput improvement purpose, and considering 8-tap DCIIF for both HE and LOCO reference configurations to achieve a single interpolation filter design for the HEVC standard.

# References

- JCTVC-C122, “A suggested starting point for HEVC Test Model”. JCTVC-C122, 3rd JCTVC meeting, Guangzhou, CN, October 2010

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-B300, 2nd. JCT-VC Meeting, Geneva, CH, July 2010.