| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | DoCTBment: JCTVC-C067 WG11 Number: m18090 |
| --- | --- |

| Title: | TE9-1 report on performance tests for different CTU and TU sizes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | 1)Jaeil Kim, Bumshik Lee, Sangsoo Ahn Munchurl Kim 2)Hui Yong Kim, Jongho Kim, Seyoon Jeong, Jinsoo Choi 3)Kazushi Sato 4)Lu Yu, Xiaolin Shen 5)Krit Panusopone, Faisal Ishtiaq 6)Kiho Choi, Euee S. Jang 7)Woo-Jin Han 8) Seoung-Jun Oh, Jieon Kim, Dong Gyu Sim, Sea-Nae Park, Hyo-Min Choi | Tel: Email: | +82-42-350-7419 mkim@ee.kaist.ac.kr |
| Source: | KAIST1), ETRI2), SONY3), Zhejiang University4), Motorola5), Hanyang University6), Samsung Electronics7), Kwangwoon University8) |  |  |

_____________________________

# Abstract

- Introduction

This report provides partial experimental results of TE9: Large Block Structure [1]. The experimental results in this report are obtained for the performance tests for different CTB and TU sizes of TMuC 0.7.2 for Table 1 in Sec. 3.1 of [1].

The purpose of the experiments for Table 1 is to assess the performance of large block structure in Test Model under Consideration [2] for different combinations of CTB and TU sizes. For the experiments, the three sets of tests are defined as follows:

Set 0: Effect of variable CTB and TU sizes

- Varying max CTB size from 16x16 to 128x128 of full depth with variable max TU sizes of full depth

Set 1: Effect of variable max CTB size (virtual experimental set)

- Varying max CTB size from 16x16 to 128x128 of full depth with max TU sizes of 16x16 and 8x8

Set 2: Effect of CTB depths

- Varying max CTB size from 16x16 to 128x128 of variable depths with variable max TU sizes of full depths

In order to accomplish the experiments with limited computational resources within a limit amount of times, the whole experiment sets with various combinations of different max CTB and max TM sizes are divided and distributed for TE9 participants. All the experimental results in this report are integrated based on the results carried by the TE9 participants.

For the experiments, TMuC software version 0.7.2 are used with modification of PADDING == 64 for which the original padding is set to PADDING == 16. This modification is inevitable because PAD==16 option is not appropriate due to the constraint that the frame width and height should be multiples of min CU and min TU. Notice that the current PAD option in TMuC simply adds padding pixels to make both width and height to be multiple of 16 (MAX_PAD_SIZE macro).

One simple way is to change MAX_PAD_SIZE macro to 64 and use PAD=1 for Set 2 to ensure that both frame width and height be a multiple of 64 to cover the extreme case, MaxCU=128, Depth=2.

Furthermore, in order to statistically analyze the effects of different CTB and TU size, a patch of codes are added into the TMuC software version 0.7.2 to collect the finally selected modes of CTB and TU sizes.

- Experiments on performance tests for different CTB and TU sizes

## Experiment Sets

| No. | Set 0 (KAIST, ETRI, Kwangwoon University) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Max CTB Sizes | CTB Depths | Max TU Sizes | TU Depths | Available TU sizes | Responsible organizations |
| s0-0 | 128x128 | 5 | 64x64 | QuadtreeTULog2MaxSize=6, QuadtreeTULog2MinSize=2 | 64x64, 32x32, 16x16, 8x8, 4x4 | KAIST |
| s0-1 |  |  | 32x32 | QuadtreeTULog2MaxSize=5, QuadtreeTULog2MinSize=2 | 32x32, 16x16, 8x8, 4x4 | KAIST |
| s0-2 |  |  | 16x16 | QuadtreeTULog2MaxSize=4, QuadtreeTULog2MinSize=2 | 16x16, 8x8, 4x4 | KAIST |
| s0-3 |  |  | 8x8 | QuadtreeTULog2MaxSize=3, QuadtreeTULog2MinSize=2 | 8x8, 4x4 | KAIST |
| s0-4 | 64x64 | 4 | 64x64 | QuadtreeTULog2MaxSize=6, QuadtreeTULog2MinSize=2 | 64x64, 32x32, 16x16, 8x8, 4x4 | ETRI/Kwangwoon University/Motorola |
| s0-5 |  |  | 32x32 | QuadtreeTULog2MaxSize=5, QuadtreeTULog2MinSize=2 | 32x32, 16x16, 8x8, 4x4 | ETRI/Kwangwoon University |
| s0-6 |  |  | 16x16 | QuadtreeTULog2MaxSize=4, QuadtreeTULog2MinSize=2 | 16x16, 8x8, 4x4 | ETRI/Kwangwoon University |
| s0-7 |  |  | 8x8 | QuadtreeTULog2MaxSize=3, QuadtreeTULog2MinSize=2 | 8x8, 4x4 | ETRI/Kwangwoon University |
| s0-8 | 32x32 | 3 | 32x32 | QuadtreeTULog2MaxSize=5, QuadtreeTULog2MinSize=2 | 32x32, 16x16, 8x8, 4x4 | ETRI/Kwangwoon University/Motorola/ Zhejiang University |
| s0-9 |  |  | 16x16 | QuadtreeTULog2MaxSize=4, QuadtreeTULog2MinSize=2 | 16x16, 8x8, 4x4 | KAIST/Zhejiang University |
| s0-10 |  |  | 8x8 | QuadtreeTULog2MaxSize=3, QuadtreeTULog2MinSize=2 | 8x8, 4x4 | KAIST/Zhejiang University |
| s0-11 | 16x16 | 2 | 16x16 | QuadtreeTULog2MaxSize=4, QuadtreeTULog2MinSize=2 | 16x16, 8x8, 4x4 | KAIST/Zhejiang University |
| s0-12 |  |  | 8x8 | QuadtreeTULog2MaxSize=3, QuadtreeTULog2MinSize=2 | 8x8, 4x4 | ETRI/Kwangwoon University/ Zhejiang University |

| No. | Set 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Max TU Sizes | TU Depth | Max CTB Sizes | CTB Depth (MaxPartitionDepth) | Available CTB size | Responsible organizations |
| s1-0 | 16x16 | QuadtreeTULog2MaxSize=4, QuadtreeTULog2MinSize=2 | 128x128 | 5 | 128x128, 64x64, 32x32, 16x16 | same as s0-2 |
| s1-1 |  |  | 64x64 | 4 | 64x64, 32x32, 16x16 | same as s0-6 |
| s1-2 |  |  | 32x32 | 3 | 32x32, 16x16 | same as s0-9 |
| s1-3 | 8x8 | QuadtreeTULog2MaxSize=3, QuadtreeTULog2MinSize=2 | 128x128 | 5 | 128x128, 64x64, 32x32, 16x16 | same as s0-3 |
| s1-4 |  |  | 64x64 | 4 | 64x64, 32x32, 16x16 | same as s0-7 |
| s1-5 |  |  | 32x32 | 3 | 32x32, 16x16 | same as s0-10 |
| s1-6 |  |  | 16x16 | 2 | 16x16 | same as s0-12 |

| No. | Set 2 (KAIST, Samsung Electronics, SONY, Hanyang University, Motorola) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Max CTB Sizes | CTB Depths | Max TU Sizes | TU Depth | Available TU size | Responsible organizations |
| s2-0 | 128x128 | 2 | 64x64 | QuadtreeTULog2MaxSize=6, QuadtreeTULog2MinSize=2 | 64x64, 32x32, 16x16, 8x8, 4x4 | KAIST |
| s2-1 |  |  | 32x32 | QuadtreeTULog2MaxSize=5, QuadtreeTULog2MinSize=2 | 32x32, 16x16, 8x8, 4x4 | KAIST |
| s2-2 |  |  | 16x16 | QuadtreeTULog2MaxSize=4, QuadtreeTULog2MinSize=2 | 16x16, 8x8, 4x4 | Samsung Electronics |
| s2-3 |  |  | 8x8 | QuadtreeTULog2MaxSize=3, QuadtreeTULog2MinSize=2 | 8x8, 4x4 | Samsung Electronics |
| s2-4 | 128x128 | 3 | 64x64 | QuadtreeTULog2MaxSize=6, QuadtreeTULog2MinSize=2 | 64x64, 32x32, 16x16, 8x8, 4x4 | Samsung Electronics |
| s2-5 |  |  | 32x32 | QuadtreeTULog2MaxSize=5, QuadtreeTULog2MinSize=2 | 32x32, 16x16, 8x8, 4x4 | Samsung Electronics |
| s2-6 |  |  | 16x16 | QuadtreeTULog2MaxSize=4, QuadtreeTULog2MinSize=2 | 16x16, 8x8, 4x4 | Samsung Electronics |
| s2-7 |  |  | 8x8 | QuadtreeTULog2MaxSize=3, QuadtreeTULog2MinSize=2 | 8x8, 4x4 | Samsung Electronics |
| s2-8 | 128x128 | 4 | 64x64 | QuadtreeTULog2MaxSize=6, QuadtreeTULog2MinSize=2 | 64x64, 32x32, 16x16, 8x8, 4x4 | SONY |
| s2-9 |  |  | 32x32 | QuadtreeTULog2MaxSize=5, QuadtreeTULog2MinSize=2 | 32x32, 16x16, 8x8, 4x4 | SONY |
| s2-10 |  |  | 16x16 | QuadtreeTULog2MaxSize=4, QuadtreeTULog2MinSize=2 | 16x16, 8x8, 4x4 | SONY |
| s2-11 |  |  | 8x8 | QuadtreeTULog2MaxSize=3, QuadtreeTULog2MinSize=2 | 8x8, 4x4 | SONY |
| s2-12 | 64x64 | 2 | 64x64 | QuadtreeTULog2MaxSize=6, QuadtreeTULog2MinSize=2 | 64x64, 32x32, 16x16, 8x8, 4x4 | SONY |
| s2-13 |  |  | 32x32 | QuadtreeTULog2MaxSize=5, QuadtreeTULog2MinSize=2 | 32x32, 16x16, 8x8, 4x4 | SONY |
| s2-14 |  |  | 16x16 | QuadtreeTULog2MaxSize=4, QuadtreeTULog2MinSize=2 | 16x16, 8x8, 4x4 | Hanyang University |
| s2-15 |  |  | 8x8 | QuadtreeTULog2MaxSize=3, QuadtreeTULog2MinSize=2 | 8x8, 4x4 | Hanyang University |
| s2-16 | 64x64 | 3 | 64x64 | QuadtreeTULog2MaxSize=6, QuadtreeTULog2MinSize=2 | 64x64, 32x32, 16x16, 8x8, 4x4 | Hanyang University |
| s2-17 |  |  | 32x32 | QuadtreeTULog2MaxSize=5, QuadtreeTULog2MinSize=2 | 32x32, 16x16, 8x8, 4x4 | Hanyang University |
| s2-18 |  |  | 16x16 | QuadtreeTULog2MaxSize=4, QuadtreeTULog2MinSize=2 | 16x16, 8x8, 4x4 | Hanyang University |
| s2-19 |  |  | 8x8 | QuadtreeTULog2MaxSize=3, QuadtreeTULog2MinSize=2 | 8x8, 4x4 | Hanyang University |
| s2-20 | 64x64 | 4 | 64x64 | QuadtreeTULog2MaxSize=6, QuadtreeTULog2MinSize=2 | 64x64, 32x32, 16x16, 8x8, 4x4 | same as s0-4 |
| s2-21 |  |  | 32x32 | QuadtreeTULog2MaxSize=5, QuadtreeTULog2MinSize=2 | 32x32, 16x16, 8x8, 4x4 | same as s0-5 |
| s2-22 |  |  | 16x16 | QuadtreeTULog2MaxSize=4, QuadtreeTULog2MinSize=2 | 16x16, 8x8, 4x4 | same as s0-6 |
| s2-23 |  |  | 8x8 | QuadtreeTULog2MaxSize=3, QuadtreeTULog2MinSize=2 | 8x8, 4x4 | same as s0-7 |
| s2-24 | 32x32 | 2 | 32x32 | QuadtreeTULog2MaxSize=5, QuadtreeTULog2MinSize=2 | 32x32, 16x16, 8x8, 4x4 | Motorola |
| s2-25 |  |  | 16x16 | QuadtreeTULog2MaxSize=4, QuadtreeTULog2MinSize=2 | 16x16, 8x8, 4x4 | Motorola |
| s2-26 |  |  | 8x8 | QuadtreeTULog2MaxSize=3, QuadtreeTULog2MinSize=2 | 8x8, 4x4 | Motorola |
| s2-27 | 32x32 | 3 | 32x32 | QuadtreeTULog2MaxSize=5, QuadtreeTULog2MinSize=2 | 32x32, 16x16, 8x8, 4x4 | same as s0-8 |
| s2-28 |  |  | 16x16 | QuadtreeTULog2MaxSize=4, QuadtreeTULog2MinSize=2 | 16x16, 8x8, 4x4 | same as s0-9 |
| s2-29 |  |  | 8x8 | QuadtreeTULog2MaxSize=3, QuadtreeTULog2MinSize=2 | 8x8, 4x4 | same as s0-10 |

## Performances

Table 1-1 Performances on two max TU sizes (64x64 and 32x32) with max CU size (128x128) of full depth (5): (s0-0, CU size = 128, full depth, 64x64~4x4 TU) vs. (s0-1, CU size = 128, full depth, 32x32~4x4 TU)

|  | Intra |  |  | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | 0.1 | 0.3 | 0.3 | 0.1 | 0.5 | 0.6 |  |  |  |
| Class B | 0.4 | 1.1 | 1.0 | 0.7 | 1.6 | 1.5 | 0.6 | 4.3 | 5.7 |
| Class C | 0.0 | 0.3 | 0.2 | 0.3 | 0.4 | 0.3 | 0.3 | 1.2 | 1.3 |
| Class D | 0.1 | 0.1 | 0.1 | 0.1 | 0.4 | 0.3 | 0.0 | 0.8 | 0.7 |
| Class E | 0.2 | 1.8 | 1.3 |  |  |  | 1.9 | 9.2 | 6.3 |
| All | 0.2 | 0.7 | 0.6 | 0.4 | 0.8 | 0.8 | 0.6 | 3.6 | 3.4 |

Table 1-2 Performances on two max TU sizes (64x64 and 16x16) with max CU size (128x128) of full depth (5): (s0-0, CU size = 128, full depth, 64x64~4x4 TU) vs. (s0-2, CU size = 128, full depth, 16x16~4x4 TU)

|  | Intra |  |  | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | 0.6 | 1.5 | 1.7 | 1.1 | 3.0 | 3.1 |  |  |  |
| Class B | 2.2 | 4.1 | 4.4 | 3.4 | 6.9 | 7.2 | 3.0 | 16.4 | 21.4 |
| Class C | 0.2 | 0.8 | 0.6 | 1.0 | 2.2 | 2.0 | 0.9 | 6.2 | 6.8 |
| Class D | 0.2 | 0.7 | 0.5 | 0.4 | 1.8 | 1.4 | 0.2 | 5.5 | 4.8 |
| Class E | 1.6 | 7.7 | 6.6 |  |  |  | 6.2 | 33.5 | 22.7 |
| All | 1.0 | 2.9 | 2.7 | 1.6 | 3.8 | 3.7 | 2.4 | 14.3 | 13.8 |

Table 1-3 Performances on two max TU sizes (64x64 and 8x8) with max CU size (128x128) of full depth (5): (s0-0, CU size = 128, full depth, 64x64~4x4 TU) vs. (s0-3, CU size = 128, full depth, 8x8~4x4 TU)

|  | Intra |  |  | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | 4.2 | 5.9 | 8.3 | 6.0 | 11.1 | 13.9 |  |  |  |
| Class B | 7.9 | 10.1 | 12.9 | 13.1 | 20.7 | 22.6 | 11.5 | 41.7 | 54.6 |
| Class C | 2.0 | 3.6 | 4.0 | 5.0 | 8.9 | 9.1 | 4.3 | 17.7 | 20.2 |
| Class D | 1.4 | 3.3 | 3.0 | 6.6 | 9.2 | 8.4 | 2.1 | 19.1 | 17.1 |
| Class E | 8.7 | 25.4 | 21.4 |  |  |  | 25.6 | 100.4 | 75.1 |
| All | 4.9 | 9.2 | 9.6 | 8.3 | 13.2 | 14.0 | 10.0 | 41.0 | 40.5 |

Table 1-4 Performances on two max CU sizes (128x128 and 64x64) with max TU size (64x64) of full depth (5): (s0-0, CU size = 128, full depth, 64x64~4x4 TU) vs. (s0-4, CU size = 64, full depth, 64x64~4x4 TU)

|  | Intra |  |  | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | 0.1 | 0.1 | 0.2 | 0.5 | 0.2 | 0.5 |  |  |  |
| Class B | 0.1 | 0.1 | 0.1 | 1.0 | 0.5 | 0.4 | 0.4 | 0.3 | 0.6 |
| Class C | 0.0 | 0.0 | 0.1 | 0.3 | 0.1 | 0.3 | 0.0 | 0.0 | 0.1 |
| Class D | 0.0 | -0.1 | -0.1 | 0.1 | 0.1 | 0.3 | -0.1 | -0.2 | 0.0 |
| Class E | 0.0 | 0.0 | 0.1 |  |  |  | 1.3 | 2.2 | 2.3 |
| All | 0.1 | 0.0 | 0.0 | 0.5 | 0.2 | 0.4 | 0.3 | 0.5 | 0.7 |

Table 1-5 Performances on two max CU sizes (128x128 and 64x64) with max TU size (32x32) of full depth (4): (s0-0, CU size = 128, full depth, 64x64~4x4 TU) vs. (s0-5, CU size = 64, full depth, 32x32~4x4 TU)

|  | Intra |  |  | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | 0.1 | 0.4 | 0.4 | 0.5 | 0.7 | 1.0 |  |  |  |
| Class B | 0.5 | 1.2 | 1.1 | 1.7 | 2.0 | 2.1 | 0.9 | 4.4 | 5.6 |
| Class C | 0.1 | 0.3 | 0.2 | 0.6 | 0.5 | 0.6 | 0.3 | 1.3 | 1.5 |
| Class D | 0.0 | 0.0 | 0.0 | 0.1 | 0.3 | 0.4 | -0.1 | 0.8 | 0.7 |
| Class E | 0.3 | 1.8 | 1.4 |  |  |  | 2.8 | 11.4 | 9.2 |
| All | 0.2 | 0.7 | 0.7 | 0.8 | 1.0 | 1.1 | 0.9 | 4.1 | 4.0 |

Table 1-6 Performances on two max CU sizes (128x128 and 64x64) with max TU size (16x16) of full depth (3): (s0-0, CU size = 128, full depth, 64x64~4x4 TU) vs. (s0-6, CU size = 64, full depth, 16x16~4x4 TU)

|  | Intra |  |  | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | 0.7 | 1.5 | 1.8 | 1.5 | 3.2 | 3.8 |  |  |  |
| Class B | 2.3 | 4.2 | 4.5 | 5.2 | 8.0 | 8.2 | 3.3 | 16.4 | 21.6 |
| Class C | 0.2 | 0.9 | 0.7 | 1.3 | 2.5 | 2.4 | 0.9 | 6.4 | 7.1 |
| Class D | 0.1 | 0.7 | 0.5 | 0.4 | 1.9 | 1.6 | 0.0 | 5.6 | 4.8 |
| Class E | 1.7 | 8.0 | 6.8 |  |  |  | 7.9 | 36.5 | 27.3 |
| All | 1.1 | 3.0 | 2.8 | 2.4 | 4.3 | 4.3 | 2.8 | 15.0 | 14.8 |

Table 1-7 Performances on two max CU sizes (128x128 and 64x64) with max TU size (8x8) of full depth (2): (s0-0, CU size = 128, full depth, 64x64~4x4 TU) vs. (s0-7, CU size = 64, full depth, 8x8~4x4 TU)

|  | Intra |  |  | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| All |  |  |  |  |  |  |  |  |  |

Table 1-8 Performances on two max CU sizes (128x128 and 32x32) with max TU size (32x32) of full depth (4): (s0-0, CU size = 128, full depth, 64x64~4x4 TU) vs. (s0-8, CU size = 32, full depth, 32x32~4x4 TU)

|  | Intra |  |  | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | 0.4 | 0.7 | 0.8 | 3.0 | 2.0 | 2.5 |  |  |  |
| Class B | 0.9 | 1.7 | 1.8 | 6.0 | 4.8 | 4.6 | 3.4 | 6.0 | 7.2 |
| Class C | -0.1 | 0.2 | 0.2 | 2.0 | 1.6 | 1.7 | 0.7 | 1.9 | 1.7 |
| Class D | 0.0 | 0.2 | 0.1 | 1.2 | 1.2 | 1.4 | 0.0 | 1.2 | 1.4 |
| Class E | 0.7 | 3.0 | 2.4 |  |  |  | 9.0 | 18.0 | 14.7 |
| All | 0.4 | 1.1 | 1.0 | 3.3 | 2.6 | 2.7 | 2.9 | 6.0 | 5.8 |

Table 1-9 Performances on two max CU sizes (128x128 and 32x32) with max TU size (16x16) of full depth (3): (s0-0, CU size = 128, full depth, 64x64~4x4 TU) vs. (s0-9, CU size = 32, full depth, 16x16~4x4 TU)

|  | Intra |  |  | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | 1.0 | 1.8 | 2.1 | 4.0 | 4.9 | 5.4 |  |  |  |
| Class B | 2.7 | 4.8 | 5.2 | 8.8 | 10.3 | 10.2 | 5.9 | 18.0 | 22.7 |
| Class C | 0.3 | 1.0 | 0.8 | 3.5 | 3.5 | 3.5 | 1.3 | 7.1 | 7.6 |
| Class D | 0.1 | 0.7 | 0.5 | 1.3 | 2.4 | 2.5 | 0.1 | 6.4 | 5.2 |
| Class E | 2.2 | 9.0 | 7.6 |  |  |  | 13.4 | 42.3 | 33.3 |
| All | 1.3 | 3.4 | 3.2 | 4.8 | 5.7 | 5.7 | 4.7 | 16.9 | 16.6 |

Table 1-10 Performances on two max CU sizes (128x128 and 32x32) with max TU size (8x8) of full depth (2): (s0-0, CU size = 128, full depth, 64x64~4x4 TU) vs. (s0-10, CU size = 32, full depth, 8x8~4x4 TU)

|  | Intra |  |  | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | 4.4 | 6.1 | 8.7 | 8.8 | 13.3 | 16.0 |  |  |  |
| Class B | 8.4 | 10.6 | 13.5 | 18.6 | 23.7 | 26.7 | 14.1 | 42.9 | 54.7 |
| Class C | 2.1 | 3.8 | 4.1 | 6.8 | 10.5 | 10.5 | 4.8 | 18.4 | 20.5 |
| Class D | 1.4 | 3.2 | 3.0 | 3.4 | 7.7 | 6.9 | 2.0 | 20.0 | 18.1 |
| Class E | 9.1 | 26.2 | 22.2 |  |  |  | 34.4 | 117.6 | 88.0 |
| All | 5.1 | 9.6 | 10.0 | 10.1 | 14.5 | 15.7 | 12.5 | 45.0 | 43.2 |

Table 1-11 Performances on two max CU sizes (128x128 and 16x16) with max TU size (16x16) of full depth (3): (s0-0, CU size = 128, full depth, 64x64~4x4 TU) vs. (s0-11, CU size = 16, full depth, 16x16~4x4 TU)

|  | Intra |  |  | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | 2.0 | 3.7 | 4.7 | 12.7 | 11.9 | 12.9 |  |  |  |
| Class B | 4.4 | 7.9 | 9.1 | 23.8 | 22.7 | 22.5 | 14.8 | 25.5 | 29.8 |
| Class C | 1.0 | 1.8 | 1.7 | 9.5 | 8.2 | 8.5 | 5.5 | 9.1 | 10.1 |
| Class D | 0.3 | 1.4 | 1.1 | 6.6 | 6.3 | 6.3 | 2.2 | 8.7 | 7.7 |
| Class E | 4.5 | 15.1 | 13.3 |  |  |  | 34.3 | 66.8 | 54.9 |
| All | 2.5 | 5.8 | 5.9 | 13.9 | 13.0 | 13.2 | 13.0 | 25.0 | 24.0 |

Table 1-12 Performances on two max CU sizes (128x128 and 16x16) with max TU size (8x8) of full depth (2): (s0-0, CU size = 128, full depth, 64x64~4x4 TU) vs. (s0-12, CU size = 16, full depth, 8x8~4x4 TU)

|  | Intra |  |  | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| All |  |  |  |  |  |  |  |  |  |

Table 2-1 Performances on two max CU sizes (128x128 and 64x64) with max TU size (16x16) of full depth (3): (s0-2, CU size = 128, full depth, 16x16~4x4 TU) vs. (s0-6, CU size = 64, full depth, 16x16~4x4 TU)

|  | Intra |  |  | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | 0.1 | 0.0 | 0.1 | 0.4 | 0.3 | 0.6 |  |  |  |
| Class B | 0.1 | 0.1 | 0.1 | 1.8 | 1.0 | 0.9 | 0.3 | 0.1 | 0.3 |
| Class C | 0.0 | 0.1 | 0.0 | 0.3 | 0.4 | 0.4 | 0.0 | 0.2 | 0.3 |
| Class D | 0.0 | 0.0 | 0.0 | 0.1 | 0.1 | 0.2 | -0.1 | 0.3 | 0.0 |
| Class E | 0.1 | 0.2 | 0.2 |  |  |  | 1.6 | 2.6 | 3.9 |
| All | 0.1 | 0.1 | 0.1 | 0.7 | 0.5 | 0.6 | 0.4 | 0.6 | 0.9 |

Table 2-2 Performances on two max CU sizes (128x128 and 32x32) with max TU size (16x16) of full depth (3): (s0-2, CU size = 128, full depth, 16x16~4x4 TU) vs. (s0-9, CU size = 32, full depth, 16x16~4x4 TU)

|  | Intra |  |  | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | 0.4 | 0.3 | 0.4 | 2.9 | 1.9 | 2.2 |  |  |  |
| Class B | 0.5 | 0.7 | 0.8 | 5.3 | 3.3 | 2.9 | 2.9 | 1.8 | 1.8 |
| Class C | 0.1 | 0.2 | 0.2 | 2.5 | 1.3 | 1.5 | 0.4 | 0.9 | 0.8 |
| Class D | -0.1 | 0.0 | 0.0 | 0.9 | 0.6 | 1.1 | -0.1 | 1.1 | 0.3 |
| Class E | 0.6 | 1.3 | 1.0 |  |  |  | 6.8 | 7.2 | 8.7 |
| All | 0.3 | 0.5 | 0.5 | 3.0 | 1.9 | 2.0 | 2.2 | 2.4 | 2.5 |

Table 2-3 Performances on two max CU sizes (128x128 and 16x16) with max TU size (16x16) of full depth (3): (s0-2, CU size = 128, full depth, 16x16~4x4 TU) vs. (s0-11, CU size = 16, full depth, 16x16~4x4 TU)

|  | Intra |  |  | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | 1.4 | 2.2 | 3.1 | 11.4 | 8.8 | 9.7 |  |  |  |
| Class B | 2.2 | 3.7 | 4.7 | 19.7 | 15.1 | 14.8 | 11.6 | 9.5 | 9.0 |
| Class C | 0.8 | 1.0 | 1.1 | 8.5 | 6.0 | 6.5 | 4.6 | 3.0 | 3.3 |
| Class D | 0.2 | 0.7 | 0.6 | 6.2 | 4.5 | 4.9 | 2.1 | 3.6 | 2.9 |
| Class E | 2.9 | 7.7 | 6.9 |  |  |  | 26.5 | 27.2 | 27.9 |
| All | 1.5 | 2.9 | 3.2 | 12.0 | 9.0 | 9.2 | 10.3 | 9.7 | 9.6 |

Table 2-4 Performances on two max CU sizes (128x128 and 64x64) with max TU size (8x8) of full depth (2): (s0-3, CU size = 128, full depth, 8x8~4x4 TU) vs. (s0-7, CU size = 64, full depth, 8x8~4x4 TU)

|  | Intra |  |  | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| All |  |  |  |  |  |  |  |  |  |

Table 2-5 Performances on two max CU sizes (128x128 and 64x64) with max TU size (8x8) of full depth (2): (s0-3, CU size = 128, full depth, 8x8~4x4 TU) vs. (s0-10, CU size = 32, full depth, 8x8~4x4 TU)

|  | Intra |  |  | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | 0.2 | 0.3 | 0.4 | 2.6 | 2.0 | 2.0 |  |  |  |
| Class B | 0.4 | 0.5 | 0.6 | 5.0 | 3.0 | 3.7 | 2.4 | 1.6 | 1.3 |
| Class C | 0.1 | 0.2 | 0.1 | 1.7 | 1.5 | 1.4 | 0.5 | 0.7 | 0.3 |
| Class D | 0.0 | 0.0 | 0.0 | -2.9 | -1.3 | -1.4 | -0.1 | 0.6 | 1.1 |
| Class E | 0.5 | 0.6 | 0.7 |  |  |  | 7.1 | 9.2 | 7.5 |
| All | 0.2 | 0.3 | 0.4 | 1.7 | 1.3 | 1.5 | 2.2 | 2.6 | 2.2 |

Table 2-6 Performances on two max CU sizes (128x128 and 64x64) with max TU size (8x8) of full depth (2): (s0-3, CU size = 128, full depth, 8x8~4x4 TU) vs. (s0-12, CU size = 16, full depth, 8x8~4x4 TU)

|  | Intra |  |  | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| All |  |  |  |  |  |  |  |  |  |

Table 3-1 Performances on two different CU depths (5 and 2) for the same max CU size (128x128) with max TU size (64x64) of full depth (5): (s0-0, CU size = 128, CU depth = 5, 64x64~4x4 TU) vs. (s2-0, CU size = 128, CU depth = 2, 64x64~4x4 TU)

|  | Intra |  |  | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | 11.7 | 13.5 | 14.0 | 35.6 | 45.3 | 41.7 |  |  |  |
| Class B | 8.2 | 9.8 | 10.2 | 20.1 | 24.1 | 23.5 | 19.5 | 16.8 | 16.0 |
| Class C | 15.2 | 14.4 | 15.2 | 42.0 | 47.5 | 50.9 | 37.3 | 37.1 | 36.8 |
| Class D | 15.3 | 13.3 | 13.7 | 47.7 | 53.8 | 53.9 | 38.3 | 36.8 | 36.2 |
| Class E | 13.8 | 13.3 | 14.4 |  |  |  | 34.2 | 28.1 | 32.6 |
| All | 12.6 | 12.6 | 13.2 | 35.3 | 41.1 | 41.4 | 31.4 | 29.0 | 29.4 |

Table 3-2 Performances on two different CU depths (5 and 2) for the same max CU size (128x128) with two different max TU sizes (64x64, 32x32) of full depths (5 and 4): (s0-0, CU size = 128, CU depth = 5, 64x64~4x4 TU) vs. (s2-1, CU size = 128, CU depth = 2, 32x32~4x4 TU)

|  | Intra |  |  | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | 11.8 | 13.9 | 14.5 | 35.8 | 46.0 | 42.8 |  |  |  |
| Class B | 8.7 | 11.4 | 11.7 | 20.7 | 27.0 | 26.6 | 20.2 | 22.8 | 23.4 |
| Class C | 15.2 | 14.8 | 15.3 | 44.0 | 48.7 | 52.2 | 37.7 | 39.2 | 39.0 |
| Class D | 15.3 | 13.4 | 13.8 | 47.7 | 54.3 | 53.9 | 38.0 | 37.5 | 36.9 |
| Class E | 14.0 | 15.6 | 15.9 |  |  |  | 36.1 | 41.7 | 42.9 |
| All | 12.9 | 13.6 | 14.0 | 36.1 | 42.6 | 42.9 | 32.0 | 34.1 | 34.3 |

Table 3-3 Performances on two different CU depths (5 and 2) for the same max CU size (128x128) with two different max TU sizes (64x64, 16x16) of full depths (5 and 3): (s0-0, CU size = 128, CU depth = 5, 64x64~4x4 TU) vs. (s2-2, CU size = 128, CU depth = 2, 16x16~4x4 TU)

|  | Intra |  |  | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | 12.8 | 15.8 | 16.2 | 37.1 | 51.6 | 48.6 |  |  |  |
| Class B | 11.3 | 16.5 | 16.9 | 24.3 | 36.8 | 36.6 | 23.3 | 40.3 | 45.6 |
| Class C | 15.8 | 15.9 | 16.1 | 44.5 | 53.9 | 57.0 | 40.5 | 49.0 | 50.1 |
| Class D | 15.7 | 14.6 | 14.6 | 49.2 | 58.2 | 57.8 | 40.1 | 47.3 | 45.6 |
| Class E | 16.5 | 24.3 | 23.5 |  |  |  | 41.5 | 78.9 | 69.2 |
| All | 14.3 | 17.2 | 17.2 | 38.0 | 49.0 | 49.3 | 35.2 | 51.5 | 51.2 |

Table 3-4 Performances on two different CU depths (5 and 2) for the same max CU size (128x128) with two different max TU sizes (64x64, 8x8) of full depths (5 and 2): (s0-0, CU size = 128, CU depth = 5, 64x64~4x4 TU) vs. (s2-3, CU size = 128, CU depth = 2, 8x8~4x4 TU)

|  | Intra |  |  | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | 17.8 | 21.4 | 24.2 | 46.1 | 67.8 | 68.6 |  |  |  |
| Class B | 19.0 | 25.3 | 29.5 | 37.6 | 61.7 | 64.9 | 35.5 | 84.0 | 106.5 |
| Class C | 19.0 | 20.9 | 21.5 | 52.7 | 71.6 | 75.7 | 49.3 | 74.5 | 78.7 |
| Class D | 18.5 | 19.1 | 19.0 | 56.6 | 74.5 | 72.4 | 49.2 | 73.6 | 70.6 |
| Class E | 27.1 | 49.9 | 47.5 |  |  |  | 66.7 | 172.2 | 143.5 |
| All | 20.1 | 26.6 | 27.8 | 47.8 | 68.6 | 70.3 | 48.2 | 95.6 | 97.5 |

Table 3-5 Performances on two different CU depths (5 and 3) for the same max CU size (128x128) with the same max TU size (64x64) of full depth (5): (s0-0, CU size = 128, CU depth = 5, 64x64~4x4 TU) vs. (s2-4, CU size = 128, CU depth = 3, 64x64~4x4 TU)

|  | Intra |  |  | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | 6.0 | 6.7 | 6.6 | 13.0 | 17.0 | 15.4 |  |  |  |
| Class B | 4.2 | 5.0 | 5.1 | 7.3 | 9.2 | 9.0 | 6.9 | 5.0 | 4.7 |
| Class C | 9.3 | 8.9 | 9.4 | 16.4 | 19.3 | 20.7 | 15.5 | 14.9 | 14.5 |
| Class D | 9.9 | 8.7 | 9.2 | 19.8 | 22.8 | 23.1 | 16.5 | 14.9 | 15.0 |
| Class E | 6.2 | 5.0 | 6.1 |  |  |  | 9.1 | 6.3 | 8.3 |
| All | 7.1 | 6.9 | 7.3 | 13.8 | 16.6 | 16.7 | 11.9 | 10.2 | 10.4 |

Table 3-6 Performances on two different CU depths (5 and 3) for the same max CU size (128x128) with two different max TU sizes (64x64, 32x32) of full depths (5 and 4): (s0-0, CU size = 128, CU depth = 5, 64x64~4x4 TU) vs. (s2-5, CU size = 128, CU depth = 3, 32x32~4x4 TU)

|  | Intra |  |  | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | 6.1 | 7.0 | 7.0 | 13.2 | 17.6 | 16.2 |  |  |  |
| Class B | 4.6 | 6.2 | 6.3 | 8.1 | 11.3 | 10.8 | 7.6 | 9.5 | 10.5 |
| Class C | 9.3 | 9.1 | 9.5 | 16.9 | 20.2 | 21.5 | 16.0 | 16.5 | 16.4 |
| Class D | 10.0 | 8.8 | 9.3 | 20.1 | 23.2 | 23.4 | 16.6 | 15.7 | 15.9 |
| Class E | 6.5 | 7.0 | 7.4 |  |  |  | 11.2 | 17.0 | 16.0 |
| All | 7.3 | 7.7 | 7.9 | 14.3 | 17.7 | 17.7 | 12.6 | 14.2 | 14.3 |

Table 3-7 Performances on two different CU depths (5 and 3) for the same max CU size (128x128) with two different max TU sizes (64x64, 16x16) of full depths (5 and 3): (s0-0, CU size = 128, CU depth = 5, 64x64~4x4 TU) vs. (s2-6, CU size = 128, CU depth = 3, 16x16~4x4 TU)

|  | Intra |  |  | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | 6.8 | 8.5 | 8.5 | 14.1 | 21.7 | 20.7 |  |  |  |
| Class B | 6.6 | 10.1 | 10.5 | 11.0 | 19.1 | 18.6 | 10.1 | 25.1 | 30.8 |
| Class C | 9.6 | 9.9 | 10.2 | 17.7 | 23.3 | 24.6 | 16.6 | 23.5 | 24.3 |
| Class D | 10.2 | 9.6 | 9.9 | 20.1 | 25.0 | 25.1 | 16.7 | 21.6 | 21.0 |
| Class E | 8.2 | 13.8 | 13.3 |  |  |  | 15.2 | 44.8 | 35.4 |
| All | 8.3 | 10.4 | 10.5 | 15.6 | 22.1 | 22.2 | 14.3 | 27.5 | 27.6 |

Table 3-8 Performances on two different CU depths (5 and 3) for the same max CU size (128x128) with two different max TU sizes (64x64, 8x8) of full depths (5 and 2): (s0-0, CU size = 128, CU depth = 5, 64x64~4x4 TU) vs. (s2-7, CU size = 128, CU depth = 3, 8x8~4x4 TU)

|  | Intra |  |  | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | 11.3 | 13.4 | 15.9 | 20.0 | 32.5 | 34.5 |  |  |  |
| Class B | 13.3 | 17.2 | 20.7 | 21.6 | 37.5 | 39.5 | 19.5 | 58.5 | 79.1 |
| Class C | 12.2 | 14.0 | 14.5 | 22.3 | 33.7 | 35.4 | 20.7 | 41.6 | 45.8 |
| Class D | 12.2 | 13.2 | 13.2 | 23.8 | 34.9 | 33.8 | 20.8 | 42.3 | 40.5 |
| Class E | 16.6 | 34.8 | 31.9 |  |  |  | 36.6 | 123.8 | 94.2 |
| All | 13.2 | 18.1 | 19.0 | 22.2 | 35.1 | 36.2 | 23.3 | 62.5 | 64.0 |

# Summary

In assessing the performance of large block structure in Test Model under Consideration for different combinations of CTB and TU sizes, the maximum CU size of 64x64 with maximum TU size of 32x32 seems to result in reasonable RD performance compared to the anchor case without significant performance degradation of less than 1% Y-BDR increment.

- max CU: up to 64 (removal of CU =128), max TU = 16

  - Y-BDR increments

    - Intra: 0.1%, LD: 0.4%, RA: 0.7%

  - U-BDR drop increments

    - Intra: 0.1%, LD: 0.6%, RA: 0.5%

  - V-BDR drop increments

    - Intra: 0.1%, LD: 0.9%, RA: 0.6%

- max CU: up to 32 (removal of CU =128, 64) , max TU = 16

  - Y-BDR increments

    - Intra: 0.3%, LD: 2.2%, RA: 3.0%

  - U-BDR drop increments

    - Intra: 0.5%, LD: 2.4%, RA: 1.9%

  - V-BDR drop increments

    - Intra: 0.5%, LD: 2.5%, RA: 2.0%

- Max CU: up to 16 (removal of CU =128, 64, 32)

  - Y-BDR increments

    - Intra: 1.5%, LD: 10.3%, RA: 12.0%

  - U-BDR drop increments

    - Intra: 2.9%, LD: 9.7%, RA: 9.0%

  - V-BDR drop increments

Intra: 3.2%, LD: 9.6%, RA: 9.2%

- Max TU: up to 32 (removal of TU =64), max CU = 128

  - Y-BDR increments

    - Intra: 0.2%, LD: 0.4%, RA: 0.6%

  - U-BDR drop increments

    - Intra: 0.7%, LD: 0.8%, RA: 3.6%

  - V-BDR drop increments

    - Intra: 0.6%, LD: 0.8%, RA: 3.4%

- Max TU: up to 16 (removal of TU = 64, 32), max CU = 128

  - Y-BDR increments

    - Intra: 1.0%, LD: 2.4%, RA: 1.6%

  - U-BDR drop increments

    - Intra: 2.9%, LD: 14.3%, RA: 3.8%

  - V-BDR drop increments

    - Intra: 2.7%, LD: 13.8%, RA: 3.7%

- Max TU: up to 8 (removal of TU = 64, 32, 16), max CU = 128

  - Y-BDR increments

    - Intra: 4.9%, LD: 10.0%, RA: 8.3%

  - U-BDR drop increments

    - Intra: 9.2%, LD: 41.0%, RA: 13.2%

  - V-BDR drop increments

    - Intra: 9.6%, LD: 40.5%, RA: 14.0%

- Max CU & TU: up to 64 & 32, removal of CU = 128 & TU = 64

  - Y-BDR increments

    - Intra: 0.2%, LD: 0.9%, RA: 0.8%

  - U-BDR drop increments

    - Intra: 0.7%, LD: 4.1%, RA: 1.0%

  - V-BDR drop increments

    - Intra: 0.7%, LD: 4.0%, RA: 1.1%

- Max CU & TU: up to 32 & 32

  - Y-BDR increments

    - Intra: 0.4%, LD: 2.9%, RA: 3.3%

  - U-BDR drop increments

    - Intra: 1.1%, LD: 6.0%, RA: 2.6%

  - V-BDR drop increments

    - Intra: 1.0%, LD: 5.8%, RA: 2.7%

- Max CU & TU: up to 32 & 16

  - Y-BDR increments

    - Intra: 1.3%, LD: 4.7%, RA: 4.8%

  - U-BDR drop increments

    - Intra: 3.4%, LD: 16.9%, RA: 5.7%

  - V-BDR drop increments

    - Intra: 3.2%, LD: 16.6%, RA: 5.7%

# Patent rights declaration(s)

(NOTE – Activities in the JCT-VC and contributions to the JCT-VC are subject to the common patent policy for ITU-T/ITU-R/ISO/IEC. A statement of that policy can be found at
http://www.itu.int/ITU-T/dbase/patent/patent-policy.html, with further information available at http://www.itu.int/ITU-T/ipr/index.html and in the ISO/IEC Directives. The form to be used for the formal reporting of patent rights to ITU-T/ITU-R/ISO/IEC can be found at http://www.itu.int/ITU-T/ipr/index.html. Contributions to the JCT-VC proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The provided informal notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form. This obligation to provide an informal notice is supplemental to, and does not replace, any existing obligations of parties with technology included in a final or draft standard to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC. An example of an informal IPR notification statement for a contribution is provided below.)

XYZ Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).