| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 07-15 October, 2010 | Document: JCTVC-C068 WG11 Number : m18091 |
| --- | --- |

| Title: | Improved side information signaling for quad-tree based transform structure of TMuC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Authors: | Bumshik Lee, Munchurl Kim, Hui Yong Kim, Jongho Kim and Jin Soo Choi |  |  |
| Contact: | Bumshik Lee and Munchurl Kim KAIST, 335 Guseong-dong, Yuseong-Gu, Daejeon, 305-701, Korea, Hui Yong Kim, Jongho Kim and Jin Soo Choi ETRI, 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, Korea | Tel: Email: | +82-42-350-7419 {lbs, mkim}@ee.kaist.ac.kr +82-42-860-5069 {hykim5, pooney, jschoi} @etri.re.kr |
| Source: | KAIST and ETRI |  |  |

_____________________________

# Abstract

In this contribution, a new syntax is proposed for the quad-tree based transform to reduce large amounts of side information of the transform in TMuC. The available maximum and minimum transform size in the current TMuC is 6464 and 44 respectively, which are implemented with the quad-tree-structure manner in current version of TMuC. The structure is well adapted to the characteristics of the input signal while the signalling for transform types may cause large amounts of side information as the depth of the quad-tree increases. Moreover, the coded_block_flags (cbf) for luma and chroma components are another overhead for the quad-tree transform structure. In the proposed syntax, more efficient syntax is proposed to reduce of the amount of the side information including split_transform_unit_flag and cbf. The encoding and decoding for split_transform_unit_flag is skipped when cbfs for luminance(Y) and chrominance (UV) are all zero. Furthermore, a new flag with a single bit is defined to signal the patterns of the quantized coefficient for luminance and chrominance components.

# Introduction

The minimum transform sizes and maximum transform depth are signaled in SPS (sequence parameter set) and TU (transform unit) is partitioned into four sub-divided transform blocks. Since the TU is implemented with the quad-tree structure, split information at the current transform depth should be signaled and coded_block_flag (cbf) is also signaled to represent the existence of the non-zero coefficient for the luminance and chrominance components in the transform block. The split_transform_unit_flag is to represent whether the current block is partitioned or not with a single bit. However, if the transform depth gets higher, the amounts of side information such as split_transform_unit_flag and coded_block_flag significantly increase. Fig. 2 illustrates the assignment of the signalling bits for split_transform_unit_flag and coded_block_flag in the transform unit when the transform partitioning is finally determined as Fig. 1. In this example, a 3232 block is partitioned into smaller transform blocks and the minimum transform size is 44. At each leaf node, the coded_block_flag is signalled for luminance component.

Fig. 1  Quad-tree transform structure in TU (AZCB : All Zero Coefficient Block)

Fig. 2  Side information signaling for the quad-tree transform structure illustrated in Fig. 1

In order to send the side information for quad-tree transform structure illustrated in Fig. 1, total 52 bits are used. Even if the transform block is all zero coefficient block where the block is not further-partitioned,  the split_transform_unit_flag is encoded, thus resulting in the redundancy of the side information.

# Proposed syntax for quad-tree transform

In the proposed syntax for quad-tree transform structure, the split_transform_unit_flag are not encoded for the all zero coefficient block by swapping the coding order of cbf and split_transform_flag. The cbf information is written prior to split_transform_flag. Hence, 5 bits in the transform structure of Fig. 1 are reduced by skipping the split_transform_unit_flag for all-zero coefficient blocks. In addition, we define a new flag cbf_yuv, which is to indicate the existence of the quantized coefficients for luminance and chrominance component with a single bit. The cbf_yuv is to set to 1 if there are no all-zero coefficients for luminance and chrominance components in current or lower transform layers; otherwise it is set to 0. In other words, cbf_yuv is set if at least one of the following conditions hold;

- when the transform units at the current or lower transform layer contain one or more non-zero coefficients of the luminance Y.

- when the transform units at the current or lower transform layer contain one or more non-zero coefficients of the chrominance U.

- when the transform units at the current or lower transform layer contain one or more non-zero coefficients of the chrominance V.

Table 1 shows the proposed syntax and Fig. 3 shows the side information signaling by the proposed syntax. Since the split_transform_unit_flag is not sent and cbfs (Y, U and V) are represented by a single bit flag for the transform block greater than the minimum transform size, total 39 bits as side information are used, which are considerably reduced compared to the syntax of current TMuC.

Table 1. Proposed syntax for the quad-tree transform

| transform_unit_tree( x0, y0, currTransformUnitSize, layer ) { |  | C | Descriptor |
| --- | --- | --- | --- |
| if( currTransformUnitSize > MinTransformUnitSize && currTransformUnitSize <= MaxTransformUnitSize) |  |  |  |
| cbf_yuv |  | 3 ¦ 4 | u(1) ¦ ae(v) |
| If(cbf_yuv){ |  |  |  |
| split_transform_unit_flag[x0][y0][layer] |  | 3 ¦ 4 | u(1) ¦ ae(v) |
| If(split_transform_unit_flag[x0][y0][layer]){ |  |  |  |
| If(currTransformUnitSize == (MinTransformUnitSize <<1) ){ |  |  |  |
| coded_block_flag_chroma_u |  |  |  |
| codedBlockFlag[x0][y0][layer][1]=coded_block_flag_chroma_u |  |  |  |
| coded_block_flag_chroma_v |  |  |  |
| codedBlockFlag[x0][y0][layer][2] =coded_block_flag_chroma_v |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| if( split_transform_unit_flag[x0][y0][layer] ) { |  |  |  |
| splitTransformUnitSize = currTransformUnitSize >> 1 |  |  |  |
| x1 = x0 + splitTransformUnitSize |  |  |  |
| y1 = y0 + splitTransformUnitSize |  |  |  |
| transform_unit_tree( x0, y0, splitTransformUnitSize, layer+1 ) |  |  |  |
| for( i=0; i<=2; i++ ) |  |  |  |
| codedBlockFlag[x0][y0][layer][i] ¦= codedBlockFlag[x0][y0][layer+1][i] |  |  |  |
| if( x1 < PicWidthInSamplesL ) { |  |  |  |
| transform_unit_tree( x1, y0, splitTransformUnitSize, layer+1 ) |  |  |  |
| for( i=0; i<=2; i++ ) |  |  |  |
| codedBlockFlag[x0][y0][layer][i] ¦= codedBlockFlag[x1][y0][layer+1][i] |  |  |  |
| } |  |  |  |
| if( y1 < PicHeightInSamplesL ){ |  |  |  |
| transform_unit_tree( x0, y1, splitTransformUnitSize, layer+1 ) |  |  |  |
| for( i=0; i<=2; i++ ) |  |  |  |
| codedBlockFlag[x0][y0][layer][i] ¦= codedBlockFlag[x0][y1][layer+1][i] |  |  |  |
| } |  |  |  |
| if( x1 < PicWidthInSamplesL && y1 < PicHeightInSamplesL ) { |  |  |  |
| transform_unit_tree( x1, y1, splitTransformUnitSize, layer+1 ) |  |  |  |
| for( i=0; i<=2; i++ ) |  |  |  |
| codedBlockFlag[x0][y0][layer][i] ¦= codedBlockFlag[x1][y1][layer+1][i] |  |  |  |
| } |  |  |  |
| } else{ |  |  |  |
| if(currTransformUnitSize != MinTransformUnitSize){ |  |  |  |
| if(cbf_yuv){ |  |  |  |
|  | coded_block_flag_chroma_u | 3 ¦ 4 | u(1) ¦ ae(v) |
|  | codedBlockFlag[x0][y0][layer][1]=coded_block_flag_chroma_u |  |  |
|  | coded_block_flag_chroma_v | 3 ¦ 4 | u(1) ¦ ae(v) |
|  | codedBlockFlag[x0][y0][layer][2]=coded_block_flag_chroma_v |  |  |
|  | coded_block_flag_luma | 3 ¦ 4 | u(1) ¦ ae(v) |
|  | codedBlockFlag[x0][y0][layer][0]= coded_block_flag_luma |  |  |
| } |  |  |  |
| }else{ |  |  |  |
| coded_block_flag_luma |  | 3 ¦ 4 | u(1) ¦ ae(v) |
| codedBlockFlag[x0][y0][layer][0]= coded_block_flag_luma |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Fig. 3 Side information signaling by the proposed syntax

# Experimental results

The proposed syntax is implemented in the TMuC0.7.3. The experiments with common test conditions in [1] were performed for random access and low delay constraints with high efficiency settings. The experimental results are shown in Table.

Table 2. RD performance on the proposed syntax for the quad-tree transforms structure

| Sequences | Low Delay (BDBR%) |  |  | Random Access (BDBR%) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| BasketballPass_416240_50 | 0.5 | -0.5 | -0.6 | 0.3 | 0.1 | -0.1 |
| BQSquare_416240_60 | 0.6 | 3.7 | 2.1 | 0.3 | 1.4 | 1.4 |
| BlowingBubbles_416240_50 | 0.6 | 0.7 | 0.1 | 0.3 | 0.2 | 0.5 |
| RaceHorses_416240_30 | 0.7 | -0.6 | -0.4 | 0.2 | 0.3 | 0.5 |
| Average QWVGA | 0.6 | 0.8 | 0.3 | 0.3 | 0.3 | 0.5 |
| BasketballDrill_832480_50 | 0.7 | 0.0 | 0.2 | 0.4 | 0.2 | 0.3 |
| BQMall_832480_60 | 1.0 | -0.1 | -0.1 | 0.4 | 0.4 | 0.5 |
| PartyScene_832480_50 | 0.3 | 0.3 | 0.7 | 0.2 | 0.4 | 0.5 |
| RaceHorses_832480_30 | 0.5 | 0.5 | 0.2 | 0.1 | 0.2 | 0.2 |
| Average WVGA | 0.6 | 0.2 | 0.3 | 0.3 | 0.3 | 0.4 |
| vidyo1_720p_60 | 2.0 | -0.1 | -0.3 |  |  |  |
| vidyo3_720p_60 | 2.0 | -0.1 | 1.2 |  |  |  |
| vidyo4_720p_60 | 1.6 | -1.3 | -0.7 |  |  |  |
| Average 720P | 1.9 | -0.5 | 0.1 |  |  |  |
| Kimono1_19201080_24 | 1.0 | -0.2 | 0.3 | 0.7 | 0.6 | 0.8 |
| ParkScene_19201080_24 | 1.0 | 1.7 | 3.3 | 0.2 | 0.7 | 0.9 |
| Cactus_19201080_50 | 0.7 | -0.4 | 0.0 | 0.4 | 0.4 | 0.4 |
| BasketballDrive_19201080_50 | 0.7 | 0.4 | 0.2 | 0.4 | 0.3 | 0.2 |
| BQTerrace_19201080_60 | 0.7 | 0.6 | 0.6 | 0.3 | 2.7 | 2.4 |
| Average 1080P | 0.8 | 0.4 | 0.9 | 0.4 | 0.9 | 0.9 |
| Traffic_25601600_30_crop |  |  |  | 0.2 | 0.5 | 0.5 |
| PeopleOnStreet_25601600_30_crop |  |  |  | 0.3 | 1.2 | 1.3 |
| Average 1600P |  |  |  | 0.3 | 0.9 | 0.9 |
| Average | 0.9 | 0.3 | 0.4 | 0.3 | 0.6 | 0.7 |

# Conclusions

The new improved side information signaling scheme for quad-tree transform structure is proposed. The proposed signaling scheme is to skip to encode the split_transform_unit_flag for all zero coefficient blocks and the single bit flag is defined to represent the existence of the all-zero coefficients all for luminance and chrominance component for current or lower layers. The proposed syntax reduces the side information for quad-tree transform structure compared to the TMuC as shown in the illustrated example.

# Reference

- Frank Bossen, “Common test conditions and software reference configurations”, Doc. JCTVC-B300, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, Switzerland, July 2010.

ACKNOWLEDGEMENT

This work was supported by the IT R&D program of MKE/KCC/KEIT. [KI001932, Development of Next Generation DTV Core Technology]

# Patent rights declaration(s)

ETRI and KAIST  may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).