| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C071 |
| --- | --- |

| Title: | Complexity analysis on Wiener-based in-loop filters |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Li Wang, Lu Yu Zheda Road No.38 Hangzhou, China, 310027 | Tel: Email: | +86-571-87953107 yul@zju.edu.cn |
| Source: | Zhejiang University |  |  |

_____________________________

# Abstract

In this contribution, the complexity of Wiener-based adaptive in-loop filter (ALF) algorithms（A121, B064, B077, A117） in the subset2 of TE10 is analyzed along with their coding efficiencies. The contribution will analyze in three aspects: i) filter features, ii) encoding time, and iii) decoding time.

- Filter features

Though all the four algorithms design the filter coefficients according to the theory of Wiener filtering, there are some differences among them. Table 1 present their differences，and the Table 2 emphasizes on their advantage and cost. The shape of the filters listed in Table 1 may not be the restriction from the algorithms, but that implemented in TMuC.

Table 1 The difference among the four algorithms

| Proposal | Available Size | Shape | × Num | ＋Num | Input signal to filters | flexibility |
| --- | --- | --- | --- | --- | --- | --- |
| A121 [2] | 5×5 | Diamond 1/2 symmetric Point-symmetric | 7 | 13 | Post-DF | frame level or GOP level |
|  | 7×7 |  | 13 | 25 |  |  |
|  | 9×9 |  | 21 | 41 |  |  |
| B064 [3] | 7×7 | Diamond 1/2 symmetric Point-symmetric | 14 | 26 | Pre-DF & Post-DF | slice level |
|  | 9×9 |  | 22 | 42 |  |  |
|  | 11×11 |  | 32 | 62 |  |  |
| B077 [4] | 5×5 | Rectangular 1/2 symmetric Point-symmetric | 13 | 25 | Post-DF | picture level & partition level |
|  | 7×7 |  | 25 | 49 |  |  |
|  | 9×9 |  | 41 | 81 |  |  |
| A117 [5] | 5×5 | Circle 1/2 symmetric Point-symmetric | 13 | 25 | Post-DF | slice level |
|  | 7×7 |  | 15 or 23 | 29 or 45 |  |  |
|  | 9×9 |  | 25 | 49 |  |  |

Size : (maximum taps of the filter in row) ×(maximum taps of the filter column).

×Num：number of additions needed for each filtered pixel (only for luminance).

+Num：number of multiplications needed for each filtered pixel (only for luminance).

Pre-DF signal: unfiltered reconstruction signal (before deblocking )

Post-DF signal: de-blocking filtered reconstruction signal. ( just the output of the De-blocking filter)

Table 2 The advantages and disadvantages of the four algorithms

| proposal | Advantage | Disadvantages |
| --- | --- | --- |
| A121 | Can choose a filter from a set of filters without any index flag cost | Need to calculate an activity measure(Var). Each pixel costs 2 absolute value operations, and at least 11 addtions for a 7×7 neighborhood. |
| B064 | Mainly use Pre-DF signal in ALF, which leads to higher parallelism | There are not only the coefficients of the Pre-DF signal, but also the coefficient of the Post-DF signal. (One coefficient more) |
| B077 | When choose LEO or LBO mode, no coefficients but only the offset need to sent to the decoder | More flag need to be sent for indicating patterns |
| A117 | Less pass |  |

*Passes are one of the main factors that dictate hardware implementation complexity.[1] In this way, less pass means low power.

* If the shape of filters isn’t rectangular, it may cause not fully use of data getting from memory.

- Encoding Time

Below is encoding time of the functions of adaptive loop filter. It can roughly reflect the complexity of each ALF algorithms.

Experimental results listed in table 3 obtained by ALFs implementation software provided by proponents. Considering of time limitation, only one intra period for the condition of Random access, and 10 frames for of Low delay have been tested.

Table 3  The Encoding Time of adaptive loop filter (Low delay, high efficiency)

| The Class of sequences | A121(anchor) |  | B077 |  |  | A117 |  |  | B064 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | total | ALF | total | ALF | % | total | ALF | % | total | ALF | % |
| D | 79.06 | 1.06 | 79.74 | 2.43 | 129.61 | 78.08 | 0.90 | -15.28 | 77.57 | 1.05 | -1.38 |
| C | 347.66 | 3.89 | 349.98 | 9.84 | 152.92 | 342.58 | 3.60 | -7.37 | 343.91 | 4.12 | 6.00 |
| B | 1745.15 | 20.70 | 1759.62 | 51.06 | 146.79 | 1738.09 | 21.10 | 1.96 | 1717.18 | 23.07 | 11.51 |
| A | 3382.00 | 41.06 | 3378.44 | 100.79 | 145.47 | 3400.04 | 45.24 | 10.19 | 3347.08 | 51.00 | 24.20 |

Table 4  The Encoding Time of adaptive loop filter (Random access, high efficiency)

| The Class of sequences | A121(anchor) |  | B077 |  |  | A117 |  |  | B064 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | total | ALF | total | ALF | % | total | ALF | % | total | ALF | % |
| D | 64.59 | 1.51 | 65.75 | 2.40 | 59.26 | 64.19 | 0.90 | -40.51 | 64.01 | 0.78 | -48.34 |
| C | 298.92 | 5.19 | 302.83 | 9.66 | 86.13 | 298.17 | 3.46 | -33.33 | 294.60 | 2.99 | -42.39 |
| B | 1473.01 | 25.69 | 1498.75 | 50.45 | 96.38 | 1482.50 | 19.57 | -23.82 | 1445.71 | 15.83 | -38.38 |

Total: the total encoding time (sec) of the encoding processing per frame.

ALF: time (sec) of the ALF processing per frame.

%: positive value means loss, use the qualcomm’s as the anchor.

(ALF-ALF_anchor)/ ALF_anchor*100

- Decoding Time

Below is decoding time of the functions of adaptive loop filter.

Table 5  The Decoding Time of adaptive loop filter processing(Low delay, high efficiency)

| The Class of sequences | A121(anchor) |  | B077 |  |  | A117 |  |  | B064 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | total | ALF | total | ALF | % | total | ALF | % | total | ALF | % |
| D | 0.07 | 0.01 | 0.07 | 0.00 | -100.00 | 0.07 | 0.01 | 0.00 | 0.07 | 0.01 | 0.00 |
| C | 0.23 | 0.03 | 0.23 | 0.02 | -33.33 | 0.23 | 0.03 | 0.00 | 0.24 | 0.03 | 0.00 |
| B | 1.14 | 0.27 | 1.03 | 0.14 | -48.15 | 1.12 | 0.24 | -11.11 | 1.04 | 0.15 | -44.44 |
| A | 2.18 | 0.43 | 2.11 | 0.35 | -18.60 | 2.17 | 0.40 | -6.98 | 2.12 | 0.36 | -16.28 |

Total: the total decoding time (sec) of the decoding processing per frame.

ALF: time (sec) of the ALF processing when decoding per frame.

%: positive value means loss, use the qualcomm’s as the anchor.

Table 6  The Decoding Time of adaptive loop filter processing (Random access, high efficiency)

| The Class of sequences | A121(anchor) |  | B077 |  |  | A117 |  |  | B064 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | total | ALF | total | ALF | % | total | ALF | % | total | ALF | % |
| D | 0.13 | 0.01 | 0.11 | 0.01 | 0.00 | 0.12 | 0.01 | 0.00 | 0.13 | 0.01 | 0.00 |
| C | 0.42 | 0.06 | 0.42 | 0.03 | -50.00 | 0.42 | 0.04 | -33.33 | 0.42 | 0.03 | -50.00 |
| B | 1.88 | 0.46 | 1.80 | 0.18 | -60.87 | 1.87 | 0.26 | -43.48 | 1.85 | 0.20 | -56.52 |

- 4  Conclusion

In this contribution, complexity of Wiener-based in-loop filters is analyzed through the number of additions and multiplications needed, encoding and decoding time, etc.

A good filter should improve coding performance with less cost. But there is always trade off among computational complexity, data bandwidth and capability of parallel, and trade off between complexity of encoding and decoding.

- 5  Reference

- In Suk Chong, Wei-Jung Chien, Naren Malayath, and Marta Karczewicz, “Encoder complexity analysis and performance report on adaptive loop filter”, JCTVC-B095, July, 2010

- Marta Karczewicz, Peisong Chen, Rajan Joshi, Xianglin Wang, Wei-Jung Chien, Rahul Panchal, “Video coding technology proposal by Qualcomm Inc.”, JCTVC-A121, April, 2010

- Tomohiro Ikai, Tomoyuki Yamamoto, Yoshihiro Kitaura, “A parallel adaptive loop filter”,  JCTVC-B064, July, 2010

- Yu-Wen Huang, Chih-Ming Fu, Ching-Yeh Chen, Chia-Yang Tsai, and so on. “In-Loop Adaptive Restoration”, JCTVC-B077, July 2010.

- Takeshi Chujoh, Akiyuki Tanizawa, Tomoo Yamakage, “Description of video coding technology proposal by TOSHIBA”, JCTVC-A121, April, 2010

[6]  Frank Bossen , “Common test conditions and software reference configurations”, JCTVC-B300, July 2010

# Patent rights declaration(s)

Zhejiang University has no IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).