| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C078r1 WG11 Number: m18101 |
| --- | --- |

| Title: | TE3 subtest 4: High Accuracy Interpolation Filter (HAIF) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Kazuyo Kanou Tomoo Yamakage 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, Japan | Tel: Email: | +81-44-549-2288 takeshi.chujoh@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

In this contribution, experimental results of high accuracy interpolation filters are reported.  This is one of proposals in subtest 4 of tool experiment 3 on inter prediction.  The purposes of this tool experiment are to improve the performance of inter prediction with enhanced motion compensation filters.  The experimental results show that the interpolation filter which derives each fractional pixel position directly and have more sharp frequency characteristics with long tap length can achieve good performance and the interpolation filter and the in-loop filter have strong relation as a filter processing technology.  At the nest stage, computational reduction for the interpolation filter should be discussed.

# Introduction

The purpose of subset 4 in TE3 [1] is to improve the performance of inter prediction with enhanced motion compensation filters.  In this contribution, according to common condition, JCTVC-B300 [2], the experimental results of several interpolation filters are reported

# High accuracy interpolation filter (HAIF)

HAIF (High-Accuracy Interpolation Filter) is a motion compensation scheme to interpolate fractional pixels according to fractional pixel motion vector with quarter-pel resolution.  The interpolation filter is defined as a 1-dimentional FIR filter. If the motion vector points out fractional pixel position both horizontally and vertically, the 1-dimentional FIR filter is performed horizontally and vertically.

In H.264/AVC, the purposes of the interpolation filter are (1) to reduce coding noise of decoded picture and (2) adjust the pixel position to fractional pixel position.  Since TMuC software adopts several image in-loop restoration filters to reduce coding noise, the purpose of the interpolation filter is concentrated to (2).  Therefore, each fractional pixel position is derived directly from pixels at integer pixel positions to minimize low pass filter characteristics.

The filter coefficients of HAIF [3] that are the same as [4] are followings.

1/4 pixel position: {-3, 12, -37, 229, 71, -21, 6,-1} // 256

1/2 pixel position: {-3, 12, -39, 158, 158, -39, 12, -3} // 256

Actually, DCT-IF on TMuC [5] has same characteristics.

Additionally, an interpolation (HAIF86) that has 8-tap filter and 6-tap filter for computational complexity reduction is tested.  In case of inner fractional position, asymmetrical 6-tap is used and symmetrical 8-tap filter is used and in case of boundary fractional position, both asymmetrical and symmetrical 8-tap filter is used.  Since the number of multiplication of symmetrical filter is smaller than that of asymmetrical filter at the same tap-length generally, the worst number of multiplication of HAIF86 is the same as that of 6-tap filter.

The filter coefficients of 6-tap filters are followings.

1/4 pixel position: {12, -38, 228, 70, -22, 6} //256.

# Experimental results

The simulation was conducted for all sequences of WQVGA, WVGA, 720p, 1080p and cropped 4kx2k based on the recommended simulation common conditions, JCTVC-B300.  Table 1, Table 2 and Table 3 indicate the average results of coding efficiency compared to each anchor.  According to JCTVC-300, the interpolation filter of high efficiency anchors is SIFO (Switched Interpolation Filter with Offset) 12-tap/8-tap and the interpolation filter of low complexity anchors is DIF (Directional Interpolation Filter) 6-tap.  ALF is included in high efficiency anchors and is not included in low complexity anchors.

Table 1	Average results of luminance BD-rate (%) on high efficiency anchor

|  |  | HAIF (8-tap) | HAIF86 (8/6-tap) | DCT-IF (6-tap) |
| --- | --- | --- | --- | --- |
| Random access | Class A | 0.32 | 0.37 | 0.44 |
|  | Class B | 0.17 | 0.50 | 1.02 |
|  | Class C | 0.93 | 1.35 | 2.21 |
|  | Class D | 1.51 | 2.55 | 4.44 |
|  | Class E | N/A | N/A | N/A |
|  | All | 0.75 | 1.26 | 2.17 |
|  | Enc Time[%] | 62.22 | 63.04 | 56.96 |
|  | Dec Time[%] | 94.25 | 100.38 | 92.01 |
| Low delay | Class A | N/A | N/A | N/A |
|  | Class B | 0.83 | 1.22 | 1.64 |
|  | Class C | 1.99 | 2.79 | 3.07 |
|  | Class D | 3.84 | 5.82 | 6.23 |
|  | Class E | 0.58 | 0.75 | 2.17 |
|  | All | 1.82 | 2.68 | 3.24 |
|  | Enc Time[%] | 67.34 | 68.33 | 61.20 |
|  | Dec Time[%] | 95.43 | 101.13 | 93.89 |

Table 2	Average results of luminance BD-rate (%) on low complexity anchor

|  |  | HAIF (8-tap) | HAIF86 (8/6-tap) | DCT-IF (6-tap) |
| --- | --- | --- | --- | --- |
| Random access | Class A | -0.74 | 0.11 | 0.53 |
|  | Class B | -1.91 | -1.31 | -0.69 |
|  | Class C | -4.54 | -4.21 | -2.94 |
|  | Class D | -9.34 | -8.79 | -6.85 |
|  | Class E | N/A | N/A | N/A |
|  | All | -4.44 | -3.89 | -2.77 |
|  | Enc Time[%] | 111.64 | 111.83 | 100.30 |
|  | Dec Time[%] | 102.51 | 102.69% | 97.93 |
| Low delay | Class A | N/A | N/A | N/A |
|  | Class B | -1.53 | -1.25 | 1.09 |
|  | Class C | -4.38 | -3.92 | -1.52 |
|  | Class D | -8.90 | -7.90 | -5.14 |
|  | Class E | -1.70 | -0.91 | 3.50 |
|  | All | -4.11 | -3.52 | -0.67 |
|  | Enc Time[%] | 112.63 | 112.94 | 101.10 |
|  | Dec Time[%] | 101.55 | 101.77 | 98.06 |

Table 3	Average results of luminance BD-rate (%) on low complexity low delay anchor

|  |  | HAIF (8-tap) | ALF | HAIF (8-tap) + ALF |
| --- | --- | --- | --- | --- |
| Random access | Class A | -0.74 | -3.92 | -5.01 |
|  | Class B | -1.91 | -4.67 | -6.41 |
|  | Class C | -4.54 | -4.39 | -8.06 |
|  | Class D | -9.34 | -5.93 | -13.00 |
|  | Class E | N/A | N/A | N/A |
|  | All | -4.44 | -4.83 | -8.42 |
|  | Enc Time[%] | 111.64 | 101.55 | 114.86 |
|  | Dec Time[%] | 102.51 | 114.28 | 118.54 |
| Low delay | Class A | N/A | N/A | N/A |
|  | Class B | -1.53 | -4.40 | -6.53 |
|  | Class C | -4.38 | -4.89 | -8.88 |
|  | Class D | -8.90 | -4.50 | -12.20 |
|  | Class E | -1.70 | -2.90 | -4.59 |
|  | All | -4.11 | -4.27 | -8.17 |
|  | Enc Time[%] | 112.63 | 101.41 | 114.99 |
|  | Dec Time[%] | 101.55 | 116.55 | 120.37 |

# Conclusions

In this contribution, the simulation results of several interpolation filters have been shown.

The experimental results show that the interpolation filter which derives each fractional pixel position directly and have more sharp frequency characteristics with long tap length can achieve good performance and the interpolation filter and the in-loop filter have strong relation as a filter processing technology.

At the nest stage, computational reduction for the interpolation filter should be discussed.

# References

[1] 	Andreas Krutz, et.al, “Tool Experiment 3: Inter Prediction in HEVC,” Joint Collaborative Team on Video Coding, JCTVC-B303r2, Geneva, July 2010.

[2] 	F. Bossen, “Common test conditions and software reference configurations,” Joint Collaborative Team on Video Coding, JCTVC-B300, Geneva, July 2010.

[3] 	T. Chujoh, A. Tanizawa and T. Yamakage, “Description of video coding technology proposal by TOSHIBA,” Joint Collaborative Team on Video Coding, JCTVC-A117, Dresden, April 2010.

[4] 	J. Ostermann, M. Narroschke, “Motion compensated prediction with 1/8-pel displacement vector resolution,” , Video Coding Experts Group, VCEG-AD09, Hangzhou, Oct. 2006.

[5] 	“Test Model under Consideration,” Joint Collaborative Team on Video Coding, JCTVC-B205, Geneva, July 2010.

# Patent rights declaration(s)

TOSHIBA Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).