| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C086 WG11 Number: m18109 |
| --- | --- |

| Title: | Experimental results of ALF on low complexity |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Tomoo Yamakage 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, Japan | Tel: Email: | +81-44-549-2288 takeshi.chujoh@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

In this contribution, experimental results of ALF (Adaptive Loop Filter) on low complexity are reported.  This is not a new proposal and only a report of experimental results of tools that have been already implemented on TMuC software.  The experimental results show that ALF and the interpolation filter have strong relation as a filter processing technology and two tools, ALF and DCT-IF (Discrete Cosine Transform – Interpolation Filter) improves the coding efficiency effectively.  For example, the coding gains of ALF and DCT-IF 6-tap on low complexity anchor are average of 4.6% and 1.7% independently; however the coding gain of ALF plus to DCT-IF 6-tap is average of 7.3% that is greater than the sum of the gain of respective two tools.  The encoding time is increased 5% and decoding time is increased 18%.

# Experimental results

According to JCTVC-B312r6 [1], the simulation was conducted for all sequences of WQVGA, WVGA, 720p, 1080p and cropped 4kx2k based on the recommended simulation common conditions, JCTVC-B302r2 [2].  The 4-point BD-rate (%) is measured according to RBJM macro by JCTVC-A031 [3].  The version of TMuC software is 0.71.  The OS is Windows XP SP2 64-bit, the CPU is Xeon® W3565, 3.2GHz and the compiler is Visual C++ 2008.  Error: Reference source not found and 2 indicate the summary results of coding efficiency, encoding and decoding times ALF compared to low complexly anchor.  In the case of low complexity anchor, the interpolation filter is DIF (Directional Interpolation Filter) 6-tap and ALF is off.

Table 1 	Y BD-rate (%), encoding time and decoding time of ALF on low complexity random access anchor

|  | ALF | DCT-IF 6-tap | ALF + DCT-IF 6-tap |
| --- | --- | --- | --- |
| Class A | -3.92 | 0.53 | -4.85 |
| Class B | -4.67 | -0.69 | -5.96 |
| Class C | -4.39 | -2.94 | -7.08 |
| Class D | -5.93 | -6.85 | -11.41 |
| Class E | N/A | N/A | N/A |
| All | -4.83 | -2.77 | -7.56 |
| Enc Time[%] | 101.55 | 100.30 | 105.22 |
| Dec Time[%] | 114.28 | 97.93 | 116.32 |

Table 2	Y BD-rate (%), encoding time and decoding time of ALF on low complexity low delay anchor

|  | ALF | DCT-IF 6-tap | ALF + DCT-IF 6-tap |
| --- | --- | --- | --- |
| Class A | N/A | N/A | N/A |
| Class B | -4.40 | 1.09 | -5.79 |
| Class C | -4.89 | -1.52 | -7.95 |
| Class D | -4.50 | -5.14 | -10.89 |
| Class E | -2.90 | 3.50 | -3.17 |
| All | -4.27 | -0.67 | -7.11 |
| Enc Time[%] | 101.41 | 101.10 | 105.28 |
| Dec Time[%] | 116.55 | 98.06 | 119.04 |

The cross verification will be conducted by Samsung.

# Conclusion

In this contribution, the experimental results of ALF were shown.  The experimental results show that ALF and the interpolation filter have strong relation as a filter processing technology and two tools, ALF and DCT-IF improves the coding efficiency effectively.  It is suggested that ALF should be introduced into Test Model.

# Reference

[1] 	K. McCann, “Tool Experiment 12: Evaluation of TMuC Tools,” Joint Collaborative Team on Video Coding, JCTVC-B312r6, Geneva, July 2010.

[2] 	F. Bossen, “Common test conditions and software reference configurations,” Joint Collaborative Team on Video Coding, JCTVC-B300, Geneva, July 2010.

[3] 	S. Pateux, “Tools for proposal evaluations,” Joint Collaborative Team on Video Coding, JCTVC-A031, Dresden, April 2010.

[4] 	A. Tanizawa, T. Chujoh, and T. Yamakage, “Synergistic Effect of High Accuracy Interpolation Filter (HAIF) and Quad-tree based Adaptive Loop Filter (QALF),” Joint Collaborative Team on Video Coding, JCTVC-B043, Geneva, July 2010.

# Patent rights declaration(s)

TOSHIBA Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).