| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C087 WG11 Number: m18110 |
| --- | --- |

| Title: | TE12.2: Experimental results of interpolation filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Kazuyo Kanou Tomoo Yamakage 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, Japan | Tel: Email: | +81-44-549-2288 takeshi.chujoh@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

In this contribution, detailed results of interpolation filters are reported.  This is one of reports in Tool Experiment 12: Evaluation of TMuC Tools.  The interpolation filter of high efficiency anchor is SIFO (Switched Interpolation Filter with Offset) and that of low complexity anchor is DIF (Directional Interpolation Filter).  In this experiment, some DCT-IFs (Discrete Cosine Transform – Interpolation Filter) are compared to anchor filters.  The experimental results show that the coding gain of DCT-IF 12-tap compared to SIFO in the case of high efficiency anchor is average of 0.1% and the coding gain of DCT-IF 6-tap compared to DIF in the case of low complexity anchor is average of 3.6%.  The encoding of DCT-IF 12-tap compared to SIFO in the case of high efficiency anchor is about 2/3 and the decoding time is decreased 5%.  The encoding and decoding times of DCT-IF 6-tap compared to DIF in the case of low complexity anchor are stable.

# Experimental results

According to JCTVC-B312r5 [1], the simulation was conducted for all sequences of WQVGA, WVGA, 720p, 1080p and cropped 4kx2k based on the recommended simulation common conditions, JCTVC-B302r2 [2].  The 4-point BD-rate (%) is measured according to RBJM macro by JCTVC-A031 [3].  The version of TMuC software is 0.71.  The OS is Windows XP SP2 64-bit, the CPU is Xeon® W3565, 3.2GHz and the compiler is Visual C++ 2008.  Error: Reference source not found and 2 indicate the summary results of coding efficiency of DCT-IF compared to anchors.

Table 1 	Y BD-rate (%), encoding time and decoding time of DCT-IF on high efficiency anchor

|  | DCT-IF 12-tap |  | DCT-IF 8-tap |  |
| --- | --- | --- | --- | --- |
|  | Random access | Low delay | Random access | Low delay |
| Class A | 0.17 | N/A | 0.19 | N/A |
| Class B | 0.01 | 0.31 | 0.15 | 0.78 |
| Class C | -0.35 | -0.10 | 0.33 | 1.33 |
| Class D | -0.78 | -0.12 | 0.23 | 1.29 |
| Class E | N/A | 0.16 | N/A | 0.98 |
| All | -0.28 | 0.07 | 0.23 | 1.08 |
| Enc Time[%] | 64.70 | 70.15 | 62.12 | 67.18 |
| Dec Time[%] | 94.01 | 94.96 | 94.43 | 95.44 |

Table 2	Y BD-rate (%), encoding time and decoding time of DCT-IF on low complexity anchor

|  | DCT-IF 6-tap |  | DCT-IF 8-tap |  |
| --- | --- | --- | --- | --- |
|  | Random access | Low delay | Random access | Low delay |
| Class A | 0.53 | N/A | 0.60 | N/A |
| Class B | -0.69 | 1.09 | -0.72 | 1.02 |
| Class C | -2.94 | -1.52 | -5.32 | -4.08 |
| Class D | -6.85 | -5.14 | -11.16 | -9.14 |
| Class E | N/A | 3.50 | N/A | 2.39 |
| All | -2.77 | -0.67 | -4.55 | -2.54 |
| Enc Time[%] | 100.30 | 101.10 | 111.53 | 112.26 |
| Dec Time[%] | 97.93 | 98.06 | 102.53 | 101.51 |

# Conclusion

In this contribution, the experimental results of interpolation filters were shown.  The experimental results show that the coding gain of DCT-IF 12-tap compared to SIFO in the case of high efficiency anchor is average of 0.1% and the coding gain of DCT-IF 6-tap compared to DIF in the case of low complexity anchor is average of 3.6%.  The encoding of DCT-IF 12-tap compared to SIFO in the case of high efficiency anchor is about 2/3 and the decoding time is decreased 5%.  The encoding and decoding times of DCT-IF 6-tap compared to DIF in the case of low complexity anchor are stable.  It is suggested that the interpolation of Test Model should be considered current anchor specifications carefully.

# Reference

[1] 	K. McCann, “Tool Experiment 12: Evaluation of TMuC Tools,” Joint Collaborative Team on Video Coding, JCTVC-B312r5, Geneva, July 2010.

[2] 	F. Bossen, “Common test conditions and software reference configurations,” Joint Collaborative Team on Video Coding, JCTVC-B300, Geneva, July 2010.

[3] 	S. Pateux, “Tools for proposal evaluations,” JCT-VC Document, JCTVC-A031, Dresden, April 2010.

# Patent rights declaration(s)

TOSHIBA Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).