| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C088 WG11 Number: m18111 |
| --- | --- |

| Title: | TE12.7: Experimental results of IBDI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Tomoo Yamakage 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, Japan | Tel: Email: | +81-44-549-2288 takeshi.chujoh@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

In this contribution, detailed results of IBDI (Internal Bit-Depth Increase) and TPE (Transform Precision Extension) are reported.  This is one of reports in Tool Experiment 12: Evaluation of TMuC Tools.  Technically, TPE is an internal bit-depth increase of transform processing and a part of IBDI.  The experimental results show that the coding gain of TPE that is also the most of gain of IBDI in case of intra is average of 1% and the coding gain of IBDI in the case of inter is additionally average of 2%.  The encoding and decoding times are stable.

# Experimental results

According to JCTVC-B312r5 [1], the simulation was conducted for all sequences of WQVGA, WVGA, 720p, 1080p and cropped 4kx2k based on the recommended simulation common conditions, JCTVC-B302r2 [2].  The 4-point BD-rate (%) is measured according to RBJM macro by JCTVC-A031 [3].  The version of TMuC software is 0.71.  The OS is Windows XP SP2 64-bit, the CPU is Xeon® W3565, 3.2GHz and the compiler is Visual C++ 2008.  Table 1 indicates the summary results of coding efficiency of IBDI and TPE compared to IBDI off and TPE off anchors.

Table 1  Y BD-rate (%), encoding time and decoding time of IBDI and TPE

|  |  | IBDI off, TPE on |  | IBDI on, TPE off |  | IBDI on, TPE on |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | High efficiency | Low complexity | High efficiency | Low complexity | High efficiency | Low complexity |
|  |  | BD-rate | BD-rate | BD-rate | BD-rate | BD-rate | BD-rate |
| Intra | Class A | -0.89 | -1.05 | -0.63 | -0.78 | -0.64 | -0.78 |
|  | Class B | -1.23 | -1.65 | -1.05 | -1.40 | -1.05 | -1.40 |
|  | Class C | -0.43 | -0.44 | -0.19 | -0.37 | -0.19 | -0.36 |
|  | Class D | -0.35 | -0.27 | -0.05 | -0.05 | -0.06 | -0.05 |
|  | Class E | -1.85 | -2.07 | -2.13 | -2.82 | -2.14 | -2.82 |
|  | All | -0.92 | -1.08 | -0.77 | -1.04 | -0.78 | -1.04 |
|  | Enc Time[%] | 100.23 | 101.21 | 100.87 | 101.70 | 101.01 | 103.05 |
|  | Dec Time[%] | 99.07 | 99.23 | 100.82 | 100.92 | 101.01 | 101.00 |
| Random access | Class A | -0.66 | -0.92 | -2.24 | -2.27 | -2.22 | -2.28 |
|  | Class B | -1.19 | -1.52 | -3.25 | -3.06 | -3.23 | -3.05 |
|  | Class C | -0.68 | -0.65 | -1.96 | -1.56 | -1.95 | -1.57 |
|  | Class D | -0.32 | -0.29 | -1.15 | -0.83 | -1.16 | -0.82 |
|  | Class E |  |  |  |  |  |  |
|  | All | -0.75 | -0.88 | -2.21 | -1.96 | -2.20 | -1.96 |
|  | Enc Time[%] | 99.79 | 100.28 | 99.69 | 100.09 | 99.51 | 100.25 |
|  | Dec Time[%] | 99.92 | 100.06 | 101.05 | 101.17 | 101.50 | 101.13 |
| Low delay | Class A |  |  |  |  |  |  |
|  | Class B | -1.85 | -1.73 | -4.84 | -3.68 | -4.84 | -3.65 |
|  | Class C | -1.02 | -0.82 | -3.01 | -2.06 | -2.99 | -2.09 |
|  | Class D | -0.57 | -0.45 | -1.76 | -1.03 | -1.80 | -1.05 |
|  | Class E | -2.29 | -2.10 | -10.41 | -6.05 | -10.47 | -6.12 |
|  | All | -1.41 | -1.25 | -4.66 | -3.06 | -4.67 | -3.07 |
|  | Enc Time[%] | 99.88 | 99.89 | 100.06 | 100.07 | 99.86 | 100.04 |
|  | Dec Time[%] | 100.20 | 100.17 | 100.69 | 101.07 | 101.29 | 101.30 |

# Conclusion

In this contribution, the experimental results of IBDI and TPE were shown.  The experimental results show that the coding gain of TPE that is also the most of gain of IBDI in case of intra is average of 1% and the coding gain of IBDI in the case of inter is additionally average of 2%.  The encoding and decoding times are stable.  It is suggested that IBDI should be introduced into Test Model.

# Reference

[1] 	K. McCann, “Tool Experiment 12: Evaluation of TMuC Tools,” Joint Collaborative Team on Video Coding, JCTVC-B312r5, Geneva, July 2010.

[2] 	F. Bossen, “Common test conditions and software reference configurations,” Joint Collaborative Team on Video Coding, JCTVC-B300, Geneva, July 2010.

[3] 	S. Pateux, “Tools for proposal evaluations,” JCT-VC Document, JCTVC-A031, Dresden, April 2010.

# Patent rights declaration(s)

TOSHIBA Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).