| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C092 WG11 Number: m18115 |
| --- | --- |

| Title: | TE12: Report on Combined Intra Prediction evaluation |  |  |
| --- | --- | --- | --- |
| Status: | [Input Document to JCT-VC] |  |  |
| Purpose: | [Report and Proposal] |  |  |
| Author(s) or Contact(s): | Keiichi Chono Kenta Senzaki Hirofumi Aoki Junji Tajime Yuzo Senda 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 431 7595 chono@ct.jp.nec.com k-senzaki@bp.jp.nec.com hao@bu.jp.nec.com tajime@ak.jp.nec.com y-senda@bc.jp.nec.com |
| Source: | [Information and Media Processing Laboratories, NEC Corporation] |  |  |

_____________________________

# Abstract

This contribution presents a report on Combined Intra Prediction (CIP) evaluation. Simulation results show that:

- BD-Rate increases caused by disabling CIP are only 0.3%, 0.2%, and 0.1% for High Coding Efficiency (HCE) All Intra, Random Access, Low Delay coding conditions;

- BD-Rate increases caused by disabling CIP are only 0.2%, 0.1%, and 0% for Low Complexity (LC) All Intra, Random Access, Low Delay coding conditions;

- Encoding time decrease observed by disabling CIP is about 20% for HCE and LC All Intra coding conditions; significant encoding time decrease is not observed by disabling CIP in other coding conditions; and

- Significant decoding time decrease is not observed in all coding conditions.

It is proposed that CIP is not used in default coding configurations in TMuC software due to its small coding gain and that CIP is re-evaluated on the top of an HEVC test model tool set determined by the TE12 results.

- Introduction

The CIP is an intra prediction tool. CIP predictions comprise a weighted combination of an Angular intra prediction together with a pixel-by-pixel local mean prediction. Its conceptual diagram is illustrated in Figure 1. The local mean prediction is the average of pixel values to the left, top-left and above the current pixel. These values are original values in the encoder (except where reconstructed pixels are available), whereas they are the reconstructed values after inverse quantization and transform in the local decoder of the encoder and the decoder. Angular intra predictions provide an overall prediction direction and local mean predictions provide local adaptation. The CIP description of TMuC document JCT-VC B205 [2] claims that the CIP provides pixel-by-pixel adaptation and improves intra prediction, especially in large blocks where a directional intra prediction may not work well.

This contribution describes CIP encoder implementation in TMuC0.7 and its simulation results conducted according to the coding conditions described in JCTVC-B300 [2] and JCTVC-B312 [3].

Figure 1	 Example of Combined Intra Prediction using weighted combination of local mean prediction and Angular intra prediction with (Intra_Vertical, +6) for 8x8 PU partition

- Description of CIP on/off decision process in TMuC

In the current TMuC implementation, a kind of early skipping mode decision methods is introduced as in P- and B- slices, inter prediction modes are evaluated prior to intra prediction modes, and the intra mode decision process is skipped if the best inter mode results in all zeros for Y and UV blocks. Therefore this section describes TMuC’s intra mode decision process in I-slice encoding and the CIP on/off decision process during that process. Then with the assumption that a search for finding the best direction of Angular intra predictions is the most time-consuming part in intra mode decision process for I-slices, we consider CIP’s encoder complexity in terms of the number of the searches.

[Intra mode decision process in I-slice encoding]
For each PU, two partition sizes (2Nx2N and NxN) are evaluated by estimating their best directions of Angular intra predictions by disabling CIP. The best PU partition size (either 2Nx2N or NxN) is determined by comparing Rate-Distortion (R-D) costs denoted by:
   J (PuSize, dir, CIP=0) = SSD(PuSize, dir, CIP=0) + lambda*Rate(PuSize, dir, CIP=0) ,
where PuSize is a PU partition size and dir is a direction of Angular intra predictions. (Note that only some representative directions of Angular intra predictions are subjected to the Rate-Distortion cost computation.) After that, Planar mode, and CIP=1 and ROT indices with the best partition size, are subjected to Rate-Distortion cost computation.

[CIP on/off decision process in intra mode decision process]
During the CIP=1 evaluation with the best partition size, the best direction of Angular intra predictions is re-estimated on the basis of comparing each of the R-D costs, J(PuSize, dir, CIP=1), for the some representative directions. If the R-D cost of CIP=1 with the best partition size and direction among those of CIP=0, Planar mode, and ROT index > 0, the encoder decides that current PU uses CIP. Otherwise, the encoder decides that current PU does not use CIP. 


[The number of searches for finding the best direction of Angular intra predictions per LCU]
The list of best-direction searches for the 2Nx2N and NxN decision, CIP on/off decision, and ROT index decision for each CU size is below.

Table 1		List of the number of best-direction searches for the 2Nx2N/NxN decision, CIP on/off decision, and ROT index decision

Since current TMuC Anchor encoding conditions use a CU size ranging from 8x8 to 64x64, the total number of best-direction searches is 22. Within 22, the best-direction search number for CIP=1 decision is 4. Therefore not-using CIP in I-slice encoding decreases the encoding time by a factor of 4/22 (about 18%).

- Simulation

3.1. Coding conditions

We evaluated CIP according to the coding conditions described in JCTVC-B300 [2] and JCTVC-B312 [3]. The attached Excel sheet (JCTVC-C092.xls) provides the results by using (a) the TMuC0.7 software as CIP1, (b) the TMuC0.7 with CIP=0 as CIP0. Note that if an encoder crash happens due to the SIFO’s bug, the test case was re-run by using BUGFIXTMP50 in TMuC0.7.3 for both Anchor and Proposal.

3.2. Computing platforms

A 64-bit version executable was used for encoding Class A, B, C, and D, and a 32-bit one was used for Class E. A 64-bit version executable was used for decoding all Classes. The executables were generated by using Microsoft Visual C++ 2008 with SP1 and ran on Windows-based computing platforms listed below.

| Computing platform | Usage |
| --- | --- |
| Windows 7 64-bit Professional Core i7 2.8GHz and Memory 12GB | Anchor and CIP=0 encoding Class A, B, C, and D. Up to 8 encodings ran at the same time. |
| Windows XP 32-bit Professional Xeon 3.33GHz and Memory 3GB | Anchor and CIP=0 encoding Class E. Up to 8 encodings ran at the same time. |
| Windows XP 64-bit Professional Opteron 2.60GHz and Memory 32GB | Anchor and CIP=0 decoding all Classes. |

As inferred from the list, CPU automated over-clocking happened for some encoding results. Therefore individual encoding time can be significantly varied. However, since decoding was run on a single core CPU, such CPU automated over-clocking effect did not happen for decoding time comparison.

3.3. BD- Rate/PSNR and Encoding/Decoding analysis

Tables 2-7 show the summary of BD-Rate (%), BD-PSNR(dB), relative encoding and decoding time (%) against Anchor.

Table 2: Results for High Coding Efficiency All Intra.

Table 3: Results for High Coding Efficiency Random Access.

Table 4: Results for High Coding Efficiency Low Delay.

Table 5: Results for Low Complicity All Intra.

Table 6: Results for Low Complicity Random Access.

Table 7: Results for Low Complicity Low Delay.

Simulation results show that:

- BD-Rate increases caused by disabling CIP are only 0.3%, 0.2%, and 0.1% for High Coding Efficiency (HCE) All Intra, Random Access, Low Delay coding conditions;

- BD-Rate increases caused by disabling CIP are only 0.2%, 0.1%, and 0% for Low Complexity (LC) All Intra, Random Access, Low Delay coding conditions;

- Encoding time decrease observed by disabling CIP is about 20% for HCE and LC All Intra coding conditions; significant encoding time decrease is not observed by disabling CIP in other coding conditions; and

- Significant decoding time decrease is not observed in all coding conditions.

Furthermore, we observed that even in low complexity coding conditions in which coding gains come from individual tool show up, CIP's coding gain is very limited (up to 0.8% in BD-Rate for ParkScene All Intra).

As for CIP’s complexity, although CIP’s impact on the decoding time is small as expected from its required arithmetic operations deduced from the CIP description of TMuC document JCT-VC B205 [2], it should be asserted that the use of CIP can affect the throughput of encoding and decoding processes due to its pixel-by-pixel dependent process.

- Cross-verification results

NEC verified the following items though the cross-verification with Sharp:

- BD-Rate values provided by Sharp were matched with NEC’s: there is little difference come from the computing platform differences (Linux and Windows).

- Encoding and decoding time on average provided by Sharp were matched with NEC’s.

- Decoded YUV MD5 files of bitstreams provided by Sharp on NEC’s computing platform exactly matched with those provided by Sharp.

Based on the verification, we have confirmed that our CIP evaluation results in TE12 are accurate.

- Conclusions

This contribution reported the result of TE12 Combined Intra Prediction (CIP) evaluation. It is recommended that CIP is not used in default coding configurations in TMuC software due to its small coding gain and that CIP is re-evaluated on the top of an HEVC test model tool set determined by the TE12 results.

- Reference

- JCT-VC, “Draft Test Model under Consideration”, JCTVC-B205, 2nd JCT-VC Meeting, Geneva, July 2010. Document: JCTVC- B205, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 2nd Meeting: Geneva, CH, 21-28 July, 2010.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-B300, 2nd JCT-VC Meeting, Geneva, July 2010. Document: JCTVC- B205, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 2nd Meeting: Geneva, CH, 21-28 July, 2010.

- JCT-VC, “Tool Experiment 12: Evaluation of TMuC Tools”, JCTVC-B312, 2nd JCT-VC Meeting, Geneva, July 2010. Document: JCTVC- B205, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 2nd Meeting: Geneva, CH, 21-28 July, 2010.

# Patent rights declaration(s)

NEC Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).