| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C093 WG11 Number: m18116 |
| --- | --- |

| Title: | TE2: 1-D DPCM-based memory compression |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Hirofumi Aoki Keiichi Chono Kenta Senzaki Junji Tajime Yuzo Senda 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 3757 5438 hao@bu.jp.nec.com chono@ct.jp.nec.com k-senzaki@bp.jp.nec.com tajime@ak.jp.nec.com y-senda@bc.jp.nec.com |
| Source: | NEC Corporation |  |  |

_____________________________

# Abstract

In this contribution, a performance report on the 1-D DPCM-based memory compression method described in JCTVC-B057 is presented. The method is designed to reduce reference frame memory size and memory access bandwidth in motion compensation, with relatively low complexity. In consideration of the tradeoff between memory accessibility and image quality, the method employs simple 1-D DPCM. Experimental results have shown that average coding losses are 10.0 % for test cases without IBDI and 1.9 % for test cases with 12-bit IBDI. Subjective quality degradation caused by such coding loss is invisible for most test cases. Average increase of decoding time is as small as 2.7 % and memory bandwidth reduction ratios are 40.6 % for test cases without IBDI and 49.0 % for test cases with 12-bit IBDI. Experimental results on bandwidth have also shown that bandwidth reduction depends on memory architecture and implementation on mapping of compressed image data onto frame memory. The proposed method can easily adapt to various situations due to its flexibility and simplicity. It is proposed that the method is adopted in TMuC software and further evaluated from various aspects.

- Introduction

Tool Experiment 2 (TE2) on IBDI and memory compression [1, 2] was established at the 1st JCT-VC meeting and approved to be as ongoing work at the 2nd JCT-VC meeting. In this TE, the following two functionalities have been addressed:

- Improvement of coding efficiency by increasing internal process of video codec while minimizing memory access bandwidth.

- Reduction of reference frame memory access bandwidth and reference frame memory size while minimizing visual quality degradation.

Reference frame memory compression is a promising approach to meet above two functionalities. It is because prediction from reconstructed pictures stored in frame memory is the most memory-consuming part of video codec, though the high compression efficiency of up-to-date video coding technologies, including Test Model under Consideration (TMuC), depends highly on the prediction. 1-D DPCM-based memory compression method [3] has been proposed as an approach to reference frame memory compression. In this contribution, the algorithm of the proposed memory compression method is reviewed and detailed experimental results with an implementation on TMuC 0.7, including memory access bandwidth reduction, are presented.

- Algorithm Description

## Overview

Reference frames are compressed before being stored in memory and they are decompressed after being read from the memory. In consideration of the tradeoff between memory accessibility and image quality, the proposed method employs simple 1-D DPCM for the compression and decompression.

Figure 1 shows an example for memory compression with the proposed 1-D DPCM. Here a 1-D DPCM block, which is hereafter referred to as a memory compression unit (MCU), consists of eight consecutive pixels. The left-most pixel is first coded with PCM and serves as a base point. The other seven pixels are then coded with DPCM from left to right, using a five-bit nonlinear quantizer. Actually the block size, i.e., base-pixel interval and bit allocation for prediction errors can be variable in addition to a nonlinear quantization matrix. In the implementation for TE2, these parameters are signaled to decoders as side information for each sequence parameter set. Detail syntax of the parameter signaling is described in section 3.

Figure 1: 1-D DPCM prediction structure and bit allocation.

## Compression

The proposed memory compression algorithm is described below.

Given a 1-D image block consisting of N consecutive pixels, let x(n) be the nth pixel value of the block. Compressed signal y(n) (n=0,.., N-1) from x(n) with the proposed 1-D DPCM is then derived by:

y(n)=x(n);n=0epqridx(n);n≠0

where epqridx(n) represents an index to a quantization representative value, and it is obtained as follows.

epqridx(n)=argmini=0,..,2M−1epqr(i)−ep(n)

ep(n)=(x(n)−xp(n)+2B) % 2B

xp(n)=xr(n−1)

xr(n)=(xp(n)+epqr(epqridx(n))+2B) % 2B

where xr(n) is the reconstructed pixel value of x(n), xp(n) is the predicted pixel value for x(n), ep(n) is the prediction error, epqr(i) is an i-th quantization representative, and B represents bit depth of input pixels.

## Decompression

Decompressed signal xd(n) from y(n) is also obtained by the following simple equation.

xd(n)=y(n);n=0(xd(n−1)+epqr(epqridx(n))+L)%L;n≠0

As shown here, in addition to simplicity of its data structure, decompression of the proposed method is simple and straightforward. Therefore its computational impact is negligibly small.

# Syntax Description

## Syntax

Syntax for embedding information about the proposed memory compression method is shown below. This is slightly changed from the previous proposal, JCTVC-B057. In the software submitted for cross-verification for TE2, memory_compression_parameter_set() is put into seq_param_set_rbsp(). Of course it can also be put into pic_param_set_rbsp(), in order to fit the quantizer characteristics to each pictures or to switch memory compression on and off.

| memory_compression_parameter_set ( ) { | C | Descriptor |
| --- | --- | --- |
| memory_compression_flag | 0 ¦ 1 | u(1) |
| if(memory_compression_flag) { |  |  |
| dpcm_bit_depth_luma_minus1 | 0 ¦ 1 | ue(v) |
| DpcmBitDepthY = dpcm_bit_depth_luma_minus+1 |  |  |
| if(DpcmBitDepthY < BitDepthY+increased_bit_depth_luma) { |  |  |
| luma_dpcm_unit_horizontal_size_minus1 | 0 ¦ 1 | ue(v) |
| luma_dpcm_unit_vertical_size_minus1 | 0 ¦ 1 | ue(v) |
| luma_dpcm_qlist_present_flag | 0 ¦ 1 | u(1) |
| if(luma_dpcm_qlist_present_flag){ |  |  |
| dpcm_qlist(0, DpcmBitDepthY, BitDepthY+increased_bit_depth_luma) |  |  |
| } |  |  |
| } |  |  |
| dpcm_bit_depth_chroma_minus1 | 0 ¦ 1 | ue(v) |
| DpcmBitDepthC = dpcm_bit_depth_chroma_minus+1 |  |  |
| if(DpcmBitDepthC < BitDepthC+increased_bit_depth_chroma) { |  |  |
| chroma_dpcm_unit_horizontal_size_minus1 | 0 ¦ 1 | ue(v) |
| chroma_dpcm_unit_vertical_size_minus1 | 0 ¦ 1 | ue(v) |
| chroma_dpcm_qlist_present_flag | 0 ¦ 1 | u(1) |
| if(chroma_dpcm_qlist_present_flag){ |  |  |
| dpcm_qlist(1, DpcmBitDepthC, BitDepthC+increased_bit_depth_chroma) |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

| dpcm_qlist ( ycIdx, dpcmBitDepth, bitDepth ) { | C | Descriptor |
| --- | --- | --- |
| dpcm_qlist_asymmetry_flag | 0 ¦ 1 | u(1) |
| sizeofDpcmQlist = 1 << dpcmBitDepth |  |  |
| deltaDpcmQscale = 1 |  |  |
| dpcmQlist[ycIdx][0] = 0 |  |  |
| if( dpcm_qtable_asymmetry_flag == 1 ) { |  |  |
| for( i = 1; i < sizeofDpcmQlist; i++ ) { |  |  |
| second_delta_dpcm_qscale | 0 ¦ 1 | se(v) |
| deltaDpcmQscale += second_delta_dpcm_qscale |  |  |
| dpcmQlist[ycIdx][i] = dpcmQlist[ycIdx][i–1] + deltaDpcmQscale |  |  |
| } |  |  |
| } else { |  |  |
| for( i = 1; i < sizeofDpcmQlist/2; i++ ) { |  |  |
| second_delta_dpcm_qscale | 0 ¦ 1 | se(v) |
| deltaDpcmQscale += second_delta_dpcm_qscale |  |  |
| dpcmQlist[ycIdx][i] = dpcmQlist[ycIdx][i-1] + deltaDpcmQscale |  |  |
| } |  |  |
| dpcmQlist[ycIdx][ sizeofDpcmQlist/2] = 1 << (bitDepth–1) |  |  |
| for( i = sizeofDpcmQlist /2+1; i < sizeofDpcmQlist; i++ ) { |  |  |
| dpcmQlist[cIdx][i] = (1 << bitDepth) – dpcmQlist[cIdx][ sizeofDpcmQlist–i] |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

## Semantics

Semantics associated with the proposed syntax structure is described below.

memory_compression_flag equal to 1 specifies that parameters for frame memory compression are present. memory_compression_flag equal to 0 specifies that these parameters are not present. When memory_compression_flag is equal to 1, reference frames are compressed by the proposed memory compression method before being stored in memory and they are decompressed after being read from the memory. When memory_compression_flag is equal to 0, no frame memory compression is done.

dpcm_bit_depth_luma_minus1 specifies the number of bits allocated for the representation of DPCM prediction errors for the luma component. The value of dpcm_bit_depth_luma_minus1 shall be in the range of 0 to BitDepthY+increased_bit_depth_luma-1, inclusive.

luma_dpcm_unit_horizontal_size_minus1 specifies the width of MCU for the luma component. The value of luma_dpcm_unit_horizontal_size_minus1 shall be equal to or greater than 0.

luma_dpcm_unit_vertical_size_minus1 specifies the height of MCU for the luma component. The value of luma_dpcm_unit_vertical_size_minus1 shall be equal to or greater than 0.

luma_dpcm_qlist_present_flag equal to 1 specifies that a quantization representative value list syntax structure is present to specify the nonlinear quantization matrix used to compress DPCM prediction errors for the luma component. luma_dpcm_qlist_present_flag equal to 0 specifies that the default nonlinear quantization matrix is used for the luma component. The default nonlinear quantization matrix is defined as follows.

For 5-bit DPCM in order to compress 8-bit signal, the default nonlinear quantization matrix composed of 32 elements is defined by:

[0, 1, 2, 3, 6, 9, 14, 18, 24, 31, 39, 48, 58, 72, 86, 103, 128, 153, 170, 184, 198, 208, 217, 225, 232, 238, 242, 247, 250, 253, 254, 255]

For 7-bit DPCM in order to compress 12-bit signal, the default nonlinear quantization matrix composed of 128 elements is defined by:

[0, 4, 8, 12, 16, 20, 24, 28, 32, 36, 40, 44, 48, 60, 72, 84, 96, 108, 120, 132, 144, 164, 184, 204, 224, 240, 256, 272, 288, 312, 336, 360, 384, 412, 440, 468, 496, 528, 560, 592, 624, 660, 696, 732, 768, 808, 848, 888, 928, 984, 1040, 1096, 1152, 1208, 1264, 1320, 1376, 1444, 1512, 1580, 1648, 1748, 1848, 1948, 2048, 2148, 2248, 2348, 2448, 2516, 2584, 2652, 2720, 2776, 2832, 2888, 2944, 3000, 3056, 3112, 3168, 3208, 3248, 3288, 3328, 3364, 3400, 3436, 3472, 3504, 3536, 3568, 3600, 3628, 3656, 3684, 3712, 3736, 3760, 3784, 3808, 3824, 3840, 3856, 3872, 3892, 3912, 3932, 3952, 3964, 3976, 3988, 4000, 4012, 4024, 4036, 4048, 4052, 4056, 4060, 4064, 4068, 4072, 4076, 4080, 4084, 4088, 4092]

dpcm_bit_depth_chroma_minus1 specifies the number of bits allocated for the representation of DPCM prediction errors for the chroma component. The value of dpcm_bit_depth_chroma_minus1 shall be in the range of 0 to BitDepthC+increased_bit_depth_chroma-1, inclusive.

chroma_dpcm_unit_horizontal_size_minus1 specifies the width of MCU for the chroma component. The value of chroma_dpcm_unit_horizontal_size_minus1 shall be equal to or greater than 0.

chroma_dpcm_unit_vertical_size_minus1 specifies the height of MCU for the chroma component. The value of chroma_dpcm_unit_vertical_size_minus1 shall be equal to or greater than 0.

chroma_dpcm_qlist_present_flag equal to 1 specifies that a quantization representative value list syntax structure is present to specify the nonlinear quantization representative values used to compress DPCM prediction errors for the chroma component. chroma_dpcm_qlist_present_flag equal to 0 specifies that the quantization representative value list for the chroma component shall be inferred to be identical to that for the luma component.

dpcm_qlist_asymmetry_flag equal to 1 specifies that the quantization representative value list syntax structure contains all quantization representative values. dpcm_qlist_aymmetry_flag equal to 0 specifies that the syntax structure for the quantization representative value list contains a half of quantization representative values and remainder values shall be inferred as shown in section 3.1.

second_delta_dpcm_qscale is used to derive the i-th element of the quantization representative value list, which is denoted by dpcmQlist[ycIdx][i], for i in the range of 0 to sizeofDpcmQlist-1, inclusive.

# Experimental Results

The proposed method has been implemented on TMuC 0.7. Since in TMuC 0.7 encoder crashes happen due to the bug in implementation of SIFO, we have also integrated the BUGFIX50TMP fix into TMuC 0.7. This fix is disabled at the default setting, but in test cases where encoder crashes happen, we enabled this fix and reran encodings.

We tested the proposed method under TE2 test conditions desribed in JCTVC-B302[2]. As for the test cases, the following notation conventions are used here.

- RA: random access constraint

- LD: low delay constraint

- HCE: high coding efficiency configuration

- LC: low complexity configuration

In this experiment, parameters of the proposed method are set as follows.

- The MCU size is set to 8x1.

- The bit depth of quantized pixels is set to 5 for the high coding efficiency case, i.e., without IBDI, and 7 for the low complexity case, i.e., with 12-bit IBDI, respectively.

- Default quantization matrices are used.

## Coding efficiency and subjective quality analysis

As coding efficiency measure, 4-point BD-Rates were computed. Table 1 and Table 2 show BD-Rate for each test sequence with each condition against each type of anchor. More details are shown in the attached spreadsheet.

These results show that average coding losses measured by BD-Rate for the luma component are 8.1 % for RA-LC, 11.9 % for LD-LC, 0.8 % for RA-HCE and 3.0 % for LD-HCE, respectively. Unfortunately we also observed that subjective quality is slightly degraded in “vidyo3” sequence encoded with the LD-LC configuration, where ringing artifacts around edges are emphasized in particular with low bit-rate cases. However, for most test cases, subjective quality degradation caused by such coding loss is invisible.

Table 1: BD-Rates % for the LC configurations (without IBDI).

| Class | Sequence | RA |  |  | LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
| A | Traffic | 8.8 | 4.1 | 3.9 | N/A | N/A | N/A |
|  | People on Street | 4.5 | 2.4 | 2.2 | N/A | N/A | N/A |
| B | Kimono | 3.1 | 0.9 | 1.2 | 3.7 | 0.7 | 0.7 |
|  | ParkScene | 5.5 | 3.5 | 2.7 | 7.4 | 3.3 | 3.0 |
|  | Cactus | 10.1 | 6.7 | 7.4 | 13.2 | 3.8 | 6.2 |
|  | BasketballDrive | 5.2 | 2.6 | 3.1 | 5.8 | 2.4 | 2.9 |
|  | BQTerrace | 16.9 | 5.5 | 4.7 | 18.0 | 7.1 | 6.1 |
| C | BasketballDrill | 8.1 | 5.5 | 4.9 | 9.0 | 4.3 | 4.6 |
|  | BQMall | 9.5 | 4.6 | 4.8 | 12.6 | 5.3 | 5.7 |
|  | PartyScene | 9.8 | 5.9 | 5.9 | 10.2 | 6.1 | 5.7 |
|  | RaceHorses WVGA | 4.9 | 4.6 | 4.3 | 6.5 | 4.5 | 4.3 |
| D | BasketballPass | 4.1 | 3.2 | 3.8 | 4.9 | 2.4 | 3.2 |
|  | BQSquare | 15.1 | 5.9 | 6.0 | 12.7 | 6.4 | 6.8 |
|  | BlowingBubbles | 10.4 | 6.7 | 6.8 | 12.2 | 7.3 | 7.1 |
|  | RaceHorses WQVGA | 5.0 | 5.2 | 4.7 | 6.4 | 4.4 | 4.3 |
| E | Vidyo1 | N/A | N/A | N/A | 19.5 | 8.1 | 9.1 |
|  | Vidyo3 | N/A | N/A | N/A | 27.5 | 10.2 | 13.4 |
|  | Vidyo4 | N/A | N/A | N/A | 20.1 | 9.9 | 11.2 |
| Class A |  | 6.7 | 3.2 | 3.1 | N/A | N/A | N/A |
| Class B |  | 8.1 | 3.8 | 3.8 | 9.7 | 3.4 | 3.8 |
| Class C |  | 8.1 | 5.2 | 5.0 | 9.6 | 5.1 | 5.1 |
| Class D |  | 8.7 | 5.2 | 5.3 | 9.1 | 5.1 | 5.4 |
| Class E |  | N/A | N/A | N/A | 22.4 | 9.4 | 11.2 |
| Overall |  | 8.1 | 4.5 | 4.4 | 11.9 | 5.4 | 5.9 |

Table 2: BD-Rates % for the HCE configurations (with 12-bit IBDI).

| Class | Sequence | RA |  |  | LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
| A | Traffic | 0.9 | 0.7 | 1.1 | N/A | N/A | N/A |
|  | People on Street | 0.4 | 0.9 | 0.9 | N/A | N/A | N/A |
| B | Kimono | 0.5 | 0.8 | 1.1 | 0.6 | 0.5 | 1.9 |
|  | ParkScene | 0.6 | 0.6 | 1.0 | 1.1 | 0.6 | 1.7 |
|  | Cactus | 1.2 | 1.2 | 1.8 | 3.7 | 3.0 | 3.6 |
|  | BasketballDrive | 0.5 | 0.7 | 0.7 | 0.9 | 1.4 | 1.0 |
|  | BQTerrace | 1.6 | 2.2 | 3.2 | 2.1 | 4.2 | 8.7 |
| C | BasketballDrill | 0.9 | 0.9 | 0.7 | 2.1 | 1.5 | 2.0 |
|  | BQMall | 1.0 | 0.5 | 1.0 | 1.9 | 0.9 | 1.4 |
|  | PartyScene | 1.0 | 0.6 | 0.8 | 1.2 | 1.3 | 1.6 |
|  | RaceHorses WVGA | 0.4 | 0.3 | 0.4 | 0.6 | 0.3 | 0.5 |
| D | BasketballPass | 0.4 | 0.5 | 0.3 | 0.7 | 0.2 | 0.8 |
|  | BQSquare | 1.6 | 1.3 | 2.0 | 2.1 | 6.1 | 7.8 |
|  | BlowingBubbles | 0.9 | 0.6 | 0.9 | 1.5 | 1.1 | 1.4 |
|  | RaceHorses WQVGA | 0.4 | 0.5 | 0.8 | 0.7 | 0.8 | 0.4 |
| E | Vidyo1 | N/A | N/A | N/A | 8.7 | 10.1 | 11.8 |
|  | Vidyo3 | N/A | N/A | N/A | 10.4 | 16.1 | 17.6 |
|  | Vidyo4 | N/A | N/A | N/A | 9.4 | 10.2 | 11.3 |
| Class A |  | 0.7 | 0.8 | 1.0 | N/A | N/A | N/A |
| Class B |  | 0.9 | 1.1 | 1.5 | 1.7 | 1.9 | 3.4 |
| Class C |  | 0.8 | 0.6 | 0.7 | 1.4 | 1.0 | 1.4 |
| Class D |  | 0.8 | 0.7 | 1.0 | 1.2 | 2.1 | 2.6 |
| Class E |  | N/A | N/A | N/A | 9.5 | 12.1 | 13.6 |
| Overall |  | 0.8 | 0.8 | 1.1 | 3.0 | 3.6 | 4.6 |

## Complexity analysis

The complexity was evaluated by encoding times and decoding times. Our computing platforms used for experiments are listed below.

- Windows 7 Professional 64-bit, Core i7 930 2.80GHz CPU (4 cores), 12GiB memory

  - This is used for encoding Class A, B, C and D.

  - Up to 8 encoding processes ran simultaneously.

- Windows XP Professional 32-bit, Xeon W5590 3.33GHz CPU (4 cores), 3GiB memory

  - This is used for encoding Class E.

  - Up to 8 encoding processes ran simultaneously.

- Windows XP Professional 64-bit, Opteron 252 2.60GHz CPU (1 core), 32GiB memory

  - This is used for decoding all bitstreams.

  - All decoding processes ran serially.

Core i7 and Xeon CPUs have features which make process speeds variable (Turbo Boost, i.e., automated over-clocking, and Hyper Threading, i.e., virtual CPU core doubling). Therefore as for encodings, individual encoding time can be significantly varied and results shown here are just rough indicators. However, decoding times are not affected at all because of the CPU in the platform (c) does not have such features.

Table 3 shows average increase ratio of encoding times and decoding times against anchors. As for encoding times, average increase ratios are regarded as roughly equal to zero for all test conditions. As for decoding times, overall average is as small as 2.7 %. Therefore it is concluded that the complexity of the proposed method is sufficiently low.

Table 3: Average increase ratio % of decoding and encoding times.

| Condition | RA-LC | LD-LC | RA-HCE | LD-HCE |
| --- | --- | --- | --- | --- |
| Encoding Time | -1.9 | 1.2 | -2.6 | -0.4 |
| Decoding Time | 3.1 | 2.1 | 2.7 | 2.9 |

Note: Encoding times are derived from encodings with 16x1 MCU. However, it is definitely clear that no time increase can be observed as for encoding times.

## Memory size and memory access bandwidth analysis

As for memory size, reduction ratios are as follows.

- 3/4 when IBDI is disabled: Each MCU has 61 bits, and its byte-aligned size is 64 bits.

- 2/3 when 12-bit IBDI is enabled: Each MCU has 43 bits, and its byte-aligned size is 48 bits.

In addition, memory access bandwidth was measured with the tool provided by memory compression AHG [4]. It should be noted that memory access bandwidth depends on implementation of MCU arrangement on frame memory, i.e., how to map 2-D compressed image plane to 1-D frame memory. The proposed method utilizes small size of 1-D MCU and has high flexibility of mappings. In this contribution, following four mappings were tested.

- 1-to-1: All MCUs are put into frame memory in raster scan order as shown in Figure 2.

- 2-to-1: MCU groups each of which consists of two vertically adjacent MCUs are put into frame memory in raster scan order as shown in Figure 3. The memory access bandwidth is same as that of the 1-to-1 mapping of 8x2 MCUs.

Note: Actually they are not exactly same because, for example, it is not necessary to read MCU(1, 1) in order to access to MCU(1, 0) in Figure 3. However, in this contribution, memory access bandwidth is calculated as that of 8x2 MCU. It is because the current bandwidth measurement tool does not assume such accesses and whether redundant MCU is read or not also depends on implementation.

- 4-to-1: MCU groups each of which consists of four vertically adjacent MCUs are put into frame memory in raster scan order as shown in Figure 4. The memory access bandwidth is same as that of the 1-to-1 mapping of 8x4 MCUs.

- 8-to-1: MCU groups each of which consists of eight vertically adjacent MCUs are put into frame memory in raster scan order. The memory access bandwidth is same as that of the 1-to-1 mapping of 8x8 MCUs.

Figure 2: MCU scan order for the 1-to-1 mapping.

Figure 3: MCU scan order for the 2-to-1 mapping.

Figure 4: MCU scan order for the 4-to-1 mapping.

Table 4 and Table 5 show average reduction ratio of memory access bandwidth against anchors. Numbers X/Y on the second row in these tables indicates that DDR alignment size is X bits and DDR burst-read size is Y bits (see [4]). In these tables, the best reduction for each test case and each DDR alignment setting is highlighted in boldface type. Red color indicates its reduction ratio is negative, i.e., memory bandwidth increases. From these results, it is observed that memory access bandwidth highly depends on MCU mapping and memory access alignment. Given that DDR alignment size is 256 bits and DDR burst-read size is 256 bits, the best mapping pattern is 4-to-1 and average reduction ratios in that case are 40.3 % for RA-LC, 41.0 % for LD-LC, 48.4 % for RA-HCE and 49.6 % for LD-HCE, respectively.

Table 4: Average bandwidth reduction ratio % for LC cases without IBDI.

|  | RA |  |  |  | LD |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCU mapping | 32/64 | 64/128 | 128/128 | 256/256 | 32/64 | 64/128 | 128/128 | 256/256 |
| 1-to-1 | 3.4 | 5.0 | 4.8 | 2.2 | 0.8 | 3.9 | 3.5 | 0.8 |
| 2-to-1 | 10.8 | 16.9 | 22.7 | 29.5 | 9.5 | 16.4 | 22.4 | 29.7 |
| 4-to-1 | 6.4 | 20.2 | 25.6 | 40.3 | 5.4 | 20.6 | 26.2 | 41.0 |
| 8-to-1 | -12.9 | 8.0 | 18.1 | 36.9 | -16.0 | 7.6 | 18.2 | 37.6 |

Table 5: Average bandwidth reduction ratio % for HCE cases with 12-bit IBDI.

|  | RA |  |  |  | LD |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCU mapping | 32/64 | 64/128 | 128/128 | 256/256 | 32/64 | 64/128 | 128/128 | 256/256 |
| 1-to-1 | 28.1 | 27.7 | 24.1 | 17.5 | 28.1 | 27.3 | 24.4 | 18.0 |
| 2-to-1 | 24.7 | 32.8 | 38.0 | 39.2 | 24.7 | 33.1 | 38.7 | 39.8 |
| 4-to-1 | 18.0 | 26.8 | 32.6 | 48.4 | 18.3 | 27.5 | 33.6 | 49.6 |
| 8-to-1 | 5.9 | 16.1 | 22.6 | 40.9 | 6.1 | 16.7 | 23.8 | 42.2 |

# Conclusions

This contribution has presented a performance report on the 1-D DPCM-based memory compression method described in JCTVC-B057 [3]. It has been tested with two constraint sets in two experimental conditions: one is roughly 3/4 memory compression without IBDI and another is memory retainment even with 12-bit IBDI. Experimental results have shown that average coding losses are 10.0 % for test cases without IBDI and 1.9 % for test cases with 12-bit IBDI. Subjective quality degradation caused by such coding loss is invisible for most test cases. Average increase of decoding time is as small as 2.7 %. Memory access bandwidth significantly varies depending codec implementation of MCU arrangement, but in the best case, total reduction ratios are 40.6 % for test cases without IBDI and 49.0 % for test cases with 12-bit IBDI. Experimental results on bandwidth have also shown that bandwidth reduction depends on memory architecture and implementation on mapping of compressed image data onto frame memory. The proposed method can easily adapt to various situations due to its flexibility and simplicity. It is proposed that the method is adopted in TMuC software and further evaluated from various aspects.

# References

[1] T. Chujoh, “Tool Experiment 2 on IBDI and memory compression,” JCT-VC document JCTVC-A302, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 1st meeting, Dresden, April 2010.

[2] T. Chujoh, “Tool Experiment 2 on IBDI and memory compression,” JCT-VC document JCTVC-B302, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 2nd meeting, Geneva, July 2010.

[3] H. Aoki, et al., “Performance report of DPCM-based memory compression on TE2,” JCT-VC document JCTVC-B057, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 2nd meeting, Geneva, July 2010.

[4] K. Chono, et al., “JCT-VC AHG report: Memory compression,” JCT-VC document JCTVC-C007 (WG11 m18047), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd meeting, Guangzhou, October 2010.

# Patent rights declaration

NEC Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).