| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C095 WG11 Number: m18118 |
| --- | --- |

| Title: | Performance improvement of DPCM-based memory compression by adaptive quantization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hirofumi Aoki Keiichi Chono Kenta Senzaki Junji Tajime Yuzo Senda 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 3757 5438 hao@bu.jp.nec.com chono@ct.jp.nec.com k-senzaki@bp.jp.nec.com tajime@ak.jp.nec.com y-senda@bc.jp.nec.com |
| Source: | NEC Corporation |  |  |

_____________________________

# Abstract

This contribution presents a method for improving coding efficiency of DPCM-based memory compression [1, 2]. In the proposed method, a nonlinear quantization matrix for each memory compression unit (MCU) is adaptively selected from multiple nonlinear quantization matrices so as to fit quantizer characteristics to image characteristics of each MCU. Each index to the selected quantization matrix is embedded into compressed data and signaled to frame memory decompressor. Actual frame memory size and memory access bandwidth does not increase since indices can be embedded into gap area for alignment. Experimental results have shown that the proposed adaptive quantization can reduce coding loss from 10.7 % to 6.9 % for test cases without IBDI, from 2.0 % to 1.3 % for test cases with 12-bit IBDI, respectively. Applied to the 2-D version presented in JCTVC-C094, more gains are achieved and coding loss can be reduced to 4.8 % for test cases without IBDI, to 1.0 % for test cases with 12-bit IBDI, respectively. Average increase of decoding times against anchors is 14.7 % for LC cases and 12.4 % for HCE cases when four matrices are used. It can be shown that the proposed adaptive quantization provides flexibility on the trade-off between coding efficiency and complexity to the DPCM-based memory compression scheme.

- Introduction

A 1-D DPCM-based memory compression method was proposed in JCTVC-B057 [1] and it has been extended to 2-D structure with ADPCM in JCTVC-C094 [2]. Due to its simplicity of the algorithm design, the computational impact both on encoder and decoder are small. However, some coding loss is introduced by DPCM-based memory compression, though subjective quality degradation is invisible for most cases.

In order to suppress such coding loss, adaptive quantization technique is introduced into the DPCM-based memory compression scheme in this contribution. The proposed method utilizes multiple nonlinear quantization matrices and adaptively select optimal one for each memory compression unit (MCU), while the previously-proposed methods use a fixed single nonlinear quantization matrix for all MCUs.

- Algorithm Description

The previous proposal [1, 2] uses a fixed nonlinear quantization matrix which is defined in sequence parameter set (SPS) or picture parameter set (PPS). In this proposal, quantization matrix is variable for every MCU. For each MCU, the quantization matrix is selected from multiple quantization matrices that are defined in SPS or PPS, so as to minimize the prediction error for the MCU. As the difference metric, Sum of Absolute Differences (SAD) is used in this contribution. The index to specify the selected matrix is embedded in compressed data for every MCU as depicted in Figure 1.

Figure 1 shows an example for storage of compressed data in frame memory. Here an MCU consists of 8x1 pixels, each pixel has originally 8-bit depth and is coded with 5-bit DPCM, and the number of predefined quantization matrices is four. In the case of fixed quantization, total bit amount in a whole MCU is 43 bits. The arrangement of data in frame memory is left to each implementation but generally they are aligned, for example, in bytes. Supposing that MCU data are byte-aligned, bit width of storing each MCU is 48 bits and therefore 5 bits are kept unused. Coding the quantization matrix index needs 2 bits but it is less than the capacity of unused area. Therefore the index can be embedded into this area and no additional bit is needed.

Figure 1: Storage of compressed data in frame memory.

# Syntax Description

## Syntax

Syntax for the proposed memory compression method and semantics associated with the proposed syntax are changed from [1] as shown below. Two syntax elements, log2_num_luma_dpcm_qtables and log2_chroma_dpcm_qtables are additionally introduced into the syntax structure of [1]. The syntax structure of dpcm_qlist() to describe quantization representative values is same as that in [1] and is therefore omitted here.

| memory_compression_parameter_set ( ) { | C | Descriptor |
| --- | --- | --- |
| memory_compression_flag | 0 ¦ 1 | u(1) |
| if(memory_compression_flag) { |  |  |
| dpcm_bit_depth_luma_minus1 | 0 ¦ 1 | ue(v) |
| DpcmBitDepthY = dpcm_bit_depth_luma_minus+1 |  |  |
| if(DpcmBitDepthY < BitDepthY+increased_bit_depth_luma) { |  |  |
| luma_dpcm_unit_horizontal_size_minus1 | 0 ¦ 1 | ue(v) |
| luma_dpcm_unit_vertical_size_minus1 | 0 ¦ 1 | ue(v) |
| log2_num_luma_dpcm_qtables | 0 ¦ 1 | ue(v) |
| numLumaDpcmQtables = 1 << log2_num_luma_dpcm_qtables |  |  |
| for( i = 0; i < numLumaDpcmQtables; i++ ){ |  |  |
| luma_dpcm_qlist_present_flag[i] | 0 ¦ 1 | u(1) |
| if(luma_dpcm_qlist_present_flag[i]){ |  |  |
| dpcm_qlist(0, i, DpcmBitDepthY, BitDepthY+increased_bit_depth_luma) |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| dpcm_bit_depth_chroma_minus1 | 0 ¦ 1 | ue(v) |
| DpcmBitDepthC = dpcm_bit_depth_chroma_minus+1 |  |  |
| if(DpcmBitDepthC < BitDepthC+increased_bit_depth_chroma) { |  |  |
| chroma_dpcm_unit_horizontal_size_minus1 | 0 ¦ 1 | ue(v) |
| chroma_dpcm_unit_vertical_size_minus1 | 0 ¦ 1 | ue(v) |
| log2_num_chroma_dpcm_qtables | 0 ¦ 1 | ue(v) |
| numLumaDpcmQtables = 1 << log2_num_luma_dpcm_qtables |  |  |
| for( i = 0; i < numChromaDpcmQtables; i++ ){ |  |  |
| chroma_dpcm_qlist_present_flag[i] | 0 ¦ 1 | u(1) |
| if(chroma_dpcm_qlist_present_flag[i]){ |  |  |
| dpcm_qlist(1, i, DpcmBitDepthC, BitDepthC+increased_bit_depth_chroma) |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

## Semantics

Semantics associated with additional syntax elements introduced in this proposal is described below.

log2_num_luma_dpcm_qtables specifies the number of nonlinear quantization matrices that are used to compress DPCM prediction errors for the luma component.

log2_num_chroma_dpcm_qtables specifies the number of nonlinear quantization matrices that are used to compress DPCM prediction errors for the chroma component.

# Experiments

The proposed method has been implemented on TMuC 0.7 and tested with TE2 conditions described in JCTVC-B302[3]. As for the test cases, the following notation conventions are used here.

- RA: random access constraint

- LD: low delay constraint

- HCE: high coding efficiency configuration

- LC: low complexity configuration

In this experiment, parameters of the proposed method are set as follows.

- The MCU size is set to 16x1 (1-D) and 4x4 (2-D).

- The bit depth of quantized pixels is set to 5 for the high coding efficiency configuration, i.e., without IBDI, and 7 for the low complexity configuration, i.e., with 12-bit IBDI, respectively.

- For each encoding, following four nonlinear quantization matrices are used.

For 5-bit DPCM in order to compress 8-bit signal, nonlinear quantization matrices composed of 32 elements are defined by:

[0, 1, 2, 3, 6, 9, 14, 18, 24, 31, 39, 48, 58, 72, 86, 103, 128, 153, 170, 184, 198, 208, 217, 225, 232, 238, 242, 247, 250, 253, 254, 255],

[0, 4, 8, 16, 24, 32, 40, 48, 56, 64, 72, 80, 88, 96, 104, 112, 128, 144, 152, 160, 168, 176, 184, 192, 200, 208, 216, 224, 232, 240, 248, 252],

[0, 1, 2, 3, 4, 5, 6, 7, 9, 11, 14, 16, 18, 21, 24, 32, 128, 224, 232, 235, 238, 240, 242, 245, 247, 249, 250, 251, 252, 253, 254, 255],

[0, 1, 3, 5, 7, 9, 11, 13, 17, 22, 28, 33, 40, 48, 60, 74, 128, 182, 196, 208, 216, 223, 228, 234, 239, 243, 245, 247, 249, 251, 253, 255].

For 7-bit DPCM in order to compress 12-bit signal, nonlinear quantization matrices composed of 128 elements are defined by:

[0, 4, 8, 12, 16, 20, 24, 28, 32, 36, 40, 44, 48, 60, 72, 84, 96, 108, 120, 132, 144, 164, 184, 204, 224, 240, 256, 272, 288, 312, 336, 360, 384, 412, 440, 468, 496, 528, 560, 592, 624, 660, 696, 732, 768, 808, 848, 888, 928, 984, 1040, 1096, 1152, 1208, 1264, 1320, 1376, 1444, 1512, 1580, 1648, 1748, 1848, 1948, 2048, 2148, 2248, 2348, 2448, 2516, 2584, 2652, 2720, 2776, 2832, 2888, 2944, 3000, 3056, 3112, 3168, 3208, 3248, 3288, 3328, 3364, 3400, 3436, 3472, 3504, 3536, 3568, 3600, 3628, 3656, 3684, 3712, 3736, 3760, 3784, 3808, 3824, 3840, 3856, 3872, 3892, 3912, 3932, 3952, 3964, 3976, 3988, 4000, 4012, 4024, 4036, 4048, 4052, 4056, 4060, 4064, 4068, 4072, 4076, 4080, 4084, 4088, 4092],

[0, 16, 32, 48, 64, 80, 96, 112, 128, 160, 192, 224, 256, 288, 320, 352, 384, 416, 448, 480, 512, 544, 576, 608, 640, 672, 704, 736, 768, 800, 832, 864, 896, 928, 960, 992, 1024, 1056, 1088, 1120, 1152, 1184, 1216, 1248, 1280, 1312, 1344, 1376, 1408, 1440, 1472, 1504, 1536, 1568, 1600, 1632, 1664, 1696, 1728, 1760, 1792, 1856, 1920, 1984, 2048, 2112, 2176, 2240, 2304, 2336, 2368, 2400, 2432, 2464, 2496, 2528, 2560, 2592, 2624, 2656, 2688, 2720, 2752, 2784, 2816, 2848, 2880, 2912, 2944, 2976, 3008, 3040, 3072, 3104, 3136, 3168, 3200, 3232, 3264, 3296, 3328, 3360, 3392, 3424, 3456, 3488, 3520, 3552, 3584, 3616, 3648, 3680, 3712, 3744, 3776, 3808, 3840, 3872, 3904, 3936, 3968, 3984, 4000, 4016, 4032, 4048, 4064, 4080],

[0, 4, 8, 12, 16, 20, 24, 28, 32, 36, 40, 44, 48, 52, 56, 60, 64, 68, 72, 76, 80, 84, 88, 92, 96, 100, 104, 108, 112, 120, 128, 136, 144, 152, 160, 168, 176, 188, 200, 212, 224, 232, 240, 248, 256, 264, 272, 280, 288, 300, 312, 324, 336, 348, 360, 372, 384, 416, 448, 480, 512, 896, 1280, 1664, 2048, 2432, 2816, 3200, 3584, 3616, 3648, 3680, 3712, 3724, 3736, 3748, 3760, 3772, 3784, 3796, 3808, 3816, 3824, 3832, 3840, 3848, 3856, 3864, 3872, 3884, 3896, 3908, 3920, 3928, 3936, 3944, 3952, 3960, 3968, 3976, 3984, 3988, 3992, 3996, 4000, 4004, 4008, 4012, 4016, 4020, 4024, 4028, 4032, 4036, 4040, 4044, 4048, 4052, 4056, 4060, 4064, 4068, 4072, 4076, 4080, 4084, 4088, 4092],

[0, 4, 8, 12, 16, 24, 32, 40, 48, 56, 64, 72, 80, 88, 96, 104, 112, 120, 128, 136, 144, 152, 160, 168, 176, 184, 192, 200, 208, 224, 240, 256, 272, 292, 312, 332, 352, 376, 400, 424, 448, 468, 488, 508, 528, 556, 584, 612, 640, 672, 704, 736, 768, 816, 864, 912, 960, 1016, 1072, 1128, 1184, 1400, 1616, 1832, 2048, 2264, 2480, 2696, 2912, 2968, 3024, 3080, 3136, 3184, 3232, 3280, 3328, 3360, 3392, 3424, 3456, 3484, 3512, 3540, 3568, 3588, 3608, 3628, 3648, 3672, 3696, 3720, 3744, 3764, 3784, 3804, 3824, 3840, 3856, 3872, 3888, 3896, 3904, 3912, 3920, 3928, 3936, 3944, 3952, 3960, 3968, 3976, 3984, 3992, 4000, 4008, 4016, 4024, 4032, 4040, 4048, 4056, 4064, 4072, 4080, 4084, 4088, 4092].

For each case, the first one is same as the default quantization matrix. These matrices are signaled to decoders by embedding them into bitstreams using the syntax structure described in section 3.1. Although two extra bits are needed for embedding quantization matrix indices, the capacity for byte-aligned storage of MCU data is preserved as follows:

- Without IBDI:
Fixed quantization: ((8+5*15)+7)/8*8 = (83+7)/8*8 = 88
Adaptive quantization: (8+5*15+2)+7)/*8 = (85+7)/8*8 = 88

- With 12-bit IBDI:
Fixed quantization: ((12+7*15)+7)/8*8 = (117+7)/8*8 = 120
Adaptive quantization: (12+7*15+2)+7)/*8 = (119+7)/8*8 = 120

Frame memory size and memory access bandwidth are therefore same as in the case with fixed quantization [1, 2] and not evaluated here.

## Coding efficiency analysis

As coding efficiency measure, 4-point BD-Rates were computed. Table 1 and Table 2 show BD-Rate for each test sequence with each condition against each type of anchor. Here only BD-Rates for the luma component are shown. More details are shown in the attached spreadsheet.

These results show that average coding losses of the proposed adaptive quantization method applied to 1-D MCU are 5.3 % for RA-LC, 8.5 % for LD-LC, 0.5 % for RA-HCE and 2.1 % for LD-HCE, respectively, while those of fixed quantization are 8.6 % for RA-LC, 12.8 % for LD-LC, 0.9 % for RA-HCE and 3.1 % for LD-HCE, respectively. By averaging results for RA and LD constraints, the proposed adaptive quantization can reduce coding loss from 10.7 % to 6.9 % for cases without IBDI, from 2.0 % to 1.3 % for cases with 12-bit IBDI, respectively. In addition, applied to 2-D MCU, more gains are achieved. On average, the method can reduce coding loss from 7.9 % to 4.8 % for cases without IBDI, from 1.5 % to 1.0 % for cases with 12-bit IBDI, respectively. It should be noted that the method have potential for more gain by encoder optimizations, since the results shown here are obtained with the same quantization matrix set for all sequences.

Table 1: BD-Rates % against anchors. (1-D MCU)

|  | Fixed |  |  |  | Adaptive |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | RA-LC | LD-LC | RA-HCE | LD-HCE | RA-LC | LD-LC | RA-HCE | LD-HCE |
| Class A | 7.1 | N/A | 0.7 | N/A | 3.8 | N/A | 0.4 | N/A |
| Class B | 8.7 | 10.4 | 0.9 | 1.8 | 4.8 | 6.5 | 0.5 | 1.1 |
| Class C | 8.6 | 10.3 | 0.9 | 1.5 | 5.6 | 7.0 | 0.5 | 0.9 |
| Class D | 9.3 | 9.7 | 1.0 | 1.3 | 6.2 | 6.8 | 0.6 | 0.8 |
| Class E | N/A | 24.2 | N/A | 9.6 | N/A | 16.4 | N/A | 6.8 |
| Overall | 8.6 | 12.8 | 0.9 | 3.1 | 5.3 | 8.5 | 0.5 | 2.1 |

Table 2: BD-Rates % against anchors. (2-D MCU)

|  | Fixed |  |  |  | Adaptive |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | RA-LC | LD-LC | RA-HCE | LD-HCE | RA-LC | LD-LC | RA-HCE | LD-HCE |
| Class A | 5.5 | N/A | 0.6 | N/A | 2.6 | N/A | 0.3 | N/A |
| Class B | 6.3 | 7.6 | 0.7 | 1.4 | 3.3 | 4.6 | 0.4 | 0.8 |
| Class C | 6.6 | 7.8 | 0.7 | 1.1 | 4.1 | 5.3 | 0.4 | 0.7 |
| Class D | 7.1 | 7.4 | 0.8 | 1.0 | 4.4 | 4.7 | 0.4 | 0.7 |
| Class E | N/A | 16.2 | N/A | 7.2 | N/A | 10.6 | N/A | 5.2 |
| Overall | 6.5 | 9.2 | 0.7 | 2.3 | 3.7 | 5.9 | 0.4 | 1.6 |

## Complexity analysis

The complexity was evaluated by decoding times. Measurement of encoding times is omitted here since the computational impact of DPCM-based memory compression on TMuC 0.7 encoder is definitely negligible as shown in JCTVC-C093. Decoding times were measured on the following computing platform.

- OS: Windows XP Professional 64-bit

- CPU: Opteron 252 2.60GHz (single core)

- Memory: 32GiB

Table 3 and Table 4 shows average increase ratio of decoding times against anchors. It is derived from Table 3 that average increase ratio is 14.7 % for LC cases and 12.4 % for HCE cases, applied to 1-D MCU. It is also derived from Table 4 that average increase ratio is 21.7 % for LC cases and 17.3 % for HCE cases, applied to 2-D MCU. Due to cost calculations for the quantization matrix decision, the complexity is up to roughly six times higher than that of the original method with fixed quantization, where four quantization matrices are used. It should be noted that the cost calculations can be easily parallelized and the computation time increase can be kept as low as a few percentages by the parallelization. Although there is a trade-off between coding efficiency and complexity, these results show that the proposed adaptive quantization provides some flexibility on this trade-off to the DPCM-based memory compression scheme.

Table 3: Average increase ratio % of decoding times. (1-D MCU)

| Quantization | RA-LC | LD-LC | RA-HCE | LD-HCE |
| --- | --- | --- | --- | --- |
| Fixed | 3.5 | 2.4 | 3.0 | 3.0 |
| Adaptive (x4) | 15.7 | 13.8 | 12.5 | 12.4 |

Table 4: Average increase ratio % of decoding times. (2-D MCU)

| Quantization | RA-LC | LD-LC | RA-HCE | LD-HCE |
| --- | --- | --- | --- | --- |
| Fixed | 4.3 | 3.2 | 4.1 | 3.5 |
| Adaptive (x4) | 21.7 | 19.2 | 17.3 | 16.8 |

# Conclusions

In this contribution, a method for improving coding efficiency of DPCM-based memory compression [1, 2] has been presented. In the proposed method, a nonlinear quantization matrix for each memory compression unit (MCU) is adaptively selected from multiple nonlinear quantization matrices so as to fit quantizer characteristics to image characteristics of each MCU. Each index to the selected quantization matrix is embedded into compressed data and signaled to frame memory decompressor. Actual frame memory size and memory access bandwidth does not increase since indices can be embedded into gap area for alignment. Experimental results have shown that the proposed adaptive quantization can reduce the coding losses from 10.7 % to 6.9 % for test cases without IBDI, from 2.0 % to 1.3 % for test cases with 12-bit IBDI, respectively. Applied to the 2-D version presented in JCTVC-C094, more gains are achieved. Average increase of decoding times against anchors is 14.7 % for LC cases and 12.4 % for HCE cases when four matrices are used. It can be shown that the proposed adaptive quantization provides flexibility on the trade-off between coding efficiency and complexity to the DPCM-based memory compression scheme.

# References

[1] H. Aoki, et al., “TE2: 1-D DPCM-based memory compression,” JCT-VC document JCTVC-C093 (WG11 m18116), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd meeting, Guangzhou, October 2010.

[2] H. Aoki, et al., “An extension of DPCM-based memory compression to 2-D structure with ADPCM,” JCT-VC document JCTVC-C094 (WG11 m18117), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd meeting, Guangzhou, October 2010.

[3] T. Chujoh, “Tool Experiment 2 on IBDI and memory compression,” JCT-VC document JCTVC-B302, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 2nd meeting, Geneva, July 2010.

[4] K. Chono, et al., “JCT-VC AHG report: Memory compression,” JCT-VC document JCTVC-C007 (WG11 m18047), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd meeting, Guangzhou, October 2010.

# Patent rights declaration

NEC Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).