| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C102 WG11 Number: m18125 |
| --- | --- |

| Title: | TE7: Symmetry-based simplification of MDDT |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Tool Experiment Report |  |  |
| Author(s) or Contact(s): | Haitao Yang Jiantong Zhou Haoping Yu Section B1, Huawei Industrial Base, Bantian Longgang Shenzhen, China | Tel: Email: | +86 755 28789935 haitaoyang@huawei.com zhoujiantong@huawei.com haopingyu@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

In the last meeting, symmetry-based simplification of MDDT (SMDDT) [1] was proposed to reduce the number of transform matrices in the current MDDT implementation. And the SMDDT technique was included in TE 7 for further investigation. According to the TE7 specification [2], SMDDT has been implemented in the TMuC software and evaluated under the common test condition [3] since the last meeting. This document reports the performance of the proposed SMDDT technique and discusses the complexity.

# Algorithm description

In TMuC, the 9 directional intra prediction modes in AVC [4] are extended to 34 modes with finer prediction angles. To adapt to this change, the MDDT firstly maps the 34 prediction modes to 9 transform modes by grouping the modes with adjacent prediction directions together as shown in Fig. 1, and then applies different pairs of separable transforms for the residual of the 9 transform modes.

(a) Prediction modes                (b) MDDT modes

Fig. 1  Directional prediction modes (a) and corresponding MDDT modes (b) in TMuC

As described in [1], the MDDT modes shown in Fig. 1 (b) can be further reduced based on the symmetry. The mapping from the MDDT modes to the proposed SMDDT modes is specified in Table 1. For some modes, block operations such as flip and transpose are applied before the transform and after the inverse transform, which is also specified in Table 1. In table 1, X means no block operation, T means transpose operation, FV and FH means flip operation around the vertical and the horizontal axis respectively.

Table 1  Mapping relationship among UIP modes, MDDT modes and SMDDT modes, and the block operations.

| UIP mode (Logical order) | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MDDT mode | 2 | 4 | 4 | 5 | 5 | 5 | 5 | 0 | 0 | 0 | 0 | 0 | 7 | 7 | 7 | 7 | 3 |
| SMDDT mode | 3 | 2 | 2 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 2 |
| Fwd blk operation | X | X | X | X | X | X | X | X | X | X | X | X | FV | FV | FV | FV | FV |
| Inv blk operation | X | X | X | X | X | X | X | X | X | X | X | X | FV | FV | FV | FV | FV |
| UIP mode (Logical order) | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| MDDT mode | 3 | 4 | 6 | 6 | 6 | 6 | 1 | 1 | 1 | 1 | 1 | 8 | 8 | 8 | 8 | 3 | 3 |
| SMDDT mode | 2 | 2 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 2 | 2 |
| Fwd blk operation | FV | X | T | T | T | T | T | T | T | T | T | FH+T | FH+T | FH+T | FH+T | FH | FH |
| Inv blk operation | FV | X | T | T | T | T | T | T | T | T | T | T+FH | T+FH | T+FH | T+FH | FH | FH |

For SMDDT mode 3, DCT is applied. For SMDDT modes 0 ~ 2, separable transform is trained for each. For SMDDT mode 2, the row transform and the column transform are the same.

# Training of transforms and scan maps

The training dataset is composed of intra prediction residuals of several CIF and 720P sequences. The sequences listed in the common test condition are excluded. The same mapping described in Table 1 is applied to get the subset of training data for each SMDDT mode. Then the eigen decomposition is applied to get the separable KLT from each subset separately. For SMDDT mode 2, the row and the column transform are forced to be the same. The scan map of each SMDDT mode is the accumulated magnitude of the transform coefficients of all samples in the data set.

# Simulation results

As required in [2], the TMuC 0.7 is used as the software platform, and the common test condition specified in [3] is used in the tests. Two tests were done for the Intra high efficiency and random access high efficiency cases.

## RD performance

The performance of SMDDT is given in Table 2. The anchor is the performance of TMuC7.0 using the default configurations, where MDDT is applied to 4x4 and 8x8 prediction blocks. The proposed SMDDT replaces the original MDDT implementation in the TMuC7.0.

Table 2. Coding performance of SMDDT

|  | Intra |  |  | Random access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.3 | 0.4 | 0.4 | 0.1 | 0.0 | -0.1 |
| Class B | 0.1 | 0.0 | 0.1 | 0.0 | -0.2 | -0.2 |
| Class C | 0.2 | 0.1 | 0.2 | 0.1 | -0.1 | 0.0 |
| Class D | 0.3 | 0.2 | 0.1 | 0.1 | 0.2 | 0.1 |
| Class E | 0.3 | 0.3 | 0.3 |  |  |  |
| Average | 0.2 | 0.2 | 0.2 | 0.1 | 0.0 | -0.1 |
| Enc Time[%] | 89% |  |  | 108% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

In our tests, the scan maps in the original MDDT implementation yield to slightly better performance. So we replace the trained scan map with the corresponding MDDT scan maps as another implementation the proposed SMDDT. The results are shown in Table 3.

Table 3. Coding performance of SMDDT with revised scan maps

|  | Intra (revised scan order) |  |  | Random access (revised scan order) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.4 | 0.4 | 0.4 | 0.1 | 0.0 | 0.1 |
| Class B | 0.1 | 0.0 | 0.1 | 0.0 | 0.1 | -0.1 |
| Class C | 0.1 | 0.0 | 0.1 | 0.1 | 0.0 | 0.2 |
| Class D | 0.1 | 0.0 | 0.0 | 0.1 | -0.1 | -0.1 |
| Class E | 0.2 | 0.2 | 0.2 |  |  |  |
| Average | 0.1 | 0.1 | 0.1 | 0.1 | 0.0 | 0.0 |
| Enc Time[%] | 79% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

As we can see from Table 2 and 3, the proposed SMDDT can achieve approximately the same coding performance compared with MDDT.

## Subjective test

We also did the subjective test to compare the decoded pictures of the anchor and the proposed SMDDT. Although they are different, no preference is found in terms of viewing experience. The decoded pictures specified in the TE description are provided along with this document.

## Complexity analysis

Memory requirements

8 directional transform modes are defined in the current MDDT implementation in TMuC. For an NxN transform mode, one pair of NxN transform matrices and one NxN scan map matrix need to be stored. Therefore, totally 8x2=16 NxN transform matrices and 8 NxN scan map matrix are required by MDDT. Supposing the number in the transform matrices and the scan map matrices is of 8-bits precision, totally 24N2 bytes memory is consumed. As for the proposed SMDDT, only 5 transform matrices and 3 scan maps are needed. Therefore totally 8N2 bytes memory is consumed.

It should be noted that for intra prediction mode 2, DCT is used in both MDDT and SMDDT. To facilitate the scan order adaptation, an extra scan map is also stored. However, as the initial scan statistics for the following adaptation.

Computational complexity

Different machines are used for the simulation because the limited resources. So the statistics related to the encoding time in the attached data sheet is meaningless. The decoding is done on the same machine, so the decoding time can be used as a reference.

Since the proposed SMDDT is an improvement of MDDT without too much change, the relative computational complexity can be analyzed easily. In both MDDT and SMDDT, the transform is executed as matrix multiplication with integer precision, so the computational complexity is the same. The flip and transpose of residual blocks can be easily implemented especially in the hardware design, so the complexity is negligible, which is also confirmed by the decoding time comparison in Table 2 and 3.

# Conclusion

The simplified MDDT presented in this proposal can achieve approximately the same coding performance as MDDT with less transform matrices and less scan orders, thus reducing the complexity of MDDT. We suggest the proposed SMDDT be adopted as an improvement of the current MDDT in TMuC.

# References

- H. Yang, J. Zhou, H. Yu, “Simplified MDDT (SMDDT) for Intra Prediction Residual”, JCTVC-B039, Geneva, July, 2010.

- R. Cohen, C. Yeo, R. Joshi, et.al., “Tool Experiment 7: MDDT Simplification”, JCTVC-B307, Geneva, July, 2010.

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-B300, Geneva, July, 2010.

- MPEG4-AVC/H.264 standard.

# Patent rights declaration(s)

Huawei Technologies Co.,Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).