| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C106 WG11 Number: m18129 |
| --- | --- |

| Title: | Symmetry-based Scan Order Sharing Scheme for MDDT |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Haitao Yang Jiantong Zhou Haoping Yu Section B1, Huawei Industrial Base, Bantian Longgang Shenzhen, China | Tel: Email: | +86 755 28789935 haitaoyang@huawei.com zhoujiantong@huawei.com haopingyu@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

- Abstract

In this proposal, a symmetry-based scan order (SSO) sharing scheme is proposed to further simplify the mode-dependent directional transform (MDDT) [1]. Based on the symmetry among all intra prediction directions, the number of scan maps can be reduced by the proposed SSO while the coding performance is kept unchanged. We suggest that the proposed SSO scheme is adopted as an improvement of the current MDDT implementation in TMuC.

- Motivation

By using multiple transform matrices MDDT [1] exploits the redundancy in the intra prediction residuals and improves the coding performance. The intra prediction modes specified in AVC standard are shown in figure 1. In the MDDT scheme, different scan maps are used for these prediction modes, so altogether 9 scan maps are used.

Figure 1. Intra prediction modes of AVC standard

In TMuC, totally 34 intra prediction modes are defined, and the MDDT implementation is changed accordingly. Specifically, the 34 prediction modes are grouped into 9 classes by merging the modes with adjacent prediction direction. The representative prediction modes in the 9 classes are the same as the 9 prediction modes in AVC. Then different transforms are used for these classes, one transform for all prediction modes in one class, and each transform has a corresponding scan map.

Be noted that the mode indices of the 9 representative modes of the 9 classes in TMuC are not the same as those in the Figure 1. In the following, the mode indices shown in Figure 1 are used for algorithm description to avoid ambiguity.

After observing the scan maps in the MDDT implementation, symmetrical feature is found among scan maps of different prediction modes. Specifically, the transposed scan map of transform mode 0 is very similar to the scan map of transform mode 1; the scan maps of mode 4, 5, 6 are very similar to the scan maps of mode 3, 7 and 8, respectively. And after transpose, scan maps of 6 and 8 are very similar to scan maps of mode 5 and 7.

Coefficient index

Scan stat

Coefficient index

Scan stat

(b)

Figure2. Comparison of scan order statistics of different prediction modes:

(a) 4x4 blocks; (b) 8x8 blocks

Figure 2 shows the similarity of the scan maps of different prediction modes after proper alignment. The scan maps are represented by the coefficient statistics, which are obtained from the MDDT implementation in the TMuC software. Green curves are the scan maps of prediction mode 0 and the transposed scan maps of mode 1; red curves are the scan maps of prediction mode 3 and 4; and blue curves are the scan maps of prediction mode 5, 7 and the transposed scan maps of mode 6, 8. Different colors indicate different groups of there scan maps. Strong similarity is observed among scan maps within the same group. Therefore, the number of scan maps can be reduced if all prediction modes in each group could share one scan map.

- Algorithm description

The only change of SSO is the scanning process of MDDT. When doing scanning, Table 1 is used to find the scan map for each prediction mode. In the SSO implementation, the scan map of each group is computed by averaging all the aligned scan maps within the group. Of course, if the prediction modes are transposing symmetric, the two components of scanning position (x, y) should be swapped so (y, x) is actually used for locating the coefficient position in a block.

Table 1. Mapping of prediction modes to scan maps

| Prediction mode | Scan map |
| --- | --- |
| 0 | 0 |
| 1 | 0 |
| 2 | 3 |
| 3 | 2 |
| 4 | 2 |
| 5 | 1 |
| 6 | 1 |
| 7 | 1 |
| 8 | 1 |

In principle, the proposed SSO can be combined with other MDDT simplification schemes. For example, the proposed SSO can be combined directly with the MDDT simplification schemes proposed in JCTVC-C037 and JCTVC-C080, because the original 9 MDDT scan maps are used in the two contributions.

Further, SSO can be combined with our symmetry based SMDDT proposed in JCTVC-C102. In this case, the flip and transpose operations on the residual blocks are not needed any more. Preliminary results of the combination are shown in the simulation section.

- Complexity analysis

Because the number of scan maps is reduced from 9 to 4, the memory storage is saved. As for the computational complexity, no extra calculation is needed.

- Simulation results

The proposed SSO was implemented in TMuC 0.7 and the test condition specified by JCTVC-B300 [2] was used for the simulation. The coding performances of SSO over MDDT are shown in table 2-1 and table2-2. From the two tables, it is observed that SSO can achieve the same coding gain as MDDT no matter the adaptive scan order update is used or not.

Table 2-1 Performance of SSO when adaptive scan order update is enabled

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.1 | 0.1 |
| All | 0.0 | 0.0 | 0.0 |
| Dec Time | 99% |  |  |

Table 2-2 Performance of SSO when adaptive scan order update is disabled

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | 0.1 |
| Class B | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 |
| All | 0.0 | 0.0 | 0.0 |
|  | 103% |  |  |

In the above experiments, we also observed that the adaptive scan order update actually has limited coding performance improvement as table 3 shows. We noticed that for the sequence with higher resolution, the adaptation tends to have better performance, while the negative performance can be seen for the sequences with lower resolution.

Table 3 Performance of adaptive scan order update (ASO off vs. ASO on)

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | 0.1 |
| Class B | 0.1 | 0.1 | 0.1 |
| Class C | 0.0 | 0.1 | 0.1 |
| Class D | -0.1 | 0.0 | 0.0 |
| Class E | -0.1 | 0.0 | 0.0 |
| All | 0.0 | 0.0 | 0.1 |
| Dec Time | 60% |  |  |

The performance of SSO when it is combined with the SMDDT proposed in JCTVC-C102 is shown below. Due to the time limitation, only results of Class C and Class D are available. Compared with SMDDT, the average performance of the Y component for the Class C and the Class D are the same.

Table 4-1 Performance of SSO when combined with the SMDDT scheme (Scan order update enabled)

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class C | 0.1 | 0.2 | 0.2 |
| Class D | 0.1 | 0.2 | 0.1 |

Table 4-2 Performance of SSO when combined with the SMDDT scheme (Scan order update disabled)

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class C | 0.0 | 0.2 | 0.2 |
| Class D | 0.1 | 0.2 | 0.2 |

- Conclusion

Based on the symmetry of the scan maps in the current MDDT implementation in TMuC, the number of the scan maps can be reduced by the proposed SSO scheme while the performance of MDDT is maintained. The memory storage can be saved by SSO, and no extra computational complexity is needed. We suggest that the proposed SSO is adopted as an improvement of the current MDDT scheme in TMuC.

- References

[1] M. Karczewicz, “Improved Intra Coding”, VCEG-AF15, San Jose, USA, April 2007.

[2] JCT-VC, “Common test conditions and software reference configurations”, JCTVC-B300, Geneva, CH, July, 2010.

[3] JCT-VC, “Mode-Dependent Fast Separable KLT for Block-based Intra Coding”, JCTVC-B024, Geneva, CH, July, 2010.

[4] JCT-VC, “Improvement of Intra Coding by Bidirectional Intra Prediction and 1 Dimensional Directional Unified Transform”, JCTVC-B042, Geneva, CH, July, 2010.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).