| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3nd Meeting: Guangzhou, P.R.CHINA, 7-15 October, 2010 | Document: JCTVC-C112_r2 WG11 Number: m18135 |
| --- | --- |

| Title: | Fast Transforms for HEVC Test Model |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Dai Madhu Krishnan Pankaj Topiwala 750 N. Atlantic Ave., #1504 Cocoa Beach, FL 32931 USA | Tel: Email: | 443-538-3782 pankaj@fastvdo.com |
| Source: | FastVDO LLC |  |  |

_____________________________

# Abstract

The use of block transforms has been a fundamental tool in all video compression standards.  And it has been demonstrated that the coding efficiency of high resolution video can be improved by the use of large transforms as well as large block sizes [1]. Beyond the 4x4 and 8x8 transform sizes already found in AVC/H.264, larger transform sizes of 16x16, 32x32 and even 64x64 are also included as core components of the current Test Model under Consideration (TMuC) for HEVC. However, since large transform sizes do entail some non-trivial computational complexity, one may ask what is the cost-benefit analysis of their inclusion in the forthcoming Test Model.  In fact, the larger transforms in TMuC [1] are all based on Chen's fast DCT algorithm because of its regular butterfly structure and its extensibility to any desired transform sizes of order N=2m with m>=1. Even then, for complexity reasons, one prefers integer rather than floating-point versions of Chen's DCT.  Indeed, fast integer transforms based on Chen's factorization are already in the current TMuC (0.8).  However, in this document, we show that the complexity can be reduced even further with NO loss of performance.  We propose fast transforms of each candidate size that not only provide virtually identical coding performance, but offer useful gains in computational complexity.  In this way, the complexity of the transforms can be kept to a minimum, even when using large transform sizes, thus effectively eliminating concerns in considering their use in the Test Model.  Thus, attention can be focused purely on their coding efficiency benefits.

# Introduction

The current Test Model under Consideration (version 0.8) derives from proposal A124 and its associated software [6] of Samsung and BBC, but incorporating various other inputs.  In particular, A124 proposed the use of transforms up to size 64.  The document itself discussed the transform notionally as floating-point transforms with rotations, while the software implementation contained two versions – both integer – one generic (default) and the other a fast approximation, with potentially lower performance.  Perhaps for perceived reasons of efficiency, to avoid duplication only one transform per size has been retained in the TMuC – the default integer transform.  Yet the fast transform has lower complexity and performs equally well.  In this proposal, we indeed show that there is NO loss of performance, and a useful complexity gain, in using fast transforms of each size.  Since the 4-pt and 8-pt transforms retained from AVC/H.264 are already well understood, we focus on transforms of size 16 and higher.  Tests are conducted for the CS1 and CS2 settings, resulting in very negligible differences to the default transforms.

# DCT Representation Methodology

As is well-known, the DCT and its approximations can be studied in terms of their signal-flow representations, see for example [7].  By systematic analysis of the structure of the Fast Fourier Transform and its associated sub-transforms, one can create fast and efficient implementations of larger transforms based on smaller ones (e.g., split-radix).  So much so, that today, we understand the DCT can be represented purely in terms of three basic components: (a) butterflies, (b) scaling factors, and (c) planar rotations.  (It is already remarkable that higher-dimensional rotations can be represented as concatenations of a finite number of planar rotations, for various sub planes in the higher dimension.) Of these components, the butterfly is already efficient, while the scaling factors may be irrational but can be approximated by rational (and even dyadic-rational ones).  However, the rotations continue to be problematic for efficient representation, on account of using transcendental functions and numbers.

Yet it is a valuable theorem that a planar rotation can be representated in terms of three shears, or lifting steps [2, 5]; see figures 1 and 2.  These shears still contain parameters that are generally irrational, but again, these can now be approximated by rational (or dyadic-rational) coefficients.

Fig 1. (a) Representation of a plane rotation by 3 lifting steps. (b) Inverse transform.

Fig. 2. (a) A general rotation; (b) The scaled lifting structure for (a);  (c) An orthogonal plane rotation; (d) The scaled lifting structure for (c); (e) The permuted version of (c); (f) The scaled lifting structure for (e).

In terms of counting computational complexity in representing Chen's fast DCT, we have the following calculations.

When N=2m, m >= 3, the number of butterflies is

(m-1)*2(m-1) for m>=3.

Furthermore, the number of rotations is

(m-1)*2(m-2)+1.

Of these, the number of type I rotations is

2(m-2)

while the number of type II rotations is

(m-2)*2(m-2)+1.

Since type I rotation needs only two multiplications, the number of multiplications is

4* ((m-2)*2(m-2)+1) +2*2(m-2)= (2m-3)*2(m-1) +4.

The number of addition operations is

2*(m-1)*2(m-1)+2* ((m-2)*2(m-2)+1) +2*2(m-2) =3*(m-1)*2(m-1)+2.

It is well-known that a plane rotation can be performed by 3 shears [2, 5].  That representation has direct complexity consequences.  A rotation, which notionally needs 4 multiplications and 2 additions, can be implemented by 3 multiplications and 3 additions. In many cases (see figure 2), this can be reduced to 2 multiplications and 2 additions. Finally, the computational complexity can be further reduced by taking into account the complexity of the individual lifting coefficients (e.g., dyadic rational) as well as their interdependency. It can be shown that the parametrization of each rotation angle can be limited to no more than two dyadic lifting steps, by approximating rotation angles jointly [3]; see again figure 2.

So with these weapons in our arsenal, the fast integer approximations can be developed.  In fact, it is unnecessary to do much work, as good transforms are already at hand.  For convenience, we reuse the size 16 and 32 fast transforms already incorporated in the A124 proposal software [6].  For size 64, we use the X32 x H2, where 'x' standards for 'tensor product', and where H2 is the 2x2 Hadamard matrix:

H2 = [1, 1; 1, -1].

We obtain the following computational simplifications compared to the integer transforms in TmuC 0.8.

Table 1 Number of operations  of 16 point one dimensional transforms

| Transform | Operations | Number | Number of |
| --- | --- | --- | --- |
| Chen's Fast DCT | + | 74 | 118 |
|  | * | 44 |  |

Table 2 Number of operations  of 32  point one dimensional transforms

| Transform | Operations | Number | Total operations |
| --- | --- | --- | --- |
| Chen's Fast DCT | + | 194 | 310 |
|  | * | 116 |  |

Table 3 Number of operations  of 16 point one dimensional transforms

| Transform | Operations | Number | Total operations | Total Ops/sample |
| --- | --- | --- | --- | --- |
| TMuC 8.0 | + | 150 | 268 | 16.75 |
|  | >> | 118 |  |  |
| Proposed | + | 106 | 167 | 10.43 |
|  | >> | 55 |  |  |

Table 4 Number of operations  of 32  point one dimensional transforms

| Transform | Operations | Number | Total operations | Total Ops/sample |
| --- | --- | --- | --- | --- |
| TMuC 8.0 | + | 484 | 871 | 27.22 |
|  | >> | 387 |  |  |
| Proposed | + | 353 | 507 | 15.84 |
|  | >> | 154 |  |  |

Table 5 Number of operations  of 64  point one dimensional transforms

| Transform | Operations | Number | Total operations | Total Ops/sample |
| --- | --- | --- | --- | --- |
| TMuC 8.0 | + | 1416 | 2600 | 40.63 |
|  | >> | 1184 |  |  |
| Proposed | + | 770 | 1078 | 16.84 |
|  | >> | 308 |  |  |

# Test Results

Tests were conducted on sequences in the classes B – E, effectively under the Constraint Set 2 (CS2), with the modification that we coded with IPPP, unlimited GOP length (rather than GOP = 4).  Our test results are included in a separate spreadsheet.  In brief, the tests conducted to date confirm that there is NO loss of performance relative to the current TMuC transforms.  On the other hand, we achieved a substantial computational complexity savings, of approximately 60% on the 16-pt transform, nearly 2:1 on the 32-pt, and nearly 3:1 on the size 64 transforms.  Further complexity gain may be realized by further detailed analysis of computational paradigms.  Test results for high-efficiency (CS1) and low-delay CS2 are provided.  On average, for CS1, there is a loss of 0.016 dB BD-PSNR; on CS2, there is a gain of 0.01 dB BD-PSNR.  These differences are negligible and can be ignored.  But the complexity savings are substantial.  Code fragments with are embedded transform code, as well as scripts for running our experiments, are also provided for ease of verification and inclusion.

# Other simplications/observations

## 64-pt Transform Computation

The 64-pt transform represents a significant processing complexity, not just in terms of operations, but in terms of data management (esp. in hardware; e.g., see JCTVC-C057).  The use of Hadamard transform, such as in our representation of T64 = T32 x H2, and which is advocated in several other proposals (such as C255),  may be able to alleviate some of the complexities, as this decomposition can be computed as a 2D transform in two stages, rather than computing it as a single 64-pt transform.

## Partial frequency Processing

A well-known technique for reducing complexity for large transforms is to perform a partial frequency transform (that is, keep low-frequency components and simply not compute the high-frequency ones).  This method is advocated in several proposals, including C209 and C237.  In this proposal, we support this modality as well.  This functionality can be supported by a flag indicating this application.

# Recommendations

1. Recommend inclusion in TMuC now, for validation and inclusion in TM by next meeting.  Can accelerate to TM now if acceptable.

2. Keep the simplified 64-pt transform in TM(uC) at this time, for further analysis; review its retention next meeting.

# References

[1] JCT-VC, “Test Model under Consideration (TMuC),” JCT-VC B205, Geneva, CH, July 2010.

[2] I. Daubechies and W. A. Pearlman, “Factoring wavelet transforms into lifting steps,” J. Fourier Anal. Appl., vol. 4, pp. 247-269, 1998.

[3] J. Liang and T. D. Tran, "Fast multiplierless approximations of the DCT with the lifting scheme," IEEE Trans. on Signal Processing, vol. 49, pp. 3032-3044, Dec. 2001.

[4] T. D. Tran, "The BinDCT: fast multiplierless approximation of the DCT," IEEE Signal Processing Letters, vol. 7, pp. 141-145, Jun. 2000.

[5] J. Liang, T. Tran, P. Topiwala, “A 16-bit architecture for H.26L, treating DCT transforms and quantization,” ITU VCEG doc M16, Austin, TX, April, 2001.

[6] K. McCann et al, “Samsung's Response to the Call for Proposals on Video Compression Technology,” JCTVC-A124, Dresden, April, 2010.

[7] P. P. Vaidyanathan, Multirate Systems and Filter Banks, Prentice-Hall, 1993.

# Patent rights declaration(s)

FastVDO LLC may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).