| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C114 WG11 Number: m18137 |
| --- | --- |

| Title: | Zigzag Scan for CABAC/PIPE |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian Lou Krit Panusopone Limin Wang 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-3427 lou@motorola.com +1-858-404-3618 krit@motorola.com +1-858-404-3893 limin.wang@motorola.com |
| Source: | Motorola Mobility |  |  |

_____________________________

# Abstract

In TMuC0.7, the macro “HHI_TRANSFORM_CODING” enables a set of transform coefficient coding tools. It is switched on by default when the entropy coding option is CABAC/PIPE. The set of transform coefficient coding tools is based on JCTVC-A116 [1]. Among these tools, an adaptive scan is applied for significance map coding. The experimental results indicate that this adaptive scan scheme achieves only negligible performance gain. But, it introduces additional memory and computational complexity as comparing to the traditional zigzag scan.  Therefore, we propose to use the zigzag scan for significance map coding when the macro “HHI_TRANSFORM_CODING” is switched on, or CABAC/PIPE is selected.

- Introduction

In TMuC0.7, the macro “HHI_TRANSFORM_CODING” is used to select the entropy coding, either SBAC from JCTVC-A124 [2] or CABAC/PIPE from JCTVC-A116 [1]. By default, it is set to use the option of CABAC/PIPE which incorporates a set of transform coefficient coding tools. The set of coding tools based on JCTVC-A116 [1] include an adaptive scan scheme which adaptively switches between two scan patterns for the significance map coding if the transform size is larger than 8x8.

With the macro “HHI_TRANSFORM_CODING” on in TMuC0.7, the scan order for coding the significance map is allowed to switch between two predefined scan patterns per diagonal line, that is, either from bottom-left to top-right or from top-right to bottom-left diagonally [1]. The switching occurs at the end of each diagonal sub-scan, and it is controlled by two counters, c1, the number of coded significant transform coefficients that are located in the bottom-left half of the transform block, and c2, the number of coded significant transform coefficients that are located in the top-right half of the transform block. Fig.1 shows an example of the adaptive scan for 16x16 blocks, where the diagonal scan can go either way.

Fig. 1 Adaptive scan for 16x16 blocks in TMuC (Dashed line divides the block into top-right half and bottom-left half; Solid diagonal lines can go either direction)

In the current implementation of significance map coding of TMuC0.7, this adaptive scan requires additional memories:

- Two scan patterns as comparing to one zigzag scan pattern.

- Two additional counters c1 and c2.

It also introduces additional computational complexity:

- Counting the number coded of significant transform coefficients located in the bottom-left half or in the top-right half.

- Branch operations and scan selection for each coefficient before the last significant coefficient.

- The context model for an element in significant map is selected based on the coded neighboring elements in the significant map. Since a diagonal scan may go either way, it is necessary to check if the top-right element or bottom-left element is available for a given current element in significant map coding. This causes additional branch operations.

The experimental results indicate that this adaptive scan scheme achieves only negligible performance gain, as shown in Table 1, but at the expense of additional memory requirements and computational complexity.

- Proposal and experimental results

We propose to use the zigzag scan for significance map coding when the macro “HHI_TRANSFORM_CODING” is switched on, or CABAC/PIPE is selected. We modified TMuC0.7 to replace the adaptive scan with the zigzag scan for transform blocks larger than 8x8 and tested the performance. As an example, Fig.2 shows the zigzag scan for 16x16 blocks. Since the scan pattern is fixed, the neighborhood in the context selection is also fixed. Additional memory requirements and computation complexity associated with the adaptive scan therefore no longer exists.

Fig. 2 Zigzag scan for 16x16 blocks

Simulations were conducted to evaluate the performance comparison of the adaptive scan and the zigzag scan. The simulations follow common test conditions [3]. The anchors from TE12 are used for comparison. Only “Intra High Efficiency”, “Lowdelay High Efficiency” and “Randomaccess High Efficiency” are tested, since PIPE is used only for high efficiency cases.

Table I gives the summary of the performance comparison between the adaptive scan and the zigzag scan. The detailed results could be found in the attached excel file (Note: the encoding and decoding time of the anchors are copied from TE12 which runs on different PCs, so the comparison of the encoding and decoding time in this proposal in meaningless).

Table I Performance of the zigzag scan replacing the adaptive scan in TMuC0.7

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.3 | 0.2 |
| Class B | 0.1 | 0.2 | 0.4 |
| Class C | 0.0 | 0.4 | 0.4 |
| Class D | 0.0 | 0.3 | 0.4 |
| Class E | 0.1 | 0.2 | 0.1 |
| All | 0.0 | 0.3 | 0.3 |
| Enc Time[%] | 110% |  |  |
| Dec Time[%] | 125% |  |  |
|  |  | Random access |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.4 | 0.1 |
| Class B | 0.1 | 0.1 | 0.1 |
| Class C | 0.1 | 0.2 | 0.1 |
| Class D | 0.1 | 0.4 | 0.0 |
| Class E |  |  |  |
| All | 0.1 | 0.2 | 0.1 |
| Enc Time[%] | 88% |  |  |
| Dec Time[%] | 78% |  |  |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.1 | 0.2 | 0.4 |
| Class C | 0.2 | 0.3 | 0.4 |
| Class D | 0.2 | 0.7 | 0.5 |
| Class E | 0.3 | 1.3 | -0.1 |
| All | 0.2 | 0.6 | 0.3 |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 77% |  |  |

- Summary

In the current TMuC, an adaptive scan scheme is applied for the significance map coding if the transform size is larger than 8x8 and the entropy coding option is CABAC/PIPE. Only negligible performance gain could be achieved with additional memory requirements and computational complexity. Therefore, we propose to apply the zigzag scan when the macro “HHI_TRANSFORM_CODING” is switched on, or or CABAC/PIPE is selected.

- References

[1] Martin Winken, Sebastian Boße, Benjamin Bross, Philipp Helle, Tobias Hinz, Heiner Kirchhoffer, Haricharan Lakshman, Detlev Marpe, Simon Oudin, Matthias Preiß, Heiko Schwarz, Mischa Siekmann, Karsten Sühring, and Thomas Wiegand, “Description of video coding technology proposal by Fraunhofer HHI,” JCT-VC document, JCTVC-A116, April 2010.

[2] K. McCann, W.-J. Han, and I.-K. Kim, “Samsung’s Response to the Call for Proposals on Video Compression Technology,” JCT-VC document, JCTVC-A124, April 2010.

[3] F. Bossen, “Common test conditions and software reference configurations,” JCT-VC document, JCTVC-B300, July 2010.

[4] K, McCann, “Tool Experiment 12: Evaluation of TMuC Tools,” JCT-VC document, JCTVC-B312, July 2010.

# Patent rights declaration(s)

Motorola Mobility may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).