| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C115 WG11 Number: m18138 |
| --- | --- |

| Title: | TE1: Summary report of TE1 activity |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yi-Jen Chiu Intel Corp. | Tel: Email: | yi-jen.chiu@intel.com |
|  | Haoping Yu Huawei Technologies Co., Ltd. |  | haopingyu@huawei.com |
|  | Yu-Wen Huang MediaTek Inc. |  | yuwen.huang@mediatek.com |
|  | Mathias Wien RWTH Aachen University |  | wien@ient.rwth-aachen.de |
| Source: | Intel Corp., Huawei Technologies Co., Ltd., Mediatek Inc., RWTH Aachen University |  |  |

_____________________________

# Abstract

This document summarizes the activities in the Tool Experiment TE1 on Decoder-side Motion Vector Derivation (DMVD). A total of 4 proponents have requested cross-checking of their TMuC0.7 based implementations, and all of them have been evaluated with the test conditions as defined in JCTVC-B301 [1].

# Introduction

The 14 companies listed in Annex have registered for participation in TE1. Between the meetings, a lively email discussion took place among the about 30 individuals representing the participating companies.

In general, the tools under TE1 evaluation are tested in two sub-experiments:

## Sub Experiment TE.1a: DMVD Inter Prediction

Definition: General performance evaluations of DMVD, the additional coding gain over other tools that are also designed to improve coding efficiency for inter frames, e.g. AMVP.

- JCTVC-C097 (m18120) TE1：Huawei report on DMVD improvements in TMuC, [M. Yang, S. Lin, D. Wang, J. Zhou, H. Yu (Huawei)]

- JCTVC-C127 (m18151) TE1: Report of self derivation of motion estimation improvement in TMuC [Y. Chiu, L. Xu, W. Zhang, H. Jiang (Intel)]

- JCTVC-C138 (m18162) TE1.a: Implementation report of refinement motion compensation using DMVD on TMuC [M. Ueda (JVC)]

## Sub Experiment TE.1b: DMVD Direct Mode

Definition: performance evaluation of DMVD in Skip/Direct mode, comparing it with other techniques such as AMVP, MRG.

- JCTVC-C097 (m18120) TE1：Huawei report on DMVD improvements in TMuC, [M. Yang, S. Lin, D. Wang, J. Zhou, H. Yu (Huawei)]

- JCTVC-C124 (m18147) TE1: Implicit direct vector derivation [S. Sekiguchi, Y. Itani (Mitsubishi Electric)]

- JCTVC-C127 (m18151) TE1: Report of self derivation of motion estimation improvement in TMuC [Y. Chiu, L. Xu, W. Zhang, H. Jiang (Intel)]

# Document List

## TE Documents

| Participant | Document | Title | Type |
| --- | --- | --- | --- |
| Huawei | JCTVC-C097 JCTVC-C098 | TE1：Huawei report on DMVD improvements in TMuC TE1: Cross-checking of DMVD results from Mitsubishi and Intel | Proposal, verified by Intel (JCTVC-C128) & Mitsubishi (JCTVC-C125) Verification |
| Intel | JCTVC-C127 JCTVC-C128 | TE1: Report of self derivation of motion estimation improvement in TMuC TE1: Cross-checking of DMVD results from Huawei | Proposal, verified by Huawei (JCTVC-C098) Verification |
| JVC | JCTVC-C138 | TE1.a: Implementation report of Refinement Motion Compensation using DMVD on TMuC | Proposal, verified by Qualcomm (JCTVC-C266) |
| Mitsubishi Electric Corp. | JCTVC-C124 JCTVC-C125 | TE1 report: implicit direct vector derivation TE1: Spatio-temporal direct mode | Proposal, verified by Huawei (JCTVC-C098) & Hitachi (JCTVC-C024) Verification |
| Hitachi | JCTVC-C024 | TE1: Cross-check result of DMVD proposal JCTVC-C124 (Mitsubishi) | Verification |
| Qualcomm | JCTVC-C266 | TE1: Cross-check result of DMVD proposal from JCTVC-C138 (JVC/Kenwood) | Verification |

# Summary of proposals

## JCTVC-C097 (Huawei)

The goal of this Tool Experiment (TE1) is to prove the performance of Decoder-side Motion Vector Derivation (DMVD) techniques in TMuC software. There are two categories in the TE1, one is TE.1a (DMVD Inter Prediction) and the other is TE.1b (DMVD Direct Mode). This contribution continues the work in [1][2], including the Template Matching based DMVD (TM-DMVD) and the Spatial and Temporal Direct Mode (STDM) techniques. STDM and TM-DMVD work together in the TE.1a and STDM works alone in the TE.1b. Both techniques have been implemented into the TMuC 0.7 software and tested under following the common test conditions in JCTVC-B300 and JCTVC-B301. This document describes these techniques in details, which includes algorithm description, software implementation, coding performance, and complexity evaluation and analysis. According to the test results, the average bit rate savings for random access and low delay are 3.90% and 2.43%, respectively in TE1.a, and 2.9% and 0.5%, respectively in TE1.b. Please note that some of the tests for Class A and Class B sequences in TE1.a are not finished yet and the final results will be updated as soon as they are available.

### Test Results for Random Access High Efficiency Configuration Relative to TE1 Anchor

| Video Sequence |  | TM-DMVD_STDM |  |  |
| --- | --- | --- | --- | --- |
|  |  | BD-rate Y(%) | BD-rate U(%) | BD-rate V(%) |
| Class A | PeopleOnStreet |  |  |  |
|  | Traffic |  |  |  |
| Class B | Kimono |  |  |  |
|  | ParkSene | -3.6 | -2.1 | -2.1 |
|  | BasketballDrive |  |  |  |
|  | BQTerrace |  |  |  |
|  | Cactus |  |  |  |
| Class C | RaceHorses | -2.1 | -2.5 | -3.0 |
|  | PartyScene | -4.6 | -3.4 | -3.1 |
|  | BQMall | -5.9 | -4.5 | -4.9 |
|  | BasketballDrill | -3.1 | -3.6 | -4.0 |
| Class D | BasketballPass | -4.0 | -4.1 | -5.0 |
|  | BQSquare | -4.0 | -1.2 | -1.3 |
|  | BlowingBubbles | -4.5 | -3.1 | -2.8 |
|  | RaceHorses | -3.2 | -3.9 | -4.2 |
| Total Average |  | -3.90 | -3.16 | -3.37 |

Table 1 - RD performance in random access high efficiency configuration

From Table 1 the total gain from the proposed techniques is about 3.90% bitrate saving.

### Test results for Low Delay High Efficiency Configuration Relative to TE1 Anchor

| Video Sequence |  | TM-DMVD_STDM |  |  |
| --- | --- | --- | --- | --- |
|  |  | BD-rate Y(%) | BD-rate U(%) | BD-rate V(%) |
| Class B | Kimono | -2.1 | -0.4 | -0.3 |
|  | ParkSene | -1.9 | 0.6 | 0.9 |
|  | BasketballDrive | -2.5 | 0.2 | -0.3 |
|  | BQTerrace | -0.8 | 5.4 | 5.6 |
|  | Cactus | -3.2 | -1.0 | -0.7 |
| Class C | RaceHorses | -1.4 | -0.8 | -0.3 |
|  | PartyScene | -1.9 | 0.6 | 1.2 |
|  | BQMall | -3.8 | -1.6 | -1.4 |
|  | BasketballDrill | -2.8 | -2.4 | -2.5 |
| Class D | BasketballPass | -2.2 | -1.2 | -1.4 |
|  | BQSquare | -1.3 | 5.6 | 4.3 |
|  | BlowingBubbles | -2.2 | 0.7 | 1.4 |
|  | RaceHorses | -2.1 | -1.7 | -1.2 |
| Class E | Vidyo1 | -4.3 | -1.3 | -1.6 |
|  | Vidyo3 | -3.1 | -1.1 | -3.6 |
|  | Vidyo4 | -3.5 | -1.8 | -1.3 |
| Total Average |  | -2.43 | 0.00 | -0.07 |

Table 2 -	RD performance in low delay high efficiency configuration

Compared with TE1 anchor, the average gain from the proposed techniques is about 2.43% bitrate saving.

### Random Access High Efficiency Configuration Relative to TE1 Anchor

| Video Sequence |  | TM-DMVD_STDM (%) |
| --- | --- | --- |
| Class A | PeopleOnStreet |  |
|  | Traffic |  |
| Class B | Kimono |  |
|  | ParkSene | 195.82% |
|  | BasketballDrive |  |
|  | BQTerrace |  |
|  | Cactus |  |
| Class C | RaceHorses | 265.65% |
|  | PartyScene | 276.38% |
|  | BQMall | 192.99% |
|  | BasketballDrill | 192.86% |
| Class D | BasketballPass | 142.33% |
|  | BQSquare | 336.28% |
|  | BlowingBubbles | 299.55% |
|  | RaceHorses | 182.07% |
| Total Average |  | 231.55% |

Table 3	- Complexity analysis in random access high efficiency configuration

From Table 3, it can be seen that TM-DMVD_STDM is about 2.3times slower than TE1 anchor.

### Low Delay High Efficiency Configuration Relative to TE1 Anchor

| Video Sequence |  | TM-DMVD_STDM (%) |
| --- | --- | --- |
| Class B | Kimono | 330.20% |
|  | ParkSene | 437.19% |
|  | BasketballDrive | 235.28% |
|  | BQTerrace | 380.47% |
|  | Cactus | 363.31% |
| Class C | RaceHorses | 432.83% |
|  | PartyScene | 469.91% |
|  | BQMall | 272.29% |
|  | BasketballDrill | 291.66% |
| Class D | BasketballPass | 361.33% |
|  | BQSquare | 358.96% |
|  | BlowingBubbles | 358.95% |
|  | RaceHorses | 285.06% |
| Class E | Vidyo1 | 278.68% |
|  | Vidyo3 | 253.27% |
|  | Vidyo4 | 232.69% |
| Total Average |  | 305% |

Table 4 -	Complexity analysis in low delay high efficiency configuration

From Table 4, it can be seen that TM-DMVD_STDM is about 3times slower than TE1 anchor.

Note: In the last meeting we applied a simplified interpolation filter in TM-DMVD and STDM (i.e. SDMVD and SSTDM) to effectively reduce the decoding complexity without sacrificing compression performance. For the case of SSTDM, its performance was even slightly better than that of STDM.

### Test Results for Random Access High Efficiency Configuration Relative to TE1 Anchor

| Video Sequence |  | STDM |  |  |
| --- | --- | --- | --- | --- |
|  |  | BD-rate Y(%) | BD-rate U(%) | BD-rate V(%) |
| Class A | PeopleOnStreet | -3.8 | -4.4 | -4.5 |
|  | Traffic | -3.4 | -3.2 | -3.1 |
| Class B | Kimono | -3.2 | -2.6 | -2.5 |
|  | ParkSene | -2.3 | -1.9 | -1.6 |
|  | BasketballDrive | -2.5 | -2.4 | -2.7 |
|  | BQTerrace | -1.8 | -1.0 | -0.6 |
|  | Cactus | -5.1 | -4.2 | -4.3 |
| Class C | RaceHorses | -0.6 | -0.9 | -1.0 |
|  | PartyScene | -3.2 | -2.6 | -2.4 |
|  | BQMall | -4.6 | -3.9 | -4.1 |
|  | BasketballDrill | -1.7 | -2.4 | -2.5 |
| Class D | BasketballPass | -2.8 | -3.3 | -3.4 |
|  | BQSquare | -2.9 | -1.8 | -1.9 |
|  | BlowingBubbles | -3.0 | -2.0 | -2.3 |
|  | RaceHorses | -2.0 | -2.3 | -2.2 |
| Total Average |  | -2.9 | -2.6 | -2.6 |

Table 5 -	RD performance in random access high efficiency configuration

From Table 5 the average gain for proposed technique is about 2.9% bit rate saving compared with TE1 anchor.

### Test results for Low Delay High Efficiency Configuration Relative to TE1 Anchor

| Video Sequence |  | STDM |  |  |
| --- | --- | --- | --- | --- |
|  |  | BD-rate Y(%) | BD-rate U(%) | BD-rate V(%) |
| Class B | Kimono | -0.9 | -0.1 | 0.0 |
|  | ParkSene | -0.5 | 1.1 | 1.2 |
|  | BasketballDrive | -0.5 | 0.4 | 0.3 |
|  | BQTerrace | 0.7 | 3.5 | 2.7 |
|  | Cactus | -1.7 | 0.0 | 0.3 |
| Class C | RaceHorses | 0.2 | 0.5 | 0.4 |
|  | PartyScene | -0.1 | 1.2 | 1.7 |
|  | BQMall | -2.0 | -0.8 | -0.2 |
|  | BasketballDrill | -0.5 | -0.6 | -0.2 |
| Class D | BasketballPass | -0.6 | -0.8 | -0.5 |
|  | BQSquare | 0.9 | 3.5 | 3.3 |
|  | BlowingBubbles | -0.5 | 1.2 | 2.2 |
|  | RaceHorses | -0.3 | -0.6 | -0.5 |
| Class E | Vidyo1 | -1.6 | 0.2 | 0.0 |
|  | Vidyo3 | -0.3 | -0.2 | -1.3 |
|  | Vidyo4 | -0.6 | 0.5 | 0.8 |
| Total Average |  | -0.5 | 0.6 | 0.6 |

Table 6 -	RD performance in low delay high efficiency configuration

Compared with TE1 anchor, the average gain for proposed technique is about 0.5% bit rate saving.

### Random Access High Efficiency Configuration Relative to TE1 Anchor

| Video Sequence |  | STDM (%) |
| --- | --- | --- |
| Class A | PeopleOnStreet | 129.48% |
|  | Traffic | 116.27% |
| Class B | Kimono | 122.78% |
|  | ParkSene | 118.77% |
|  | BasketballDrive | 117.21% |
|  | BQTerrace | 113.02% |
|  | Cactus | 127.09% |
| Class C | RaceHorses | 95.71% |
|  | PartyScene | 135.55% |
|  | BQMall | 127.43% |
|  | BasketballDrill | 107.83% |
| Class D | BasketballPass | 114.91% |
|  | BQSquare | 120.57% |
|  | BlowingBubbles | 123.50% |
|  | RaceHorses | 116.01% |
| Total Average |  | 118% |

Table 7 - Complexity analysis in random access high efficiency configuration

From Table 7 STDM is about 1.18 times slower than TE1 anchor.

### Low Delay High Efficiency Configuration Relative to TE1 Anchor

| Video Sequence |  | STDM (%) |
| --- | --- | --- |
| Class B | Kimono | 133.15% |
|  | ParkSene | 175.20% |
|  | BasketballDrive | 129.96% |
|  | BQTerrace | 114.22% |
|  | Cactus | 121.91% |
| Class C | RaceHorses | 142.37% |
|  | PartyScene | 128.08% |
|  | BQMall | 167.59% |
|  | BasketballDrill | 134.28% |
| Class D | BasketballPass | 124.98% |
|  | BQSquare | 97.84% |
|  | BlowingBubbles | 165.17% |
|  | RaceHorses | 104.33% |
| Class E | Vidyo1 | 131.64% |
|  | Vidyo3 | 115.27% |
|  | Vidyo4 | 113.76% |
| Total Average |  | 129% |

Table 8 - Complexity analysis in low delay high efficiency configuration

From Table 8 STDM is about 1.29 times slower than TE1 anchor.

## JCTVC-C127 (Intel)

This contribution presents the techniques of decoder-side motion vector derivation (DMVD) side to increase coding efficiency of B pictures. With the motion vector (MV) information self derived at video decoder side, the transmission of these MVs from video encoder side to video decoder side is skipped and thus better coding efficiency can be achieved. The proposed self derivation of motion estimation (SDME) techniques are friendly to the parallel implementation by utilizing on the temporal correlation among the available pixels in the previously-decoded reference pictures, instead of operating on the previously reconstructed pixels of the neighboring area of the current picture, which poses an inherent decoding ordering causal relationship among decoding blocks of current picture. Experiments have demonstrated that an overall 4.8% BD bitrate improvement for the test of random access (high efficiency) category under the TE1 test conditions with 73% increase in encoding time and 150% increase in decoding time increase on TMuC 0.7 Reference Software platform. A fast version of the direct-candidate simplified SDME has demonstrated an overall 3.1% BD bitrate improvement with a surprising 26% reduction in encoding time and 18% increase in decoding time when compared to TE1 anchor. Another version of the rounded-candidate simplified SDME has demonstrated an overall 3.7% BD bitrate improvement with 10% increase in encoding time and 13% increase in decoding time when compared to TE1 anchor. Also, the proposed SDME demonstrates 2.7% BD bitrate improvement over the anchor of TE12 under random access test category (high efficiency) on the shorter version of the common test video sequence defined for TE12. A description about the normative syntax change in the form of TMuC specification is included.

### Test Results for Random Access High Efficiency Configuration Relative to TE1 Anchor – Candidate-based SDME

Table 9 shows the SDME performance compared with the TE1 anchors. The BD measurements in bitrate reduction were employed. The average BD bitrate reduction for luma component is about 4.8% with about 73% increase in encoding time and about 150% increase in decoding time.

Table 9 - Candidates based SDME performance compared with TE1 anchors

|  | Sequences | BD-rate Y (%) | BD-rate U (%) | BD-rate V (%) | Enc-time ratio | Dec-time ratio |
| --- | --- | --- | --- | --- | --- | --- |
| Class A | Traffic | -5.6 | -5.1 | -4.9 |  |  |
|  | PeopleOnStreet | -6.0 | -7.1 | -6.9 |  |  |
| Class B | Kimono1 | -5.8 | -4.9 | -4.6 |  |  |
|  | ParkScene | -4.7 | -4.0 | -3.9 |  |  |
|  | Cactus | -7.2 | -6.1 | -6.2 |  |  |
|  | BasketballDrive | -4.4 | -4.3 | -4.4 |  |  |
|  | BQTerrace | -3.1 | -2.6 | -2.5 |  |  |
| Class C | BasketballDrill | -3.3 | -3.8 | -3.9 |  |  |
|  | BQMall | -7.0 | -6.3 | -6.7 |  |  |
|  | PartyScene | -4.2 | -3.8 | -3.8 |  |  |
|  | RaceHorses | -2.2 | -3.0 | -3.4 |  |  |
| Class D | BasketballPass | -5.1 | -5.5 | -5.8 |  |  |
|  | BQSquare | -4.0 | -2.8 | -2.8 |  |  |
|  | BlowingBubbles | -4.7 | -3.6 | -3.9 |  |  |
|  | RaceHorses | -4.2 | -5.1 | -5.2 |  |  |
| Average of Class A |  | -5.8 | -6.1 | -5.9 |  |  |
| Average of Class B |  | -5.1 | -4.4 | -4.3 |  |  |
| Average of Class C |  | -4.1 | -4.2 | -4.4 |  |  |
| Average of Class D |  | -4.5 | -4.2 | -4.4 |  |  |
| Overall Average |  | -4.8 | -4.5 | -4.6 | 173% | 250% |

### Test Results for Random Access High Efficiency Configuration Relative to TE1 Anchor – Direct-Candidate Simplified SDME

Table 10.a shows the performance of the direct-candidate simplified SDME compared with the TE1 anchors. The average BD bitrate reduction for luma component is about 3.1% with a surprising 26% reduction in encoding time and about 18% increase in decoding time when compared to TE1 anchor.

Table 10.a – Direct-candidate Simplified SDME performance compared with TE1 anchors

|  | Sequences | BD-rate Y (%) | BD-rate U (%) | BD-rate V (%) | Enc-time ratio | Dec-time ratio |
| --- | --- | --- | --- | --- | --- | --- |
| Class A | Traffic | -4.1 | -3.9 | -3.9 |  |  |
|  | PeopleOnStreet | -4.1 | -4.9 | -5.0 |  |  |
| Class B | Kimono1 | -4.0 | -3.4 | -3.4 |  |  |
|  | ParkScene | -2.8 | -2.7 | -2.5 |  |  |
|  | Cactus | -5.2 | -4.3 | -4.3 |  |  |
|  | BasketballDrive | -2.8 | -2.7 | -2.9 |  |  |
|  | BQTerrace | -2.1 | -1.9 | -2.2 |  |  |
| Class C | BasketballDrill | -2.0 | -2.7 | -2.6 |  |  |
|  | BQMall | -4.7 | -4.3 | -4.7 |  |  |
|  | PartyScene | -2.6 | -2.6 | -2.5 |  |  |
|  | RaceHorses | -1.0 | -1.8 | -1.8 |  |  |
| Class D | BasketballPass | -3.3 | -3.6 | -3.9 |  |  |
|  | BQSquare | -2.0 | -1.7 | -1.3 |  |  |
|  | BlowingBubbles | -2.9 | -2.5 | -2.8 |  |  |
|  | RaceHorses | -2.4 | -3.1 | -3.2 |  |  |
| Average of Class A |  | -4.1 | -4.4 | -4.5 |  |  |
| Average of Class B |  | -3.4 | -3.0 | -3.1 |  |  |
| Average of Class C |  | -2.6 | -2.8 | -2.9 |  |  |
| Average of Class D |  | -2.6 | -2.7 | -2.8 |  |  |
| Overall Average |  | -3.1 | -3.1 | -3.1 | 74% | 118% |

### Test Results for Random Access High Efficiency Configuration Relative to TE1 Anchor – Rounded-Candidate Simplified SDME

Table 10.b shows the performance of the rounded-candidate simplified SDME with the rounded M. The average BD bitrate reduction for luma component is about 3.7% with a 10% increase in encoding time and about 13% increase in decoding time when compared to TE1 anchor.

Table 10.b – Rounded-candidate Simplified SDME performance compared with TE1 anchors

|  | Sequences | BD-rate Y (%) | BD-rate U (%) | BD-rate V (%) | Enc-time ratio | Dec-time ratio |
| --- | --- | --- | --- | --- | --- | --- |
| Class A | Traffic | -4.8 | -4.4 | -4.3 |  |  |
|  | PeopleOnStreet | -5.0 | -5.9 | -5.9 |  |  |
| Class B | Kimono1 | -4.8 | -4.1 | -4.1 |  |  |
|  | ParkScene | -3.5 | -3.2 | -3.1 |  |  |
|  | Cactus | -5.6 | -4.6 | -4.9 |  |  |
|  | BasketballDrive | -3.5 | -3.5 | -3.7 |  |  |
|  | BQTerrace | -2.8 | -2.1 | -1.9 |  |  |
| Class C | BasketballDrill | -2.6 | -3.0 | -3.3 |  |  |
|  | BQMall | -5.6 | -5.2 | -5.4 |  |  |
|  | PartyScene | -3.1 | -3.1 | -3.1 |  |  |
|  | RaceHorses | -1.6 | -2.3 | -2.7 |  |  |
| Class D | BasketballPass | -4.0 | -4.5 | -4.6 |  |  |
|  | BQSquare | -2.5 | -2.3 | -2.0 |  |  |
|  | BlowingBubbles | -3.4 | -3.0 | -3.4 |  |  |
|  | RaceHorses | -3.1 | -3.8 | -4.1 |  |  |
| Average of Class A |  | -4.9 | -5.2 | -5.1 |  |  |
| Average of Class B |  | -4.0 | -3.5 | -3.5 |  |  |
| Average of Class C |  | -3.2 | -3.4 | -3.6 |  |  |
| Average of Class D |  | -3.2 | -3.4 | -3.5 |  |  |
| Overall Average |  | -3.7 | -3.7 | -3.8 | 110% | 113% |

### Test Results for Random Access High Efficiency Configuration Relative to MRG and AMVP – Candidate-based SDME

The common test condition of TE1 disables MRG and AMVP modes (MRG=0, AMVP=0). For reference, the SDME is tested with the different testing settings of MRG and AMVP and the result is listed in Table 11. To reduce the simulation time, a shorter version of the common test sequence is applied for considering only the pictures from the first intra frame to the second intra frame for each testing point. The coding gain of SDME with “MRG = 0 && AMVP off” achieves 4.1% BD bitrate improvement in the first column of Table 11 (Referred to 4.8% improvement for the complete test sequence). The coding gain of SDME with “MRG = 1 && AMVP on” is 2.7% BD bitrate improvement which demonstrates SDME improvement over the anchor of TE12. For reference, the experiment of the combined (MRG=1 & AMVP=1) compared to TE1 anchor is investigated and a 2% BD bitrate improvement is listed in the last column of the table. (This is in fact the comparison between the TE12 anchor and TE1 anchor for the test case of Random Access (high efficiency))

The results in Table 11 show that SDME achieved sizeable coding gain (>2.3%) with all kinds of settings of MRG and AMVP for test category of Random Access (high efficiency) on the shorter version of the common test video sequence.

Table 11 – BD-rate Y of SDME under different MRG and AMVP settings

|  | Sequences | MRG = 0 AMVP off | MRG = 0 AMVP on | MRG = 1 AMVP off | MRG = 1 AMVP on | TE12 toTE1 |
| --- | --- | --- | --- | --- | --- | --- |
| Class A | Traffic | -4.0 | -2.1 | -3.4 | -2.5 | -2.5 |
|  | PeopleOnStreet | -5.1 | -3.2 | -4.6 | -3.6 | -2.0 |
| Class B | Kimono1 | -4.2 | -2.0 | -3.1 | -2.3 | -2.4 |
|  | ParkScene | -4.1 | -2.4 | -2.7 | -2.1 | -2.5 |
|  | Cactus | -5.9 | -3.2 | -5.2 | -4.4 | -2.1 |
|  | BasketballDrive | -5.0 | -1.9 | -3.8 | -2.9 | -3.3 |
|  | BQTerrace | -1.7 | -0.7 | -1.4 | -1.1 | -1.7 |
| Class C | BasketballDrill | -3.0 | -2.1 | -2.7 | -2.0 | -1.5 |
|  | BQMall | -5.5 | -1.4 | -4.7 | -3.6 | -2.5 |
|  | PartyScene | -3.3 | -3.3 | -3.1 | -2.4 | -1.3 |
|  | RaceHorses | -2.0 | -2.1 | -1.7 | -1.2 | -1.9 |
| Class D | BasketballPass | -6.1 | -1.1 | -4.5 | -4.1 | -1.6 |
|  | BQSquare | -3.2 | -4.0 | -3.0 | -2.4 | -1.4 |
|  | BlowingBubbles | -4.5 | -2.2 | -3.9 | -3.3 | -1.2 |
|  | RaceHorses | -3.8 | -2.6 | -3.1 | -2.3 | -2.3 |
| Average of Class A |  | -4.6 | -2.7 | -4.0 | -3.0 | -2.3 |
| Average of Class B |  | -4.2 | -2.0 | -3.3 | -2.5 | -2.4 |
| Average of Class C |  | -3.5 | -2.0 | -3.1 | -2.3 | -1.8 |
| Average of Class D |  | -4.4 | -2.8 | -3.6 | -3.1 | -1.6 |
| Overall Average |  | -4.1 | -2.3 | -3.4 | -2.7 | -2.0 |

## JCTVC-C138 (JVC)

In TE1 activity, the tool experiment targets at exploring the performance and value of Decoder-side Motion Vector Derivation (DMVD) and at analyzing the interaction of DMVD with other tools under consideration for HEVC.

This proposal focuses on the tool “Refinement Motion Compensation (RMC) using DMVD” which belongs to TE1.a: DMVD Inter Prediction. The feature in RMC is that bi-predictive MC blocks can be obtained by transmitting only one motion vector using inter-reference DMVD.

In this contribution, the implementation of basic RMC tool on TMuC0.7 software and result of evaluating the performance are reported. The simulation results show that the proposed technique provides 1.8% BD-rate gain for random access case and 0.0% for low delay case as against TE1 common anchor under TE1’s test conditions.

### Test Results Summary - Coding Efficiency

The BD-rate summary of this proposal is shown in Table 12.

The results of random access case for each sequence are shown in Table 13 to Table 16.

The results of low delay case for each sequence are shown in Table 17 to Table 20.

### Table 12 - BD-rate summary of this proposal

### Table 13 - Results of random access (Class A)

### Table 14 - Results of random access (Class B)

### Table 15 - Results of random access (Class C)

### Table 16 - Results of random access (Class D)

### Table 17 - Result of low delay (Class B)

### Table 18 - Result of low delay (Class C)

### Table 19 - Result of low delay (Class D)

### Table 20 - Result of low delay (Class E)

### Test Results Summary - Complexity

The results of complexity assessment for RMC are as follows.

The results show the ratio of execution time between RMC and the anchor

A mean value of three times is used for measuring the decoding time without output of yuv file.

The complexity comparison summary of this proposal is shown in Table 21.

### Table 21 - Complexity comparison summary of this proposal

RMC simply performs motion estimation for deriving second motion vector. Therefore the complexity does not increase too much even in random access condition.

## JCTVC-C138 (Mitsubishi)

This contribution is to report results of performance verification on implicit direct vector derivation technique that was approved as a part of TE1 at the last Geneva meeting. The evaluated technique provides a new efficient design of B-skip and B-direct modes with locally adaptive direct MV derivation relying on a simple decoder-side decision. A performance evaluation using TMuC software was conducted according to the test condition defined in TE1 and the results show 1.8% coding gain in average and up to 3.5% at maximum gain. The increase of decoding time was about 10% relative to TE1 anchor and encoding time was almost same as that of the anchor. This contribution also provides notes on normative text in the form of modification of TMuC specification.

### Test Results Summary

B-skip and B-direct modes using the proposed technique has been implemented into TMuC version0.7 software that was made available to the group.

The BD rate reduction obtained by the proposed direct mode is listed in Table 22, 23. It is observed that the proposed direct mode can achieve consistent coding gain especially for the sequences where spatially detailed texture and stable background scene co-exist, such as PeopleOnStreet or cactus. In such cases, spatially adjacent motion vector may not work as sufficient estimator for direct vector, but temporal candidate  could contribute to improvement of direct vector accuracy.

Table 22 - Performance in BD measurements (low delay)

|  | BD-PSNR[dB] |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Kimono | 0.1 | -0.3 | 0.2 |
| ParkScene | -0.1 | -0.6 | -0.4 |
| Cactus | -0.5 | -1 | -1.3 |
| BasketballDrive | -0.1 | 0 | -0.2 |
| BQTerrace | -0.5 | -0.9 | -1.1 |
| BasketballDrill | -0.1 | -0.1 | -0.2 |
| BQMall | -0.4 | -0.6 | -0.4 |
| PartyScene | -0.4 | -0.3 | -0.2 |
| RaceHorses | -0.1 | -0.3 | -0.4 |
| BasketballPass | -0.3 | -0.6 | -0.4 |
| BQSquare | -1.5 | 0.4 | -0.4 |
| BlowingBubbles | -0.3 | -0.7 | -0.2 |
| RaceHorses | -0.3 | -0.8 | -0.7 |
| Vidyo1 | -1.4 | -1.7 | -1.5 |
| Vidyo3 | 0.2 | -1.6 | -1.4 |
| Vidyo4 | -0.7 | -2.1 | -1.2 |
| Average Class B | -0.2 | -0.6 | -0.6 |
| Average Class C | -0.3 | -0.3 | -0.3 |
| Average Class D | -0.6 | -0.4 | -0.4 |
| Average Class E | -0.6 | -1.8 | -1.4 |
| Average All | -0.4 | -0.7 | -0.6 |

Table 23 - Performance in BD measurements (random access)

|  | BD-PSNR[dB] |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Traffic | -2.4 | -1.9 | -1.9 |
| PeopleOnStreet | -2.3 | -2.3 | -2.2 |
| Kimono | -1.9 | -1.2 | -1.1 |
| ParkScene | -1.3 | -0.9 | -0.7 |
| Cactus | -3.5 | -2.5 | -2.9 |
| BasketballDrive | -1 | -0.5 | -0.7 |
| BQTerrace | -0.9 | -0.5 | -1.1 |
| BasketballDrill | -0.8 | -0.9 | -0.8 |
| BQMall | -2.7 | -2.3 | -2.4 |
| PartyScene | -2.3 | -1.8 | -1.8 |
| RaceHorses | -0.3 | -0.4 | -0.5 |
| BasketballPass | -1.6 | -1.8 | -1.6 |
| BQSquare | -2.5 | -2.2 | -1.8 |
| BlowingBubbles | -2.5 | -1.7 | -2 |
| RaceHorses | -1.1 | -1 | -1.2 |
| Average Class A | -2.3 | -2.1 | -2.0 |
| Average Class B | -1.7 | -1.1 | -1.3 |
| Average Class C | -1.5 | -1.3 | -1.4 |
| Average Class D | -1.9 | -1.7 | -1.7 |
| Average All | -1.8 | -1.4 | -1.5 |

Table 24, 25 lists encoding and decoding time those are measured using the provided software. The “time” command in Linux platform was used to obtain these values. Decoding time and encoding time were also compared with that of anchor streams using the same platform (Fedora x86_64). In average, the increase of decoding time was about 10% relative to anchor and encoding time was almost same as that of the anchor.

Table 24 - Encoding and decoding time (low delay)

|  |  | QPI | Encoding time [sec] | Decoding time [sec] | Encoding time [sec] | Decoding time [sec] | Ratio of encoding time [%] | Ratio of decoding time [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class B | S03 | 22 | 142648.27 | 132.61 | 141458.82 | 137.24 | 99.17 | 103.49 |
| 1080p | Kimono | 27 | 132263.69 | 128.35 | 132603.70 | 135.04 | 100.26 | 105.21 |
|  |  | 32 | 125070.48 | 125.27 | 123941.50 | 132.26 | 99.10 | 105.57 |
|  |  | 37 | 119392.61 | 119.97 | 118422.91 | 125.73 | 99.19 | 104.80 |
|  | S04 | 22 | 127517.80 | 128.91 | 127363.69 | 137.88 | 99.88 | 106.96 |
|  | ParkScene | 27 | 114456.18 | 122.33 | 119235.29 | 126.85 | 104.18 | 103.70 |
|  |  | 32 | 106107.48 | 118.12 | 104959.25 | 123.41 | 98.92 | 104.47 |
|  |  | 37 | 101332.44 | 114.64 | 101176.31 | 122.33 | 99.85 | 106.71 |
|  | S05 | 22 | 250656.69 | 195.36 | 248857.15 | 211.77 | 99.28 | 108.40 |
|  | Cactus | 27 | 219604.78 | 176.45 | 216203.20 | 178.45 | 98.45 | 101.13 |
|  |  | 32 | 204587.20 | 159.96 | 211573.39 | 173.47 | 103.41 | 108.45 |
|  |  | 37 | 196329.29 | 163.63 | 193742.69 | 178.25 | 98.68 | 108.94 |
|  | S06 | 22 | 290746.09 | 223.59 | 304461.92 | 229.11 | 104.72 | 102.47 |
|  | BasketballDrive | 27 | 263284.95 | 205.75 | 261172.48 | 203.12 | 99.20 | 98.72 |
|  |  | 32 | 243647.65 | 192.26 | 241989.22 | 196.04 | 99.32 | 101.97 |
|  |  | 37 | 227824.13 | 194.12 | 227284.58 | 199.29 | 99.76 | 102.66 |
|  | S07 | 22 | 331773.68 | 314.34 | 331423.28 | 318.56 | 99.89 | 101.34 |
|  | BQTerrace | 27 | 263950.70 | 240.56 | 262497.73 | 246.35 | 99.45 | 102.41 |
|  |  | 32 | 237326.59 | 212.93 | 236147.60 | 246.58 | 99.50 | 115.81 |
|  |  | 37 | 227034.65 | 206.42 | 225309.78 | 235.99 | 99.24 | 114.32 |
| Class C | S08 | 22 | 61047.49 | 65.15 | 61440.21 | 66.92 | 100.64 | 102.73 |
| WVGA | BasketballDrill | 27 | 55634.65 | 62.11 | 56577.24 | 62.83 | 101.69 | 101.16 |
|  |  | 32 | 51848.57 | 59.86 | 51960.80 | 62.34 | 100.22 | 104.14 |
|  |  | 37 | 49043.48 | 57.46 | 49006.40 | 58.85 | 99.92 | 102.41 |
|  | S09 | 22 | 79283.60 | 77.09 | 79294.44 | 80.53 | 100.01 | 104.47 |
|  | BQMall | 27 | 73037.73 | 73.35 | 72475.60 | 76.70 | 99.23 | 104.56 |
|  |  | 32 | 68965.86 | 69.06 | 68199.30 | 73.67 | 98.89 | 106.68 |
|  |  | 37 | 65558.61 | 66.51 | 64571.92 | 71.13 | 98.49 | 106.95 |
|  | S10 | 22 | 66484.18 | 73.09 | 66622.42 | 76.10 | 100.21 | 104.12 |
|  | PartyScene | 27 | 57490.21 | 67.78 | 57692.69 | 71.20 | 100.35 | 105.05 |
|  |  | 32 | 50507.74 | 62.27 | 50495.17 | 66.60 | 99.98 | 106.95 |
|  |  | 37 | 45978.08 | 55.99 | 46292.26 | 62.17 | 100.68 | 111.03 |
|  | S11 | 22 | 50308.71 | 46.03 | 50763.98 | 47.45 | 100.90 | 103.09 |
|  | RaceHorses | 27 | 46046.21 | 43.86 | 46365.92 | 44.17 | 100.69 | 100.69 |
|  |  | 32 | 42107.82 | 40.58 | 42288.12 | 41.36 | 100.43 | 101.93 |
|  |  | 37 | 38952.50 | 37.90 | 38899.41 | 38.47 | 99.86 | 101.49 |
| Class D | S12 | 22 | 15830.10 | 22.07 | 15930.46 | 22.86 | 100.63 | 103.61 |
| WQVGA | BasketballPass | 27 | 14778.31 | 21.18 | 14709.20 | 21.98 | 99.53 | 103.76 |
|  |  | 32 | 13718.95 | 19.84 | 13660.55 | 20.99 | 99.57 | 105.79 |
|  |  | 37 | 12821.18 | 18.74 | 12844.68 | 19.87 | 100.18 | 106.04 |
|  | S13 | 22 | 17622.12 | 28.72 | 17715.39 | 30.00 | 100.53 | 104.47 |
|  | BQSquare | 27 | 15141.08 | 25.96 | 15247.94 | 27.77 | 100.71 | 106.96 |
|  |  | 32 | 12996.19 | 23.64 | 13018.77 | 25.75 | 100.17 | 108.95 |
|  |  | 37 | 11781.70 | 21.48 | 11915.55 | 23.93 | 101.14 | 111.44 |
|  | S14 | 22 | 14795.96 | 22.28 | 14876.23 | 22.72 | 100.54 | 101.96 |
|  | BlowingBubbles | 27 | 12686.88 | 20.00 | 12747.73 | 21.29 | 100.48 | 106.42 |
|  |  | 32 | 11313.37 | 18.64 | 11364.64 | 19.93 | 100.45 | 106.89 |
|  |  | 37 | 10419.02 | 17.72 | 10510.85 | 19.31 | 100.88 | 108.99 |
|  | S15 | 22 | 10893.95 | 14.60 | 10909.63 | 14.80 | 100.14 | 101.34 |
|  | RaceHorses | 27 | 9959.22 | 13.91 | 9936.26 | 13.95 | 99.77 | 100.35 |
|  |  | 32 | 9283.67 | 13.13 | 9157.53 | 13.36 | 98.64 | 101.75 |
|  |  | 37 | 8430.42 | 11.81 | 8392.18 | 12.34 | 99.55 | 104.43 |
| ClassE | S16 | 22 | 120894.71 | 145.26 | 120187.69 | 152.94 | 99.42 | 105.28 |
| WQVGA | Vidyo1 | 27 | 112875.26 | 137.95 | 113512.90 | 148.47 | 100.56 | 107.62 |
|  |  | 32 | 111595.10 | 125.74 | 111348.55 | 140.57 | 99.78 | 111.79 |
|  |  | 37 | 109895.65 | 119.73 | 110086.69 | 132.09 | 100.17 | 110.32 |
|  | S17 | 22 | 124806.52 | 148.12 | 124893.46 | 151.87 | 100.07 | 102.53 |
|  | Vidyo3 | 27 | 116147.40 | 140.14 | 116626.63 | 148.44 | 100.41 | 105.93 |
|  |  | 32 | 111678.53 | 133.64 | 112324.65 | 140.66 | 100.58 | 105.26 |
|  |  | 37 | 110029.90 | 127.01 | 110316.64 | 138.37 | 100.26 | 108.95 |
|  | S18 | 22 | 124867.69 | 147.32 | 126162.61 | 151.03 | 101.04 | 102.52 |
|  | Vidyo4 | 27 | 116653.29 | 138.86 | 117426.73 | 143.72 | 100.66 | 103.50 |
|  |  | 32 | 112722.99 | 130.65 | 112967.85 | 141.60 | 100.22 | 108.38 |
|  |  | 37 | 111432.85 | 124.72 | 112122.16 | 135.00 | 100.62 | 108.24 |
|  | Average Class B |  | 196277.77 | 173.78 | 196491.22 | 182.89 | 100.07 | 105.38 |
|  | Average Class C |  | 56393.46 | 59.88 | 56434.12 | 62.53 | 100.14 | 104.22 |
|  | Average Class D |  | 12654.51 | 19.61 | 12683.60 | 20.68 | 100.18 | 105.20 |
|  | Average Class E |  | 115299.99 | 134.93 | 115664.71 | 143.73 | 100.32 | 106.69 |
|  | Average all |  | 100217.54 | 99.48 | 100370.07 | 104.90 | 100.16 | 105.29 |

Table 25 - Encoding and decoding time (random access)

|  |  | QPI | Encoding time [sec] | Decoding time [sec] | Encoding time [sec] | Decoding time [sec] | Ratio of encoding time [%] | Ratio of decoding time [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class A | S01 | 22 | 90485.50 | 114.88 | 90797.47 | 127.86 | 100.34 | 111.30 |
| 4K | Traffic | 27 | 81978.07 | 111.79 | 82126.20 | 124.63 | 100.18 | 111.49 |
|  |  | 32 | 79471.40 | 105.05 | 78334.14 | 118.07 | 98.57 | 112.40 |
|  |  | 37 | 78499.40 | 99.54 | 77867.18 | 116.74 | 99.19 | 117.29 |
|  | S02 | 22 | 132167.72 | 133.82 | 133101.38 | 137.47 | 100.71 | 102.73 |
|  | PeopleOnStreet | 27 | 115925.59 | 121.18 | 116639.66 | 130.97 | 100.62 | 108.08 |
|  |  | 32 | 107319.87 | 115.92 | 107781.91 | 128.92 | 100.43 | 111.22 |
|  |  | 37 | 102208.89 | 114.40 | 101668.37 | 129.62 | 99.47 | 113.31 |
| Class B | S03 | 22 | 102940.58 | 128.83 | 104102.50 | 144.79 | 101.13 | 112.39 |
| 1080p | Kimono | 27 | 94991.16 | 126.69 | 92233.04 | 142.67 | 97.10 | 112.61 |
|  |  | 32 | 89205.66 | 122.26 | 89010.26 | 140.51 | 99.78 | 114.92 |
|  |  | 37 | 86239.05 | 111.56 | 85547.28 | 134.08 | 99.20 | 120.19 |
|  | S04 | 22 | 88852.04 | 126.84 | 89550.40 | 145.06 | 100.79 | 114.36 |
|  | ParkScene | 27 | 79877.03 | 123.56 | 79834.12 | 141.79 | 99.95 | 114.76 |
|  |  | 32 | 75521.00 | 123.84 | 75522.40 | 137.01 | 100.00 | 110.63 |
|  |  | 37 | 73624.59 | 111.29 | 72678.41 | 130.96 | 98.71 | 117.67 |
|  | S05 | 22 | 184739.41 | 188.11 | 185899.18 | 219.21 | 100.63 | 116.53 |
|  | Cactus | 27 | 158441.44 | 176.99 | 159030.78 | 201.14 | 100.37 | 113.65 |
|  |  | 32 | 148663.98 | 165.23 | 147190.75 | 186.70 | 99.01 | 112.99 |
|  |  | 37 | 143384.54 | 154.62 | 141995.49 | 179.61 | 99.03 | 116.17 |
|  | S06 | 22 | 225686.80 | 210.36 | 226174.41 | 237.59 | 100.22 | 112.94 |
|  | BasketballDrive | 27 | 197257.28 | 195.65 | 195732.84 | 230.29 | 99.23 | 117.70 |
|  |  | 32 | 184352.32 | 189.16 | 181788.26 | 223.87 | 98.61 | 118.35 |
|  |  | 37 | 172536.64 | 178.23 | 172392.65 | 221.27 | 99.92 | 124.15 |
|  | S07 | 22 | 228719.87 | 262.21 | 229005.31 | 299.21 | 100.12 | 114.11 |
|  | BQTerrace | 27 | 178587.62 | 242.37 | 179760.16 | 272.76 | 100.66 | 112.54 |
|  |  | 32 | 164882.83 | 216.15 | 164747.06 | 264.51 | 99.92 | 122.37 |
|  |  | 37 | 161640.73 | 202.46 | 161531.31 | 257.78 | 99.93 | 127.32 |
| Class C | S08 | 22 | 47313.81 | 62.45 | 47534.56 | 64.93 | 100.47 | 103.97 |
| WVGA | BasketballDrill | 27 | 42111.50 | 57.15 | 42448.29 | 60.73 | 100.80 | 106.26 |
|  |  | 32 | 38697.45 | 53.60 | 38748.95 | 57.07 | 100.13 | 106.47 |
|  |  | 37 | 36851.82 | 50.85 | 36833.66 | 54.03 | 99.95 | 106.25 |
|  | S09 | 22 | 59660.96 | 76.14 | 60085.37 | 84.35 | 100.71 | 110.78 |
|  | BQMall | 27 | 54378.97 | 71.07 | 54397.53 | 81.05 | 100.03 | 114.05 |
|  |  | 32 | 51657.56 | 64.25 | 51266.99 | 75.83 | 99.24 | 118.02 |
|  |  | 37 | 50003.09 | 62.00 | 49348.30 | 73.36 | 98.69 | 118.32 |
|  | S10 | 22 | 48147.40 | 68.47 | 48406.20 | 79.36 | 100.54 | 115.91 |
|  | PartyScene | 27 | 41139.71 | 64.94 | 41313.48 | 76.10 | 100.42 | 117.18 |
|  |  | 32 | 36832.99 | 60.34 | 36986.53 | 70.79 | 100.42 | 117.31 |
|  |  | 37 | 34344.85 | 54.86 | 34530.59 | 68.05 | 100.54 | 124.04 |
|  | S11 | 22 | 41200.67 | 42.57 | 41782.11 | 45.14 | 101.41 | 106.05 |
|  | RaceHorses | 27 | 36241.49 | 39.44 | 36754.72 | 41.62 | 101.42 | 105.54 |
|  |  | 32 | 32838.88 | 36.65 | 33258.95 | 39.34 | 101.28 | 107.34 |
|  |  | 37 | 30453.27 | 33.67 | 30487.67 | 36.89 | 100.11 | 109.56 |
| Class D | S12 | 22 | 13693.28 | 21.26 | 13917.66 | 22.51 | 101.64 | 105.92 |
| WQVGA | BasketballPass | 27 | 12495.02 | 19.70 | 12616.39 | 21.40 | 100.97 | 108.62 |
|  |  | 32 | 11498.08 | 18.79 | 11610.33 | 20.32 | 100.98 | 108.10 |
|  |  | 37 | 10825.63 | 18.09 | 10851.71 | 19.92 | 100.24 | 110.13 |
|  | S13 | 22 | 12472.28 | 26.53 | 12575.87 | 29.89 | 100.83 | 112.67 |
|  | BQSquare | 27 | 10208.90 | 25.19 | 10261.85 | 28.77 | 100.52 | 114.19 |
|  |  | 32 | 9410.61 | 23.62 | 9453.26 | 27.62 | 100.45 | 116.90 |
|  |  | 37 | 8971.85 | 21.82 | 8959.39 | 26.19 | 99.86 | 120.01 |
|  | S14 | 22 | 10493.71 | 21.94 | 10518.20 | 24.19 | 100.23 | 110.27 |
|  | BlowingBubbles | 27 | 9040.27 | 20.19 | 9103.22 | 22.76 | 100.70 | 112.71 |
|  |  | 32 | 8160.65 | 18.29 | 8850.86 | 21.53 | 108.46 | 117.73 |
|  |  | 37 | 7719.34 | 17.52 | 7782.54 | 20.33 | 100.82 | 116.00 |
|  | S15 | 22 | 9087.91 | 13.68 | 9187.10 | 14.32 | 101.09 | 104.70 |
|  | RaceHorses | 27 | 8181.06 | 12.83 | 8213.09 | 13.52 | 100.39 | 105.44 |
|  |  | 32 | 7355.31 | 11.80 | 7452.17 | 12.71 | 101.32 | 107.75 |
|  |  | 37 | 6938.38 | 11.26 | 6915.44 | 12.61 | 99.67 | 112.03 |
|  | Average Class A |  | 98507.05 | 114.57 | 98539.54 | 126.79 | 99.94 | 110.98 |
|  | Average Class B |  | 142007.23 | 167.82 | 141686.33 | 195.54 | 99.71 | 116.32 |
|  | Average Class C |  | 42617.15 | 56.15 | 42761.49 | 63.04 | 100.39 | 111.69 |
|  | Average Class D |  | 9784.52 | 18.91 | 9891.82 | 21.16 | 101.14 | 111.45 |
|  | Average all |  | 74443.80 | 91.23 | 74408.27 | 104.54 | 100.30 | 113.07 |

The derivation process of direct vector with our proposed technique requires doubled memory access to reference pictures to obtain all prediction block candidates and computation of two SAD calculations per PU at decoder-side, relative to default condition. Our proposed technique requires derivation of the temporal co-located vector and the spatial neighboring vector, which does not have any additional impact to memory size required for motion vector storage if direct vector derivation process relies on temporal motion vector reference.

# Conclusion

Based on the observed results, it is suggested to review the result of the proposal with the JCT-VC committee, and to make recommendation to adopt the proposed technologies onto the HEVC Test Model under Consideration (TMuC) software and the test model (TM) software for further investigation.

The TE partners also suggest the formation of an AhG on decoder-side parameter derivation in which the TE on DMVD and other activities are coordinated.

# References

- Yi-Jen Chiu, Haoping Yu, Yu-Wen Huang, and Mathias Wien, “Tool Experiment 1: Decoder-Side Motion Vector Derivation”, Meeting Document JCTVC-B301, Geneva, Switzerland, July 2010.

# Annex: Participants List

| Participant | Person, Email address |  |
| --- | --- | --- |
| Hitachi | Tomokazu Murakami, tomokazu.murakami.xr@hitachi.com |  |
| Huawei | Haoping Yu, haopingyu@huawei.com Mingyuan Yang, myyang@huawei.com Sixin Lin, linsx@huawei.com |  |
| Intel | Yi-Jen Chiu, yi-jen.chiu@intel.com Lidong Xu, lidong.xu@intel.com Wenhao Zhang, wenhao.zhang@intel.com |  |
| JVC | Satoru Sakazume, sakazume.satoru@jk-holdings.com Shigeru Fukushima, fukushima.shigeru@jk-holdings.com Motoharu Ueda, ueda.motoharu@jk-holdings.com |  |
| Leibniz Univ. Hannover | Sven Klomp, klomp@tnt.uni-hannover.de Holger Meuel, meuel@tnt.uni-hannover.de |  |
| MediaTek | Shawmin Lei, shawmin.lei@mediatek.com Yu-Wen Huang, yuwen.huang@mediatek.com Chih-Wei Hsu, cw.hsu@mediatek.com Ching-Yeh Chen, chingyeh.chen@mediatek.com Jian-Liang Lin, jl.lin@mediatek.com |  |
| Mitsubishi Electric Corp. | Shun-ichi Sekiguchi, Sekiguchi.Shunichi@eb.MitsubishiElectric.co.jp Yusuke Itani, Itani.Yusuke@dx.MitsubishiElectric.co.jp |  |
| Peking University | Yue Wang, wangyue@jdl.ac.cn Siwei Ma, swma@jdl.ac.cn Li Zhang, zhanglili@jdl.ac.cn Wen Gao, wgao@pku.edu.cn |  |
| Qualcomm | Rajan Joshi, rajanj@qualcomm.com Wei-Jung Chien, wchien@qualcomm.com |  |
| RWTH Aachen University | Mathias Wien, wien@ient.rwth-aachen.de |  |
| Samsung | Woo-Jin Han, wjhan.han@samsung.com |  |
| Skype Technologies SA | Lazar Bivolarski, lazar.bivolarsky@skype.net |  |
| Technische Univ. Berlin | Alexander Glantz, glantz@nue.tu-berlin.de Andreas Krutz, krutz@nue.tu-berlin.de Michael Tok, tok@nue.tu-berlin.de |  |
| Zhejiang University | Lu Yu, yul@zju.edu.cn Shangwen Li, dylanwenster@gmail.com Binbin Yu, zjuybb@zju.edu.cn |  |