| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-14 October, 2010 | Document: JCTVC-C119 WG11 Number: m18142 |
| --- | --- |

| Title: | TE12.2 report on MV prediction AMVP/IMVP by Mitsubishi |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kazuo Sugimoto Akira Minezawa Shun-ichi Sekiguchi Information Technology R&D Center, Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-467-41-2463 Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

The purpose of this document is to report the result of tool experiment on MV prediction according to the conditions specified in JCTVC-B312[1].

# Verification

Simulation results has been generated by exercising turning on IMVP instead of AMVP in the four inter-coding configurations specified for TMuC 0.7 in JCTVC-B00[2]. We have used TMuC version 0.7 software, and simulation has been done by using various Linux x86_64 workstations typically having Intel Xeon processor around 3GHz clock. For each Class, we have used the same workstations so that time measurement can be consistent, and we measured the encoding/decoding time using “time” command of Linux system.

# Results

We evaluated PSNR as well as encoding/decoding time for each sequences, and the summary of the results are shown below. More detailed results are also shown in the attached excel file.

The tested is the TMuC version 0.7 with IMVP enabled by setting “IMP : 1” in configuration files and setting the Macro “HHI_AMVP_OFF” as “1” to disable AMVP.

- When the BD-rate is negative, it means IMVP improves coding efficiency.

- When the BD-rate is positive, it means IMVP decreases coding efficiency.

- When the encoding/decoding time is larger than 100%, it means IMVP slow the processing speed.

- When the encoding/decoding time is smaller than 100%, it means IMVP accelerate the processing speed.

According to the tables, we can see that IMVP decreases coding performance smaller than 1 % for each condition. The encoding time with IMVP is slightly slower than that of Anchor settings for each condition. The decoding time is almost the same for all conditions.

# Conclusion

It is observed that IMVP decreases coding performance while encoding time is slightly increased compared to AMVP. The performance loss is more than 1% for Class A sequences.

# References

| [1] | Ken McCann, “Tool Experiment 12: Evaluation of TMuC Tools,” Doc. JCTVC-B312, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, CH, July 2010 |
| --- | --- |
| [2] | Frank Bossen, “Common test conditions and software reference configurations,” Doc. JCTVC-B300, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, CH, July 2010 |