| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C121 WG11 Number: m18144 |
| --- | --- |

| Title: | Suggested approach toward HEVC Test Model creation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shun-ichi Sekiguchi (Mitsubishi) Kazuo Sugimoto (Mitsubishi) | Tel: Email: | +81-467-41-2463 Sekiguchi.Shunichi@eb.MitsubishiElectric.co.jp |
|  | Shawmin Lei (MediaTek) | Tel: Email: | +886 3 567 0766 ext.25555 shawmin.lei@mediatek.com |
|  | Krit Panusopone (Motorola) Ajay Luthra (Motorola) | Tel: Email: Tel: Email: | +1 858 404 3618 krit@motorola.com +1 858 404 3470 aluthra@motorola.com |
|  | Keiichi Chono (NEC) | Tel: Email: | +81 50 431 7595 chono@ct.jp.nec.com |
|  | Atsuro Ichigaya (NHK) | Tel: Email: | +81 3 5494 3350 ichigaya.a-go@nhk.or.jp |
|  | Teruhiko Suzuki (Sony) | Tel: Email: | +81-3-5435-3308 teruhikos@jp.sony.com |
|  | Minhua Zhou (Texas Instruments Inc.) | Tel: Email: | + 214 480 3816 zhou@ti.com |
| Source: | Mitsubishi Electric Corporation, MediaTek, Motorola, NEC, NHK, Sony, Texas Instruments |  |  |

_____________________________

# Abstract

This joint contribution proposes a suggested approach toward definition of HEVC Test Model (HM) and addresses some software related issues, which needs to be considered in test model discussions based on all TE reports.

# Introduction

As the result of test model discussion in the last Geneva meeting, the group agreed to define HEVC Test Model (HM) with only the minimum set of well-tested tools that together form a coherent design that is confirmed to show good capability [1]. Given this agreement, this joint contribution proposes a suggested approach toward definition of HM based on contributions available at Guangzhou meeting. Significant efforts on TE12 will be discussed at the Guangzhou meeting to identify how much coding efficiency and complexity can be seen per each TMuC tool. And results of other established TEs should also be discussed for evaluation of other tools those are currently not included into TMuC. However, these tool basis performance reports will be assessments of each individual tool relative to B300 condition and it is still difficult to identify what is the best minimum combination of available tools. Meanwhile, there are several contributions each of those reports performance evaluation of candidate tool set [2-5]. It can be found that each tool set evaluated in these separate contributions has some sort of overlap on common core set of coding tools, which means that those core tools have intensively been validated as tool combination via cross-verification by multiple parties.

This joint contribution recommends the group to review these tool set basis performance evaluations, to identify a base structure of HM first, and to define HM by adding other individual tools those would be identified as efficient to the base structure. This contribution also addresses some software related issues, which needs to be considered in test model discussions based on all TE reports.

# List of contributions on tool set basis performance evaluation

The following contributions on tool set basis performance evaluation are subject to the discussion made in this joint contribution.

- JCTVC-C065 “Testing results on the simplified TMuC configurations “ [2]

- JCTVC-C066 “Cross verification of Mitsubishi 0.7.4 configurations” [3]

- JCTVC-C122 “A suggested tool set for starting point of HEVC Test Model” [4]

- JCTVC-C269 “A study on the impact of intra smoothing” [5]

# TMuC configuration for core set of coding tools

The tool sets evaluated in the contributions listed in the previous section have high degree of overlap, which can be called “core set of coding tools”. The following tables show required TMuC configurations to achieve the core set of coding tools by comparing the configuration for TE12 anchor defined in B300 condition. We believe that this core set can follow “one tool per one functionality” concept, which is ideal shape of video coding standard and thus is relevant to form a base structure of HM.

Note: It is further recommended to make careful assessments on MV prediction scheme and adaptive scanning part of transform coefficient coding by reviewing relevant contributions [7-10], in terms of performance/complexity trade-off.

Table 1. Configurations for High-Efficiency Condition

| Tool name | TMuC default setting (JCTVC-B300) | Proposed setting | Parameters for proposed setting |
| --- | --- | --- | --- |
| Coding unit | Min size: 8x8 Max size: 64x64 | Same as B300 | Same as B300 |
| Transform unit | Quadtree | Samsung based | QuadTreeTUFlag : 0 |
| Large transform | Max size: 64 | Max size: 32 | MaxTrSize : 32 |
| Angular Intra Prediction | I64: 5 modes I32: 34 modes I16: 34 modes I8: 34 modes I4: 17 modes I2: 3 modes | I64: 5 modes I32: 17 modes I16: 17 modes I8: 17 modes I4: 17 modes I2: 3 modes | #if UNIFIED_DIRECTIONAL_INTRA const UChar g_aucIntraModeNumAng[7] = { 3, // 2x2 17, // 4x4 17, // 8x8 17, // 16x16 17, // 32x32 5, // 64x64 5 // 128x128 }; const UChar g_aucIntraModeBitsAng[7] = { 2, // 2x2 5, // 4x4 5, // 8x8 5, // 16x16 5, // 32x32 3, // 64x64 3 // 128x128 }; |
| Rotational transform (ROT) | Enabled | Disabled | ROT : 0 |
| Mode depend. directional transform (MDDT) | Enabled | Disabled | #define QC_MDDT 0 |
| Adaptive Intra Smoothing (AIS) on/off | Enabled | Disabled | AIS : 0 |
| Combined Intra Prediction (CIP) | Enabled | Disabled | CIP : 0 |
| Planar prediction | Enabled | Disabled | #define PLANAR_INTRA 0 |
| Edge-based prediction | Enabled | Disabled | EdgePredictionEnable : 0 |
| Deblocking filter | Enabled | Same as B300 | Same as B300 |
| Adaptive loop filter (ALF) | Diamond | Same as B300 | Same as B300 |
| Transform coefficient coding | HHI | Same as B300 | Same as B300 |
| Internal Bit-Depth Increase (IBDI) | 4 bits | Same as B300 | Same as B300 |
| Rate Distortion Optimized Quantization (RDOQ) | On | Same as B300 | Same as B300 |
| CABAC-based RDO | On | Same as B300 | Same as B300 |
| Asymmetric Motion Prediction Unit (AMP) | Enabled | Disabled | AMP: 0 |
| Block Merging (MRG) | Enabled | Disabled | MRG: 0 |
| Motion vector prediction (AMVP/IMVP) | AMVP method | Same as B300 | Same as B300 |
| Adaptive motion vector resolution (AMVRES) | Enabled | Disabled | AMVRES : 0 |
| Transform Skip Flag | Disabled | Same as B300 | Same as B300 |
| PU-Based Merging | Disabled | Same as B300 | Same as B300 |
| Partition-based intensity compensation | Disabled | Same as B300 | Same as B300 |
| Transform precision extension | Enabled | Disabled | undefine TRANS_PRECISION_EXT |

Table 2. Configurations for Low-Complexity Condition

(only the difference from High Efficiency Settings are listed)

| Tool name | TMuC default setting (JCTVC-B300) | Proposed setting | Parameters for proposed setting |
| --- | --- | --- | --- |
| Adaptive loop filter (ALF) | Disabled | Same as B300 | Same as B300 |
| Entropy coder | LCEC | Same as B300 | Same as B300 |
| Internal Bit-Depth Increase (IBDI) | Off | Same as B300 | Same as B300 |
| CABAC-based RDO | Off | Same as B300 | Same as B300 |
| Adaptive motion vector resolution (AMVRES) | Disabled | Same as B300 | Same as B300 |
| LCEC Phase 2 | Disabled | Enabled | #define LCEC_PHASE2 (1) |
| Transform precision extension | Enabled | Same as B300 | Same as B300 |
| Shift bits for transform precision extension | 8x8: 7 bit 16x16: 6 bit 32x32: 5 bit 64x64: 4 bit | 8x8: 4 bit 16x16: 4 bit 32x32: 4 bit 64x64: 4 bit | const int g_iShift8x8 = 4; const int g_iShift16x16 = 4; const int g_iShift32x32 = 4; const int g_iShift64x64 = 4; |

# Software issues to be considered in test model discussion

It has been found that the current TMuC software has various unclear implementations listed below, which should affect performance result to be presented in all TE reports. We propose to consider these issues carefully when the group reviews TE reports.

- When AMVRES is turned on, which is the default setting of B300, it automatically turns on QC_AMVRES_LOW_COMPLEXITY macro. This means that a specific 8-tap interpolation filter, which has not been documented into TMuC, is always applied for 1/8-pel generation. When evaluating a single MC interpolation filter combined with AMVRES feature, it should be noted that adaptive interpolation filter is invoked depending on MV resolution.

- When the macro DEFAULT_IS is turned on, which is the default setting of B300, it always applies (1 2 1) low-pass filtering to reference samples for intra prediction. The performance of this default setting is worse than the case when DEFAULT_IS is turned off.  ( see [4,5] for more information)

- The default setting of RDO-Q process for low-complexity condition is unstable. ( see [6] for more information)

- ROUNDING_CONTROL_BIPRED is activated as default in the B300 condition and complex RDO based rounding offset decision is implemented in the current software. However, this feature has not been tested at all.

# Suggested approach for test model creation

- To review all TE reports considering software related issues as discussed in section 4 of this document

- To review all tool set basis performance evaluation reports available at this meeting, such as the ones listed in section 2 of this document

- To define a base structure of HM first that follows “one tool per one functionality”

- To define HM by adding only the currently available tools, which will be evaluated by TEs and provides sufficient coding gain at agreeable complexity, to the base structure of HM

References:

| [1] | Gary J.Sullivan and Jens-Rainer Ohm, “Report of 2nd JCT-VC Geneva meeting,” Doc. JCTVC-B200, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, CH, July 2010 |
| --- | --- |
| [2] | Minhua Zhou, “Testing results on the simplified TMuC configurations,” Doc. JCTVC-C065, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Guangzhou, CN, October 2010 |
| [3] | Minhua Zhou, “Cross verification of Mitsubishi 0.7.4 configurations,” Doc. JCTVC-C066, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Guangzhou, CN, October 2010 |
| [4] | Kazuo Sugimoto, et.al., “A suggested starting point for HEVC Test Model,” Doc. JCTVC-C122, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Guangzhou, CN, October 2010 |
| [5] | Ali Tabatabai, et.al., “A study on the impact of intra smoothing,” Doc. JCTVC-C269, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Guangzhou, CN, October 2010 |
| [6] | Kazuo Sugimoto, et.al., “On selecting coding tools for HEVC Test Model l,” Doc. JCTVC-C123, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Guangzhou, CN, October 2010 |
| [7] | Yi-Jen Chiu, et.al., “TE1: Summary report of TE1 activity,” Doc. JCTVC-C115, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Guangzhou, CN, October 2010 |
| [8] | Kazuo Sugimoto, et.al., “TE12.2 report on MV prediction AMVP/IMVP by Mitsubishi,” Doc. JCTVC-C119, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Guangzhou, CN, October 2010 |
| [9] | Joel. Jung, “TE11: Summary report for TE11 on motion vector coding,” Doc. JCTVC-C293, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Guangzhou, CN, October 2010 |
| [10] | J. Lou, “Zigzag scan for CABAC/PIPE,” Doc. JCTVC-C114, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Guangzhou, CN, October 2010 |