| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C122 WG11 Number: m18145 |
| --- | --- |

| Title: | A suggested starting point for HEVC Test Model |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazuo Sugimoto, Shun-ichi Sekiguchi (Mitsubishi) | Tel: Email: | +81-467-41-2463 Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp |
|  | Shawmin Lei (MediaTek) | Tel: Email: | +886 3 567 0766 ext.25555 shawmin.lei@mediatek.com |
|  | Keiichi Chono (NEC) | Tel: Email: | +81 50 431 7595 chono@ct.jp.nec.com |
|  | Atsuro Ichigaya (NHK) | Tel: Email: | +81 3 5494 3354 ichigaya.a-go@nhk.or.jp |
|  | Teruhiko Suzuki (Sony) | Tel: Email: | +81-3-5435-3308 teruhikos@jp.sony.com |
|  | Minhua Zhou (Texas Instruments Inc.) | Tel: Email: | + 214 480 3816 zhou@ti.com |
| Source: | Mitsubishi Electric Corporation, MediaTek, NEC, NHK, Sony, Texas Instruments |  |  |

_____________________________

# Abstract

This contribution is to propose a suggested set of TMuC coding tools for both high-efficiency and low-complexity conditions, as a starting point of HEVC Test Model (HM) with experimental data jointly verified by multiple companies. The proposed toolset follows “one tool per one functionality” concept and achieves reasonable coding efficiency relative to TMuC anchor used for TE12 assessment, while reducing encoding and decoding time significantly. It is recommended to construct a base structure of HM based on the suggested toolset and consider further additions of other tools via careful assessment of all TE results for creation of HM.

# Introduction

As the result of test model discussion in the last Geneva meeting, the group agreed to define HEVC Test Model (HM) with only the minimum set of well-tested tools that together form a coherent design that is confirmed to show good capability [1]. Given this agreement, we provide performance verification results of a suggested tool set that should fulfill the requirement of HM’s starting point based on the one discussed in JCTVC-B122 [2]. This tool set follows “one tool per one functionality” concept and shows reasonable coding performance relative to TMuC anchor used for TE12 assessment that follows B300 configuration [3], while reducing encoding and decoding time significantly. In summary, for high-efficiency inter coding condition, coding gain around 4% less than the TE12 anchor while reducing encoding and decoding time around 65%. For low-complexity inter coding condition, gain is around 3% higher than an anchor, which improves the one of TE12 low-complexity condition, while reducing encoding time around 30% and make decoding time to be around 1/5.

# TMuC configurations for the suggested toolsets

The following tables show required TMuC configurations to achieve the suggested toolsets by comparing the configuration defined for TE12 anchor. The different settings from B300 are mainly coming from choosing one tool per each functionality, and we apply selection criteria such that:

- The tool was commonly used in many CfP submissions.

- Performance of the tool should be promising in terms of experience in conventional standards, related exploration studies such as KTA, and evaluation of CfP submissions.

- Specification of the tool should be as simple as possible.

Table 1. Configurations for High-Efficiency Condition

| Tool name | TMuC default setting (JCTVC-B300) | Proposed setting | Parameters for proposed setting |
| --- | --- | --- | --- |
| Coding unit | Min size: 8x8 Max size: 64x64 | Same as B300 | Same as B300 |
| Transform unit | Quadtree | Samsung based | QuadTreeTUFlag : 0 |
| Large transform | Max size: 64 | Max size: 32 | QuadTreeTULog2MaxSize : 5 MaxTrSize : 32 |
| Angular Intra Prediction | I64: 5 modes I32: 34 modes I16: 34 modes I8: 34 modes I4: 17 modes I2: 3 modes | I64: 5 modes I32: 17 modes I16: 17 modes I8: 17 modes I4: 17 modes I2: 3 modes | #if UNIFIED_DIRECTIONAL_INTRA const UChar g_aucIntraModeNumAng[7] = { 3, // 2x2 17, // 4x4 17, // 8x8 17, // 16x16 17, // 32x32 5, // 64x64 5 // 128x128 }; const UChar g_aucIntraModeBitsAng[7] = { 2, // 2x2 5, // 4x4 5, // 8x8 5, // 16x16 5, // 32x32 3, // 64x64 3 // 128x128 }; |
| Rotational transform (ROT) | Enabled | Disabled | ROT : 0 |
| Mode depend. directional transform (MDDT) | Enabled | Disabled | #define QC_MDDT 0 |
| Adaptive Intra Smoothing (AIS) on/off | Enabled | Disabled | AIS : 0 |
| Combined Intra Prediction (CIP) | Enabled | Disabled | CIP : 0 |
| Planar prediction | Enabled | Disabled | #define PLANAR_INTRA 0 |
| Edge-based prediction | Enabled | Disabled | EdgePredictionEnable : 0 |
| Deblocking filter | Enabled | Same as B300 | Same as B300 |
| Adaptive loop filter (ALF) | Diamond | Same as B300 | Same as B300 |
| Entropy coder | PIPE | Same as B300 | Same as B300 |
| Transform coefficient coding | HHI | Same as B300 | Same as B300 |
| Internal Bit-Depth Increase (IBDI) | 4 bits | Same as B300 | Same as B300 |
| Rate Distortion Optimized Quantization (RDOQ) | On | Same as B300 | Same as B300 |
| CABAC-based RDO | On | Same as B300 | Same as B300 |
| Asymmetric Motion Prediction Unit (AMP) | Enabled | Disabled | AMP: 0 |
| Block Merging (MRG) | Enabled | Disabled | MRG: 0 |
| Motion vector prediction (AMVP/IMVP) | AMVP method | Same as B300 | Same as B300 |
| Interpolation filter | SIFO | DCT-IF 12 tap | InterpFilterType: 0 DIFTap: 12 |
| Adaptive motion vector resolution (AMVRES) | Enabled | Disabled | AMVRES : 0 |
| Transform Skip Flag | Disabled | Same as B300 | Same as B300 |
| PU-Based Merging | Disabled | Same as B300 | Same as B300 |
| Partition-based intensity compensation | Disabled | Same as B300 | Same as B300 |
| Transform precision extension | Enabled | Disabled | undefine TRANS_PRECISION_EXT |

Table 2. Configurations for Low-Complexity Condition

(only the difference from High Efficiency Settings are listed)

| Tool name | TMuC default setting (JCTVC-B300) | Proposed setting | Parameters for proposed setting |
| --- | --- | --- | --- |
| Adaptive loop filter (ALF) | Disabled | Same as B300 | Same as B300 |
| Entropy coder | LCEC | Same as B300 | Same as B300 |
| Internal Bit-Depth Increase (IBDI) | Off | Same as B300 | Same as B300 |
| CABAC-based RDO | Off | Same as B300 | Same as B300 |
| Interpolation filter | DIF | DCT-IF 6 tap | InterpFilterType: 0 DIFTap: 6 |
| Adaptive motion vector resolution (AMVRES) | Disabled | Same as B300 | Same as B300 |
| LCEC Phase 2 | Disabled | Enabled | #define LCEC_PHASE2 (1) |
| Transform precision extension | Enabled | Same as B300 | Same as B300 |
| Shift bits for transform precision extension | 8x8: 7 bit 16x16: 6 bit 32x32: 5 bit 64x64: 4 bit | 8x8: 4 bit 16x16: 4 bit 32x32: 4 bit 64x64: 4 bit | const int g_iShift8x8 = 4; const int g_iShift16x16 = 4; const int g_iShift32x32 = 4; const int g_iShift64x64 = 4; |

# Performance Verification

All signed companies have conducted cross-verification experiments with the configurations specified in the previous section. All simulation conditions other than the changes to the TMuC configuration as shown in Table 1 and 2 are the same as those defined in JCTVC-B300 [3] with one exception for low-complexity condition. Regarding the LCEC, it was found that TMuC before version 0.7.4 has not optimally been implemented especially when MDDT is turned off. Thus, we used TMuC 0.7.4 with LCEC_PHASE2 Macro enabled for both the anchor and the suggested tool set. Readers can obtain the proposed settings by overwriting the attached modified source code and configuration files on TMuC 0.7.4 software.

Additionally, some of co-author companies have conducted cross-verification experiments using CABAC instead of PIPE with the same configurations for other settings.

The simulation results with PIPE for high efficiency conditions and LCEC for low complexity conditions are shown in Table 3, and the results with CABAC for high efficiency conditions are shown in Table 4. According to these results, it is observed that the suggested tool set provides good coding performance such as around 4% BD-rate degradation for high-efficiency inter coding conditions relative to the TE12 anchor while reducing encoding/decoding time almost one-third of the anchor. For low-complexity case, it provides better coding performance while reducing encoding time, while decoding time is less than one-fifth of the one of the corresponding improved anchor as specified above.

Table 3. Experimental results with PIPE/LCEC

Table 4. Experimental results with CABAC

Some of co-author companies have also conducted two additional cross-verification experiments using the proposed settings as follows:

  - Performance verification with "DEFAULT_IS" to 0

With adaptive intra smoothing (AIS) disabled, low-pass filter is always applied to predictor in default configurations which degrades coding performance. By setting DEFAULT_IS to 0, such default low-pass filtering is disabled for all intra predictions.

Some of co-author companies conducted the performance verification tests using the proposed base settings with “DEFAULT_IS” set to 0, and the test results are shown in Table 5.

According to these results, it is observed that setting “DEFAULT_IS” to 0 improves the performance around 2% for intra only conditions, and around 0.5% for inter coding conditions with the almost the same encoding/decoding time.

Table 5. Experimental results with “DEFAULT_IS  0”

  - Performance verification on SHV test sequences

Some of co-author companies conducted the performance verification tests using the SHV test sequences using the proposed base settings. We used 300 frames of each “Nebuta festival “ and “Steam locomotive train” sequence[4] cropped to Class A size[5], and the test results are shown in Table 6. According to the results, the performance with the proposed settings provides only 2.5-4.5% performance loss from B300 anchor while encoding/decoding time are significantly decreased.

Table 6. Experimental results on SHV sequences

Conclusion

The performance results on a suggested set of TMuC tools as the starting point of HEVC Test Model (HM) were reported in this contribution, which have been cross-verified by multiple co-author companies. This tool set follows “one tool per one functionality” concept and shows reasonable coding performance relative to the anchor used for TE12 assessment, while reducing encoding and decoding time significantly. It is recommended to take the suggested toolset as a base structure of HM and consider further additions of other tools via careful assessment of all TE results for creation of HM.

References:

| [1] | Gary J.Sullivan and Jens-Rainer Ohm, “Report of 2nd JCT-VC Geneva meeting”, Doc. JCTVC-B200, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, CH, July 2010 |
| --- | --- |
| [2] | Minhua Zhou, et. al., “Proposed Approach on Test Model Development”, Doc. JCTVC-B122, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, CH, July 2010 |
| [3] | Frank Bossen, “Common test conditions and software reference configurations”, Doc. JCTVC-B300, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, CH, July 2010 |
| [4] | Shinichi Sakaida, et. al., “7680 x 4320 format test sequences for JCT-VC”, Doc. JCTVC-A023, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC M PEG, Dresden, DE, April 2010 |
| [5] | Kazuhisa Iguchi, et. al., “Performance report of TMuC for Super Hi-Vision”, Doc. JCTVC-C055, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Guangzhou, CN, October 2010 |