| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C127 WG11 Number: m18151 |
| --- | --- |

| Title: | TE1: Report of self derivation of motion estimation improvement in TMuC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Jen Chiu 2200 Mission College Blvd. Santa Clara CA USA 95054 | Tel: Email: | 408-765-9494 yi-jen.chiu@intel.com |
|  | Lidong Xu 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171425 lidong.xu@intel.com |
|  | Wenhao Zhang 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171657 wenhao.zhang@intel.com |
|  | Hong Jiang 2200 Mission College Blvd. Santa Clara CA USA 95054 | Tel: Email: | 916-356-0457 hong.h.jiang@intel.com |
| Source: | Intel Corp. |  |  |

_____________________________

# Abstract

This contribution presents the techniques of decoder-side motion vector derivation (DMVD) side to increase coding efficiency of B pictures. With the motion vector (MV) information self derived at video decoder side, the transmission of these MVs from video encoder side to video decoder side is skipped and thus better coding efficiency can be achieved. The proposed self derivation of motion estimation (SDME) techniques are friendly to the parallel implementation by utilizing on the temporal correlation among the available pixels in the previously-decoded reference pictures, instead of operating on the previously reconstructed pixels of the neighboring area of the current picture, which poses an inherent decoding ordering causal relationship among decoding blocks of current picture. Experiments have demonstrated that an overall 4.8% BD bitrate improvement for the test of random access (high efficiency) category under the TE1 test conditions with 73% increase in encoding time and 150% increase in decoding time increase on TMuC 0.7 Reference Software platform. A fast version of the direct-candidate simplified SDME has demonstrated an overall 3.1% BD bitrate improvement with a surprising 26% reduction in encoding time and 18% increase in decoding time when compared to TE1 anchor. Another version of the rounded-candidate simplified SDME has demonstrated an overall 3.7% BD bitrate improvement with 10% increase in encoding time and 13% increase in decoding time when compared to TE1 anchor. Also, the proposed SDME demonstrates 2.7% BD bitrate improvement over the anchor of TE12 under random access test category (high efficiency) on the shorter version of the common test video sequence defined for TE12. A description about the normative syntax change in the form of TMuC specification is included.

# Introduction

Various decoder-side motion vector derivation (DMVD) techniques have been proposed to improve the coding efficiency of hybrid based video encoder [1-5]. In our response to the HEVC CFP [6], a mirror-based self derivation of motion estimation (SDME) technique was presented to improve the coding efficiency of the bi-prediction modes of B pictures. The fast techniques of candidate-based motion vector prediction were proposed to improve the mirror-based SDME in [7]. This contribution reports the improvement of the fast candidate-based SDME technique in TMuC. The following of this contribution has Section 2 to provide an introduction of the proposed candidates based SDME, and Section 3 to describe the simplifications of SDME and mode decision to reduce the encoder and decoder complexities. A description about the normative syntax change in the form of TMuC specification is included in Section 4. The performance data of the coding efficiency and coding complexity of the proposed scheme are contained in Section 5. Section 6 concludes this contribution.

# Overview of SDME

The candidates based SDME is proposed to improve the coding efficiency of B picture for the next generation of video coding standard. Figure 1 shows the block diagram of a typical hybrid-based video encoder with SDME module being added. The SDME module performs motion search only on reference pictures, and the produced motion vector could be applied to current target prediction unit (PU) for motion compensation when the best RDO performance is achieved with SDME mode during the phase of coding mode selection. On the video decoder side, the decoder performs the identical SDME operation to produce the motion compensated predictions for the target PU when the said target PU is encoded in SDME mode.

Figure 1 - Block diagram of hybrid video encoding with SDME being supported

## Candidates based SDME

In [7], candidates based SDME was proposed to reduce the complexity of video encoder and video decoder, in which, the SDME module first checks 9 candidate motion vector pairs, and then perform a small range fractional pixel refinement ME around the best candidate motion vector pair. This contribution reduced the number of candidates from 9 to 6 to accommodate the new coding structure in TMuC reference software. The 6 candidates are selected from the MVs of spatial and temporal neighboring PUs, as depicted in Table 1 and Figure 2, and one candidate from the collocated PUs. All candidate MVs are scaled (by POC distances) to the nearest forward and backward reference pictures. Each candidate consists of one forward MV and one backward MV. Since the proposed SDME is performed between the two reference pictures, the decoding process does not have dependency on the neighbor PU reconstructions.

Figure 2 – Candidate MVs from spatial neighboring PUs

Table 1 - Description of the candidates

| Base MV | 0 | the zero MVs |
| --- | --- | --- |
| Spatial neighbor MVs scaled by POC distances | 1 | MVs of left neighbor PU A: a. If it is bi-predicted, use the two MVs b. If it is FW predicted, get the BW MV by mirroring the FW MV c. If it is BW predicted, get the FW MV by mirroring the BW MV d. If it is intra coded, this candidate will be unavailable |
|  | 2 | MVs of top neighbor PU B a. If it is bi-predicted, use the two MVs b. If it is FW predicted, get the BW MV by mirroring the FW MV c. If it is BW predicted, get the FW MV by mirroring the BW MV d. If it is intra coded, this candidate will be unavailable |
|  | 3 | MVs of top-right neighbor PU C (as described in 1) - If PU C is not available, use that of bottom left PU E - If PU E is also not available, use that of top-left PU D - If PU D is also not available, this candidate will be unavailable |
|  | 4 | Mdian filtered MVs of the above three candidates 1, 2 and 3 |
| Temporal neighbor MVs scaled by POC distances | 5 | MVs of collocated neighbor PU in BW reference a. If it is bi-predicted, use the two MVs b. If it is FW predicted, get the BW MV by mirroring the FW MV c. If it is BW predicted, get the FW MV by mirroring the BW MV d. If it is intra coded, this candidate will be unavailable |

A two-step refining ME is performed around the best candidate. First, the eight half-pel positions with the offsets (-2, -2)/4, (-2, 0)/4, (-2, 2) /4, (0, -2) /4, (0, 2) /4, (2, -2) /4, (2, 0) /4 and (2, 2) /4 to the best candidate are checked, and then eight positions with the offsets (-1, -1)/4, (-1, 0)/4, (-1, 1) /4, (0, -1) /4, (0, 1) /4, (1, -1) /4, (1, 0) /4 and (1, 1) /4 to the best half-pel candidate position are checked.

## SDME based coding modes

The SDME based motion vector derivation method can be used as a new coding mode in addition to the existing coding modes of the regular ME based coding structure. In bitstream syntax, a SDME flag is added for each PU to signal the occurrence of motion vector derivation method if the SDME is available for this PU coding mode. The SDME is applied to the bi-prediction coding modes of all CU and PU sizes. For 2Nx2N PUs, SDME is applied only to Skip/Direct modes. Also, the SDME is only applied for traditional B pictures. (When the coding parameter GPB is set to1, the P pictures will also be coded as B pictures. But SDME is currently not applied for these special B pictures.)

# Simplification of SDME

## Direct-candidate

To reduce the coding complexity, a simplified SDME is proposed by disabling the motion estimation involved in the 2-step refinement of precision for half-pel and quarter-pel. All of the PUs inside a CU shares the same DMVD flag. On the experiment of the common test sequence, the computing time of video encoding is surprisingly reduced by 26%, and the decoding time is significantly reduced from “150% increase” down to “18% increase” with an overall sizeable BD bitrate improvement of 3.1% as shown in Table 3.

## Rounded-candidate

On top of the direct-candidate simplified SDME version to disable the motion estimation involved in the 2-step refinement of precision for half-pel and quarter-pel, a process to round the fractional MV of the predictor to the nearest integer location is proposed to reduce the complexity in preparation of the calculation of the sum of absolute difference (SAD). With this, the SAD is calculated based on the rounded integer-location pixels. The best MV candidate is selected with the smallest SAD obtained from the rounded integer location. The associated fractional MV is in use for the reconstruction phase of motion compensation. Table 4 shows an overall 3.7% BD bitrate improvement is obtained with 10% increase in encoding time and 13% increase in decoding time when compared to TE1 anchor.

# Syntax changes

A new entry for “BiDMVDFlag” is added into the SPS syntax table to signal if the proposed DMVD scheme is applied. And then for each PU, the syntax changes are shown as follows. The part in red is related to the simplified SDME.

| prediction_unit( x0, y0, currPredUnitSize ) { | C | Descriptor |
| --- | --- | --- |
| if( slice_type != I ) |  |  |
| skip_flag |  |  |
| if( skip_flag ) { |  |  |
| if(BiDMVDFlag = 1&& slicetype==B && !slice_is_GPB) |  |  |
| skip_dmvd_flag | 2 | ue(v) ¦ ae(v) |
| if( mv_competition_flag ) { |  |  |
| if( inter_pred_idc != Pred_L1 && NumMVPCand( L0 ) > 1 ) |  |  |
| mvp_idx_l0 | 2 | ue(v) ¦ ae(v) |
| if( inter_pred_idc != Pred_L0 && NumMVPCand( L1 ) > 1 ) |  |  |
| mvp_idx_l1 | 2 | ue(v) ¦ ae(v) |
| } |  |  |
| } |  |  |
| else { |  |  |
| if(!entropy_coding_mode_flag) |  |  |
| mode_table_idx |  | vlc(n,v) |
| else { |  |  |
| if( slice_type != I ) |  |  |
| pred_mode | 2 | u(1) ¦ ae(v) |
| } |  |  |
| if( PredMode == MODE_INTRA ) { |  |  |
| planar_flag | 2 | u(1) ¦ ae(v) |
| if(planar_flag) { |  |  |
| planar_delta_y = getPlanarDelta() |  |  |
| planar_delta_uv_present_flag | 2 | u(1) ¦ ae(v) |
| if(planar_delta_uv_present_flag) { |  |  |
| planar_delta_u = getPlanarDelta() |  |  |
| planar_delta_v = getPlanarDelta() |  |  |
| } |  |  |
| } else { |  |  |
| if(entropy_coding_mode_flag) |  |  |
| intra_split_flag | 2 | ae(v) |
| combined_intra_pred_flag | 2 | u(1) ¦ ae(v) |
| for( i = 0; i < ( intra_split_flag ? 4 : 1 ); i++ ) { |  |  |
| prev_intra_luma_pred_ flag | 2 | u(1) ¦ ae(v) |
| if( !prev_intra_pred_luma_flag ) |  |  |
| rem_intra_luma_pred_mode | 2 | ue(v) ¦ ae(v) |
| if( adaptive_intra_smoothing_flag ) |  |  |
| intra_luma_filt_flag | 2 | u(1) ¦ ae(v) |
| } |  |  |
| } |  |  |
| if( chroma_format_idc != 0 ) |  |  |
| intra_chroma_pred_mode | 2 | ue(v) ¦ ae(v) |
| } |  |  |
| } |  |  |
| else if( PredMode == MODE_INTER ) { |  |  |
| if(entropy_coding_mode_flag) |  |  |
| inter_partitioning_idc | 2 | ue(v) ¦ ae(v) |
| for( i= 0; i < NumPuParts( inter_partitioning_idc ); i++ ) { |  |  |
| if( NumMergeCandidates > 0 ) { |  |  |
| merge_flag[ i ] | 2 | ue(1) ¦ ae(v) |
| if( merge_flag[ i ] && NumMergeCandidates > 1 ) |  |  |
| merge_left_flag[ i ] | 2 | ue(1) ¦ ae(v) |
| } |  |  |
| if( !merge_flag[i] ) { |  |  |
| if(BiDMVDFlag = 1 && slice_type == B && !slice_is_GPB && PU_size != 2Nx2N && is_first_pu_in_cu) |  |  |
| pu_dmvd_flag | 2 | ue(v) ¦ ae(v) |
| if(BiDMVDFlag = 0 ¦¦ !pu_dmvd_flag) { |  |  |
| if( slice_type == B) |  |  |
| inter_pred_idc[ I ] | 2 | ue(v) ¦ ae(v) |
| if( inter_pred_idc[ i ] != Pred_L1 ) { |  |  |
| if( entropy_coding_mode_flag ) { |  |  |
| if( num_ref_idx_l0_active_minus1 > 0) |  |  |
| ref_idx_l0[ i ] | 2 | ue(v) ¦ ae(v) |
| mvres_l0 [ i ] | 2 | ue(1) ¦ ae(v) |
| } |  |  |
| else { |  |  |
| ref_idx_mvres_l0[ i ] | 2 | ue(v) ¦ ae(v) |
| } |  |  |
| mvd_l0[ i ][ 0 ] | 2 | se(v) ¦ ae(v) |
| mvd_l0[ i ][ 1 ] | 2 | se(v) ¦ ae(v) |
| if( mv_competition_flag && NumMVPCand( L0, i ) > 1 ) |  |  |
| mvp_idx_l0[ i ] | 2 | ue(v) ¦ ae(v) |
| } |  |  |
| if( inter_pred_idc[ i ] != Pred_L0 ) { |  |  |
| if( entropy_coding_mode_flag ) { |  |  |
| if( num_ref_idx_l1_active_minus1 > 0) |  |  |
| ref_idx_l1[ i ] | 2 | ue(v) ¦ ae(v) |
| mvres_l1 [ i ] | 2 | ue(1) ¦ ae(v) |
| } |  |  |
| else { |  |  |
| ref_idx_mvres_l1[ i ] | 2 | ue(v) ¦ ae(v) |
| } |  |  |
| mvd_l1[ i ][ 0 ] | 2 | se(v) ¦ ae(v) |
| mvd_l1[ i ][ 1 ] | 2 | se(v) ¦ ae(v) |
| if( mv_competition_flag && NumMVPCand( L1, i ) > 1 ) |  |  |
| mvp_idx_l1[ i ] | 2 | ue(v) ¦ ae(v) |
| } |  |  |
| } |  |  |
| else { |  |  |
| if( mv_competition_flag && NumMVPCand( L0, i ) > 1 ) |  |  |
| mvp_idx_l0[ i ] | 2 | ue(v) ¦ ae(v) |
| if( mv_competition_flag && NumMVPCand( L1, I ) > 1 ) |  |  |
| mvp_idx_l1[ I ] | 2 | ue(v) ¦ ae(v) |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| else if( PredMode == MODE_DIRECT ) { |  |  |
| if( (BiDMVDFlag = 1 && slicetype==B && !slice_is_GPB) |  |  |
| direct_dmvd_flag | 2 | ue(v) ¦ ae(v) |
| if( slice_type == B && ! direct_dmvd_flag) |  |  |
| inter_pred_idc | 2 | ue(v) ¦ ae(v) |
| if( mv_competition_flag ) { |  |  |
| if( inter_pred_idc != Pred_L1 && NumMVPCand( L0 ) > 1 ) |  |  |
| mvp_idx_l0 | 2 | ue(v) ¦ ae(v) |
| if( inter_pred_idc != Pred_L0 && NumMVPCand( L1 ) > 1 ) |  |  |
| mvp_idx_l1 | 2 | ue(v) ¦ ae(v) |
| } |  |  |
| } |  |  |
| } |  |  |

BiDMVDFlag specifies the DMVD method is used in bitstream.

- If 1, DMVD derivation method is enabled. Otherwise, it is disabled.

skip_dmvd_flag specifies the MV derivation method for Skip mode. Default value is 0.

- If 1, DMVD derivation method is used.

- Otherwise TMuC original derivation method is used.

pu_dmvd_flag specifies the MV derivation method for the PU. It is not applied to 2Nx2N PUs. Default value is 0.

- If 1, DMVD derivation method is used.

- Otherwise TMuC original derivation method is used.

direct_dmvd_flag specifies the MV derivation method for Direct mode. Default value is 0.

- If 1, DMVD derivation method is used.

- Otherwise TMuC original derivation method is used.

# Performance

The encoder is complied in Linux OS and is tested in a compute farm consisting of hundred of CPU cores. The computing cores in the system may not be identical, and information of the log of encoding time ratio may produce slightly different result in each occurrence of the simulation. The decoder was tested on a single workstation with the following configurations: WindowXP x64 edition, Intel Xeon 3.16GHz CPU, 16GB RAM, and Visual Studio 2008.

To reduce the simulation time, a shorter version of the common test video sequence is applied for considering only the pictures from the first intra frame to the second intra frame for each testing point. This short version of the common test video sequence is applied to the tests described in Section 5.4.

## Candidates based SDME

The proposed SDME techniques were implemented onto TMuC 0.7 Reference Software and evaluated under the test conditions defined by the TE1 group [8]. The experiment is investigated on the test case of random access (high efficiency) because SDME is for improving the coding efficiency of B picture. Some key testing configurations of TE1 are listed as follows.

- Hierarchical coding structure of IbBbBbBbP

- QPI=22, 27, 32, and 37; QPP=QPI+1; QPB= QPI+2

- TE12 anchor test conditions but MRG = 0 and HHI_AMPV_OFF on

Table 2 shows the SDME performance compared with the TE1 anchors. The BD measurements in bitrate reduction were employed. The average BD bitrate reduction for luma component is about 4.8% with about 73% increase in encoding time and about 150% increase in decoding time.

Table 2 - Candidates based SDME performance compared with TE1 anchors

|  | Sequences | BD-rate Y (%) | BD-rate U (%) | BD-rate V (%) | Enc-time ratio | Dec-time ratio |
| --- | --- | --- | --- | --- | --- | --- |
| Class A | Traffic | -5.6 | -5.1 | -4.9 |  |  |
|  | PeopleOnStreet | -6.0 | -7.1 | -6.9 |  |  |
| Class B | Kimono1 | -5.8 | -4.9 | -4.6 |  |  |
|  | ParkScene | -4.7 | -4.0 | -3.9 |  |  |
|  | Cactus | -7.2 | -6.1 | -6.2 |  |  |
|  | BasketballDrive | -4.4 | -4.3 | -4.4 |  |  |
|  | BQTerrace | -3.1 | -2.6 | -2.5 |  |  |
| Class C | BasketballDrill | -3.3 | -3.8 | -3.9 |  |  |
|  | BQMall | -7.0 | -6.3 | -6.7 |  |  |
|  | PartyScene | -4.2 | -3.8 | -3.8 |  |  |
|  | RaceHorses | -2.2 | -3.0 | -3.4 |  |  |
| Class D | BasketballPass | -5.1 | -5.5 | -5.8 |  |  |
|  | BQSquare | -4.0 | -2.8 | -2.8 |  |  |
|  | BlowingBubbles | -4.7 | -3.6 | -3.9 |  |  |
|  | RaceHorses | -4.2 | -5.1 | -5.2 |  |  |
| Average of Class A |  | -5.8 | -6.1 | -5.9 |  |  |
| Average of Class B |  | -5.1 | -4.4 | -4.3 |  |  |
| Average of Class C |  | -4.1 | -4.2 | -4.4 |  |  |
| Average of Class D |  | -4.5 | -4.2 | -4.4 |  |  |
| Overall Average |  | -4.8 | -4.5 | -4.6 | 173% | 250% |

## Direct-candidate simplified SDME

Table 3 shows the performance of the direct-candidate simplified SDME compared with the TE1 anchors. The average BD bitrate reduction for luma component is about 3.1% with a surprising 26% reduction in encoding time and about 18% increase in decoding time when compared to TE1 anchor.

Table 3 – Direct-candidate Simplified SDME performance compared with TE1 anchors

|  | Sequences | BD-rate Y (%) | BD-rate U (%) | BD-rate V (%) | Enc-time ratio | Dec-time ratio |
| --- | --- | --- | --- | --- | --- | --- |
| Class A | Traffic | -4.1 | -3.9 | -3.9 |  |  |
|  | PeopleOnStreet | -4.1 | -4.9 | -5.0 |  |  |
| Class B | Kimono1 | -4.0 | -3.4 | -3.4 |  |  |
|  | ParkScene | -2.8 | -2.7 | -2.5 |  |  |
|  | Cactus | -5.2 | -4.3 | -4.3 |  |  |
|  | BasketballDrive | -2.8 | -2.7 | -2.9 |  |  |
|  | BQTerrace | -2.1 | -1.9 | -2.2 |  |  |
| Class C | BasketballDrill | -2.0 | -2.7 | -2.6 |  |  |
|  | BQMall | -4.7 | -4.3 | -4.7 |  |  |
|  | PartyScene | -2.6 | -2.6 | -2.5 |  |  |
|  | RaceHorses | -1.0 | -1.8 | -1.8 |  |  |
| Class D | BasketballPass | -3.3 | -3.6 | -3.9 |  |  |
|  | BQSquare | -2.0 | -1.7 | -1.3 |  |  |
|  | BlowingBubbles | -2.9 | -2.5 | -2.8 |  |  |
|  | RaceHorses | -2.4 | -3.1 | -3.2 |  |  |
| Average of Class A |  | -4.1 | -4.4 | -4.5 |  |  |
| Average of Class B |  | -3.4 | -3.0 | -3.1 |  |  |
| Average of Class C |  | -2.6 | -2.8 | -2.9 |  |  |
| Average of Class D |  | -2.6 | -2.7 | -2.8 |  |  |
| Overall Average |  | -3.1 | -3.1 | -3.1 | 74% | 118% |

## Rounded-candidate Simplified SDME

Table 4 shows the performance of the rounded-candidate simplified SDME with the rounded MV as described in Section 3. The average BD bitrate reduction for luma component is about 3.77% with a 103% increase in encoding time and about 130% increase in decoding time when compared to TE1 anchor.

Table 4 – Rounded-candidate Simplified SDME performance compared with TE1 anchors

|  | Sequences | BD-rate Y (%) | BD-rate U (%) | BD-rate V (%) | Enc-time ratio | Dec-time ratio |
| --- | --- | --- | --- | --- | --- | --- |
| Class A | Traffic | -4.8 | -4.4 | -4.3 |  |  |
|  | PeopleOnStreet | -5.0 | -5.9 | -5.9 |  |  |
| Class B | Kimono1 | -4.8 | -4.1 | -4.1 |  |  |
|  | ParkScene | -3.5 | -3.2 | -3.1 |  |  |
|  | Cactus | -5.6 | -4.6 | -4.9 |  |  |
|  | BasketballDrive | -3.5 | -3.5 | -3.7 |  |  |
|  | BQTerrace | -2.8 | -2.1 | -1.9 |  |  |
| Class C | BasketballDrill | -2.6 | -3.0 | -3.3 |  |  |
|  | BQMall | -5.6 | -5.2 | -5.4 |  |  |
|  | PartyScene | -3.1 | -3.1 | -3.1 |  |  |
|  | RaceHorses | -1.6 | -2.3 | -2.7 |  |  |
| Class D | BasketballPass | -4.0 | -4.5 | -4.6 |  |  |
|  | BQSquare | -2.5 | -2.3 | -2.0 |  |  |
|  | BlowingBubbles | -3.4 | -3.0 | -3.4 |  |  |
|  | RaceHorses | -3.1 | -3.8 | -4.1 |  |  |
| Average of Class A |  | -4.9 | -5.2 | -5.1 |  |  |
| Average of Class B |  | -4.0 | -3.5 | -3.5 |  |  |
| Average of Class C |  | -3.2 | -3.4 | -3.6 |  |  |
| Average of Class D |  | -3.2 | -3.4 | -3.5 |  |  |
| Overall Average |  | -3.7 | -3.7 | -3.8 | 110% | 113% |

  - Performance with MRG and AMVP

The common test condition of TE1 disables MRG and AMVP modes (MRG=0, AMVP=0). For reference, the SDME is tested with the different testing settings of MRG and AMVP and the result is listed in Table 5. To reduce the simulation time, a shorter version of the common test sequence is applied for considering only the pictures from the first intra frame to the second intra frame for each testing point. The coding gain of SDME with “MRG = 0 && AMVP off” achieves 4.1% BD bitrate improvement in the first column of Table 4.1 (Referred to 4.8% improvement for the complete test sequence). The coding gain of SDME with “MRG = 1 && AMVP on” is 2.7% BD bitrate improvement which demonstrates SDME improvement over the anchor of TE12. For reference, the experiment of the combined (MRG=1 & AMVP=1) compared to TE1 anchor is investigated and a 2% BD bitrate improvement is listed in the last column of the table. (This is in fact the comparison between the TE12 anchor and TE1 anchor for the test case of Random Access (high efficiency))

The results in Table 5 show that SDME achieved sizeable coding gain (>2.3%) with all kinds of settings of MRG and AMVP for test category of Random Access (high efficiency) on the shorter version of the common test video sequence.

Table 5 – BD-rate Y of SDME under different MRG and AMVP settings

|  | Sequences | MRG = 0 AMVP off | MRG = 0 AMVP on | MRG = 1 AMVP off | MRG = 1 AMVP on | TE12 toTE1 |
| --- | --- | --- | --- | --- | --- | --- |
| Class A | Traffic | -4.0 | -2.1 | -3.4 | -2.5 | -2.5 |
|  | PeopleOnStreet | -5.1 | -3.2 | -4.6 | -3.6 | -2.0 |
| Class B | Kimono1 | -4.2 | -2.0 | -3.1 | -2.3 | -2.4 |
|  | ParkScene | -4.1 | -2.4 | -2.7 | -2.1 | -2.5 |
|  | Cactus | -5.9 | -3.2 | -5.2 | -4.4 | -2.1 |
|  | BasketballDrive | -5.0 | -1.9 | -3.8 | -2.9 | -3.3 |
|  | BQTerrace | -1.7 | -0.7 | -1.4 | -1.1 | -1.7 |
| Class C | BasketballDrill | -3.0 | -2.1 | -2.7 | -2.0 | -1.5 |
|  | BQMall | -5.5 | -1.4 | -4.7 | -3.6 | -2.5 |
|  | PartyScene | -3.3 | -3.3 | -3.1 | -2.4 | -1.3 |
|  | RaceHorses | -2.0 | -2.1 | -1.7 | -1.2 | -1.9 |
| Class D | BasketballPass | -6.1 | -1.1 | -4.5 | -4.1 | -1.6 |
|  | BQSquare | -3.2 | -4.0 | -3.0 | -2.4 | -1.4 |
|  | BlowingBubbles | -4.5 | -2.2 | -3.9 | -3.3 | -1.2 |
|  | RaceHorses | -3.8 | -2.6 | -3.1 | -2.3 | -2.3 |
| Average of Class A |  | -4.6 | -2.7 | -4.0 | -3.0 | -2.3 |
| Average of Class B |  | -4.2 | -2.0 | -3.3 | -2.5 | -2.4 |
| Average of Class C |  | -3.5 | -2.0 | -3.1 | -2.3 | -1.8 |
| Average of Class D |  | -4.4 | -2.8 | -3.6 | -3.1 | -1.6 |
| Overall Average |  | -4.1 | -2.3 | -3.4 | -2.7 | -2.0 |

# Conclusions

This contribution reports the improvement of SDME technique on TMuC 0.7 Reference Software. The proposed candidate-based SDME techniques aim to increase coding efficiency of B pictures. It is friendly to the parallel implementation by utilizing the temporal correlation among the available pixels in the previously-decoded reference pictures, instead of operating on the previously reconstructed pixels of the neighboring area of the current picture, which poses an inherent decoding ordering causal relationship among decoding PUs of current picture. Experiments have demonstrated the BD bitrate improvement on top of TMuC platform with an overall 4.8% improvement on the test case of Random Access (high efficiency) mode under the common TE1 test conditions, while the average encoding time and decoding time are increased by about 73% and 150% respectively. A fast version of the direct-candidate simplified SDME has shown an overall 3.1% improvement in BD bitrate reduction with a surprising 26% reduction in encoding time and about 18% increase in decoding time when compared to TE1 anchor. Another version of the rounded- candidate simplified SDME has demonstrated an overall 3.77% BD bitrate improvement with 10% increase in encoding time and 13% increase in decoding time when compared to TE1 anchor. Also, the proposed SDME achieves 2.7% BD bitrate improvement over the anchor of TE12 for the Random Access test category (high efficiency) on the shorter version of the common test video sequence.

# References

- M. Ueda, S. Fukushima, “TE1: Refinement Motion Compensation using Decoder-side Motion Estimation”, Meeting Document JCTVC-A032, April 2010.

- S. Lin, M. Yang, J. Zhou, and et.al, “TE1: Huawei report on DMVD improvements”, Meeting Document JCTVC-A037, April 2010.

- S. Sekiguchi, Y. Itani, “Performance evaluation on implicit direct vector derivation”, Meeting Document JCTVC-A068, April 2010.

- Y.-W. Huang, C.-Y. Chen, C.-W. Hsu, J.-L. Lin, Y.-P. Tsai, J. An, S. Lei, “TE1: Decoder-side motion vector derivation with switchable template matching”, Meeting Document JCTVC-A076, April 2010.

- W.-J. Chien, M. Karczewicz, P. Chen, “TE1: Decoder-side motion vector derivation report from Qualcomm”, Meeting Document JCTVC-A097, April 2010.

- Y. Chiu, L. Xu, W. Zhang, H. Jiang, “Description of video coding technology proposal: self derivation of motion estimation and adaptive (Wiener) loop filtering”, Meeting Document JCTVC-A106, April 2010.

- Y. Chiu, L. Xu, W. Zhang, H. Jiang, “TE1: Fast techniques to improve self derivation of motion estimation”, Meeting Document JCTVC-B047, July 2010.

- Yi-Jen Chiu, Haoping Yu, Yu-Wen Huang, and Mathias Wien, “Tool Experiment 1: Decoder-Side Motion Vector Derivation”, Meeting Document JCTVC-B301, July 2010.

# Patent rights declaration(s)

(NOTE – Activities in the JCT-VC and contributions to the JCT-VC are subject to the common patent policy for ITU-T/ITU-R/ISO/IEC. A statement of that policy can be found at
http://www.itu.int/ITU-T/dbase/patent/patent-policy.html, with further information available at http://www.itu.int/ITU-T/ipr/index.html and in the ISO/IEC Directives. The form to be used for the formal reporting of patent rights to ITU-T/ITU-R/ISO/IEC can be found at http://www.itu.int/ITU-T/ipr/index.html. Contributions to the JCT-VC proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The provided informal notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form. This obligation to provide an informal notice is supplemental to, and does not replace, any existing obligations of parties with technology included in a final or draft standard to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC. An example of an informal IPR notification statement for a contribution is provided below.)

Intel Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).