| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C134 WG11 Number: m18158 |
| --- | --- |

| Title: | Comments on V2V coding for TM/TMuC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dake He, Gergely Korodi, Paul Imthurn, Joel Jamias, Daniel O’Loughlin, Gaelle Martin-Cocher 175 Columbia St. West Waterloo, Ontario N2L 5Z5 Canada | Tel: Email: | +1-519-888-7465 {dhe, gkorodi, pimthurn, jjamias, doloughlin, gmartincocher}@ rim.com |
| Source: | Research In Motion Ltd. |  |  |

_____________________________

# Abstract

This document provides information about V2V coding to facilitate the discussion of TM/TMuC on therelevant subject. It is shown that the design of a V2V tree directly impacts, not only its compression performance, but its throughput, in both software and hardware implementations. Further evaluation of V2V codes by measuring their compression performance, encoding and decoding throughput, sizes, and power consumption in hardware is recommended.

- Background

V2V coding for HEVC (high efficiency video coding) was proposed in JCTVC-A116 and JCTVC-A120, and subsequently implemented in the TMuC (Test Model under Consideration) [1] reference software. Specifically, by setting symbol_mode to 2, a method called PIPE is selected which makes use of 12 V2V codes for entropy coding. In TE12 [2], the compression performance of PIPE is tested against BAC(binary arithmetic coding) and LCEC (Low-complexity entropy coding). In TE8 [3], PIPE V2V codes (Codeset 1) are evaluated against another set of V2V codes proposed by RIM (Codeset 2) in both software and hard implementations, where the two codesets’ compression efficiency is further measured against encoding and decoding time in software, and against gate counts, throughput in bins/cycle, and power consumption in hardware.

- Tool Experiment Results

## Software

In this section, we review the Bernoulli results collected in TE8. All these results are cross-verified.

The test environment is configured as follows.

CPU: Intel(R) Core(TM) i7 CPU 860@2.80GHz

g++: 4.4.1

Operating System: Linux

The following table summarizes the performance loss of BAC, Codeset 1, and Codeset 2 compared to the entropy in percentages (column “Loss”), processing 1 billion bins per code. The encoding and decoding throughput values show the million bin symbols processed per second.

| Coding method | Loss (%) | Encoding Throughput | Decoding Throughput | Cross-checked |
| --- | --- | --- | --- | --- |
| BAC (JM11/KTA) | 0.159 | 57.7 | 144.4 | Yes |
| BAC (JM 17.2) | 0.159 | 130.7 | 136.1 | No |
| Codeset 1 | 0.147 | 68.7 | 110.4 | Yes |
| Codeset 2 | 0.122 | 222.0 | 251.1 | Yes |

Detailed results are provided below.

BAC JM11/KTA

LPS Prob  Entropy    Samples   Optimal  Average     Loss  Enc Thru  Dec Thru

0.5000000  1.00000  1000000000  1.00000  1.00000    0.000%    52.8     128.5

0.4500000  0.99277  1000000000  0.99278  1.00000    0.727%    61.2     136.9

0.4300000  0.98582  1000000000  0.98582  0.98665    0.084%    44.8     110.1

0.4050000  0.97380  1000000000  0.97381  0.97521    0.144%    45.5     112.9

0.3450000  0.92952  1000000000  0.92953  0.93097    0.154%    47.6     117.4

0.3150000  0.89886  1000000000  0.89888  0.89999    0.124%    47.7     122.1

0.2550000  0.81911  1000000000  0.81912  0.81963    0.062%    54.1     135.1

0.2050000  0.73182  1000000000  0.73182  0.73235    0.072%    52.1     136.6

0.1650000  0.64614  1000000000  0.64616  0.64676    0.094%    58.2     164.0

0.0850000  0.41956  1000000000  0.41955  0.41975    0.047%    75.9     189.3

0.0400000  0.24229  1000000000  0.24231  0.24241    0.042%    96.9     252.0

0.0200000  0.14144  1000000000  0.14145  0.14153    0.055%   123.2     302.0

=============================================================================

=== SUMMARY ===    12000000000  0.73177  0.73294    0.159%    57.7     144.4

BAC JM17.2

LPS Prob  Entropy    Samples   Optimal  Average     Loss  Enc Thru  Dec Thru

0.5000000  1.00000  1000000000  1.00000  1.00000    0.000%   111.1     109.7

0.4500000  0.99277  1000000000  0.99278  1.00000    0.727%   119.6     113.7

0.4300000  0.98582  1000000000  0.98582  0.98665    0.084%   100.9     109.1

0.4050000  0.97380  1000000000  0.97381  0.97521    0.144%   107.1     111.9

0.3450000  0.92952  1000000000  0.92953  0.93097    0.154%   113.2     107.2

0.3150000  0.89886  1000000000  0.89888  0.89999    0.124%   114.3     109.5

0.2550000  0.81911  1000000000  0.81912  0.81963    0.062%   117.9     130.7

0.2050000  0.73182  1000000000  0.73182  0.73235    0.072%   135.3     141.8

0.1650000  0.64614  1000000000  0.64616  0.64676    0.094%   141.9     156.8

0.0850000  0.41956  1000000000  0.41955  0.41975    0.047%   182.6     211.6

0.0400000  0.24229  1000000000  0.24231  0.24241    0.042%   207.1     243.1

0.0200000  0.14144  1000000000  0.14145  0.14153    0.055%   228.6     267.0

=============================================================================

=== SUMMARY ===    12000000000  0.73177  0.73294    0.159%   130.7     136.1

Codeset 1

LPS Prob  Entropy    Samples   Optimal  Average     Loss  Enc Thru  Dec Thru

0.5000000  1.00000  1000000000  1.00000  1.00000    0.000%    35.5     140.1

0.4500000  0.99277  1000000000  0.99278  0.99541    0.265%    61.3      93.6

0.4300000  0.98582  1000000000  0.98582  0.98784    0.205%    61.8      82.7

0.4050000  0.97380  1000000000  0.97381  0.97490    0.112%    61.0      85.5

0.3450000  0.92952  1000000000  0.92953  0.93052    0.106%    70.5     111.7

0.3150000  0.89886  1000000000  0.89888  0.90002    0.127%    62.6      95.0

0.2550000  0.81911  1000000000  0.81912  0.82050    0.168%    71.0      93.3

0.2050000  0.73182  1000000000  0.73182  0.73251    0.094%    65.1     111.8

0.1650000  0.64614  1000000000  0.64616  0.64691    0.117%    76.5     102.5

0.0850000  0.41956  1000000000  0.41955  0.42018    0.149%   100.1     135.2

0.0400000  0.24229  1000000000  0.24231  0.24335    0.429%   121.7     178.6

0.0200000  0.14144  1000000000  0.14145  0.14202    0.401%   149.3     191.4

=============================================================================

=== SUMMARY ===    12000000000  0.73177  0.73285    0.147%    68.7     110.4

Codeset 2

LPS Prob  Entropy    Samples   Optimal  Average     Loss  Enc Thru  Dec Thru

0.5000000  1.00000  1000000000  1.00000  1.00000    0.000%   297.2     450.7

0.4500000  0.99277  1000000000  0.99278  0.99519    0.242%   198.7     197.0

0.4300000  0.98582  1000000000  0.98582  0.98759    0.179%   208.5     226.8

0.4050000  0.97380  1000000000  0.97381  0.97470    0.092%   218.0     199.2

0.3450000  0.92952  1000000000  0.92953  0.93046    0.099%   220.3     242.8

0.3150000  0.89886  1000000000  0.89888  0.89955    0.075%   191.0     206.4

0.2550000  0.81911  1000000000  0.81912  0.82020    0.132%   196.9     222.8

0.2050000  0.73182  1000000000  0.73182  0.73291    0.148%   209.1     176.5

0.1650000  0.64614  1000000000  0.64616  0.64670    0.084%   220.8     264.3

0.0850000  0.41956  1000000000  0.41955  0.42015    0.143%   238.4     280.8

0.0400000  0.24229  1000000000  0.24231  0.24260    0.118%   232.3     350.6

0.0200000  0.14144  1000000000  0.14145  0.14188    0.302%   276.9     571.0

=============================================================================

=== SUMMARY ===    12000000000  0.73177  0.73266    0.122%   222.0     251.1

## Hardware

Due to time constraints, the hardware test in TE8 is not cross verified. The following results are provided as information only.

Note that, although Figure 1 indicates that the Codeset 2 hardware has a larger gate-count than than the Codeset 1 hardware, the gate-counts involved are very small when compared to the size of the entire chip on which these decoders are likely to appear.  These chips will likely contain millions of gate-equivalents, whereas Codeset 2 hardware is never more than about 15K gates.  Furthermore, Codeset 2 hardware has a large advantage in average output bins per clock cycle (see Table 1), which manifests itself as a an advantage over Codeset 1 hardware in power consumption at a given throughput rate (Figure 2).  Furthermore, the Codeset 2 hardware is capable of much greater maximum throughput than the Codeset 1 hardware.

Figure 1. Gate Count versus Clock Rate

Figure 2. Total Power versus Performance

At 50 Mbps, power consumption is about 50% greater for Codeset 1 versus Codeset 2.  At > 50 Mbps, Codeset 2 becomes even more power efficient.

Table 1. 95% Confidence Interval for Bins per Clock Cycle

| Design | Mean | High Range | Low Range |
| --- | --- | --- | --- |
| Codeset 1 | 1.68 | 2.56 | 0.80 |
| Codeset 2 | 8.18 | 8.70 | 7.66 |

# Comments

## Implementation

The Bernoulli test results above show that the average decoding throughput of Codeset 1 used by PIPE is at times worse than that of BAC. Such results, thought seem to be counter-intuitive at first sight, are not surprising. Indeed, carefully examining the implementation of Codeset 1 reveals that the decoding is performed by processing one input bit at a time, which is exactly the same as in BAC decoding. In this manner, the fact of knowing the tree structure in V2V coding is not used to improve its throughput.

In contrast, the decoder of Codeset 2 greedily processes as many input bits as possible. For example, at the beginning of a decoding a codeword, the decoder, knowing the V2V code, knows the smallest codeword length of all the codewords. As such, it can read in and process that many bits. To proceed, the decoder again knows the smallest length of all the remaining valid codewords, and thus can read in and process as many bits (minus the number of bits processed so far). Such process is repeated until a complete codeword is decoded.

Though the greedy method described above in general provides better decoding throughput than the corresponding implementation of Codeset 1 in the current TMuC software, the improvement can be quite limited if the underlying V2V codes are not properly designed: one could easily come up with an example where the decoding may have to progress one bit at a time even with the latter implementation.

## Tree Design

Assume that the decoder operates in the greedy manner. The problem of designing a V2V code then becomes a search problem where we would like to strike a good tradeoff among

- Compression efficiency

- Decoding throughput

- Size

In the above, decoding throughput can be indirectly measured as the average number of bins per a single bit. The V2V codes in Codeset 2 were indeed designed by approximately solving the above search problem. The test results in Section 2 reflect directly our design choices.

In addition to the three performance measures above, the design of a V2V code might also take into consideration the following parameters.

- Encoding throughput

- Power consumption

Note that our discussion here is focused on the underlying codeset. As such, a well designed codeset will benefit both operational modes of V2V coding in TMuC: parallel or interleaved.

## Interaction with Context Modeling

V2V coding was proposed as a simpler, faster alternative to BAC. In order to match the throughput improvement offered by V2V coding, various techniques [3] were proposed to improve the throughput of context modeling. Since V2V coding and context modeling are tightly connected to each other, it is important to carefully investigate how these two processes interact, and what design choices made to one would affect the other, in particular, those choices concerning throughput. For example, as the throughput of context modeling improves, the current implementation of Codeset 1 in TMuC might lag behind and thus throttles the whole decoding process.

Furthermore, it should also be noted that in the interleaved operational mode of V2V coding, whenever the context model needs data from a V2V code whose output buffer is empty, the context model must stall until the decoder provides more data. The greater the number of bins per clock cycle that the decoder produces, the fewer the number of stalls. In other word even with a context model with low throughput capability a higher throughput is typically required of V2V coding.

# Proposal

Based on the information provided above, we make the following proposals.

- Consider an alternative set of V2V codes, e.g., Codeset 2 tested in TE8 for applications demanding high throughput. These codes can be used in both operational modes: parallel or interleaved.

- Define throughput requirements, possibly depending upon profiles.

- Define size requirements, possibly depending upon profiles.

- Adopt the implementation of Codeset 2 for improved throughput.

- Consider the context model solutions of [3] for an overall throughput improvement.

# References

[1] Test model under consideration. JCTVC-A205, Dresden, Germany, 2010.

[2] F. Bossen, “Common test conditions and software reference configurations,” JCTVC-B300, Geneva, Switzerland, 2010.

[3] M. Budagavi and G. Martin-Cocher, ``Tool experiment 8: Parallel entropy coding,’’ JCTVC-B308, Geneva, Switzerland, 2010.

# Patent rights declaration(s)

Research In Motion Ltd may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).