| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C137 WG11 Number: m18161 |
| --- | --- |

| Title: | Decoder-side Block Boundary Decision (DBBD) with OBMC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shigeru Fukushima JVC KENWOOD Holdings, Inc. 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-6597 fukushima.shigeru@jk-holdings.com |
| Source: | JVC Kenwood Holdings, Inc. / Victor Company of Japan, Limited |  |  |

_____________________________

# Abstract

This proposal focuses on the tool “Decoder-side Block Boundary Decision (DBBD)” which was proposed in the JCTVC-A108 as one of tools for HEVC CfP response. DBBD aims to perform motion compensation with various block patterns without additional coding bits by deciding the MC boundary on the decoder-side. In this proposal, DBBD with OBMC is proposed for improving the quality of MC block in addition to the previous proposal. The simulation results show that the proposed technique provides 3.1% BD-rate gain for random access, and 2.2% for low delay as against JM16.2 anchor under HEVC common test conditions.

# Introduction

In AVC, fixed block patterns such as 16x16/16x8/8x16/8x8 can be used for motion compensation and a coding mode which indicates the block pattern is signaled as ‘mb_type’. It can achieve high coding performance in the case that two or more different motions are included in the macroblock. However it is difficult to achieve high coding performance in the case of the prediction with a mismatch between block boundary and actual motion boundary as shown in Figure 1. In order to minimize the influence of this problem, some techniques which increase the number of fixed block patterns compared to AVC have been already proposed. In those techniques, the prediction error can be reduced by defining new fixed block patterns as new coding modes. However such approach is not always effective because the additional coding bits that indicate new coding modes are required. For this reason, it is desirable to support various block patterns without additional coding bits for motion compensation.

In this proposal, DBBD (Decoder-side Block Boundary Decision) with OBMC (Overlapped Block Motion Compensation) is proposed. DBBD was proposed in JCTVC-A108 [1] as one of tools for HEVC CfP response. OBMC is described in H.263 [2] as a tool for improving the quality of MC block. DBBD and OBMC have good compatibility each other since DBBD prefers the vague boundary and both tools need to have the overlapped region of two prediction blocks. By using OBMC in DBBD, high quality MC block can be obtained with minimum additional processing.

Figure 1  Problem of fixed block patterns

# Algorithm Description

## Overview

The overview of DBBD is shown in Figure 2. It is an example in the case of Nx16 mode.

Figure 2  Overview of DBBD

DBBD can perform motion compensation with various block patterns without additional coding bits because the block boundary is decided on the decoder-side. DBBD achieves high coding performance to exploit the correlation between actual motion boundary and object boundary on prediction blocks.

## Procedure

Figure 3 shows the flowchart of DBBD.

Figure 3  Flowchart of DBBD

### Decoding procedure

The decoding procedure of DBBD is as follows.

- Generate two prediction blocks based on two motion vectors by the size of macroblock.

- Evaluate each candidate boundary on two prediction blocks in order to decide the real boundary.

- Perform OBMC along the real boundary.

- Obtain the final MC block by replacing the pixels around the real boundary with OBMC filtered samples.

Figure 4 shows how to evaluate the boundary. The candidate boundary is evaluated by three activities and NEV (Neighboring Efficiency Value). SAD (Sum of Absolute Difference) is used for all evaluation elements.

Activity1 is an activity on the candidate boundary in prediction block1. Activity2 is an activity on the candidate boundary in prediction block2. Larger value of Activity1 and Activity2 is in better because there is strong correlation between actual motion boundary and object boundary.

Activity3 is an activity on the candidate boundary in the block which is connected by two prediction blocks. Smaller value of Activity3 is in better because it indicates that the final MC block becomes smoother.

NEV shows the prediction efficiency in neighboring region if the prediction blocks are extended to the neighboring region. Smaller value of NEV is in better because there is strong correlation on coding efficiency between in the neighboring region and in the target block.

The better value for best boundary is summarized in Table 1. The real boundary is decided by combination of the four elements.

Figure 4  Boundary evaluation

Table 1  Better value for best boundary

| Element | Activity1 | Activity2 | Activity3 | NEV |
| --- | --- | --- | --- | --- |
| Better value | Larger | Larger | Smaller | Smaller |

Figure 5 shows OBMC process in DBBD. OBMC is performed after decision of the real boundary in order to improve the quality of MC block.

Figure 5  OBMC in DBBD

Figure 6 shows OBMC filtering details. OBMC is performed along the real boundary and the range of OBMC is set to two. The mixed ratio depends on the distance from the real boundary. The influence from another prediction block is stronger in the position that is closer to the real boundary.

Figure 6  OBMC filtering details

### Encoding procedure

The encoding procedure of DBBD is as follows.

- Set a provisional boundary for motion estimation.

- Perform motion estimation using the block divided by the provisional boundary.

- Simulate the decoding procedure of DBBD in order to decide the real boundary and to obtain the final MC block.

- Evaluate the final MC block for mode decision.

## Implementation

### Software

DBBD is implemented into JM16.2 AVC reference software [3].

### Syntax structure

The syntax is the same as AVC perfectly.

### Prediction mode

Only 16x8 and 8x16 prediction mode is replaced by 16xN and Nx16, respectively. The other prediction modes are the same as AVC.

### Others

The candidate boundary N is restricted to 4, 6, 8, 10, and 12.

Only luma prediction samples are used to evaluate the boundary.

The decided boundary is also used for chroma prediction samples in MC process.

Motion vector prediction of 16xN and Nx16 are obtained with the same rule as in the case of 16x8 and 8x16, respectively.

Selectable transform sizes are the same as AVC. (Only 8x8 or 4x4 can be used)

It can be applied to bi-prediction in B-frame to perform the process of DBBD after generating bi-predictive block of each region.

# Simulation Results

## Test conditions

We set up the test conditions to match with HEVC common test conditions [4] as much as possible.

### Test sequences

All common test sequences are used.

All frames are encoded for all sequences and test cases.

### Coding structure

Random access and low delay configuration are simulated.

Random access configuration that has hierarchical B coding structure is constructed of I/P/B frame. (We tried to use B-frame instead of P-frame, but could not use it on JM16.2 correctly.)

Low delay configuration that has no hierarchical coding structure is constructed of I/B frame.

### Reference frames

Two reference frames can be used on P-frame.

One reference frame can be used for each list on B-frame.

The B-frames for hierarchical B coding structure can use a reference frame that is temporally the nearest one for each direction.

The B-frames for non-hierarchical coding structure can use a reference frame, that is temporally the nearest one for list0, and that is the secondary nearest one for list1.

### Intra period

The intra refresh period for random access is dependent on the frame rate of the source: a value 24 is used for sequences with a frame rate equal to 24fps, 32 for 30fps, 48 for 50fps, and 64 for 60fps.

There is no intra refresh for low delay. (Only the first frame is coded as I-frame.)

### Quantization parameter

For each test sequence four quantization parameter values are to be used: 22, 27, 32 and 37. These define the QP values used for I-frames in a sequence.

## BD-rate results

Table 2 shows the summary results of this proposal.

Table 3 shows the results of random access for each sequence.

Table 4 shows the results of low delay for each sequence.

Table 2  Summary of results

|  | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -3.9 | -3.4 | -3.2 |  |  |  |
| Class B | -2.6 | -1.8 | -1.8 | -1.8 | -1.9 | -1.7 |
| Class C | -3.4 | -2.7 | -2.7 | -2.6 | -2.3 | -2.3 |
| Class D | -3.0 | -2.5 | -2.5 | -2.3 | -2.5 | -2.5 |
| Class E |  |  |  | -2.3 | -0.1 | -1.5 |
| All | -3.1 | -2.4 | -2.4 | -2.2 | -1.8 | -2.0 |

Table 3  Results of random access

|  |  |  | JM 16.2 reference |  |  | Proposal |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | QPISlice | kbps | Y psnr |  |  | kbps | Y psnr | BD-rate Y |
| Class A | S01 | 22 | 17562.98 | 41.342 |  |  | 17303.34 | 41.38 | -2.8 |
| 4K | Traffic | 27 | 7267.89 | 38.679 |  |  | 7134.89 | 38.722 |  |
|  |  | 32 | 3542.56 | 35.926 |  |  | 3487.12 | 35.976 |  |
|  |  | 37 | 1937.22 | 33.176 |  |  | 1914.02 | 33.216 |  |
|  | S02 | 22 | 41325.09 | 40.055 |  |  | 40185.43 | 40.069 | -4.9 |
|  | PeopleOnStreet | 27 | 20039.15 | 37.037 |  |  | 19318.45 | 37.095 |  |
|  |  | 32 | 10634.84 | 34.059 |  |  | 10277.82 | 34.173 |  |
|  |  | 37 | 6136.11 | 31.251 |  |  | 5972.02 | 31.393 |  |
| Class B | S03 | 22 | 6582.48 | 41.228 |  |  | 6498.76 | 41.249 | -1.8 |
| 1080p | Kimono | 27 | 3044.76 | 39.107 |  |  | 3004.54 | 39.137 |  |
|  |  | 32 | 1532.06 | 36.526 |  |  | 1519.55 | 36.561 |  |
|  |  | 37 | 822.29 | 33.905 |  |  | 818.31 | 33.931 |  |
|  | S04 | 22 | 9529.8 | 39.792 |  |  | 9391.99 | 39.814 | -2.3 |
|  | ParkScene | 27 | 4178.2 | 37.162 |  |  | 4105.91 | 37.189 |  |
|  |  | 32 | 1973.28 | 34.35 |  |  | 1945.59 | 34.383 |  |
|  |  | 37 | 960.82 | 31.703 |  |  | 953.55 | 31.735 |  |
|  | S05 | 22 | 24843.07 | 38.268 |  |  | 24574.49 | 38.295 | -2.6 |
|  | Cactus | 27 | 7897.65 | 36.56 |  |  | 7783.36 | 36.589 |  |
|  |  | 32 | 3744.31 | 34.485 |  |  | 3690.49 | 34.518 |  |
|  |  | 37 | 1950.82 | 32.12 |  |  | 1926.64 | 32.148 |  |
|  | S06 | 22 | 23848.89 | 38.945 |  |  | 23574.63 | 38.975 | -3.0 |
|  | BasketballDrive | 27 | 8478.32 | 37.046 |  |  | 8336.08 | 37.081 |  |
|  |  | 32 | 4117.38 | 34.989 |  |  | 4050.66 | 35.036 |  |
|  |  | 37 | 2274.89 | 32.732 |  |  | 2245.97 | 32.788 |  |
|  | S07 | 22 | 52938.79 | 37.422 |  |  | 52304.02 | 37.443 | -3.2 |
|  | BQTerrace | 27 | 10457.91 | 34.882 |  |  | 10318.28 | 34.918 |  |
|  |  | 32 | 3355.45 | 33.212 |  |  | 3300.54 | 33.248 |  |
|  |  | 37 | 1511.58 | 31.074 |  |  | 1500.64 | 31.128 |  |
| Class C | S08 | 22 | 4812.7 | 39.891 |  |  | 4713.26 | 39.913 | -3.2 |
| WVGA | BasketballDrill | 27 | 2335.87 | 36.654 |  |  | 2283.09 | 36.694 |  |
|  |  | 32 | 1151.24 | 33.806 |  |  | 1127.67 | 33.863 |  |
|  |  | 37 | 607.18 | 31.253 |  |  | 598.22 | 31.317 |  |
|  | S09 | 22 | 4771.18 | 39.931 |  |  | 4661.48 | 39.964 | -3.7 |
|  | BQMall | 27 | 2243.72 | 37.263 |  |  | 2182.78 | 37.305 |  |
|  |  | 32 | 1144.34 | 34.357 |  |  | 1114.28 | 34.417 |  |
|  |  | 37 | 622.56 | 31.458 |  |  | 610.05 | 31.525 |  |
|  | S10 | 22 | 9166.34 | 37.969 |  |  | 9028.55 | 37.99 | -2.2 |
|  | PartyScene | 27 | 4043.2 | 34.219 |  |  | 3977.52 | 34.251 |  |
|  |  | 32 | 1862.86 | 31.007 |  |  | 1833.55 | 31.037 |  |
|  |  | 37 | 854.17 | 28.037 |  |  | 847.71 | 28.072 |  |
|  | S11 | 22 | 5978.99 | 39.077 |  |  | 5851.33 | 39.095 | -4.3 |
|  | RaceHorses | 27 | 2615.54 | 35.698 |  |  | 2531.61 | 35.739 |  |
|  |  | 32 | 1244.09 | 32.665 |  |  | 1204.2 | 32.755 |  |
|  |  | 37 | 626.67 | 29.828 |  |  | 612.33 | 29.913 |  |
| Class D | S12 | 22 | 1885.09 | 40.457 |  |  | 1852.52 | 40.472 | -3.0 |
| WQVGA | BasketballPass | 27 | 959.74 | 36.645 |  |  | 938.65 | 36.683 |  |
|  |  | 32 | 482.49 | 33.234 |  |  | 472.05 | 33.297 |  |
|  |  | 37 | 252.46 | 30.352 |  |  | 247.98 | 30.414 |  |
|  | S13 | 22 | 2427.04 | 37.675 |  |  | 2377.15 | 37.694 | -2.6 |
|  | BQSquare | 27 | 896.17 | 33.855 |  |  | 880.22 | 33.9 |  |
|  |  | 32 | 376.47 | 30.891 |  |  | 370.71 | 30.915 |  |
|  |  | 37 | 186.33 | 28.218 |  |  | 183.67 | 28.23 |  |
|  | S14 | 22 | 2085.46 | 38.054 |  |  | 2054.17 | 38.073 | -2.2 |
|  | BlowingBubbles | 27 | 937.6 | 34.521 |  |  | 921.88 | 34.545 |  |
|  |  | 32 | 433.77 | 31.269 |  |  | 426.59 | 31.302 |  |
|  |  | 37 | 198.16 | 28.322 |  |  | 195.3 | 28.342 |  |
|  | S15 | 22 | 1489.48 | 39.403 |  |  | 1449.07 | 39.424 | -4.3 |
|  | RaceHorses | 27 | 746.59 | 35.646 |  |  | 720 | 35.695 |  |
|  |  | 32 | 368.19 | 32.136 |  |  | 357.31 | 32.227 |  |
|  |  | 37 | 184.82 | 29.185 |  |  | 180.57 | 29.254 |  |

Table 4  Results of low delay

|  |  |  | JM 16.2 reference |  |  | Proposal |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | QPISlice | kbps | Y psnr |  |  | kbps | Y psnr | BD-rate Y |
| Class B | S03 | 22 | 8368.39 | 41.597 |  |  | 8291.2 | 41.615 | -1.0 |
| 1080p | Kimono | 27 | 4074.08 | 39.56 |  |  | 4051.87 | 39.59 |  |
|  |  | 32 | 2157.95 | 36.978 |  |  | 2164.28 | 37.029 |  |
|  |  | 37 | 1178.2 | 34.011 |  |  | 1189.78 | 34.065 |  |
|  | S04 | 22 | 12905.05 | 40.074 |  |  | 12730.96 | 40.082 | -1.6 |
|  | ParkScene | 27 | 5354.77 | 36.982 |  |  | 5273.22 | 36.997 |  |
|  |  | 32 | 2406.62 | 33.913 |  |  | 2374.13 | 33.928 |  |
|  |  | 37 | 1150.07 | 30.891 |  |  | 1148.39 | 30.908 |  |
|  | S05 | 22 | 36244.86 | 39.081 |  |  | 35711.48 | 39.085 | -2.3 |
|  | Cactus | 27 | 10249.58 | 36.679 |  |  | 10055.48 | 36.693 |  |
|  |  | 32 | 4747.06 | 34.42 |  |  | 4672.1 | 34.451 |  |
|  |  | 37 | 2350.84 | 31.898 |  |  | 2336.45 | 31.936 |  |
|  | S06 | 22 | 35364.26 | 39.812 |  |  | 34857.84 | 39.816 | -2.3 |
|  | BasketballDrive | 27 | 12577.05 | 37.52 |  |  | 12356.71 | 37.538 |  |
|  |  | 32 | 6273.53 | 35.459 |  |  | 6183.06 | 35.498 |  |
|  |  | 37 | 3388.06 | 32.997 |  |  | 3359.8 | 33.047 |  |
|  | S07 | 22 | 101813.21 | 39.969 |  |  | 100390.8 | 39.943 | -2.0 |
|  | BQTerrace | 27 | 17919.48 | 35.538 |  |  | 17554.94 | 35.553 |  |
|  |  | 32 | 5166.49 | 32.994 |  |  | 5116.57 | 33.021 |  |
|  |  | 37 | 2262.83 | 30.186 |  |  | 2276.98 | 30.229 |  |
| Class C | S08 | 22 | 6151.51 | 39.649 |  |  | 6060.75 | 39.669 | -2.8 |
| WVGA | BasketballDrill | 27 | 3059.89 | 36.511 |  |  | 2995.85 | 36.535 |  |
|  |  | 32 | 1504.66 | 33.7 |  |  | 1466.83 | 33.729 |  |
|  |  | 37 | 747.17 | 30.912 |  |  | 731.36 | 30.958 |  |
|  | S09 | 22 | 6688.62 | 40.327 |  |  | 6561.76 | 40.341 | -3.0 |
|  | BQMall | 27 | 3122.1 | 37.264 |  |  | 3049.69 | 37.288 |  |
|  |  | 32 | 1552.87 | 34.041 |  |  | 1513.9 | 34.082 |  |
|  |  | 37 | 801.48 | 30.741 |  |  | 784.43 | 30.786 |  |
|  | S10 | 22 | 14968.09 | 39.018 |  |  | 14777.25 | 39.015 | -1.4 |
|  | PartyScene | 27 | 7208.06 | 34.878 |  |  | 7114.39 | 34.88 |  |
|  |  | 32 | 3137.84 | 30.917 |  |  | 3098.23 | 30.926 |  |
|  |  | 37 | 1275.62 | 27.283 |  |  | 1256.51 | 27.295 |  |
|  | S11 | 22 | 8392.91 | 40.776 |  |  | 8272.43 | 40.782 | -3.0 |
|  | RaceHorses | 27 | 3975.13 | 37.259 |  |  | 3886.27 | 37.276 |  |
|  |  | 32 | 1748.98 | 33.718 |  |  | 1697.3 | 33.746 |  |
|  |  | 37 | 796.86 | 30.354 |  |  | 773.99 | 30.391 |  |
| Class D | S12 | 22 | 2472.01 | 41.044 |  |  | 2444.36 | 41.055 | -2.3 |
| WQVGA | BasketballPass | 27 | 1355.62 | 37.457 |  |  | 1332.58 | 37.466 |  |
|  |  | 32 | 701.65 | 34.008 |  |  | 683.44 | 34.029 |  |
|  |  | 37 | 351.19 | 30.874 |  |  | 342.91 | 30.911 |  |
|  | S13 | 22 | 4366.92 | 39.078 |  |  | 4292.75 | 39.082 | -1.9 |
|  | BQSquare | 27 | 2034.03 | 34.836 |  |  | 1993.53 | 34.843 |  |
|  |  | 32 | 817.91 | 30.866 |  |  | 806.68 | 30.887 |  |
|  |  | 37 | 273.79 | 27.064 |  |  | 274.11 | 27.097 |  |
|  | S14 | 22 | 3312.48 | 38.819 |  |  | 3257.81 | 38.816 | -1.9 |
|  | BlowingBubbles | 27 | 1483.77 | 34.557 |  |  | 1457.11 | 34.561 |  |
|  |  | 32 | 628.83 | 30.683 |  |  | 617.38 | 30.7 |  |
|  |  | 37 | 259.69 | 27.302 |  |  | 255.49 | 27.309 |  |
|  | S15 | 22 | 1945.37 | 40.473 |  |  | 1913.34 | 40.481 | -3.0 |
|  | RaceHorses | 27 | 1005.01 | 36.605 |  |  | 977.13 | 36.607 |  |
|  |  | 32 | 494.35 | 32.918 |  |  | 477.36 | 32.93 |  |
|  |  | 37 | 234.31 | 29.73 |  |  | 228.54 | 29.752 |  |
| ClassE | S16 | 22 | 3340.39 | 43.067 |  |  | 3276.96 | 43.105 | -1.8 |
| WQVGA | Vidyo1 | 27 | 1278.78 | 40.36 |  |  | 1261.65 | 40.39 |  |
|  |  | 32 | 637.2 | 37.455 |  |  | 635.6 | 37.51 |  |
|  |  | 37 | 359.12 | 34.535 |  |  | 361.44 | 34.574 |  |
|  | S17 | 22 | 4550.67 | 43.158 |  |  | 4459.66 | 43.187 | -3.3 |
|  | Vidyo3 | 27 | 1572.66 | 40.333 |  |  | 1532.88 | 40.387 |  |
|  |  | 32 | 696.86 | 37.178 |  |  | 684.76 | 37.247 |  |
|  |  | 37 | 342.29 | 33.945 |  |  | 341.2 | 33.995 |  |
|  | S18 | 22 | 4030.83 | 43.008 |  |  | 3971.38 | 43.041 | -1.9 |
|  | Vidyo4 | 27 | 1293.56 | 40.17 |  |  | 1279.24 | 40.205 |  |
|  |  | 32 | 612.8 | 37.297 |  |  | 608.97 | 37.345 |  |
|  |  | 37 | 345.81 | 34.457 |  |  | 346.01 | 34.491 |  |

The simulation results show that DBBD provides 3.1% BD-rate [5] gain up to 4.9% for random access, and 2.2% up to 3.3% for low delay as against JM16.2 anchor.

It is observed that DBBD is more effective for the sequence that contains large motion region such as PeopleOnStreet, RaceHorses, and BQMall.

# Conclusions

The simulation results show that DBBD can improve coding efficiency in spite of restrictive implementation. It will be improved by supporting other prediction modes and transform sizes.

We think that DBBD is useful for HEVC which will support various block patterns in order to achieve high coding performance.

Therefore, we would like to show the effectiveness of DBBD on TMuC [6]/TM software through participating in some TE/CE as a proponent.

# References

[1] “Description of video coding technology proposal by JVC”,

JCTVC-A108, Dresden, April 2010.

[2] ITU-T Recommendation H.263.

[3] AVC Reference Software, http://iphome.hhi.de/suehring/tml/download/.

[4] “Common test conditions and software reference configurations”,

JCTVC-B300, Geneva, July 2010.

[5] G. Bjøntegaard, “Improvements of the BD-PSNR model,” ITU-T SG16 Q.6 Document,

VCEG-AI11, Berlin, July 2008.

[6] “Test Model under Consideration”,

JCTVC-B205, Geneva, July 2010

# Patent rights declaration(s)

JVC KENWOOD Holdings, Inc. and Victor Company of Japan, Limited may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).