| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C140 WG11 Number: m18164 |
| --- | --- |

| Title: | TE9-2 Report on Performance Tests for Different sets of PU modes by Fraunhofer HHI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Tool experiment result report |  |  |
| Author(s) or Contact(s): | Benjamin Bross Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: | +49 30 31002 622 benjamin.bross@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports the results of testing different sets of PU modes within the scope of tool experiment 9 on large block structures in TMuC [1]. The first set defines tests forcing a subdivision of residual quadtrees (RQTs) such that transform blocks don't span over PU borders. This is separately tested for quadratic, rectangular and asymmetric shapes. Another Test switches (virtually) redundant shapes off.

# Simulation environment

## Software

The software used to run the simulations is TMuC 0.7.4 [2]. The binaries were compiled on a 64 bit Ubuntu 9.04 Linux using the included make files and gcc 4.3.3. The coding conditions used for the simulations are specified in [3]. The anchor used for all tests uses these configuration files settings. Only in compile-time settings, the transform skip flag macro is enabled in TypeDef.h:

#define HHI_RQT_ROOT 1

Test specific changes from this anchor configuration are listed in the different test sections.

## Hardware

Three different types of computers have been used to run the simulations  all running a 64 bit Linux operating system. Their hardware is listed in Error: Reference source not found.

Table 1-1-1 Computers used to run the simulations.

| Type | Cores | CPU | RAM | OS |
| --- | --- | --- | --- | --- |
| 1 | 24 | Intel(R) Xeon(R) CPU X7460 @ 2.66GHz | 96 GB | Ubuntu 9.04, 64 bit |
| 2 | 8 | Intel(R) Xeon(R) CPU X5460 @ 3.16GHz | 32 GB | Ubuntu 9.04, 64 bit |
| 3 | 8 | Intel(R) Xeon(R) CPU X5570 @ 2.93GHz | 32 GB | Ubuntu 9.04, 64 bit |

# Force RQT split according to PUs

## Force split for NxN shapes

### Test setup

Table 2-2 shows the config file and compile time settings used to perform the test.

Table 2-2 Test setup for 'Force split for NxN shapes' test.

| Anchor Setting | Disabled |
| --- | --- |
| Tested Setting | Enabled |
| Tested config file settings | n.a. |
| Tested compile-time settings | #define HHI_RQT_FORCE_SPLIT_NxN 1 |

### Results

The coding efficiency results in this section are given in terms of BD-rate [4][5]. The values are averaged over all sequences from one class. BD-PSNR, PSNR values and bit rates for every test point can be found in the attached Excel spreadsheet JCTVC-C140_forceSpl_NxN.xls.

The results are presented in two tables: Table 2-3 and Table 2-4. Each table represents a test for a constraint set using the high efficiency (HE) and low complexity (LC) settings.

Table 2-3 Average BD-rates for random access constraint set when forcing split for NxN shapes.

|  | Random Access HE |  |  | Random access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.1 |
| Class C | 0.0 | -0.1 | 0.1 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |

Table 2-4 Average BD-rates for low delay constraint set when forcing split for NxN shapes.

|  | Low delay HE |  |  | Low delay LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.1 |
| Class C | 0.0 | 0.1 | -0.1 | 0.0 | 0.2 | 0.0 |
| Class D | 0.1 | 0.4 | -0.1 | 0.0 | 0.1 | 0.0 |
| Class E | 0.0 | -0.3 | 0.1 | 0.0 | -0.7 | -0.3 |
| All | 0.0 | 0.1 | 0.0 | 0.0 | -0.1 | 0.0 |

## Force split for rectangular shapes

### Test setup

Table 2-5 shows the config file and compile time settings used to perform the test.

Table 2-5 Test setup for 'Force split for rectangular shapes' test.

| Anchor Setting | Disabled |
| --- | --- |
| Tested Setting | Enabled |
| Tested config file settings | n.a. |
| Tested compile-time settings | #define HHI_RQT_FORCE_SPLIT_RECT 1 |

### Results

The coding efficiency results in this section are given in terms of BD-rate [4][5]. The values are averaged over all sequences from one class. BD-PSNR, PSNR values and bit rates for every test point can be found in the attached Excel spreadsheet JCTVC-C140_forceSpl_RECT.xls.

Furthermore relative encoding and decoding times in percentage of the anchor times are given. The numbers are averaged over all sequences. Due to the heterogeneous structure of the computer cluster consisting of three different computer types and non-constant workloads, these numbers can only be taken as a rough indication of the complexity.

The results are presented in two tables: Table 2-6 and Table 2-7. Each table represents a test for a constraint set using the high efficiency (HE) and low complexity (LC) settings.

Table 2-6 Average BD-rates for random access constraint set when forcing split for rectangular shapes.

|  | Random Access HE |  |  | Random access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | -0.1 | 0.0 | 0.0 | 0.1 | 0.1 |
| Class B | 0.0 | 0.1 | 0.1 | 0.1 | 0.3 | 0.3 |
| Class C | 0.0 | 0.0 | 0.1 | 0.1 | 0.1 | 0.1 |
| Class D | 0.0 | 0.1 | 0.3 | 0.0 | 0.2 | 0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.1 | 0.1 | 0.2 | 0.2 |

Table 2-7 Average BD-rates for low delay constraint set when forcing split for rectangular shapes.

|  | Low delay HE |  |  | Low delay LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.1 | 0.1 | 0.0 | 0.1 | 0.2 | 0.3 |
| Class C | 0.0 | 0.1 | 0.1 | 0.1 | 0.2 | 0.2 |
| Class D | 0.0 | 0.1 | 0.3 | 0.1 | 0.3 | 0.2 |
| Class E | -0.1 | 0.4 | 0.2 | 0.2 | -0.2 | 0.0 |
| All | 0.0 | 0.2 | 0.1 | 0.1 | 0.2 | 0.2 |

## Force split for asymmetric shapes

### Test setup

Table 2-8 shows the config file and compile time settings used to perform the test.

Table 2-8 Test setup for 'Force split for asymmetric shapes' test.

| Anchor Setting | Disabled |
| --- | --- |
| Tested Setting | Enabled |
| Tested config file settings | n.a. |
| Tested compile-time settings | #define HHI_RQT_FORCE_SPLIT_ASYM 1 |

### Results

The coding efficiency results in this section are given in terms of BD-rate [4][5]. The values are averaged over all sequences from one class. BD-PSNR, PSNR values and bit rates for every test point can be found in the attached Excel spreadsheet JCTVC-C140_forceSpl_ASYM.xls.

Furthermore relative encoding and decoding times in percentage of the anchor times are given. The numbers are averaged over all sequences. Due to the heterogeneous structure of the computer cluster consisting of three different computer types and non-constant workloads, these numbers can only be taken as a rough indication of the complexity.

The results are presented in two tables:Table 2-9 and Table 2-10. Each table represents a test for a constraint set using the high efficiency (HE) and low complexity (LC) settings.

Table 2-9 Average BD-rates for random access constraint set when forcing split for asymmetric shapes.

|  | Random Access HE |  |  | Random access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | -0.1 | -0.1 | 0.2 | 0.2 | 0.2 |
| Class B | 0.2 | 0.0 | 0.1 | 0.3 | 0.5 | 0.5 |
| Class C | 0.2 | 0.0 | 0.1 | 0.2 | 0.2 | 0.3 |
| Class D | 0.1 | 0.0 | -0.1 | 0.1 | 0.3 | 0.3 |
| Class E |  |  |  |  |  |  |
| All | 0.2 | 0.0 | 0.0 | 0.2 | 0.3 | 0.3 |

Table 2-10 Average BD-rates for low delay constraint set when forcing split for asymmetric shapes.

|  | Low delay HE |  |  | Low delay LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.3 | 0.4 | 0.2 | 0.3 | 0.5 | 0.8 |
| Class C | 0.2 | 0.3 | 0.3 | 0.3 | 0.5 | 0.5 |
| Class D | 0.1 | 0.2 | 0.1 | 0.2 | 0.7 | 0.7 |
| Class E | 0.0 | 0.9 | 0.6 | 0.4 | 0.8 | 0.4 |
| All | 0.2 | 0.4 | 0.3 | 0.3 | 0.6 | 0.6 |

# Disable Inter NxN PUs

## Test setup

Table 3-11 shows the config file and compile time settings used to perform the test.

Table 3-11 Test setup for 'Disable Inter NxN PUs' test.

| Anchor Setting | Disabled |
| --- | --- |
| Tested Setting | Enabled |
| Tested config file settings | n.a. |
| Tested compile-time settings | #define HHI_DISABLE_INTER_NxN_SPLIT 1 #define HHI_RMP_SWITCH 1 |

## Results

The coding efficiency results in this section are given in terms of BD-rate [4][5]. The values are averaged over all sequences from one class. BD-PSNR, PSNR values and bit rates for every test point can be found in the attached Excel spreadsheet JCTVC-C140_disable_inter_NxN.xls.

The results are presented in two tables: Error: Reference source not found, and Error: Reference source not found. Each table represents a test for a constraint set using the high efficiency (HE) and low complexity (LC) settings.

Table 3-12 Average BD-rates for random access constraint set when forcing split for asymmetric shapes.

|  | Random Access HE |  |  | Random access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 |
| Class B |  |  |  | 0.0 | -0.1 | 0.0 |
| Class C | 0.0 | 0.0 | 0.2 | 0.0 | 0.0 | -0.1 |
| Class D | 0.0 | -0.1 | 0.1 | 0.0 | 0.1 | 0.0 |
| Class E |  |  |  |  |  |  |
| All |  |  |  | 0.0 | 0.0 | 0.0 |

Table 3-13 Average BD-rates for low delay constraint set when forcing split for asymmetric shapes.

|  | Low delay HE |  |  | Low delay LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B |  |  |  | -0.1 | -0.2 | -0.1 |
| Class C | 0.0 | 0.1 | 0.2 | 0.0 | -0.1 | -0.1 |
| Class D | 0.0 | 0.3 | 0.2 | 0.0 | 0.0 | -0.3 |
| Class E | -0.1 | -0.3 | 0.7 | 0.1 | -0.2 | -0.3 |
| All |  |  |  | 0.0 | -0.1 | -0.2 |

# References

- JCT-VC, “Tool Experiment 9: Large Block Structure”, JCTVC-B309, JCT-VC Meeting, Geneva, July 2010.

- JCT-VC, “Test Model under Consideration Software 0.7.4”,

https://hevc.hhi.fraunhofer.de/svn/svn_TMuCSoftware/tags/0.7.4

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-B300, JCT-VC Meeting, Geneva, July 2010.

- Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.

- Gisle Bjøntegaard, “Improvements of the BD-PSNR model”, ITU-T SG16/Q6, 35th VCEG Meeting, Berlin, Germany, July 2008, Doc. VCEG-AI11.