| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C143 |
| --- | --- |

| Title: | TE10 Subtest2: Coding Unit Synchronous Picture Quadtree-based Adaptive Loop Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chia-Yang Tsai, Chih-Ming Fu, Ching-Yeh Chen, Yu-Wen Huang, and Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Contact: Shawmin Lei +886-3-567-0766 ext.25555 shawmin.lei@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution describes MediaTek’s work on adaptive loop filter (ALF), namely coding unit synchronous picture quadtree-based adaptive loop filter (CS-PQALF). The proposed method uses multi-level quadtree partitions to allow local adaptivity for Wiener loop filtering. The partitions boundaries are aligned with the boundaries of the largest coding units (LCUs). Each partition is a basic filter unit (FU) and can be enhanced by different Wiener filters to reduce mean square error between the reconstructed picture and the original picture. In this proposal, special attention is paid to reduce the number of encoding passes from 16, for the ALF in TMuC, to 2, for our CS-PQALF, while maintaining the good coding efficiency performance of the anchor. This reduction of encoding passes can dramatically reduce the power consumption and access time involved in external memory accesses. Simulation results are compared with the anchor adopted in the subtest 2 of tool experiment 10 (TE 10) [1]. The proposed CS-PQALF increases 0.1% and 0.3% BD-rates for high efficiency random access (HE-RA) configuration and high efficiency low delay (HE-LD) configuration, respectively. The encoder execution times are increased by 1% for both HE-RA and HE-LD. The decoder execution times are increased by 3% and 1% for HE-RA and HE-LD, respectively. Note that the software execution time cannot really reflect the reduction of hardware external memory access overhead. Additional experiments on combining four techniques including controlled clipping (JCTVC-C146), improved deblocking filter (JCTVC-C142), quadtree-based adaptive offset (JCTVC-147), and CS-PQALF show 2.2% and 2.5% BD-rate reductions for HE-RA and HE-LD, respectively.

# Introduction

In prior, we proposed the quadtree-based adaptive restoration (QAR) [2] to enhance the coding efficiency of the in-loop filtering on KTA2.6r1 [3]. In this work, we adopt the concept of picture quadtree partitioning in QAR and propose the coding unit synchronous picture quadtree-based adaptive loop filter (CS-PQALF). The CS-PQALF is developed on TMuC0.7 [4]. In CS-PQALF, the chroma processing is the same as that of TMuC0.7. The luma picture is split into several multi-level quadtree partitions, and each partition boundary is aligned to the boundaries of the largest coding units (LCUs). Each partition has its own filtering process and thus be called as a filter unit (FU). Besides, to effectively reduce the encoding passes, fast filtering distortion estimation (FFDE) is proposed to estimate filtering distortion without real filtering.

Besides CS-PQALF, we also propose three other in-loop filtering tools in this meeting, including improved deblocking filter (IDF) [5], controlled clipping (CC) [6], and quadtree-based adaptive offset (QAO) [7]. We will provide the experimental results of the combined four tools in this contribution.

The rest of this contribution is organized as follows. The proposed CS-PQALF algorithm will be described in Section 2, compression performance will be shown in Section 3, complexity analysis will be discussed in Section 4, additional experimental results on combined in-loop filtering tools will be shown in Section 5, and finally Section 6 will give a conclusion.

# Algorithm description

In prior, many in-loop filter algorithms are proposed to achieve high coding efficiency, but most of them require high computation complexity and large external memory access for reading or writing current/reconstructed/filtered frame buffers [8]. Therefore, in this work, we focus on reduction of encoding complexity and external memory access without sacrificing compression performance. To effectively reduce external memory access, fast filtering distortion estimation (FFDE) is developed. The proposed FFDE can estimate filtering distortion without filtering operations. By using FFDE, the number of encoding passes can be effectively reduced from 16, for the ALF in TMuC, to 2, for our CS-PQALF, and the number of external memory accesses is reduced from more than 20 to only 3 in this proposal when no on-chip frame buffer is available.

The 2-pass encoding flow is described as follows. At the first pass, the quadtree split pattern and the best filter of each FU are decided. The filtering distortions are estimated by FFDE during the decision process. According to the decided quadtree split pattern and the selected filters of all FUs, the reconstructed picture is filtered. At the second pass, the CU synchronous ALF on/off control is performed. According to the ALF on/off results, the first filtered picture is partially recovered by the reconstructed picture.

Details of CS-PQALF are introduced as follows. Top-down quadtree split and filter decision are illustrated in Section 2.1. In Section 2.2, the FFDE is introduced. The filter properties, time-delayed filtering and the 2-step filter coefficient prediction are in Section 2.3, Section 2.4, and Section 2.5, respectively. In Section 2.6, the syntax of CS-PQALF is described.

## Top-down quadtree split and filter decision

A top-down splitting strategy is adopted to divide a picture into multi-level quadtree partitions by using a rate-distortion criterion. Each partition is called a filter unit (FU). The splitting process aligns quadtree partitions with LCU boundaries. The encoding order of FUs follows the z-scan order. For example, in Figure 1, the picture is split into 10 FUs, and the encoding order is FU0, FU1, FU2, FU3, FU4, FU5, FU6, FU7, FU8, and FU9.

Figure 1  Example of LCU-aligned picture quadtree splitting

To indicate the picture quadtree split pattern, split flags are encoded and transmitted in z-order. Figure 2 shows the quadtree split pattern in correspondence with Figure 1.

Figure 2  Quadtree split flags encoded in z-order

The filter of each FU is selected from two filter sets based on the rate-distortion criterion. The first set has 1/2-symmetric square-shaped and rhombus-shaped filters newly derived for the current FU. The second set comes from time-delayed filter buffers; the time-delayed filter buffers store the filters previously derived for FUs of prior pictures. The filter with the minimum rate-distortion cost of these two sets is chosen for the current FU. Similarly, if the current FU is not the smallest FU and can be further split into 4 children FUs, the rate-distortion costs of the 4 children FUs are calculated. By comparing the rate-distortion cost of the split and non-split cases recursively, the picture quadtree split pattern can be decided.

In our current implementation, the maximum quadtree split level is 2, which means the maximum number of FUs is 16. During the quadtree split decision, the correlation values for deriving Wiener coefficients of the 16 FUs at the bottom quadtree level (smallest FUs) can be reused. The rest FUs can derive their Wiener filters from the correlations of the 16FUs at the bottom quadtree level. Therefore, there is only one frame buffer access for deriving the filter coefficients of all FUs.

After the quadtree split pattern is decided, to further reduce the filtering distortion, the CU synchronous ALF on/off control is performed. The operation of the CU synchronous ALF on/off in CS-PQALF is basically the same as that in TMuC0.7 [4]. By comparing the filtering distortion and non-filtering distortion, the leaf CU can explicitly switch ALF on/off in its local region. The coding efficiency may be further improved by redesigning the filter coefficients according to the ALF on/off results. However, the redesigning process needs additional frame buffer accesses. In the proposed CS-PQALF encoder design, there is no redesign process after the CU synchronous ALF on/off decision in order to minimize the number of frame buffer accesses.

## Fast filtering distortion estimation (FFDE)

To evaluate the rate-distortion trade-off during the filter selection of one FU, the filtering distortion and filter side information bitrate have to be calculated repetitively. The filtering distortion is the mean squared error between the input original picture and the filtered picture. Therefore, in a conventional way, in order to obtain the filtering distortion, the picture has to be filtered. However, the filtering process leads to the frame buffer access which may cause an extra heavy burden on hardware implementation. Therefore, we propose fast filtering distortion estimation (FFDE) to obtain the filtering distortion and to avoid real filtering.

For the current FU, there are K pixels. For the k-th pixel in the FU, and represent the input original signal and the reconstructed signal (the signal after deblocking). To derive the Wiener filter coefficients, the auto-correlation matrix  of  and the cross-correlations  of and are calculated. Then the Wiener filter coefficients, {w0, w1, w2,…, wN-1}, are derived. Therefore, the filtering distortion D of the current FU can be estimated by

D=k=0K−1i=0N−1wi⋅x(k−i)−s(k)2.                                         (1)

After a series of substitutions, (1) can be modified to

D=i=0N−1wi.                                  (2)

In (2), Rxx,K, Rxs,K, and Rss,Krepresent the followings, respectively.

Rxx,K: the sum of allRxx(k) in the current FU.

Rxs,K: the sum of all Rxs(k) in the current FU.

Rss,K: the sum of squared values of the all K original pixels in the current FU.

The filtering distortion can be estimated by (2). Furthermore, the candidate filters of the same FU share the sameRss,K. That is,Rss,Kis irrelevant when comparing the rate-distortion costs. Therefore, in the filter selection process of the current FU, the rate-distortion trade-off can be estimated by the correlation values and Wiener filter coefficients without real filtering.

## Filter properties

There are two kinds of filter shapes in CS-PQALF. One is the symmetric square filter, and the allowed filter sizes are 3x3, 5x5, 7x7, and 9x9. The other one is the symmetric rhombus filter, and the allowed filter sizes are 5x5, 7x7, and 9x9. One flag is encoded to indicate the filter shape selection.

All of the above 7 filters can be regarded as the subsets of the symmetric 9x9 square filter. Therefore, for one FU, the correlation values for deriving the filter coefficients of the symmetric 9x9 square filter are first calculated from the input original picture and the reconstructed picture. Then the other 6 filter coefficients can be derived by reusing the correlation values.

## Time-delayed filtering

It was designed to transmit filter coefficients of every FU. However, sometimes the optimal filter of a partition can be quite similar to those of prior pictures. If the filter coefficients of prior pictures are already stored in a buffer, it is not necessary to transmit filter coefficients for such a partition. Therefore, the bitrate for coding filter coefficients can be effectively reduced. In our current implementation, there are 7 groups of buffers for storing the time-delayed 7 types of filters introduced in Section 2.3; each group has 8 buffers.

## 2-step filter coefficient prediction

A 2-step filter coefficient prediction scheme is adopted as follows.

(1) 1st Step: DC coefficient prediction.

The DC coefficient (or say, central filter coefficient) is implicitly predicted by the rest coefficients of the same filter.

(2) 2nd Step: Time-delayed prediction.

The filter coefficients in the time-delayed buffer are chosen to perform differential prediction. If the coded bitrate of the 1st step predicted residual coefficients can be further reduced, the time-delay buffer index of the predictor and the 2nd step predicted residual coefficients will be transmitted. Otherwise, this 2nd step prediction is switched off. An additional one-bit flag is encoded to indicate the switch status of the time-delayed prediction.

## Syntax description

The ALF related parameters, alf_param(), are located in the slice header. In CS-PQALF, we modified parts of the luma syntax in alf_param(). The rest parts are consistent with the anchor in TMuC0.7. The modified parts are highlighted in Table 1.

Table 1  CS-PQALF parameter syntax

| alf_param() { | C | Descriptor |
| --- | --- | --- |
| alf_td_buffer_reset_flag | 2 | u(1) |
| adaptive_loop_filter_flag | 2 | u(1) |
| if ( adaptive_loop_filter_flag ) { |  |  |
| for ( i = 0; i < number of FUs; i++ ) { |  |  |
| alf_zordered_split_flag[i] | 2 | u(1) |
| } |  |  |
| for ( i = 0; i < number of FUs; i++ ) { |  |  |
| alf_zordered_FU_param(i) |  |  |
| } |  |  |
| alf_chroma_idc | 2 | ue(v) |
| if ( alf_chroma_idc ) { |  |  |
| alf_length_chroma_minus_5_div2 | 2 | ue(v) |
| for( i = 0; i< AlfLengthChroma; i++ ) { |  |  |
| alf_coeff_chroma [i] |  | ge(v) |
| } |  |  |
| } |  |  |
| alf_cu_control_flag | 2 | u(1) |
| if ( alf_cu_control_flag ) { |  |  |
| alf_cu_control_max_depth | 2 | ue(v) |
| } |  |  |
| } |  |  |
| } |  |  |

| alf_zordered_FU_param() { | C | Descriptor |
| --- | --- | --- |
| alf_filter_shape | 2 | u(1) |
| alf_filter_size | 2 | ue(v) |
| if ( number of time-delayed filter buffers > 0 ) { |  |  |
| alf_td_filter_flag | 2 | u(1) |
| } |  |  |
| if ( alf_td_filter_flag == 1 ) { |  |  |
| b = number of bits to indicate the time-delayed buffer index |  |  |
| alf_td_filter_buffer_index | 2 | u(b) |
| } |  |  |
| else { |  |  |
| if ( number of time-delayed filter buffers > 0 ) { |  |  |
| alf_td_prediction_flag | 2 | u(1) |
| } |  |  |
| if ( alf_td_prediction_flag == 1 ) { |  |  |
| b = number of bits to indicate the time-delayed buffer index |  |  |
| alf_td_prediction_buffer_index | 2 | u(b) |
| } |  |  |
| for ( i = 0; i < ALFLengthLuma; i++ ) { |  |  |
| alf_coeff_luma[i] | 2 | ge(v) |
| } |  |  |
| } |  |  |
| } |  |  |

### Semantics

Table 2  Relationship between alf_filter_shape and alf_filter_size

| alf_filter_size | alf_filter_shape = 1 (rhombus shape) | alf_filter_shape = 0 (square shape) |
| --- | --- | --- |
| 0 | 5x5 | 5x5 |
| 1 | 7x7 | 7x7 |
| 2 | 9x9 | 9x9 |
| 3 |  | 3x3 |

alf_td_buffer_reset_flag equals to 1 to denote that the time-delayed buffers are reset.

alf_zordered_split_flag equals to 1 to denote that this FU is split.

alf_filter_shape equals to 1 or 0 to denote rhombus- or square-shaped filter is selected.

alf_filter_size denotes the filter size. The relationship with alf_filter_shape is shown in Table 2.

alf_td_filter_flag equals to 1 to denote that the time-delayed filtering is enabled.

alf_td_filter_buffer_index denotes that the time-delayed buffer index for time-delayed filtering.

alf_td_prediction_flag equals to 1 to denote that the time-delayed prediction is enabled.

alf_td_prediction_buffer_index denotes the time-delayed buffer index for time-delayed prediction.

# Compression performance discussion

We integrated the CS-PQALF into TMuC0.7. There are two test configurations on Wiener-based in-loop filters defined in the JCTVC-B310 [1]. They are the high efficiency random access (HE-RA) and high efficiency low delay (HE-LD) configurations. Table 3 shows detailed BD-rates of each sequence in different configurations. The BD-rate increases are 0.1% and 0.3% for HE-RA and HE-LD, respectively. The BD-rate performance is comparable with the anchor.

Table 3  BD-rates of the proposed CS-PQALF

## Cross verification

MediaTek thanks Intel and Qualcomm for performing the cross-verification for this proposal. From the cross-check report, JCTVC-C229 [9], the cross-verification results match the experimental results provided by MediaTek.

# Complexity analysis

## Relative encoding/decoding time against the anchor

Table 4 shows the encoding time and decoding time against the anchor. The encoding time and decoding time for HE-RA are slightly increased by 1% and 3%, respectively. The encoding time and decoding time for HE-LD are both slightly increased by 1%. Please note that these numbers are averaged from all test points of all simulation sequences instead of a particular case. Due to the computing farm overhead, the small execution time differences can be ignored.

Table 4  Encoding/decoding time against the anchor

|  | Random access | Low delay |
| --- | --- | --- |
| Enc Time[%] | 101% | 101% |
| Dec Time[%] | 103% | 101% |

In any case, the encoding/decoding time may not effectively show the complexity reduction by the proposed CS-PQALF. The PC/Workstation computing farm is too powerful to reveal some problems in real embedded system hardware design. For example, frequent external memory accesses may cause execution delay. The discussion about the external memory access in ALF will be described in Section 4.3.

## Number of encoding passes

In JCTVC-C083 [10], the anchor uses 16 encoding passes. Following the same definition, we calculate the number of encoding passes of the proposed CS-PQALF. The first encoding pass occurs in the Wiener filtering after the picture quadtree split is decided. The second encoding pass occurs in the possible reconstructed frame write-back after the CU synchronous ALF on/off switch is decided. Compared with the anchor, the encoding passes of CS-PQALF are effectively reduced and also keep comparable BD-rate performance, as shown in Table 3.

## Memory requirement analysis

The number of frame buffer accesses is also considered as a metric to analyze encoding complexity. In real hardware design, the on-chip memory is very limited [8]. During the ALF encoding process, any frame-scale frame buffer accessing, for example, calculating correlations, filtering picture, and writing filtered results, should be counted as one external memory access. If we assume there is no on-chip frame buffer and consider off-chip frame buffer accesses instead of encoding passes, the number of the off-chip frame buffer accesses for the proposed CS-PQALF is only 3 times, while the anchor needs more than 20 times of off-chip frame buffer accesses.

Another significant improvement of our proposal is to use block-wise local adaptation, instead of pixel-wise adaptation in the anchor. The TMuC anchor adopts the pixel-wise ALF classification approach, which needs additional memory to record the pixel-wise ALF classes. This additional memory usage can be regarded as another frame buffer. This buffer may have to be allocated in external memory. The memory access to this buffer occurs when filtering picture, redesigning filter, and writing filtered results. On the other hand, the proposed CS-PQALF adopts the block-wise local adaptation as ALF classification approach. The block-wise method does not need to record the classification information at pixel level. If this type of frame buffer access is also counted, the number of the off-chip frame buffer accesses for the proposed CS-PQALF remains 3 times, while the anchor needs much more than 20 times, increased from 16. Furthermore, the block-wise approach also totally avoids the complexity involved in pixel-wise classification, which has complexity of 7x7 summations for each pixel.

In real hardware design, the external memory access may dominate the execution time. Compared with the TMuC anchor, the proposed CS-PQALF shows its great benefit on saving the number of frame buffer accesses.

# Additional experimental results on combined in-loop filtering tools

In prior proposal [2], we tried to make a joint design among multiple in-loop filtering tools. In this contribution, we combine four in-loop filtering tools including CS-PQALF, improved deblocking filter (IDF) [5], controlled clipping (CC) [6], and quadtree-based adaptive offset (QAO) [7]. Figure 3 shows the encoder diagram with the combined in-loop filtering tools. The preprimary result on our combined in-loop filtering tools under TE10 test configurations is provided in Table 5. The average BD-rate reduction is about 2.4%. As shown in Table 6, the increase of encoder time is 1.5%, and that of decoder time is 7.5%.

Figure 3  Encoder diagram with combined in-loop filtering tools (CS-PQALF, QAO, IDF, and CC)

Table 5  BD-rates of the combined in-loop filtering tools

Table 6  Encoding/decoding time against the anchor

|  | Random access | Low delay |
| --- | --- | --- |
| Enc Time[%] | 102% | 101% |
| Dec Time[%] | 107% | 108% |

# Conclusion

MediaTek developed the coding unit synchronous picture quadtree-based adaptive loop filter (CS-PQALF). To improve the coding efficiency, picture quadtree partitions for multiple filters per picture, adaptation between square-shaped and rhombus-shaped filters, 2-step filter coefficients prediction, and time-delayed filtering are adopted. To reduce the number of encoding passes, the fast filtering distortion estimation (FFFDE) is also proposed. Experimental results show that the proposed CS-PQALF increases 0.1% and 0.3% BD-rates for high efficiency random access and high efficiency low delay configurations, respectively. The number of encoding passes is 2. The encoder execution time and the decoder execution time are slightly increased by 1%-3%. We conclude that CS-PQALF effectively reduces the number of encoding passes from 16 (for anchor) to 2 and avoids the complexity of the pixel-wise adaptation in the anchor while maintaining comparable compression performance as the anchor. It makes the ALF techniques much more feasible.

Besides, the combined in-loop filtering tools, including IDF, CS-PQALF, CC, and QAO, show 2.2% and 2.5% BD-rate reductions for high efficiency random access and high efficiency low delay configurations, respectively. The BD-rate reductions do not fully add up but the combined tools are still better than each individual one.

# Patent rights declaration

MediaTek Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Tomoo Yamakage, Takeshi Chujo, Shawmin Lei, Keiichi Chono, Marta Karczewicz ,Andrew Segall, Tomoyuki Yamamoto, Byeungwoo Jeon, Jeongyeon Lim, and Jizheng Xu, “Tool Experiment 10: In-Loop Filtering,” Document of Joint Collaborative Team on Video Coding, JCTVC-B310, Jul. 2010.

- Yu-Wen Huang, Chih-Ming Fu, Ching-Yeh Chen, Chia-Yang Tsai, Yongying Gao, JichengAn, Kai Zhang, and Shawmin Lei, “In-Loop Adaptive Restoration,” Document of Joint Collaborative Team on Video Coding, JCTVC-B077, Jul. 2010.

- VCEG KTA2.6r1 Software, http://iphome.hhi.de/suehring/tml/download/KTA/jm11.0kta2.6r1.zip

- “Test Model under Consideration,” Document of Joint Collaborative Team on Video Coding, JCTVC-B205, Jul. 2010.

- Jicheng An, Kai Zhang, Yongying Gao, Xun Guo, Chih-Ming Fu, Yu-Wen Huang and Shawmin Lei, “TE10 Subtest1: Improved Deblocking Filter,” Document of Joint Collaborative Team on Video Coding, JCTVC-C142, Oct. 2010.

- Yu-Lin Chang, Chih-Ming Fu, Andrew Segall, Yeping Su, Ching-Yeh Chen, Yu-Wen Huang and Shawmin Lei, “TE10 Subtest3: Controlled Clipping,” Document of Joint Collaborative Team on Video Coding, JCTVC-C146, Oct. 2010.

- Chih-Ming Fu, Ching-Yeh Chen, Yu-Wen Huang, and Shawmin Lei, “TE10 Subtest3: Quadtree-based Adaptive Offset,” Document of Joint Collaborative Team on Video Coding, JCTVC-C147, Oct. 2010.

- In Suk Chong, Wei-Jung Chien, Naren Malayath, and Marta Karczewicz, “Encoder Complexity Analysis and Performance Report on Adaptive Loop Filter,” Document of Joint Collaborative Team on Video Coding, JCTVC-B095, Jul. 2010.

- In Suk Chong and Marta Karczewicz, “TE10 Subtest2: Cross check result of MediaTek ALF,” Document of Joint Collaborative Team on Video Coding, JCTVC-C229, Oct. 2010.

- Keiichi Chono and Tomoo Yamakage, “TE10: Summary of TE10 on In-Loop Filtering,” Document of Joint Collaborative Team on Video Coding, JCTVC-C083, Oct. 2010.

Joint proposal of MediaTek and SHARP.