| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C148 |
| --- | --- |

| Title: | TE12: Crosscheck on Deblocking Filter in TMuC0.7 by MediaTek |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |  |  |
| Purpose: | Information |  |  |  |  |
| Author(s) or Contact(s): | Yu-Lin Chang, Yu-Wen Huang, and Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Contact: Shawmin Lei +886-3-567-0766 ext.25555 shawmin.lei@mediatek.com |  |  |
| Source: | MediaTek Inc. |  |  |  | MediaTek Inc. |

_____________________________

# Abstract

The purpose of this document is to test the deblocking filter in TMuC0.7 as defined in JCTVC-B312. The deblocking filter is turned on and off to see the difference of coding performance. Simulation results show that disabling deblocking filter increases average bitrate by 0.4%, 1.1%, 0.4%, 0.2%, and 0.6% for random access, random access LoCo, low delay, low delay LoCo, and intra LoCo configurations, respectively. However, the bitrate increases by 1.1% for intra configuration. The reduction of encoding time and decoding time is about 3% when deblocking is disabled.

# Introduction

JCTVC-B312 [1] defines the experiment of deblocking filter. The settings are shown in Table 1. There is one parameter needed to be set in the encoder configuration: LoopFilterDisable. If LoopFilterDisable equals to 0, the deblocking filter is turned on. Else if the LoopFilterDisable equals to 1, the deblocking filter is turned off.

Table 1  Deblocking filter experiment settings

| Tool name | Default setting | Alternative setting | Parameters for Alternative setting |
| --- | --- | --- | --- |
| Deblocking filter | Enabled | Disabled | LoopFilterDisable : 1 |

# Tool experiment on deblocking filter

The source code we used in this tool experiment is TMuC0.7. We use GCC 3.4.6-9(x86_64) and Intel CPUs on Redhat Linux(x86_64) for the tests. Six configurations (intra, intra LoCo, random access, random access LoCo, low delay, low delay LoCo) were conducted for the tool experiment of deblocking filter. The parameter LoopFilterDisable is set to the default setting, 1, in the anchor and is set to 0 in the alternative experiment.

# Results

The BD-rates from our experimental results were confirmed with Qualcomm’s results. Most of our BD rates match those provided by Qualcomm. However, the average U BD-rate in the random access LoCo condition is increased by 0.1% in Qualcomm’s results. This might be caused by different versions of TMuC and different compilers.

## Compression performance discussion

There are six test configurations for tool experiment on deblocking filter defined in the JCTVC-B312 [1]. They are the intra, intra LoCo, random access, random access LoCo, low delay, and low delay LoCo condition. Table 2 shows the detailed BD-rate of each sequence in different configurations. The reference of the tool experiment is the anchor defined in JCTVC-B300 [2]. The objective rate-distortion measurement uses the BD rate defined in VCEG-AI11 [3]. The bitrate reductions are 0.6%, 0.4%, 1.1%, 0.4%, and 0.2% for the intra LoCo, random access, random access LoCo, low delay, and low delay LoCo configurations, respectively. However, 1.1% bitrate increase is observed for the intra configuration. Bugs may exist in the intra deblocking filter of TMuC0.7. MediaTek has improved the coding performance for intra configurations, and detailed results can be referred to JCTVC-C142 [4].

| Table 2 Coding performance for anchor and deblocking filter off |  |
| --- | --- |
| (a) Intra | (b) Intra LoCo |
| (c) Random access | (d) Random access LoCo |
| (e) Low delay | (f) Low delay LoCo |

## Complexity analysis

The encoding and decoding time comparison is shown in Table 3. Due to the computing farm network overhead, there might be some variations in the encoding time and decoding time. However, the encoding time and decoding time in most of the conditions are reduced by 3-4% while turning off the deblocking filter. Please note that these numbers are averaged from all test points of all simulation sequences instead of a particular case.

Table 3  Encoding/decoding time comparison

|  | Intra | Intra Loco | Random access | Random access LoCo | Low delay | Low delay LoCo |
| --- | --- | --- | --- | --- | --- | --- |
| Enc Time[%] | 100% | 100% | 99% | 99% | 100% | 103% |
| Dec Time[%] | 97% | 96% | 100% | 97% | 100% | 98% |

# Conclusion

Deblocking filter plays an important role in current video coding system. It improves the subjective view quality and objective coding performance. We found that the BD rate increase is around 0.3% after turning off the deblocking filter in the current TMuC0.7. The encoding time and decoding time are reduced by 3~4% in most of the conditions if the deblocking filter is turned off. However, the current deblocking filter in TMuC seems to have some bugs for the intra configuration causing 1.1% bitrate increase.

# References

- Ken McCann, “Tool Experiment 12: Evaluation of TMuC Tools,” Document of Joint Collaborative Team on Video Coding, JCTVC-B312, Geneva, Switzerland, Jul. 2010

- Frank Bossen, “Common Test Conditions and Software Reference Configurations, “ Document of Joint Collaborative Team on Video Coding, JCTVC-B300, Geneva, Switzerland, Jul. 2010

- Gisle Bjøntegaard, “Improvements of the BD-PSNR Model”, ITU-T SG16/Q6, 35th VCEG Meeting, Berlin, Germany, Jul. 2008, Doc. VCEG-AI11.

- Jicheng An, Kai Zhang, Yongying Gao, Xun Guo, Chih-Ming Fu, Yu-Wen Huang, and Shawmin Lei, “TE10 Subtest1: Improved Deblocking Filter,” Document of Joint Collaborative Team on Video Coding, JCTVC-C142, Guangzhou, China, Oct. 2010