| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C155 WG11 Number: m18181 |
| --- | --- |

| Title: | TMuC 0.7 results with sequence dependent QPs |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kenneth Andersson, Rickard Sjöberg Färögatan 6 16480 Stockholm Sweden | Tel: Email: | +46 10 7141538 kenneth r andersson @ericsson com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution investigates performance when targeting CfP bitrates using sequence specific QP’s compared to common conditions (QP=22, 27, 32 and 37) for low delay and random access configurations. The sequence specific QPs gives very similar coding efficiency as for the common conditions and gives bitrates generally closer to application bitrates and thus better suited for subjective assessment. A positive side effect is that the worst case encoding time is reduced by about 20%.

- Background

Using QP=22 generates high bitrates for some sequences, far from applications bitrates for broadcast and video communication. For example the 1080p/60Hz sequence BQTerrace is encoded at 41Mbps at QP=22 for random access high-efficiency configuration and at 81Mbps for low-delay high-efficiency configuration using the common conditions [1]. At low QPs it is very hard to distinguish coding artifacts for some sequences. Optimization of codec performance towards high rates may not be desired. It is possibly better to have rate points better suited for subjective assessments.

- Generation of sequence dependent QPs

To target bitrates defined in the CfP [4] the sequence specific QPs from contribution [2] have been used as a starting point. Those QPs have then been adjusted to fit the range between the max and the min QP with only 4 QPs for each sequence. The in-between QPs were then obtained at equal distance from the max and min QPs and rounded to integer values. For all intra coding the sequence specific QPs for random access minus 1 is used to get better quality of the intra frames to enable subjective viewing with reduced flickering. All other settings are according to the common conditions [1].

- Results

The results were obtained using TMuC 0.7 both for the anchor using common conditions [1] and using sequence dependent QPs. The encoders and decoders were compiled on Linux using gcc 4.3.3. The simulations were performed on a Linux cluster using Intel processors of 2.9GHz.

A summary of the results is shown below. BD-rate is according to [3]. Encoding time and decoding time are ratios of time measures of sequence dependent QPs and common conditions both using TMuC 0.7. Time is measured as user time.

Table I: BD-rate and encoding and decoding time ratios for common conditions and sequence dependent QPs

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0,1 | 0,1 | 0,2 | 0,1 | 0,0 | 0,1 |
| Class B | -0,1 | -0,1 | 0,2 | 0,0 | -0,3 | -0,1 |
| Class C | -0,2 | 0,5 | 0,6 | -0,3 | 0,3 | 0,2 |
| Class D | -0,2 | -0,1 | -0,1 | -0,1 | -0,5 | -0,3 |
| Class E | -0,3 | 0,7 | 0,8 | -0,1 | 0,3 | 0,2 |
| All | -0,2 | 0,2 | 0,3 | -0,1 | -0,1 | 0,0 |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 96% |  |  | 98% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0,3 | 0,3 | 0,3 | -0,3 | -0,3 | 0,0 |
| Class B | -0,4 | -0,1 | -0,2 | -0,5 | -0,2 | -0,4 |
| Class C | 0,1 | -1,1 | -1,1 | 0,1 | -0,9 | -0,9 |
| Class D | 0,0 | -0,3 | -0,5 | 0,0 | -0,5 | 0,0 |
| Class E |  |  |  |  |  |  |
| All | -0,1 | -0,4 | -0,5 | -0,1 | -0,5 | -0,4 |
| Enc Time[%] | 98% |  |  | 97% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0,4 | 0,0 | 0,1 | -0,2 | 0,1 | -0,2 |
| Class C | 0,1 | 0,6 | 0,8 | 0,2 | 0,3 | 0,3 |
| Class D | 0,0 | 0,4 | 0,2 | 0,1 | 0,1 | 0,2 |
| Class E | 0,2 | -0,5 | 0,5 | 0,0 | -1,8 | -0,1 |
| All | -0,1 | 0,2 | 0,4 | 0,0 | -0,2 | 0,1 |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |

Table II: Comparison of average bitrates for high-efficiency and low complexity configurations for common conditions and sequence dependent QPs

|  | Intra |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- |
|  | anchor | seq QPs | anchor | seq QPs |  |
|  | rate [kbps] | rate [kbps] | rate [kbps] | rate [kbps] |  |
| Class A | 55780,82 | 52801,53 | 59892,57 | 56843,47 |  |
| Class B | 40783,02 | 39025,17 | 42926,15 | 41411,19 |  |
| Class C | 14230,39 | 9394,013 | 15181 | 10061,89 |  |
| Class D | 4888,911 | 5976,728 | 5184,165 | 6338,288 |  |
| Class E | 12305,1 | 13108,81 | 13562,87 | 14427,48 |  |
| All | 23826,07 | 22307,68 | 25364,73 | 23868,11 |  |
|  | Random access |  | Random access LoCo |  |  |
|  | anchor | seq QPs | anchor | seq QPs | CfP |
|  | rate [kbps] | rate [kbps] | rate [kbps] | rate [kbps] | rate [kbps] |
| Class A | 10975,84 | 7111,679 | 12041,57 | 7963,065 | 8250 |
| Class B | 6637,275 | 4218,081 | 6936,614 | 4704,447 | 5000 |
| Class C | 2176,126 | 1048,038 | 2449,662 | 1168,549 | 1192 |
| Class D | 693,5393 | 766,6257 | 807,9576 | 892,2079 | 878 |
| Class E |  |  |  |  |  |
| All | 4441,115 | 2838,161 | 4786,446 | 3179,426 | 3318,667 |
|  | Low delay |  | Low delay LoCo |  |  |
|  | anchor | seq QPs | anchor | seq QPs | CfP |
|  | rate [kbps] | rate [kbps] | rate [kbps] | rate [kbps] | rate [kbps] |
| Class A |  |  |  |  |  |
| Class B | 10150,52 | 4530,713 | 10023,87 | 6112,433 | 5000 |
| Class C | 3001,806 | 1350,127 | 3552,469 | 1589,851 | 1192 |
| Class D | 988,0381 | 1060,644 | 1237,969 | 1356,182 | 878 |
| Class E | 1167,315 | 919,5454 | 1375,011 | 1133,055 | 878 |
| All | 4388,37 | 2190,955 | 4587,884 | 2859,092 | 2244,625 |

In Table I it is shown that the BD-rate is marginally affected by the use of sequence dependent QPs although the average bitrates in Table II are significantly lower for sequence dependent QPs than for the common conditions. Using sequence dependent QPs, bitrates closer to the bitrates that were defined in CfP [4] are obtained except for low delay configurations for Class D. The average encoding and decoding time are a little shorter for sequence dependent QPs compared to the common conditions.

The full results can be found in attached excel sheet. There can it also be seen that the worst case encoding time for one rate point (high-efficency low-delay) is reduced from 118 hours to 94 hours, e.g. by 20%.

# Conclusions

It has been shown that the use of sequence dependent QPs has marginal effect on the BD-rate. One advantage of sequence dependent QPs is that it avoids optimization on bitrates far from application bitrates and thus also in most cases gives rate points better suited for subjective assessment.

- References

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-B300, Geneva, Switzerland, July 2010.

- Ken McCann, Woo-Jin Han, Il-Koo Kim, “Samsung’s Response to the Call for Proposals on Video Compression Technology”, JCTVC-A124, Dresden, Germany, April 2010.

- Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.

- “Joint call for proposals on video compression technology,” ISO/IEC MPEG 91st meeting, N11113, Kyoto, Japan, January 2010