| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C166r1 |
| --- | --- |

| Title: | TE11: Study on motion vector coding (experiment 3.3.a & 3.3.c) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazushi Sato W12F 1-11-1 GCO Osaki, Shinagawa-ku, Tokyo 141-0032 Japan | Tel: Email: | +81 3 5435 3308 Kazushi.Sato@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

Messages on JCTVC-B081 are:

- Swapping of code number assignment for motion vector predictor may help improvement.

- Temporal predictor as well as spatial predictor is important.

- In current TMuC motion vector predictor configuration is in SPS but it should rather be in SliceHeader.

Related to these messages this contribution reports result of 2 experiments as follows:

Experiment -1: Comparison of with & without Temporal Predictor

Experiment -2: Code Number Swapping of Motion Predictors

- Introduction

# Experiment -1: Comparison of with & without Temporal Predictor

In AVC temporal predictor is employed only with direct mode. AMVP in TMuC allows having a set of multiple candidate of motion vector predictor including the temporal predictor.

In this section the performance of having a temporal predictor as a pmv candidate is examined.

## Simulation Condition

In Table 2.1.1 simulation conditions for experiment -1 is shown.

Table 2.1.1

| Simulation Software | TMuC0.7.3 - Anchor: without temporal predictor - To be compared: with temporal predictor |
| --- | --- |
| Setting | Based on encoder_lowdelay.cfg encoder_lowdelay_loco.cfg encoder_randomaccess.cfg encoder_randomaccess_loco.cfg |
| QP | 22,27,32,37 |
| Sequences | ClassB,C,D&E |

## Simulation Result

Results of Experiment -1 is shown in Table 2.2.1.

Gain obtained with the temporal predictor ranges around 0 – 1.8%.

Table 2.2.1

[Lowdelay]

| sequences | bd_r |
| --- | --- |
| B_BQTerrace | -1.77076 |
| B_BasketballDrive | -0.83673 |
| B_Cactus | -0.94437 |
| B_Kimono1 | -0.1792 |
| B_ParkScene | -0.23797 |
| C_BQMall | -1.62612 |
| C_BasketballDrill | -0.55808 |
| C_PartyScene | -1.12531 |
| C_RaceHorses | -0.63322 |
| D_BQSquare | -2.42429 |
| D_BasketballPass | -0.72455 |
| D_BlowingBubbles | -1.021 |
| D_RaceHorses | -0.66231 |
| E_vidyo1 | -0.80283 |
| E_vidyo3 | 0.056739 |
| E_vidyo4 | -0.76922 |

[Lowdelay_Loco]

| sequences | bd_r |
| --- | --- |
| B_BQTerrace | -1.51502 |
| B_BasketballDrive | -1.10516 |
| B_Cactus | -1.10341 |
| B_Kimono1 | -0.66844 |
| B_ParkScene | -0.55099 |
| C_BQMall | -1.64576 |
| C_BasketballDrill | -0.85627 |
| C_PartyScene | -0.60031 |
| C_RaceHorses | -1.01017 |
| D_BQSquare | -0.56225 |
| D_BasketballPass | -1.01264 |
| D_BlowingBubbles | -0.50781 |
| D_RaceHorses | -1.32326 |
| E_vidyo1 | -0.51066 |
| E_vidyo3 | -0.10842 |
| E_vidyo4 | -0.39004 |

[Randomaccess]

| sequences | bd_r |
| --- | --- |
| B_BQTerrace | -0.93764 |
| B_BasketballDrive | -0.89957 |
| B_Cactus | -1.29456 |
| B_Kimono1 | -0.89329 |
| B_ParkScene | -0.5629 |
| C_BQMall | -1.21931 |
| C_BasketballDrill | -0.5863 |
| C_PartyScene | -0.91151 |
| C_RaceHorses | -0.65957 |
| D_BQSquare | -1.58954 |
| D_BasketballPass | -0.88194 |
| D_BlowingBubbles | -1.04684 |
| D_RaceHorses | -0.94534 |

[Randomaccess_Loco]

| sequences | bd_r |
| --- | --- |
| B_BQTerrace | -0.6841 |
| B_BasketballDrive | -0.82233 |
| B_Cactus | -1.00935 |
| B_Kimono1 | -0.83504 |
| B_ParkScene | -0.47812 |
| C_BQMall | -0.97794 |
| C_BasketballDrill | -0.60618 |
| C_PartyScene | -0.47291 |
| C_RaceHorses | -0.7115 |
| D_BQSquare | -0.63241 |
| D_BasketballPass | -0.85285 |
| D_BlowingBubbles | -0.50131 |
| D_RaceHorses | -1.05303 |

In Table 2.2.2, psnry and generated bits for each frame with the result “B_Cactus” QP=22, random access case is shown.

As can be seen the data of psnry with (a) without temporal predictor and (b) with temporal predictor is almost same for each frame so we can compare coding efficiency with the generated bits.

In this case gain differs by frame as follows: the closer in temporal direction the current frame and the reference frame are, the more gain obtained with the temporal predictor. This is generally true with other sequences and with other QPs.

Employing the temporal predictor as a mv predictor candidate improves coding efficiency but it also cause increase in complexity or power consumption. By not employing the temporal predictor, memory access for the motion vector derivation is not necessary, and cost function calculation for less number of candidates will be necessary. Reduction of power consumption will be important to develop ecological HEVC products especially like DSC or camcorders.

However, not applying the temporal predictor for all frames causes loss in coding efficiency. So, applying it just with the frames whose temporal distance is close to the reference frame may be a good trade-off between coding efficiency and complexity or power consumption.

For such complexity reduction it will be preferable that pmv configuration can be set not at SPS-level but at PPS- or frame-level. Current TMuC specification contains “mv_competition_temporal_flag” in the slice header and we support this.

Table 2.2.2

| POC | Off bit | Off PSNRY | On bit | On PSNRY | On_bit/Off_bit |
| --- | --- | --- | --- | --- | --- |
| 0 | 2843176 | 41.7507 | 2843176 | 41.7507 | 1 |
| 8 | 1690936 | 40.4026 | 1690936 | 40.4026 | 1 |
| 4 | 555280 | 38.9003 | 557432 | 38.9029 | 1.003875522 |
| 2 | 264368 | 38.4 | 263968 | 38.4045 | 0.998486958 |
| 6 | 271248 | 38.3301 | 272376 | 38.3429 | 1.004158556 |
| 1 | 52704 | 37.8715 | 52264 | 37.8757 | 0.991651488 |
| 3 | 46920 | 37.8813 | 46736 | 37.8903 | 0.996078431 |
| 5 | 38920 | 37.8469 | 38840 | 37.8623 | 0.997944502 |
| 7 | 35824 | 37.7917 | 33776 | 37.7921 | 0.942831621 |

.

.

.

# Experiment -2: Code Number Swapping of Motion Predictors

Current AMVP in TMuC assigns largest code number for the temporal predictor.

In this section performance of code number swapping (smallest code number is assigned for the temporal predictor) is examined. Right now configuration of motion vector predictors is still in SPS, not in SliceHeader.

## Simulation Condition

In Table 3.1.1 simulation conditions for experiment -2 is shown.

Table 3.1.1

| Simulation Software | TMuC0.7.3 (a) Largest code number is assigned for the temporal predictor (b) Smallest code number is assigned for the temporal predictor |
| --- | --- |
| Setting | Based on encoder_lowdelay_loco.cfg encoder_randomaccess_loco.cfg |
| QP | 22,27,32,37 |
| Sequences | ClassC (50frs) |

## Simulation Result

Results of Experiment -1 is shown in Table 3.2.1. This result is verified by LG.

Table 3.2.1

| Sequence | BDR | BDH | BDR |
| --- | --- | --- | --- |
| C_BasketballDrill_LD_Loco | 0.054169 | -0.02464 | 0.038935 |
| C_BQMall_LD_Loco | -0.20791 | -0.27945 | -0.08717 |
| C_PartyScene_LD_Loco | -0.04549 | 0.034009 | -0.17182 |
| C_RaceHorses_LD_Loco | 0.106271 | -0.04523 | 0.269816 |
| C_BasketballDrill_RA_Loco | -0.03159 | -0.04369 | -0.0428 |
| C_BQMall_RA_Loco | -0.04245 | -0.02528 | -0.03215 |
| C_PartyScene_RA_Loco | 0.002228 | -0.04099 | 0.088258 |
| C_RaceHorses_RA_Loco | 0.026939 | 0.088976 | -0.0502 |

In Table 3.2.2, psnry and generated bits for each frame with the result “C_BQMall” QP=22, low delay case is shown.

As can be seen the data of psnry with (a) without temporal predictor and (b) with temporal predictor is almost same for each frame so we can compare coding efficiency with the generated bits.

If we compare coding efficiency of (a) and (b) at sequence level not so much gain can be observed. However if we compare at frame level, for some frames (a) provides better result while for other frames (b) performs better. So switching configuration at frame-level or slice-level better it will potentially improves coding efficiency.

Table 3.2.2

| POC | _0 bit | _0 psnry | _1 bit | _1 psnry |
| --- | --- | --- | --- | --- |
| 0 | 533576 | 41.5191 | 533576 | 41.5191 |
| 1 | 64648 | 39.7358 | 64648 | 39.7358 |
| 2 | 72640 | 39.725 | 72592 | 39.7231 |
| 3 | 74584 | 39.7131 | 74432 | 39.7144 |
| 4 | 74616 | 39.7188 | 75672 | 39.7391 |
| 5 | 80848 | 39.7239 | 81632 | 39.7435 |
| 6 | 83384 | 39.7364 | 83008 | 39.7366 |
| 7 | 80064 | 39.7195 | 79704 | 39.7316 |
| 8 | 75864 | 39.7246 | 74376 | 39.7124 |
| 9 | 77192 | 39.7309 | 76880 | 39.7375 |
| 10 | 79704 | 39.7263 | 79320 | 39.7307 |
| 11 | 92008 | 39.7917 | 90896 | 39.7777 |
| 12 | 88784 | 39.7956 | 91344 | 39.8217 |
| 13 | 89888 | 39.7471 | 88128 | 39.7325 |
| 14 | 91536 | 39.7331 | 91248 | 39.7459 |
| 15 | 94280 | 39.7423 | 93816 | 39.7175 |

.

.

.

# Conclusion

In this contribution we have reported results with 2 kinds of experiments related to our proposal JCTVC-B081: Experiment -1 is a comparison of with & without temporal predictor, and Experiment -2 is code number configuration of motion predictors.

The result with Experiment -1 shows that the temporal predictor contributes to coding efficiency by 0 –2.2 %.

Especially with the random access case different amount of gain can be observed frame-by-frame: the closer in the temporal distance between the current frame and the reference frame is, the more coding efficiency improvement can be observed. So applying the temporal predictor only with the frames whose temporal distance with the reference frame is closer will be a good trade-off between coding efficiency and complexity or power consumption.

The result with Experiment -2 shows that swapping the code number at SPS level will improve coding efficiency by up to 0.20% and further gain can be expected if it can be set at Slice level.

# References

[1] “Test Model under Consideration”, JCTVC-B205_draft005, July 2010

[2] “On Motion Vector Coding”, JCTVC-B081, July 2010

# Patent rights declaration(s)

Sony Corp. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).