| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C169 WG11 Number: m18198 |
| --- | --- |

| Title: | TE6.b: Experimental results of DCIM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E. Maani, L. Dong, W. Liu 1730 N 1st Street San Jose, CA 95112 | Tel:(408) 352-4710 Email:ehsan.maani@am.sony.com |  |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This document present the experimental results of Differential Coding of Intra Modes (DCIM) [1] under the test conditions defined for Tool Experiment 6 Error: Reference source not found.

DCIM uses neighborhood edge estimation to predict Intra prediction direction and differentially encodes the modes with respect to the predicted direction. This enables a higher precision in the Intra prediction directions without substantially increasing the mode signaling overhead. An additional bit is transmitted per block (i.e., PU) to signal to the decoder weather DCIM is used.

# Software

The test was implemented into the TMuC software 0.7.  Due to incompatibility with MDDT and Edge-based Intra prediction , when DCIM is used, MDDT and Edge-based prediction [3] is set to off.

# Default test condition results

Table 1 shows the summary of the results compared with the TMuC 0.7 software for both high efficiency and low complexity settings. It has recently come to our attention that the results on intra low complexity configuration is affected by a VLC bug when MDDT is turned off (Ticket #79).  The bug fix has recently been incorporated into the implementation and tests are being redone. The results for intra low complexity configuration will thus be updated once the test results become available.

For both profiles, the modified software that includes DCIM roughly increases the encoding time to 200% and decreases the decoding time to 50%.  Nevertheless, the encoding and decoding times are affected due the fact that MDDT and Edge-based prediction are present in the anchor (TMuC 0.7) but not in the modified software. By setting MDDT to off, Rotational Transform (ROT) will be used for all block sizes; thus, increasing the encoding time and decreasing the decoding time. As a result an accurate evaluation of DCIM complexity is not reflected in these numbers. Furthermore, TE 12 results show that there is 1.9% gain achieved on chroma component by MDDT (Table 2). Because MDDT gain on chroma components is only present in the anchor (default settings), we observe a small coding efficiency loss for chroma components in the results presented in this section. To obtain a more realistic complexity and performance evaluation, additional tests are carried out in the next section where MDDT is set to off on both the anchor and the test.

- Summary of BD Rates and Encoding/Decoding Time.

|  | Intra High Efficiency |  |  | Intra Low Complexity |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.8 | 1.2 | 1.7 | -1.1 | -0.3 | -0.5 |
| Class B | -1.9 | 0.2 | 0.5 | -0.8 | 0.0 | 0.0 |
| Class C | -2.5 | -0.5 | -0.6 | -2.3 | -1.6 | -1.7 |
| Class D | -1.4 | 0.4 | 0.0 | -1.1 | -1.1 | -1.5 |
| Class E | -2.3 | 1.9 | 1.2 | -2.6 | -0.3 | -0.9 |
| All | -1.9 | 0.5 | 0.4 | -1.5 | -0.7 | -0.9 |
| Enc Time[%] | 214% |  |  |  |  |  |
| Dec Time[%] | 62% |  |  |  |  |  |

- Summary of BD Rates of MDDT [4]

|  | Intra High Efficiency |  |  | Intra Low Complexity |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.3 | 2.1 | 2.6 | 0.2 | 0.1 | 0.1 |
| Class B | -0.1 | 1.7 | 2.1 | 0.9 | 0.9 | 1.1 |
| Class C | -0.4 | 1.2 | 1.1 | -0.1 | -0.5 | -0.5 |
| Class D | -0.3 | 1.1 | 0.9 | 0.0 | -0.6 | -0.9 |
| Class E | 0.3 | 3.9 | 3.3 | 0.5 | 1.8 | 0.9 |
| All | -0.1 | 1.9 | 1.9 | 0.3 | 0.3 | 0.1 |

# Additional test

In this section we present results with respect to a slightly modified anchor. This new setting allows the tools to be added one at a time without any compatibility issue to correctly evaluate their performance and complexity.

In this modified test condition the anchor is TMuC 0.7 default setting with the following tools turned off:

MDDT, edge-based Intra prediction, AIS, and CIP. The compile time setting DEFAULT_IS is changed from 1 to 0 since it has significantly better coding performance. DCIM is then turned on and compared against this anchor. Table 2 shows the test results for high efficiency profile. As shown in the table DCIM has about 1.8% bit savings in this settings. The encoding and decoding time is 160% and 120% of the modified anchor, respectively.

- Summary of BD Rate and Encoding/Decoding Time for modified Test Condition.

|  | Intra High Efficiency |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.3 | -0.9 | -0.8 |
| Class B | -1.6 | -1.3 | -1.3 |
| Class C | -2.1 | -1.3 | -1.3 |
| Class D | -1.1 | -0.6 | -0.6 |
| Class E | -2.7 | -1.9 | -1.9 |
| All | -1.8 | -1.2 | -1.2 |
| Enc Time[%] | 162% |  |  |
| Dec Time[%] | 124% |  |  |

# References

- E. Maani, W. Liu, “Differential Coding of Intra Modes (DCIM)”, Doc. JCTVC-B109, Geneva, Switzerland, Jul 2010

- Ali Tabatabai, “Tool Experiment 6: Intra Prediction Improvement”, Doc. JCTVC-B306_r3, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, Switzerland, July 2010.

- I. Amonou et al., “Description of video coding technology proposal by France Telecom, NTT, NTT DOCOMO, Panasonic and Technicolor”, Doc. JCTVC-A114, Dresden, Germany, Apr. 2010

- E. Alshina, A. Alshin, W.-J. Han, R. Joshi, M. Coban, “TE12.4: Experimental results of MDDT and ROT by Samsung and Qualcomm”, Doc. JCTVC-C202, Guangzhou, China, October 2010.

# Patent rights declaration(s)

(NOTE – Activities in the JCT-VC and contributions to the JCT-VC are subject to the common patent policy for ITU-T/ITU-R/ISO/IEC. A statement of that policy can be found at
http://www.itu.int/ITU-T/dbase/patent/patent-policy.html, with further information available at http://www.itu.int/ITU-T/ipr/index.html and in the ISO/IEC Directives. The form to be used for the formal reporting of patent rights to ITU-T/ITU-R/ISO/IEC can be found at http://www.itu.int/ITU-T/ipr/index.html. Contributions to the JCT-VC proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The provided informal notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form. This obligation to provide an informal notice is supplemental to, and does not replace, any existing obligations of parties with technology included in a final or draft standard to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC. An example of an informal IPR notification statement for a contribution is provided below.)

Sony Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).