| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C172 WG11 Number: m18201 |
| --- | --- |

| Title: | TE6: Summary report of intra prediction improvements tool experiments |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Ali Tabatabai | Tel: +1-408-352-4715 Email:ali.tabatabai@am.sony.com |  |
| Source: | TE06 Coordinator |  |  |

_____________________________

# Abstract

This contribution is a summary of Tool Experiment 6, Intra Prediction Improvements.  From a total of 6 proposed toolsets 4 have been evaluated by at least on organization. For cross checking we used the recommended test conditions of intra-only prediction mode for both high compression efficiency and low complexity as defined in the document B306_r3 [1].

Introduction

T06 is organized in 3 tool subsets:

## TE06.a: Bidirectional Intra Prediction

Proposed tools in this tool subset are:

- JCTVC-B042 (Toshiba):  Weighted Bidirectional Prediction (WBP) and Adaptive Sub-block Coding Order (ASCO)

- JCTVC-B040 (Huawei and Hisilicon):  Short Distance Intra Prediction Using Correlation between Lines & Pixels

- JCTVC-A109 (MediaTek):  Overlapped Block Intra Prediction (OBIP), Multiple Model Karhunen Loeve Transform (MMKLT), Context Dependent Mode Representation (CDMR)

- JCTVC-A118 (Microsoft):  Line/Pixel Based Prediction Modes

## TE06.b: Edge Based Intra Prediction

- JCTVC-B109 (Sony):  Differential Coding of Intra Prediction Mode

## TE06.c: Parallel Intra Coding

- JCTVC-B112 (Sharp Labs. of America)

# Cross Verification

| Tool | Proponent | Cross-checker(s) |
| --- | --- | --- |
| TE6.a.1 | Toshiba | Renesas, Sharp |
| TE6.a.2 | Huawei, Hisilicon | Microsoft, DOCOMO, LGE, Ericsson |
| TE6.a.4 | Microsoft | Huawei |
| TE6.b.1 | Sony | Sharp, Panasonic |

# Experimental Results

Table 1: TE6.a.1 BD Rate Summary

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.7 | -2.1 | -2.1 | -2.5 | -1.8 | -1.6 |
| Class B | -2.1 | -1.4 | -1.4 | -2.1 | -1.3 | -1.2 |
| Class C | -1.9 | -1.5 | -1.4 | -2.4 | -1.5 | -1.4 |
| Class D | -2.0 | -1.4 | -1.4 | -2.4 | -1.4 | -1.4 |
| Class E | -2.2 | -2.0 | -1.9 | -2.5 | -2.2 | -2.2 |
| All | -2.1 | -1.6 | -1.6 | -2.3 | -1.6 | -1.5 |
| Enc Time[%] | 94% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 104% |  |  |

Table 2: TE06.a.4 BD Rate Summary

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.3 | -0.6 | -0.7 | -1.2 | -0.4 | -0.6 |
| Class B | -2.3 | -1.0 | -0.9 | -2.4 | -0.6 | -0.8 |
| Class C | -2.7 | -1.1 | -1.3 | -3.3 | -1.1 | -1.2 |
| Class D | -2.1 | -0.9 | -0.9 | -2.7 | -0.8 | -0.8 |
| Class E | -1.9 | -0.7 | -0.6 | -1.9 | -1.2 | -1.1 |
| All | -2.1 | -0.9 | -0.9 | -2.4 | -0.8 | -0.9 |
| Enc Time[%] | 172% |  |  | 229% |  |  |
| Dec Time[%] | 208% |  |  | 301% |  |  |

Table 3:  TE6.b.1 BD Rate Summary

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.8 | 1.2 | 1.7 | 2.3 | 0.9 | 2.8 |
| Class B | -1.9 | 0.2 | 0.5 | 3.9 | 2.9 | 3.9 |
| Class C | -2.5 | -0.5 | -0.6 | -0.1 | -2.5 | -2.2 |
| Class D | -1.4 | 0.4 | 0.0 | 0.2 | -2.4 | -2.7 |
| Class E | -2.3 | 1.9 | 1.2 | 1.8 | 8.6 | 6.2 |
| All | -1.9 | 0.5 | 0.4 | 1.7 | 1.2 | 1.4 |
| Enc Time[%] | 214% |  |  | 191% |  |  |
| Dec Time[%] | 62% |  |  | 48% |  |  |

# Visual Evaluation

No independent visual information was reported

# Conclusion

Experimental results of 3 of the tool subsets that have been cross verified has been presented.

# References

- Ali Tabatabai, “Tool Experiment 6: Intra Prediction Improvement”, Doc. JCTVC-B306_r3, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, Switzerland, July 2010.